3GPP TSG-RAN WG2 Meeting #131bis                                                                      R2-2507580

Prague, Czech Republic, Oct. 13th-17th , 2025

Agenda item:	10.3.3

Source:	China Telecom

Title:	Initial Considerations on the Impact of AS Security and New Services on 6G Common CP and UP

Document for:	Discussion

- Introduction

A new study item on next generation RAT was approved [1]. The 6GR Study Item aims to develop one non-backward compatible radio access technology to meet a broad range of use cases, and requirements as defined during the RAN requirements study. The detailed objectives of the study on radio interface protocol are as follow:

(3) Radio interface pro tocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and pro tocol design [RAN2] - Control Plane architecture (including RRC states) and pro tocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

The latest RAN#109 meeting agreed the following text proposal for TR38.914 regarding 6G architecture [2].

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

Based on the above progress, this paper provides our initial considerations on the impact of access stratum security and AI/ML framework on 6G common user plane and control plane.

- Discussion

The figures below show the existing NR radio protocol architecture for the user plane and the control plane[3]. The 5G protocol stack includes multiple layers such as MAC, RLC, PDCP, and, optionally, SDAP. The architecture was originally designed to support a wide range of applications and verticals.

Figure 1: User Plane Protocol Stack

Figure 2: Control Plane Protocol Stack

In 6G, however, emerging service requirements such as immersive communication, massive communication (IoT), sensing, and AI are expected to have direct impact on both CP and UP. Since service requirements drive protocol design, the necessity and role of each protocol layer (e.g., whether SDAP remains needed) should be carefully analyzed.

To fully capture the needs of 6G, functional requirements and design targets for the user plane should first be identified, including latency targets that account for XR, AI/ML, sensing, NTN, and other new services. Therefore, the existing 5G CP/UP architecture can be taken as a baseline for 6G, and further evaluate the impact of new service requirements on new protocol layer functionalities.

Proposal 1: To take the existing 5G CP/UP protocol layer architecture as a baseline, and further evaluate the impact of new service requirements on new protocol layer functionalities.

  - AS Security

In the RAN#109 meeting, an LS was sent to SA3 to invite SA to consider adding a check point for the alignment between SA3 and RAN2 on any security requirements that impact the AS protocol design [4]. Therefore, it is necessary to take into account SA3’s ongoing work and its implications for both control plane and user plane security. Other potential security impacts should wait for SA3’s final conclusions before further consideration.

The current 5G system uses dedicated algorithms for encryption and integrity protection, negotiated independently. 3GPP SA3 have recently completed specifications for 256-bit cryptographic algorithms. Furthermore, the 3GPP Authenticated Encryption with Associated Data (AEAD) mechanism is being studied in both 5G-A and 6G [5]. Unlike the traditional separated encryption and integrity protection algorithm, AEAD combines encryption and integrity into a single algorithm with one key, simplifying key management, reducing power consumption, and improving efficiency through single-pass processing.

AEAD adoption has direct implications for AS procedures. For intra-RAT mobility, AEAD simplifies handover as all security endpoints support it. However, inter-RAT mobility and legacy interworking are challenging, since legacy 4G/5G systems lack AEAD support, requiring reconciliation of key derivation and security contexts. Another issue is security order: 5G uses MtE for AS and Encrypt-then-MAC (EtM) for NAS. AEAD aligns with EtM, so adapting it to AS may require less efficient two-pass operation. Therefore, if AEAD is finally specified in 6G system, it will impact the whole security selection and negotiation procedure between network and UE. For example, PDCP procedures may be impacted and could require updates.

Observation 1: The new security mechanism AEAD significantly impacts security related procedures in RAN.

In 6G, parameters once seen as non-sensitive may pose privacy risks. PHY signaling and channel information could be exploited, leading to data leakage or UE access failures for example, sensing signals revealing user health or MIB/SIB spoofing disrupting access. To mitigate these risks, security should be incorporated early in the design of reference signals, channels, MAC CE, and system information broadcasts. It is necessary to take into account SA3’s ongoing work and its implications for both control plane and user plane security. Other potential security impacts should wait for SA3’s final conclusions before further consideration. Considering these aspects together with AEAD adoption, comprehensive security solutions across PHY, MAC, PDCP, and RRC layers may need to be studied.

Proposal 2: To study the impact of new security/privacy protection solutions based on the conclusion of SA3 on RAN CP/UP protocol layers.

2.2 Integrated and Unified AI/ML Framework

In 5G-Advance, AI/ML frameworks such as model lifecycle management (LCM), unified data collection, and traffic differentiation were not considered in the architecture. As a result, AI/ML support in 5G-Advanced is fragmented, retrofitted, and limited in scalability. For 6G, an integrated AI/ML framework should be designed from Day-1 to ensure efficiency, security, and interoperability across UE, RAN, and Core.

For 6G, an integrated AI/ML framework should be designed from the outset to ensure interoperability, security, and efficiency across UE, RAN, and Core. Such a framework should feature unified and flexible LCM, consistent data collection with MNO visibility, and scalability for future use cases. Differentiating AI/ML data traffic from user plane and control plane traffic will be critical to guarantee performance and enable efficient training and deployment.

Proposal 3: To adopt an integrated AI/ML framework from Day-1 in 6G.

The 6G Radio SID outlines several AI-related objectives:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

AI/ML previous studies in 3GPP have already addressed in different contexts, for example SA2 on NWDAF, RAN3 on AI/ML aspects for NG-RAN architecture, and RAN1/2/4 on AI/ML for the air interface. In R19, SA2 made efforts to promote AI/ML cross-domain coordination [6]. However, the AI/ML framework in 5G remains fragmented. For 6G, it is necessary to adopt a unified AI/ML framework across TSGs and WGs to ensure optimal operation and interoperability.

Proposal 4: To address AI/ML in a unified way across TSGs and WGs for optimal operation.

The functional framework for AI/ML for NR air interface are shown in figure 1.

Figure 3: Functional framework for AI/ML for NR Air Interface

Based on the TR 38.843, AI/ML data collection serves multiple purposes within the model lifecycle (LCM), e.g., model training, model inference, model monitoring, model selection, model update, etc [7]. Each function has distinct requirements and may impact specifications differently. Offline model training (i.e., UE- /NW-/ two-sided model training) does not impose latency constraints. In contrast, model inference and real-time performance monitoring require timely collection of data from other entities to meet latency requirements and ensure accurate results. These distinctions are essential for defining data handling, scheduling, and processing mechanisms in 6G AI/ML framework studies.

Based on the functions and procedures identified in 5G, the interactions between them can be further analyzed using the TR 38.843 functional framework as a starting point for 6G studies.

Proposal 5: To use the AI/ML functional framework defined in 5G (TR 38.843) as a starting point for 6G studies.

2.3 TCP Impact

In 6G networks, TCP will continue to face critical challenges, magnified by the emergence of new application domains such as AI/ML, integrated sensing and communication, UAV-based connectivity, and immersive XR. While TCP remains the dominant transport protocol, its limitations become more severe in ultra-high-speed, low-latency, and highly dynamic environments. For AI/ML workloads relying on rapid data exchange between devices, edge, and cloud, delayed acknowledgments due to lack of prioritization can significantly hinder training and inference performance. Similarly, in XR and holographic communication, massive downlink traffic inevitably produces large volumes of uplink ACKs, which, if not intelligently suppressed or aggregated, will congest uplink capacity and degrade immersive user experience. In UAV and sensing-enabled 6G systems, reliance on fragile terahertz and ultra-high-frequency links means that even brief blockages can disrupt TCP flows, as the protocol misinterprets interruptions as congestion, resulting in drastic throughput degradation. To unlock the full potential of 6G, TCP must be enhanced through cross-layer mechanisms that ensure timely and efficient handling of ACKs, incorporate intelligent suppression strategies, and enable rapid recovery and multi-link robustness, so that AI/ML, UAV control, sensing collaboration, and XR applications can thrive on the next-generation network.

Proposal 6: To study RAN-assisted TCP enhancements for 6G to ensure efficient ACK handling, robust recovery, and optimal performance for AI/ML, sensing, UAV, and XR applications.

- Conclusions

Based on the discussion in the previous sections, we propose the following:

Proposal 1: To take the existing 5G CP/UP protocol layer architecture as a baseline, and further evaluate the impact of new service requirements on new protocol layer functionalities.

Observation 1: The new security mechanism AEAD significantly impacts security related procedures in RAN.

Proposal 2: To study the impact of new security/privacy protection solutions based on the conclusion of SA3 on RAN CP/UP protocol layers.

Proposal 3: To adopt an integrated AI/ML framework from Day-1 in 6G.

Proposal 4: To address AI/ML in a unified way across TSGs and WGs for optimal operation.

Proposal 5: To use the AI/ML functional framework defined in 5G (TR 38.843) as a starting point for 6G studies.

Proposal 6: To study RAN-assisted TCP enhancements for 6G to ensure efficient ACK handling, robust recovery, and optimal performance for AI/ML, sensing, UAV, and XR applications.

- References

- RP-251881 New SID: Study on 6G Radio, NTT DOCOMO (Moderator), Prague, Czech Republic

- RP-252870 pCR for 38.914 on requirements for architecture and migration for 6G

- TS 38.300

- RP-252891   LS on Early Alignment on Access Stratum security aspects.

- SP- 251247 New SID on supporting AEAD algorithms

- SP-241936, New SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)