3GPP TSG-RAN-WG2 Meeting #131bis	R2-2507591

Prague, Czech Republic, 13th– 17th Oct, 2025

Agenda item	:		8.13.3

Source	: 	Sharp

Title	: 	(SRAP-6) discussion on remaining issues related to U-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

RAN2 discussed SRAP issue via post email discussion and Rapporteur suggested to discuss one remaining issues [1]:

Discussion

(SRAP-6): intermediate relay UE behaviour for SRB1

The issue SRAP-6 is below:

RAN2 to discuss the RLC channel configuration to be used for SRB1 in multi-hop U2N Relay:

-	Option-1: Similar to SRB0, the default SL-RLC1 can be used on the PC5-hop between remote UE and the First U2N Relay, dedicated RLC channel configuration is used on the other hops.

-	Option-2: The default SL-RLC1 can be used on all the hops, Additionally, define the behavior of intermediate relay UEs depending on whether the RLC channel configuration for SRB1 is present or absent.

We think it is natural to follow Rel-17 behaviour: UE can use default SL-RLC1 as fallback, but PC5 RLC channel for SRB1 is configured via dedicated signalling, i.e., RRCResume, by gNB.

Furthermore, we provided proposed change in J011 as below[2]. Rapp indicated that the sl-EgressRLC-Channel-DL is used for DL transmission. UE also needs to transmit SRB1 data on UL. Then, proposed change in J011 can be further updated.

[Proposed Change]: 1> for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition): 2> configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList if applicable; 2> for L2 U2N Relay UE in case of single hop or L2 Last U2N Relay UE: 3> if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured: 4> release SL-RLC1, if established; 4> associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1; 3> else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured) 4> if SL-RLC1 is not established: 5> apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1; 2> for L2 Intermediate U2N Relay UE: 3> if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-Channel-DL is configured: 4> release SL-RLC1, if established; 4> associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-Channel-DL with SRB1; 3> else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-Channel-DL is not configured) 4> if SL-RLC1 is not established: 5> apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

- (SRAP-6) Add procedural text in the RRC specification to describe the SRB1 behaviour for Intermediate U2N Relay UE, dedicated signalling can be used to configure RLC channel for SRB1 for both of a remote UE and an intermediate relay UE.

Conclusion

In this paper, we made following observations and proposals:

- (SRAP-6) Add procedural text in the RRC specification to describe the SRB1 behaviour for Intermediate U2N Relay UE, dedicated signalling can be used to configure RLC channel for SRB1 for both of a remote UE and an intermediate relay UE.

Reference

- R2-250xxxx, Report of [Post131][411][Relay] Rel-19 relay 38.351 CR (OPPO)

- R2-250xxxx, SLRelay related RIL from ASN.1 review