3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507600

Prague, Czech Republic, Oct 13th - 17th 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining MAC Issues of UEIBM

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

According to the post email discussion [Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung), there are still some remaining MAC issues on the UEIBM, In this paper we would like to share our views on these remaining issues.

- Discussion

In this chapter, we first discuss PUCCH release upon TAT expiry, then shares our views on BWP deactivation, and finally provides a brief discussion on the dormant BWP case.

2.1 PUCCH Release upon TAT Expiry

In the post-email discussions, one remaining issue concerns PUCCH release upon TAT expiry. Contributions are expected based on the following proposal:

Proposal 1: For mode-B UEI reporting, PUCCH and type-1 CG PUSCH resource can be associated with different TAGs. Discuss UE behavior when the TAT for PUSCH is expired while the TAT for PUCCH is running. - Option-1: release the associated PUCCH; - Option-2a: not release PUCCH and transmit the Report Indication on PUCCH; - Option 2b: not release PUCCH and not transmit the Report Indication on PUCCH.

Based on company feedback, the resolution of this issue primarily depends on whether the PUCCH resource would be used further, for example:

- Case 1: The PUCCH resource is still needed for HARQ-ACK or other CSI reporting.

- Case 2: The network might restore uplink transmission on this SCell later.

Case 1 requires further confirmation from RAN1. Case 2 can be discussed in RAN2. For instance, if the uplink capacity of the pTAG becomes insufficient, the network may need to restore uplink transmission on this SCell. If the PUCCH resource has not been released, the network would not need to reconfigure the UE.

From the network's perspective, even if the UE does not release the corresponding PUCCH resource, the network is aware that this PUCCH will not be used for UE Mode B report indication transmission if Option 2b is selected. Whether this PUCCH resource is reallocated for other purposes or to other UEs can be entirely handled by network implementation. Therefore, Option 2b is slightly preferred from the network side as it offers greater flexibility.

Proposal 1: For Mode-B UEI reporting, if the PUSCH resource of a UEI report configuration points to a SCell whose sTAG's TAT has expired, the UE should not release the corresponding PUCCH resource and should not transmit the Report Indication on PUCCH.

On the TP part, the first thing is to clarify the companies’ understanding on the current spec.

2> else: 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells;

In the current spec, the wording “for” is used instead of the “on”, thus before any specification modification, RAN2 should first confirm whether such a cross-carrier case is already covered.

Proposal 1a:  RAN2 should first confirm whether the current specification includes such a cross-carrier case.

2.2 Deactivated BWP

In the post-meeting email discussion, another remaining issue concerns the transmission of the UEI Report Indication for the deactivated BWP case. Contributions are expected based on the following proposal:

Proposal 2: Discuss whether to transmit UEI Report Indication on PUCCH if the BWP for the associated mode-B CG PUSCH is deactivated.

If the BWP carrying the Mode-B CG PUSCH is deactivated, the initial PUCCH-based Report Indication need not be transmitted. The key question for RAN2 is to determine whether this scenario can be managed entirely through network implementation. For instance, the network could avoid configuring a Mode-B CG PUSCH on a SCell BWP that it might deactivate later.

Proposal 2: If the BWP carrying the Mode-B CG PUSCH is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH. RAN2 is to confirm whether this should be specified as a defined behavior or left to network implementation to avoid.

2.3 UEIBR for the Dormant BWP

For the dormant BWP, in the previous meetings, it has been agreed that

If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

However it’s not clear on whether it’s allowed the UE to perform mode-B beam reporting for this BWP. There are 2 different understandings:

- Understanding 1: The UE can perform mode-B beam reporting for this BWP.

- Understanding 2: The UE can’t perform mode-B beam reporting for this BWP.

## 5.15.1 Downlink and Uplink 1> if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP: 2> stop the bwp-InactivityTimer of this Serving Cell, if running. 2> not monitor the PDCCH on the BWP; 2> not monitor the PDCCH for the BWP; 2> not receive DL-SCH on the BWP; 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

If we take the legacy semi-persistent CSI report behaviour, it means the UE can report it for this dormant BWP, e.g. report it on the SpCell. However, for the UEI mode B CSI report for the dormant BWP, the question is how to determine the current beam. Even in the last RAN1 meeting a new parameter was introduced for some special cases, it’s still hard to determine the current beam for the dormant BWP when this new parameter is absent.

| CCofIndicatedTCI-r19 | New | This parameter is used to indicate a CC index of the serving cell on which the indicated TCI state used to determine the current beam RS is applied | ServCellIndex |
| --- | --- | --- | --- |

Based on the above analysis, the understanding 2 is preferred for the UEI mode B CSI report.

Proposal 3: When the BWP in a SCell is a dormant BWP, the UE should not perform UEI mode B CSI report for that BWP.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: For Mode-B UEI reporting, if the PUSCH resource of a UEI report configuration points to a SCell whose sTAG's TAT has expired, the UE should not release the corresponding PUCCH resource and should not transmit the Report Indication on PUCCH.

Proposal 1a:  RAN2 should first confirm whether the current specification includes such a cross-carrier case.

Proposal 2: If the BWP carrying the Mode-B CG PUSCH is deactivated, the UE shall not transmit the UEI Report Indication on PUCCH. RAN2 is to confirm whether this should be specified as a defined behavior or left to network implementation to avoid.

Proposal 3: When the BWP in a SCell is a dormant BWP, the UE should not perform UEI mode B CSI report for that BWP.