3GPP TSG-RAN WG2 Meeting #131bis	R2-2506715

Prague, 13 – 17 October 2025

Agenda item:	10.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	6GR Common Aspects

WID/SID:	FS_6G_Radio- Release 20

Document for:	Discussion and Decision

# 1	Introduction

In this paper we consider various aspects affecting both user and control plane including:

Quality of Service (QoS) is a fundamental pillar of the RAN, crucial for the optimal utilization of resources (hardware, energy, and spectrum).

Joint network and UE energy savings are one of the key factors in the 6G design to allow energy cost efficient deployments as well keeping end user experience seamless.

Also security is a key area in 3GPP radio systems: It ensures privacy and confidentiality of communications. While security is mainly in the domain of SA3, RAN2 is responsible for RRC operation and should also consider the overall AS security procedures and consider if there are ways to avoid the most impactful and obvious omissions in the 5G protocol design.

Carrier Aggregation (CA) and Bandwidth adaptation are also aspects to consider in the design for 6G. A unified framework for Carrier Aggregation (CA) would provide a simplified approach to managing this feature and allow for shorter setup times. It is also beneficial to consider the varied bandwidth capabilities of the UEs in the design for bandwidth adaptation and management such that we simplify the design and reduce signalling overhead while improving switching times.

Non-Terrestrial Networks (NTN) is another key feature of 6G to provide connectivity in remote, rural, and areas where terrestrial networks are not economically viable, thereby enabling ~100% population coverage in 6G.

Lastly we consider 6G specification writing from testability point of view.

# 2	Quality of Service

The envisioned QoS framework for 6G requires intelligence to deal with all the challenges and future scenarios foreseen in the coming decade. To achieve this, we have identified four key RAN capabilities: identification, characterisation, monitoring and adjustments:

-	The identification of services by the RAN is expected to rely on QoS flows from the CN (with the configuration of appropriate traffic flow templates) as in 5G.

-	The characterisation is expected to be based on existing 5G mechanisms where 5QI is used to define the specific quality profile for a service by the CN. A potential enhancement under discussion in SA2 involves introducing an adaptive resource type to better support interactive services like XR and streaming. This addresses current RAN limitations in efficiently managing applications that demand real-time adjustments to both throughput and latency without over-provisioning resources and thus reducing network capacity [RP-251986]. That discussion is to be taken place in SA2 though.

-	The monitoring mechanisms currently defined in the QoE Measurement Collection (QMC) framework are good for network optimization and benchmarking purposes but not designed for immediate enhancement of a UE's user experience. This ought to be enhanced and can be generally referred to as observability [RP-252227]

-	The adjustments RAN2 needs to discuss are the ones related to uplink i.e. how to ensure that the QoS requirements of a given QoS flow or a group of QoS flows are fulfilled, in other word, how to control the dynamic mapping of QoS flows onto uplink transport blocks.

We have identified five essential requirements for the control of uplink QoS flow(s):

1.	A QoS flow must have the possibility to go through first no matter what (e.g. for SRB);

NOTE:	SRBs are not QoS flows but let’s assume they are for the sake of reasoning.

2.	The RAN must be able to configure the UE in order to guarantee a bit rate (e.g. GBR for voice);

3.	The RAN must be able to configure the UE in order to avoid starving low priority traffic (e.g. using PBR);

4.	The priority of a QoS flow must be allowed to vary according to the looming possibility of discard (e.g. as for XR in Rel-19);

5.	The RAN must be able to configure the UE in order to limit a bit rate (e.g. MFBR).

The LCP framework of 5G in Rel-19 fulfils the first four requirements but not the fifth one (at least not without artificial segregation of resources and mapping restrictions on those). This is something that must be enhanced in 6G to efficiently support e.g. Slicing, QoS control and also make SCONE relevant.

Proposal 1: the QoS framework of 6G is based on that of 5G, with a control of uplink QoS flows that shall allow absolute prioritisation, guarantee a bit rate, avoid starvation, adjust the priority based on the looming possibility of discard, and enforce bit rate limits, while being observable.

# 3	Reinforced AS security for 6G

In 5G AS security, the PDCP layer plays a crucial role in ensuring the confidentiality and integrity of both user plane and control plane packets. It is responsible for ciphering and integrity protection, which secures the data transmitted between the user equipment and the network from unauthorized access and tampering. The PDCP layer provides integrity and confidentiality protection for the RRC layer, utilizing a 128-bit integrity key (KRRCint) and a 128-bit cipher key (KRRCenc) derived from the security key hierarchy. While the PDCP layer secures the data packets, the headers for PDCP and lower layers are not ciphered or integrity-protected, meaning certain control elements are not protected within the user plane layers.

The lack of protection for MAC CEs presents several security vulnerabilities. Attackers can exploit these unprotected elements to carry out various malicious activities. For instance, they can manipulate the SCell Activation/Deactivation MAC CE as part of Carrier Aggregation attacks to maliciously activate unnecessary secondary cells—causing increased UE power consumption and faster battery drain—or deactivate required ones, leading to reduced throughput and degraded user experience. Another exploitation involves the Timing Advance Command, where attackers can listen to these commands to partially locate users within a cell, posing a risk to user privacy. Additionally, attackers can manipulate reference signal activation MAC CEs, such as Channel State Information (CSI) reporting, to disrupt service configurations or track user movements within a cell. Spoofing and sniffing attacks can force UEs to perform wideband UL transmissions, causing collisions and jamming legitimate UE transmissions. Buffer Status Report attacks can lead to bandwidth theft, where attackers send false buffer status reports, causing the network to allocate fewer resources to victim UEs and more to the attacker. Discontinuous Reception (DRX) and Power Headroom (PHR) attacks result in service degradation, where spoofing low recommended bitrate requests can degrade user QoS, and Timing Advance (TA) MAC CE injections can desynchronize uplink transmissions. Tracking and localization using reference signals can disrupt channel estimation, as disabling semi-persistent SRS transmissions can lower UE throughput, especially in beamforming scenarios. Active attacks, such as Denial of Service, can block initial access to a cell, disconnect users, and severely impact throughput. Amplified Distributed DoS attacks can spoof uplink grants, tricking UEs into constant transmission, jamming legitimate communications, and causing a sharp drop in performance.

During the initial access phase, the RRC connection setup is vulnerable to various attacks because security is not activated until after the RRC connection is established. This vulnerability exposes the network to potential spoofing, jamming, and sniffing attacks. The initial access phase involves several messages exchanged between the UE and the network, such as Msg3, Msg4, and Msg5, which are not protected by AS security. This unprotected exchange can be targeted by attackers to disrupt the connection setup process or to gain unauthorized access to network resources.

Besides, broadcasting essential network parameters to all UEs in a cell can be seen as a security risk. The system information must be accessible to all UEs, making traditional encryption and integrity protection difficult. The main threat lies in the potential manipulation of system information, which could disrupt the RRC connection establishment. To counter this, the proposal suggests a secure broadcast mechanism that ensures the integrity of system information while maintaining accessibility for all UEs.

Finally, quantum-safe algorithms are essential to safeguard against the potential threats posed by quantum computing. The transition from 128-bit to 256-bit keys in the future, as discussed in the NIST, is a significant step towards enhancing encryption and integrity protection, making them resilient to quantum attacks. This shift requires interaction with SA3 to ensure that the proposed algorithms are standardized and adopted across the industry.

Improving 6G security in the RAN thus involves addressing several critical areas:

1.	Quantum-safe algorithms;

2.	The initial access phases of the RRC of connection setup and resume;

3.	MAC CEs;

4.	System Information broadcast.

As usual, RAN2 should collaborate with SA3 on AS security matters, especially given the June 2026 checkpoint in the SA3 6G security SID [SP-251233].

Proposal 2: the security framework of 6GR shall be based on Quantum-safe algorithms and also aim at protecting the initial access phase of RRC connection setup and resume, MAC CEs and system information broadcast.

# 4	Energy efficiency in 6G

5G NR’s design offers improved network energy saving from the beginning. Unlike LTE’s CRSs in every DL slot, NR avoids this always-on signalling. In addition, the NR design integrates PSS/SSS and PBCH in one single SS/PBCH block, optimizing beam sweeping. The default SS/PBCH block transmission periodicity in NR is 20 ms, and this is the setting used by virtually all deployments.

In 5G-A, further work on NES has been conducted in Rel-18 and Rel-19. The Rel-18 study item phase on NES identified several beneficial NES adaptation schemes categorized by time, frequency, spatial and power domains, as summarized in TS 38.864. RAN1 addressed time, spatial and power domain techniques primarily in the Rel-18 NES WI [3], targeting cells with RRC CONNECTED UEs. Rel-19 NES WI [4] further considers time and frequency domain techniques, targeting primarily cells with RRC IDLE/INACTIVE UEs.

The 5G and 5G-A NES features are shown in Figure 1. As can be seen from Figure 1, many different NES techniques are defined in 5G. However, 5G has been adding NES features on top of the NR baseline (Rel-15), which has inherent limitations on applicability of such techniques given the presence of legacy UEs in the field. For 6G, the intent should be to have NES by design from the first release, so that the entire network energy-saving toolbox, incorporating our learnings from LTE and NR, can be applied across all cells and all 6G devices from day-one.

Figure 1. List of NR network energy saving features

Energy efficiency is one of the important aspects for the design of 6G networks. However, independent design of network energy saving (NES) and UE energy saving (ES) mechanisms may limit the targeted benefits from both. In some cases, UE energy saving mechanisms may increase power consumption on the network side. Similarly, designing NES operations without taking the UE ES into account may lead to UE energy consumption issues. In previous 5G NR studies, the alignment between network and UE energy-saving functionalities in different RRC states has been deemed beneficial. For 6G, the network energy saving impact on UE energy consumption and vice versa need to be evaluated and taken into account when designing 6G NES operations and UE power saving mechanisms.

Furthermore, it is critical to enable energy saving in the most relevant practical network deployments. For example, also in scenarios where operators have limited band capability and it may not be feasible to consider multi carrier scenarios, but rather the one available band is used for all the purposes. Thus, we need to consider energy saving design and features applicable to single-band deployments in which such coverage layer cannot be switched off. Basically, we need to enable operators that need to keep the single-band active all the time still to obtain energy saving compared to 4G/5G. This necessitates that energy-saving designs and features should be applicable to the coverage layer, rather than being limited to the capacity layer only or mainly.

For initial 6G deployments, largely characterized by low or no-load conditions, the default operational mode should be “energy-saving”, while additional capacity, network transmissions, and operations should be activated dynamically based on actual demand. Such default mode is beneficial also considering that a significant portion of base stations in mature networks will frequently operate under low or no-load conditions as around 70% of base stations only carry around 20% of traffic, therefore their utilization is low. Such default mode means for example infrequent transmission of always-on signals and control channels, and reduced PRACH and paging occasions to ensure NES gain, also in the coverage layer which cannot be switched off. A 6G cell should be able to switch smoothly from the NES-default operation mode to a less energy conserving mode when the load increases, for example by adapting the periodicity of the broadcasting signals and channels. It may be beneficial to enable a more aggressive default NES operation mode for capacity layer cells than for coverage layer cells. This could involve more aggressive relaxation or omission of broadcast channels and signals in the capacity cells, e.g. only transmitting relaxed signals not enough for camping as such or just by turning off those carriers when there is no need for extra capacity.

Proposal 3: From day one 3GPP should design 6G energy saving mechanisms taking into account joint network and UE optimizations for both coverage and capacity layers, single and multiple carrier deployments and allowing reduced transmissions of always-on signals.

Similarly, as for network energy saving also for UE power saving many different techniques are defined in NR as can be seen from Figure 2.

Figure 2. UE power saving mechanisms in 5G.

For UE power saving in NR DCP and Low Power Wake-Up Signal (LP-WUS) both serve the same purpose in the RRC_CONNECTED state, while Paging Early Indication (PEI) and LP-WUS serve the same purpose in the RRC_IDLE and RRC_INACTIVE states. To maintain efficiency and avoid redundancy, 3GPP should strive to minimize the definition of parallel solutions that achieve the same functionality.

Proposal 4: RAN2 to discuss and decide which UE power saving features are inherited from 5G to 6G while avoiding specifying multiple features for the same purpose.

# 5	Spectrum aggregation and bandwidth

In this section we discuss the spectrum management aspects, specifically addressing carrier aggregation and bandwidth adaptation.

Carrier aggregation (CA): The baseline 5G CA framework inherited from LTE-Rel-10, where each uplink (UL) is paired to a specific downlink (DL) serving cell, with the UL CA being a subset of the configured DL CA. However, other multi-carrier features of 5G such as supplementary UL, UL TX switching, and low-band carrier aggregation switching operate differently, not allowing simultaneous transmission across all carriers and thus being configured differently. Furthermore, the setup and activation of these carriers require additional time-consuming such as initial carrier measurement, precise synchronization, and channel acquisition.

For 6GR, a single(unified) framework for multi-carrier usage based on carrier aggregation should be studied, which incorporates flexible UL/DL band pairing and UL TX switching. This approach would simplify the management of multi-carrier features into a cohesive architecture and allow higher data rates with increased capacity demands. Additional studies should also be considered to optimize the setup times for the multiple carriers. 6GR requires all these features to ensure that the network can quickly establish and utilize the aggregated bandwidth, leading to improved user experience and network efficiency.

Bandwidth adaptation: 5G defined the Bandwidth part (BWP) as a contiguous set of frequency resources within a carrier that can be configured for a UE within a single cell. It was specified with the intention to support different UE BW capabilities while providing networks the flexibility to assign UEs to different part of the available carrier bandwidth. The BWP framework also supports UE power saving by allowing network to use dynamic switching between varying UE bandwidths. However, the current system is considerably complex and has a significant signalling overhead caused by various physical parameters e.g subcarrier spacing, CORESETs, search spaces, reference signal configurations etc. This individual flexibility not only leads to various BWP types, but also different switching delays.

For 6GR, we should continue supporting UE bandwidth capabilities with adaptability for increased bandwidths and additional use cases. 6G should also focus on clearly defining the use cases and scenarios for bandwidth adaptation with the aim for a simpler design and reduced RRC overhead. Developing more reliable switching mechanism over faster switching is crucial, which requires involving RAN4 in the design process, to optimize RF requirements along with bandwidth adaptation functionality.

Proposal 5: RAN2 to study 6GR spectrum management, focusing on scenarios for bandwidth adaptation and a unified carrier aggregation framework that natively supports flexible uplink/downlink pairing and uplink Tx switching. The study shall investigate optimal configurations and signalling mechanisms that minimize overhead, and support rapid SCell activation mechanisms natively from the initial 6G release.

# 6	Non-Terrestrial Networks (NTN)

The 6GR SID tasks the WGs to “Aim at a harmonized 6G Radio design for TN and NTN, including their integration.”

NTN has already seen quite a large body of work over the previous 3GPP releases, evaluating and introducing optimizations on 5G NR, eMTC and NB-IoT radios for NTN. A multitude of satellite deployments in terms of architecture, satellite altitude and satellite footprint (cell sizes) are supported. For example, the cell radius can be signalled to be in the range of 10 km to 2560 km while system timers facilitate satellite altitudes ranging from LEO (few hundred km) to GEO (about 36000 km). The supported frequency bands range from L & S bands to Ku & Ka bands and in combination with handheld, IoT and VSAT terminals the supported scenarios include non-delay sensitive IoT, messaging, voice, video, and mobile & fixed broadband. 5G NTN mobility has been enhanced with several NTN-specific solutions, such as distance- or time-based measurement event reporting or Conditional Handover triggering. Release 18 has also introduced satellite switching with resynchronization to make the cell change procedure invisible to the UE.

While most of the features defined for 5G IoT NTN and NR NTN seem to be well-justified, we do not think all shall be blindly copied to 6G (e.g. consider carefully the usefulness and feasibility of satellite switching with resynchronization). Therefore, for 6G NTN, it shall provide support of key performance requirements defined for 5G NR NTN and IoT NTN but maybe not necessarily all features. The exact list of features to be inherited from 5G NTN to 6G NTN can be further discussed. Furthermore, following the learnings of the 5G NR NTN studies, the primary focus on the 6G NTN related studies should be on FDD operation.

For 6G, as indicated in the SID, a special emphasis in the NTN design for 6GR should be placed on commonality with TN so that the same HW platforms developed for TN can be configured to NTN operation with minimum NTN-specific software extensions. For example, solutions developed for improved coverage critical for NTN should be seen as 6GR solutions applicable to both NTN and TN, including the optimizations defined to meet the LPWA-originating requirements. This will enable devices to be able to support TN and NTN with the same base chipset and also offer service continuity to NTN when TN is not available. In general, 3GPP should avoid defining parallel solutions for NTN, TN and LPWA, so the solutions defined should take all these different use cases into account – e.g. wrt. NTN the large RTT impact needs to be considered in the base design from 6G day one. Exceptionally, certain features developed for NTN, such as the mobility solutions based on time and distance that account for satellite movement, are inherently tied to the nature of satellites. These features are technically applicable in NTN but not in TN and therefore should be discussed exclusively in the context of 6G NTN.

Additionally, in 5G, NTN is relying on GNSS, and the UE is aware of its own position. This is used for instance for initial access, mobility procedures and PLMN selection. As GNSS can be spoofed, jammed or become unavailable due to radio conditions, this makes the system vulnerable. Furthermore, the GNSS module also adds cost, power consumption and complexity to the UE. Therefore the 6G NTN system should be able to function without the need for the UE to have GNSS capability. However, the 5G NTN approach of UE-based pre-compensation, where the UE autonomously adjusts the timing advance and frequency offset is more power & signalling efficient than the use of closed loop commands (similar to TN timing advance commands), when considering the need for frequent updates due to the satellite movement. The pre-compensation is based on the UE being aware of its geo-location and the satellite’s location, and therefore the 6G NTN UE shall still be geo-location aware, but it should not be mandated that the location awareness is achieved via GNSS. Below table is a summary of our understanding on the key requirements for 6G NTN.

Table 1: Key requirements for 6G NTN

| NTN aspect | Support |
| --- | --- |
| Satellite architecture | Regenerative & transparent architecture. The architecture impact of support for UE-satellite-UE communication needs further discussion. |
| Satellite altitude | The range from VLEO (few hundred km) to GSO (about 36000 km) and possibly support for multi-orbit deployments. |
| Satellite footprint | A cell radius of a few tens of km and up to maximum cell range supported in terrestrial 6G network. |
| Frequency bands | At least existing 5G & IoT NTN bands in L, S, Ka and Ku range. |
| UE types | Handheld LPWA & MBB devices and VSAT for FWA. The power classes, bandwidth, number of antennas, number of HARQ processes, and duplexing schemes need further discussion. |
| Traffic types | At least non-delay sensitive IoT, messaging, voice, video, and mobile & fixed broadband. |
| Mobility | Seamless mobility for NTN-NTN and NTN-TN also needs to be considered for certain traffic types. |
| GNSS requirement | The 6G NTN system shall function without the need for UEs to have GNSS capability. |

Proposal 6: RAN2 study and agree on designing 6G NTN in a harmonized manner from 6G day one to ensure feature applicability to both terrestrial (TN) and non-terrestrial network (NTN) where technically applicable, guarantee support for UEs without GNSS capability, and fully uphold the key performance requirements defined for 5G NR NTN and IoT NTN, with FDD operation as the primary duplexing mode.

# 7	Testable features in 6G

3GPP RAN5 is responsible for defining and maintaining conformance testing specifications for User Equipment. The primary objective of RAN5 is to ensure that UEs meet the technical requirements defined by other 3GPP working groups, such as RAN2, which focuses on protocol specifications. These conformance tests are critical to ensuring that UEs comply with 3GPP standards and function reliably within the network. For RAN2 specifications, RAN5 has developed test cases for key protocol layers, including MAC, RLC, PDCP, and RRC signaling.

Several features in 3GPP specifications have been defined with the note “up to UE implementation”. This provides implementation flexibility beyond capabilities but creates difficulties for testing and interoperability. Conformance testing can only validate the aspects explicitly defined in the protocol, while the remaining behavior is unspecified. Different vendors may therefore exhibit divergent behavior under identical traffic conditions, making deterministic testing impossible.

RAN5 specifies test cases exclusively for clear UE requirements, where the verbal form “shall” is used. The term "shall" indicates mandatory requirements (possibly within a capability) that must be met for compliance. According to the Specification Drafting Rules outlined in [3GPP TR 21.801], the verbal forms listed in Table 7.1 below are strictly used to indicate requirements that must be followed to conform to the standard, with no deviations permitted.

Table 7.1: Requirement

| Verbal form | Equivalent expressions for use in exceptional cases (see clause 6.6.1) |
| --- | --- |
| shall | is to is required to it is required that has to only ... is permitted it is necessary |
| shall not | is not allowed [permitted] [acceptable] [permissible] is required to be not is required that ... be not is not to be |
| Do not use "must" as an alternative for "shall". (This will avoid any confusion between the requirements of a standard and external statutory obligations). Do not use "may not" instead "shall not" to express a prohibition. To express a direct instruction, for example referring to steps to be taken in a test method, use the imperative mood (e.g. "switch on the recorder"). |  |

RAN5 does not specify test cases for requirements expressed using other verbal forms such as “may,” “should,” “can” or “up to UE implementation”. These terms introduce optionality or recommendations beyond capabilities, leading to untestable and thus unpredictable UE behaviour.  RAN5 focuses solely on testable and mandatory requirements to ensure consistency and compliance.

In addition, some features depend heavily on UE application-layer interfaces. These interfaces are outside the scope of 3GPP but critical for end-to-end service realization. Even if the RAN and Core signaling behave correctly, missing or vendor-specific application interfaces can prevent the feature from being validated in practice. This gap between standardized protocol behavior and application-layer integration complicates interoperability and service assurance.

If a feature includes UE capability and the UE supports that feature, RAN2 should aim to specify with clear and precise requirements for the UE. For example: 'If the UE supports capability X, then the UE shall [specific requirement].' This ensures consistency and eliminates ambiguity in the implementation of UE capabilities.

To enhance clarity and ensure predictable UE behaviour, the following is proposed:

Proposal 7: Prioritize in specifications the use of “UE shall” to define testable UE requirements and avoid using “UE may,” “UE should,”, “UE can” or “up to UE implementation” as much as possible, as these terms introduce ambiguity and hinder the creation of RAN5 test cases. For optional features, use 'UE shall' to define requirements clearly, for example: “if the UE supports capability X, then the UE shall [specific requirement]”.

# 5	Conclusions

We have discussed the energy efficiency, NTN, QoS, AS security, BWP&CA and testability for 6G, with the following conclusions

Proposal 1: the QoS framework of 6G is based on that of 5G, with a control of uplink QoS flows that shall allow absolute prioritisation, guarantee a bit rate, avoid starvation, adjust the priority based on the looming possibility of discard, and enforce bit rate limits, while being observable.

Proposal 2: the security framework of 6GR shall be based on Quantum-safe algorithms and also aim at protecting the initial access phase of RRC connection setup and resume, MAC CEs and system information broadcast.

Proposal 3: From day one 3GPP should design 6G energy saving mechanisms taking into account joint network and UE optimizations for both coverage and capacity layers, single and multiple carrier deployments and allowing reduced transmissions of always-on signals.

Proposal 4: RAN2 to discuss and decide which UE power saving features are inherited from 5G to 6G while avoiding specifying multiple features for the same purpose.

Proposal 5: RAN2 to study 6GR spectrum management, focusing on scenarios for bandwidth adaptation and a unified carrier aggregation framework that natively supports flexible uplink/downlink pairing and uplink Tx switching. The study shall investigate optimal configurations and signalling mechanisms that minimize overhead, and support rapid SCell activation mechanisms natively from the initial 6G release.

Proposal 6: RAN2 study and agree on designing 6G NTN in a harmonized manner from 6G day one to ensure feature applicability to both terrestrial (TN) and non-terrestrial network (NTN) where technically applicable, guarantee support for UEs without GNSS capability, and fully uphold the key performance requirements defined for 5G NR NTN and IoT NTN, with FDD operation as the primary duplexing mode.

Proposal 7: Prioritize in specifications the use of “UE shall” to define testable UE requirements and avoid using “UE may,” “UE should,”, “UE can” or “up to UE implementation” as much as possible, as these terms introduce ambiguity and hinder the creation of RAN5 test cases. For optional features, use 'UE shall' to define requirements clearly, for example: “if the UE supports capability X, then the UE shall [specific requirement]”.