3GPP TSG RAN WG2#131bis	R2-2507618

Prague, Czech, 13th – 17th Oct. 2025

Agenda item:	8.4.4

Source:	Nokia

Title:	Discussion on open issues in 38.304 for LP-WUS

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The discussion paper focuses on the following open issue:

38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS.

# 2	Discussion

It was discussed that a moving UE in an urban environment cannot be permitted to suspend measurements for up to 20 seconds, as this would negatively impact cell reselection and paging performance. In contrast, a stationary UE can be allowed to suspend neighbor cell measurements until it begins moving again. Therefore, an LP-WUS UE should be enabled to utilize LR measurements to assess the "low mobility" and "not-at-cell-edge" criteria.

In Release 16, the low mobility criterion and the "not at cell edge" criterion were specified for neighbor cell RRM measurement relaxation. These criteria are evaluated solely based on MR measurement results. In Release 19, new functionalities were introduced, including serving cell RRM measurement offloading and serving/neighbour cell RRM measurement relaxation based on both LR and MR measurement results. Some companies raised concerns that if a UE supports both R16 RRM relaxation and R19 serving cell RRM offloading and relaxation, there could be potential impacts on the R16 criteria, particularly in scenarios where serving cell offloading occurs and MR measurement results are unavailable.

In R16, the concept of neighbor cell measurement relaxation was introduced, allowing the network to configure either the "not at cell edge" criterion, the low mobility criterion, or both. The low mobility criterion was primarily designed for UEs with low mobility, enabling power-saving benefits for such devices. It is also advantageous to extend the low mobility criterion to LP-WUS UEs. This would allow an evaluation of their capability to perform serving cell offloading, serving cell relaxation, and neighbor cell relaxation effectively. In Rel-16, the low mobility criterion is defined as below:

##### 5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

If the relaxed measurement criterion outlined in clause 5.2.4.9.1 is satisfied for a duration of TSearchDeltaP, the UE may opt to perform relaxed measurements for intra-frequency cells, inter-frequency cells, and inter-RAT frequency cells.

In scenarios where offloading occurs, the LR remains active while the MR is deactivated when the low mobility criteria are met. Under these conditions, the UE can rely solely on LR measurement results to determine whether the low mobility criteria is fulfilled.

Proposal 1: The UE that is not low mobile is not allowed to use R19 serving cell offloading, serving cell measurement relaxation and neighbour cell measurement relaxation.

Proposal 2: NW can configure low mobility criterion to start/stop R19 serving cell offloading, serving cell measurement relaxation and neighbour cell measurement relaxation.

Proposal 3: Re-use R16 relaxed measurement criterion for UE with low mobility for R19 serving cell offloading, serving cell measurement relaxation and neighbour cell measurement relaxation. Use LR measurement results instead of MR measurement results in low mobility criteria

# 3	Conclusion

Based on the discussion the following is proposed:

Proposal 1: The UE that is not low mobile is not allowed to use R19 serving cell offloading, serving cell measurement relaxation and neighbour cell measurement relaxation.

Proposal 2: NW can configure low mobility criterion to start/stop R19 serving cell offloading, serving cell measurement relaxation and neighbour cell measurement relaxation.

Proposal 3: Re-use R16 relaxed measurement criterion for UE with low mobility for R19 serving cell offloading, serving cell measurement relaxation and neighbour cell measurement relaxation. Use LR measurement results instead of MR measurement results in low mobility criteria

# 4	References