3GPP TSG-RAN WG2 #131-bis	R2-2507623

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	8.10.3

Source: 	Ericsson

Title:	Stage-2 corrections for SONMDT features

Document for:	Discussion and Decision

# 1 Introduction

This paper discusses the stage-2 description of MRO for LTM as well as MDT enhancements in release 19. In particular, it deals with the SHR for LTM and logging NSAG information in the MDT are discussed in the following.

# 2 Discussion

It has been agreed in Rel-19 to cover LTM aspects in the SHR for further analysis and optimization of the handover parameters. However, the stage-2 description of SHR does not reflect that it also applies to LTM cell switches.

RAN2 to discuss changes to stage-2 description of SHR according to the TP in Annex 1.

Furthermore, it has been agreed in Rel-19 to include information regarding UE not being able to re-select to a cell according to NSAG priorities in logged MDT report. However, the stage-2 description does not reflect that.

RAN2 to discuss changes to stage-2 description of logged MDT according to the TP in Annex 2.

# 3 Conclusion

Stage-2 aspects of MRO were discussed in this paper. Based on the above discussion, we propose that:

Proposal 1	RAN2 to discuss changes to stage-2 description of SHR according to the TP in Annex 1.

Proposal 2	RAN2 to discuss changes to stage-2 description of logged MDT according to the TP in Annex 2.

# Annex 1: Text proposal to TS 38.300 (Successful HO)

First change, ommited text not changed

#### 15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful LTM cell switches, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover/LTM cell switch, the UE may collect Successful Handover Report (SHR) based on configuration by network, if stored, and makes the SHR available to the network as specified in TS 38.331 [12]. The UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded.

For SHR collected during intra-NR handover/LTM cell switch, if the target NR node fetches the SHR from the UE and the trigger of SHR is T310/T312, it may forward the information to the source NR node, i.e. the node handling the cell reported as source cell in this SHR, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

If the NG-RAN node that fetches the SHR from the UE is neither the source node nor the target node of the handover, it forwards the information to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case of failure shortly after successful Handover/LTM cell switch, the same mobility event may generate both a SHR and a RLF report. In this case, the node(s), which configured the SHR trigger causing the SHR, may take the duplication into account e.g. ignore the SHR.

Upon retrieval of an SHR, the receiving node may analyse whether its mobility configuration needs adjustment.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.

The SHR report can be collected for intra-NR handover/LTM cell switch and for handover from NR to E-UTRA.

Remaining text not changed

# Annex 2: Text proposal to TS 37.320 (logged MDT)

First change, ommited text not changed

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AICH	Acquisition Indicator CHannel

BLER	Block Error Rate

BSSID	Basic Service Set Identifier

CA	Carrier Aggregation

CDMA	Code Division Multiple Access

CHO	Conditional Handover

CN	Core Network

CPICH	Common Pilot CHannel

DAPS	Dual Active Protocol Stack

DCH	Dedicated CHannel

DL	Downlink

DRX	Discontinuous Reception

ECGI	E-UTRAN Cell Global Identifier

E-CID	Enhanced Cell-ID (positioning method)

E-DCH	Enhanced Uplink DCH

EDGE	Enhanced Data rates for GSM Evolution

E-RUCCH	E-DCH Random Access Uplink Control CHannel

eNB	Evolved NodeB

EPLMN	Equivalent PLMN

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FACH	Forward Access CHannel

FDD	Frequency Division Duplex

FIFO	First Input First Output

FPACH	Fast Physical Access CHannel

GERAN	GSM EDGE Radio Access Network

gNB	Next Generation Node B

GNSS	Global Navigation Satellite System

HESSID	Homogenous Extended Service Set Identifier

HOF	Handover Failure

IMEI-SV	International Mobile Equipment Identity Software Version

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISCP	Interference on Signal Code Power

LA	Location Area

LTE	Long Term Evolution

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBSFN	MBMS Single Frequency Network

MDT	Minimization of Drive-Tests

NG-RAN	Next Generation RAN

NPN	Non-Public Network

NR	New Radio

NSAG	Network Slice Access Group

OAM	Operation and Maintenance

P-CCPCH	Primary Physical Common Control CHannel

PCH	Paging CHannel

PCI	Physical Cell Id

PDCP	Packet Data Convergence Protocol

PH	Power Headroom

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

PS	Packet Switched

QCI	QoS Class Identifier

QoS	Quality of Service

RA	Routing Area

RAB	Radio Access Bearer

RAT	Radio Access Technology

RB	Radio Bearer

RF	Radio Frequency

RLC	Radio Link Control

RLF	Radio Link Failure

RNC	Radio Network Controller

RPLMN	Registered PLMN

RRC	Radio Resource Control

RRM	Radio Resource Management

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RTT	Round Trip Time

RTWP	Received Total Wideband Power

SCell	Secondary Cell

SIR	Signal to Interference Ratio

SINR	Signal to Noise plus Interference Ratio

SNPN	Stand-alone Non-Public Network

SNR	Signal to Noise Ratio

SON	Self Organizing/Optimizing Network

SRB	Signalling Radio Bearer

SRNC	Serving RNC

SSB	Synchronization Signal Block

SSID	Service Set Identifier

TA	Tracking Area

TCE	Trace Collection Entity

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunication System

UPH	Uplink PH

URA	UTRAN Registration Area

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

Next Change

#### 5.4.2.X	Logging of NSAG related information

For NR, upon UE failing to re-select to a suitable cell that supports the NSAG ID with the highest priority and selecting to a different cell, following information are logged:

- The NSAG ID with the highest priority;

- Cell ID of the re-selected cell;