3GPP TSG-RAN2#131bis	R2-2507641

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	9.7.2

Source:	Ericsson

Title:	NB-IoT NTN voice over GSO

Document for:	Discussion, Decision

# 1	Introduction

The new Rel-20 WID [1] aims to specify the RAN related enhancements for NB-IoT NTN to support IMS voice call over GSO.

The instructions in the agenda is to focus on the down-selection between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 and any responses to other WG LSs.

The down-selection between CP and UP solutions is discussed in section 2.1, and the LSs from other WGs is discussed in section 2.2.

In the reminder of the introduction, we discuss the WID and the WID objectives.

## 1.1	Service and system aspects

The SA groups have studied this scenario, as summarized in the RAN WID [1]:

SA1 has studied the use case of IMS voice call using GSO satellite access, and the results are documented in TR 22.887. Normative service requirements and KPIs on IMS voice call using GSO satellite access is captured in TS 22.261, as shown in Table 1 below. Table 1: Performance requirements for satellite access Scenario; Experienced data rate (DL); Experienced data rate (UL); Area traffic capacity (DL) (note 1); Area traffic capacity (UL) (note 1); Overall user density; Activity factor; UE speed; UE type; Others. IMS voice call using GEO (note 7); [1-3] kbit/s; [1-3] kbit/s; N/A; N/A; N/A; N/A; TBD; Handheld; Call set up time (note 8) ≤30 s (note 9). Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57];. SA has approved a new SA4-led SID on Ultra Low Bitrate Speech Codec in SP-250378. According to the agreed contribution in SA4 in S4-251123, the PHY bite rate is set to 1~5kbps and codec bite rate is set to 0.8~4.8kbps. SA also approved a new SA2-led SID on IMS voice call over GSO via NB-IoT-NTN in SP-250400. As WT#1 of the SID, both CP and UP based solutions are being studied and the SI is be concluded by TSG#110 (Dec. 2025), including the down-selection between CP and UP solutions.

The latest approved SA4-led SID is SP-250635 (revision of SP-250378).

Further SP-250787 (revised from SP-250400) is the latest approved SA2-led SID.

We note that there are not yet requirements on the delay, and that the data rates are very low, only 1 to 3 kbps. SA1 have not considered voice activity factor (that is when voice in one direction is silent while the other direction is talking), which may affect the capacity for voice (nr of users).

## 1.2	Objective of this work item

The objectives of this WI are:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

We give some preliminary comments to each of the objectives.

-	Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

-	Coordination with SA is required

-	The objective can be updated with further details based on the outcome of the above down-selection

The first point is to decide between UP and CP solution and present to RAN#110 in December. This has a sub-objective to coordinate with SA. We discuss this issue in section 2.1 below.

-	Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

As voice codecs have predictable packet creation rates and predictable packet size, semi persistent scheduling (SPS) is suitable. Especially for the UL, the extra delay of dynamic scheduling may lead to bad quality and SPS may be required to achieve acceptable quality.

-	Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

This is just business as usual for UP solution, while for CP we may need to add possibility of RRC reconfiguration to add/delete bearers and SPS grants/assignments (for example due to link adaptation).

-	Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

This means adding establismentCause emergency, and possibly more bearers per UE (for IMS signalling and for the emergency PDN gateway).

-	Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

We note that UE specific Koffset may be an issue in NGSO as TA reporting MAC CEs in UL and Differential Koffset MAC CEs in DL would mess up the transport block sizes of DL and UL SPS, and may therefore not be desirable, at least not in NGSO where there will be frequent report/commands.

# 2	Discussion

## 2.1	Selecting between CP and UP solutions

### 2.1.1	RLC UM vs. AM

In regular LTE (non-NB-IoT LTE), RLC UM is used for voice bearers.

Voice data that has much higher delay than other voice data do not contribute to the QoS and may be discarded by the voice codec.

In GSO, the RTT is about 542 ms for transparent payload [9] 38.821 Table 4.2-2, thus the delay for one RLC AM retransmission is the RTT plus time for RLC to discover missing RLC data (estimated at 1.5 s for one RLC retransmission in transparent architecture [9] 38.821 clause 7.2.2.2). For RLC to discover there is an outstanding RLC SN, a later SN must arrive at the UE, thus the retransmitted data may be discarded by the voice codec.

Therefore, RLC retransmissions will arrive so late that the data is useless and the transmissions wasted precious radio resources.

Using RLC AM for voice data in GSO will waste radio resources.

RLC UM for voice over GSO is supported regardless of whether UP or CP solution is selected.

### 2.1.2	Network architecture

23.401:

Figure 4.2.1-1: Non-roaming architecture for 3GPP accesses

There are deployments of the NB-IoT Control plane CIoT EPS optimization, but not of the NB-IoT User plane CIoT EPS optimization. However, there is no voice or other real time data support in current NB-IoT, while there are deployments of eMTC with User plane CIoT optimization that support real time voice over the user plane.

The NB-IoT User plane CIoT EPS optimization need to add real time user data to the S1-U interface. The S1-U interface carries real time user data in regular LTE (non-NB-IoT LTE).

The UP solution with real time data traffic needs updating the S1-U reference point to handle real time traffic. The S1-U interface can handle real time traffic in non-NB-IoT LTE.

To transfer real time data using the NB-IoT Control plane optimization for CIoT, we need to update the architecture such that S1-MME and S11-U can carry real time user data. Further, the data path for the real time data includes one extra node (the MME) that do not handled real time traffic today.

The CP solution with real time data traffic needs updating the S1-MME and S11-U reference points to handle real time traffic. These interfaces cannot carry real time traffic today.

The CP solution with real time data traffic needs one extra network node, the MME, compared to the UP solution. The MME cannot handle real time data traffic today.

### 2.1.3	QoS model

The QoS framework (36.300 clause 13, 23.401 clause 4.2, 23.203 clause 6.1.7, see excerpts in Appendix A) clearly specifies that for one EPS bearer, the packets shall get the same packet treatment.

36.300:

An EPS bearer/E-RAB is the level of granularity for bearer level QoS control in the EPC/E-UTRAN. That is, SDFs mapped to the same EPS bearer receive the same bearer level packet forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.), as specified in TS 23.401 [17].

…

Figure 13.1-1: EPS Bearer Service Architecture

The QoS framework only support QoS differentiation between bearers.

It may be possible to support some kind of “hack” to have sub-bearer QoS handling for the voice traffic, however, we think we shall not break the QoS framework.

As voice data likely needs to be on an RLC UM bearer and there is a need for a signalling radio bearer (SRB) using RLC AM for other traffic (RRC Release etc.), we think voice must be on a separate bearer in both CP and UP solution. The IMS signalling (or if SA decides on not using IMS, some other signalling to set up the voice call) can be discussed later, whether it needs a separate bearer or if it can share another bearer (for example SRB).

The NB-IoT NTN voice over GSO solution shall not break the QoS framework regardless of whether UP or CP solution is selected. That is, all data of one bearer gets the same packet treatment.

There can only be one PDN for each bearer. To support emergency voice, the UE must have one PDN for regular voice and one for emergency because IMS for signalling for emergency and IMS signalling for regular voice uses different PDNs (likely separate PDNs will also be needed if SA selects to not use the IMS for voice). When a voice call starts, the IMS may send the first few voice packets over the IMS signalling bearer before the IMS voice bearer is established, to decrease the initial delay.

The UP solution requires adding DRBs to separate the IMS signalling bearer (QCI = 5) for regular voice and IMS signalling bearer for emergency voice and the voice bearer itself (QCI = 1, or a new QCI being discussed in SA2).

The UP solution will need support for more than two bearers per UE.

The CP solution will require to add at least one new type of bearer for the voice traffic, and we must have RRC QoS signalling (to use RLC UM instead of AM) for this. The support for emergency may require one more SRB, that is one SRB for the emergency voice PDN and one SRB for regular voice PDN.

The CP solution needs implementation of RLC UM for SRB data; this means the voice SRB must be on a separate SRB from the SRB1 in existing architecture.

### 2.1.4	RRC signalling

The CP solution does not support RRC Reconfiguration today, RRC reconfiguration is likely needed for supporting the addition of SPS and a voice SRB.

For the UP solution we will need to add SPS to the existing RRC reconfiguration.

To configure SPS and addition/removal of voice bearer, CP solution needs RRC reconfiguration which is not supported today for CP. UP solution already has RRC reconfiguration where SPS configuration can be added.

### 2.1.5	Summary CP and UP selection

Below we have summarized the observations above

|  | Core data path updates | RAN update to SRBs | RAN update to DRBs | Existing deployments | RRC updates |
| --- | --- | --- | --- | --- | --- |
| CP solution | S1-MME and S11-U must support real time traffic. The MME on the voice path must support real time traffic | More than one SRB is needed. SRB must support QoS and RLC UM |  | None with real time data support. There are deployments for non-real time data. | RRC Reconfiguration, not supported today, needed for SPS and bearer additions |
| UP solution | S1-U must support real time traffic, which it does for eMTC User plane CIoT EPS optimization |  | More than two DRBs is needed | None for NB-IoT. There are deployments of eMTC with User plane CIoT EPS optimization. | SPS |

Considering the table above, CP solution can be supported but with larger changes in RAN compared to the UP solution and likely with performance that, in best case, can be on par with the UP solution.

There are no existing deployments of CP solution that supports voice traffic, while there are eMTC deployments of voice for User plane CIoT EPS optimization.

There are no existing deployments of NB-IoT UP nor CP solution with real time data traffic support.

There are deployments of voice for eMTC with User plane CIoT EPS optimization.

The CP solution has more drastic changes compared to the UP solution.

There are neither market reasons (no deployed CP voice networks) nor technical reasons (CP solution changes are major compared to UP solution) to support CP solution. There are market reasons (deployed eMTC UP voice networks) and technical reasons (UP solution changes are small compared to CP solution) to support UP solution.

Implementing the CP voice solution is like implementing the UP solution without calling it UP solution and makes NB-IoT have two similar methods for the same thing.

RAN2 supports the UP based solution for NB-IoT NTN voice over GSO.

Send LS to inform SA2 that RAN2 has selected the UP solution and ask whether there are any major issues with that, see draft in Appendix B.

## 2.2	LSs in

RAN2 has received six LSs for this WI issues [2] - [7], two with actions for RAN2.

### 2.2.1	LS from SA4: S4-251584 [2] LS on the RAN simulation assumptions for ULBC

The LS has these actions for RAN2:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

#### 2.2.1.1	Assumptions in LS

The LS has an attachment with this figure:

##### 5.2.2.3	Frame structure

For dynamic scheduling, an example frame structure for Half-duplex FDD for the 80ms bundling period is shown in Figure 5.2.2.3-1. The duration of NPDSCH is 4ms and can take a different value depending on the DL SNR.

Figure 5.2.2.3-1 An example frame structure for 80ms bundling period and dynamic scheduling

The figure assumes a perfect Koffset tuned to the UEs TA – that is, the UE must report the TA during RRCConnectionSetup and eNB must send a Differential Koffset to set the UE specific Koffset. The figure does not indicate any HARQ feedback, thus HARQ feedback is disabled. Both TA reporting and HARQ feedback disabling are optional features in NB-IoT NTN. Disabling HARQ feedback is beneficial for the voice traffic because retransmissions will arrive too late to be useful, same as for RLC retransmissions, and then not sending HARQ feedback enables the UE to have UL SPS closer in time to the DL SPS.

The LS S4-251584 assumes use of the optional features HARQ feedback disabling and TA reporting, further the UE specific Koffset is perfectly tuned to the UEs timing advance.

If a TA report is triggered during an RRC connection, the TB of the SPS grant may be filled by the TA report MAC CE instead of voice data. If a Differential Koffset is sent in DL, the TB of the SPS assignment may be filled by the Differential Koffset MAC CE instead of voice data. To avoid TA reports that override voice data in the SPS grants, a UE specific Koffset must be sat according to the first TA report with a margin so that UE or satellite movement do not result in a UE specific Koffset being too small.

If the UE do not support TA reporting, or if TA reporting is not used, we must have a part of the SPS period to account for the spread in TA for different UEs in a cell. The delay spread in GSO cells can be up to 10.3 ms (3500 km cell radius at minimum elevation angle see [9] Table 4.2-2 or 6.44/3.26 ms for 1000/500 km cell radius see Table 7.2.1.1.1.2-1).

A figure that correctly reflect using the Cell specific Koffset to build the SPS periodicity was included in [8]:

If non-perfect Koffset is used, there must be a part of the SPS period to account for the difference between the UEs actual TA and Koffset. If cell specific Koffset is used then the minimum compensation is equal to Koffset minus minimum TA in the cell.

We think UE that do not support TA reporting must be supported for this feature.

NB-IoT UEs without TA reporting capability are supported in voice over GSO.

#### 2.2.1.2	MAC overhead

First, a MAC header of 1 byte is possible in some special cases, for example if the MAC PDU has only data from one radio bearer and there are no MAC CEs. However, in general we cannot say the packet overhead in MAC is one byte.

If TA reporting, or CQI reporting or PH reporting is used there may be UL reports to send in the next MAC PDU making the overhead larger. In case of UP solution, if multiple bearers have data, the MAC overhead is more than one byte. Likely, in CP solution, we must have RLC UM for the voice traffic instead of RLC AM (as RLC retransmissions will arrive when the data is too old and useless, see further section 2.1 above), possibly one SRB is needed for regular signalling and one SRB for the voice traffic – in that case also CP solution may have issues that both SRBs has traffic and MAC overhead is more than one byte. The network may need to send MAC CEs in DL, for example TA commands; UL Transmission Extension Update or Differential Koffset – in all cases MAC overhead is more than one byte.

Only in special cases, the MAC overhead will be one byte.

Reply to SA4 that a packet overhead of 1 byte of MAC header is not realistic.

#### 2.2.1.3	Overall overhead

The radio protocols are PDCP, RRC, RLC, MAC and PHY, see 36.300 clause 4.0, and are, except PHY, in RAN2s scope. The NAS, RTP/UDP/IP and Non-IP protocols are not in RAN2 scope, SA2 have been looking into these overheads [3].

Reply to SA4 that RTP, UDP, IP and Non-IP headers are outside of RAN2 scope.

The PDCP layer has this functional view, see 36.323:

Figure 4.2.2.1 - PDCP layer, functional view

If the header for RTP is 12 bytes, UDP is 8 bytes, IPv4 20 bytes, headers are totally 40 bytes. With ROHC the 40 bytes of RTP/UDP/IP header can be compressed to 4 bytes, but how that works is outside of RAN2 scope besides the supported ROHC profiles which are specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE:

In this version of the specification the support of the following profiles is described:

Table 5.5.1.1: Supported ROHC protocols and profiles

| Profile Identifier | Usage: | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 5795 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

headerCompression-r13		CHOICE {

notUsed						NULL,

rohc						SEQUENCE {

maxCID-r13					INTEGER (1..16383)				DEFAULT 15,

profiles-r13				SEQUENCE {

profile0x0002				BOOLEAN,

profile0x0003				BOOLEAN,

profile0x0004				BOOLEAN,

profile0x0006				BOOLEAN,

profile0x0102				BOOLEAN,

profile0x0103				BOOLEAN,

profile0x0104				BOOLEAN

},

...

}

},

profiles The profiles used by both compressor and decompressor in both UE and E-UTRAN. The field indicates which of the ROHC profiles specified in TS 36.323 [8] are supported, i.e. value true indicates that the profile is supported. Profile 0x0000 shall always be supported when the use of ROHC is configured. If support of two ROHC profile identifiers with the same 8 LSB's is signalled, only the profile corresponding to the highest value shall be applied.

Thus, signalling the RTP/UDP/IP profiles (0x0001 and 0x0101) are not supported for NB-IoT, but it is a small change to add that support.

The RoHC is outside RAN2 scope besides the supported RoHC profiles (specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE).

Reply to SA4 that RAN2 supports signalling of RoHC profiles, as specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE, but the functions and compression is outside of RAN2 scope. Note that the compression profiles for RTP are not supported but can be added for Rel-20.

The general protocol structure for CP and UP is, 36.300:

Figure 4.3.2-1: Control-plane protocol stack

Figure 4.3.1-1: User-plane protocol stack

The (minimum) headers are:

UP traffic: PDCP 1 byte, RLC 1 byte, MAC 2 bytes; totally 4 bytes.

CP traffic RRC 2 bytes, RLC 2 bytes, MAC 2 bytes, totally 6 bytes.

Reply to SA4 that packet overhead for UP solution in PDCP, RLC, and MAC is 4 bytes and packet overhead for CP solution in RRC, RLC, and MAC is 6 bytes.

### 2.2.2	LS from SA2 S2-2507636 [4]: LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

We go through and answer the questions for RAN2 one by one.

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

RAN2 only specifies the configuration of RoHC profiles (see 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE). The functions of RoHC are outside of RAN2 scope.

Reply to SA2 that RAN2 supports signalling of RoHC profiles, as specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE, but the functions and compression is outside of RAN2 scope. Note that the compression profiles for RTP are not supported but can be added for Rel-20.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

This depends on the amount of variation and how often the size changes. If there are large and random variations in size, we must use dynamic scheduling which will have severe drawback for UL data as it adds at least one RTT for the UE to indicate the size of the data while for DL it can work to schedule the UE dynamically. Dynamic scheduling will use much more resources for NPDCCH, and decoding of the DCIs. If many repetitions are needed, the UL and DL data plus UL and DL DCI and decoding time for DCI may fill all available time between voice packets while if SPS can be used we get a little better possibility for repetitions in the same time period.

If the variations are small, say that there are two packet sizes that has two distinct periodicities (similar top VoLTE with a talk state with voice packets every 20 ms and a silent state with SIDs of smaller size than the voice packets every 160 ms) – we can tailor a special SPS to fit these packets. For example, if voice packets are every 40 ms, we can define an SPS with periodicity 40 ms and with UL skipping (see skipUplinkTxSPS in MAC), then the UE can send the smaller SIDs using the regular SPS grants and skip the UL transmissions when the UE is in silent state without SID to transmit.

Reply to SA2 that dynamic scheduling can handle dynamic varying sizes, but will add at least one RTT of 542 ms for UL traffic.

Reply to SA2 that if the size change between two sizes and two periodicities that evenly divides each other, then likely a specially tuned semi-persistent scheduling can support IMS voice without an extra RTT of delay for UL.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

The additions of more DRBs is not complex to specify.

Reply to SA2 it is feasible from a specifications point of view to add more than 2 DRBs per UE in Rel-20.

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

This we discussed above; it is possible but has some drawbacks.

Reply to SA2 that it is possible to define a new SRB for voice media but it has the drawback of needing RRC reconfiguration for CP solution which is not supported today.

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

This we discussed in 2.1 above; it is possible but has the drawback that the RAN2 changes are larger than for the UP solution.

Reply to SA2 that it is possible to define a new SRB for voice media with RLC-UM, but it has the drawback of needing RRC reconfiguration for CP solution, which is not supported today, and the CP solution changes are larger compared to the UP solution.

### 2.2.3	LS where RAN2 is on cc: S2-2507578, C1-255650, R4-2511782, and R1-2506541

RAN2 is on the cc for these LSs and there are no RAN2 actions.

# 3	Conclusion

In this paper we have made these observations:

Observation 1	Using RLC AM for voice data in GSO will waste radio resources.

Observation 2	The UP solution with real time data traffic needs updating the S1-U reference point to handle real time traffic. The S1-U interface can handle real time traffic in non-NB-IoT LTE.

Observation 3	The CP solution with real time data traffic needs updating the S1-MME and S11-U reference points to handle real time traffic. These interfaces cannot carry real time traffic today.

Observation 4	The CP solution with real time data traffic needs one extra network node, the MME, compared to the UP solution. The MME cannot handle real time data traffic today.

Observation 5	The QoS framework only support QoS differentiation between bearers.

Observation 6	The UP solution will need support for more than two bearers per UE.

Observation 7	The CP solution needs implementation of RLC UM for SRB data; this means the voice SRB must be on a separate SRB from the SRB1 in existing architecture.

Observation 8	To configure SPS and addition/removal of voice bearer, CP solution needs RRC reconfiguration which is not supported today for CP. UP solution already has RRC reconfiguration where SPS configuration can be added.

Observation 9	There are no existing deployments of NB-IoT UP nor CP solution with real time data traffic support.

Observation 10	There are deployments of voice for eMTC with User plane CIoT EPS optimization.

Observation 11	The CP solution has more drastic changes compared to the UP solution.

Observation 12	The LS S4-251584 assumes use of the optional features HARQ feedback disabling and TA reporting, further the UE specific Koffset is perfectly tuned to the UEs timing advance.

Observation 13	If non-perfect Koffset is used, there must be a part of the SPS period to account for the difference between the UEs actual TA and Koffset. If cell specific Koffset is used then the minimum compensation is equal to Koffset minus minimum TA in the cell.

Observation 14	Only in special cases, the MAC overhead will be one byte.

Observation 15	The RoHC is outside RAN2 scope besides the supported RoHC profiles (specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE).

We propose:

Proposal 1	RLC UM for voice over GSO is supported regardless of whether UP or CP solution is selected.

Proposal 2	The NB-IoT NTN voice over GSO solution shall not break the QoS framework regardless of whether UP or CP solution is selected. That is, all data of one bearer gets the same packet treatment.

Proposal 3	RAN2 supports the UP based solution for NB-IoT NTN voice over GSO.

Proposal 4	Send LS to inform SA2 that RAN2 has selected the UP solution and ask whether there are any major issues with that, see draft in Appendix B.

Proposal 5	NB-IoT UEs without TA reporting capability are supported in voice over GSO.

Proposal 6	Reply to SA4 that a packet overhead of 1 byte of MAC header is not realistic.

Proposal 7	Reply to SA4 that RTP, UDP, IP and Non-IP headers are outside of RAN2 scope.

Proposal 8	Reply to SA4 that RAN2 supports signalling of RoHC profiles, as specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE, but the functions and compression is outside of RAN2 scope. Note that the compression profiles for RTP are not supported but can be added for Rel-20.

Proposal 9	Reply to SA4 that packet overhead for UP solution in PDCP, RLC, and MAC is 4 bytes and packet overhead for CP solution in RRC, RLC, and MAC is 6 bytes.

Proposal 10	Reply to SA2 that RAN2 supports signalling of RoHC profiles, as specified in 36.323 clause 5.5 and 36.331 PDCP-Config-NB IE, but the functions and compression is outside of RAN2 scope. Note that the compression profiles for RTP are not supported but can be added for Rel-20.

Proposal 11	Reply to SA2 that dynamic scheduling can handle dynamic varying sizes, but will add at least one RTT of 542 ms for UL traffic.

Proposal 12	Reply to SA2 that if the size change between two sizes and two periodicities that evenly divides each other, then likely a specially tuned semi-persistent scheduling can support IMS voice without an extra RTT of delay for UL.

Proposal 13	Reply to SA2 it is feasible from a specifications point of view to add more than 2 DRBs per UE in Rel-20.

Proposal 14	Reply to SA2 that it is possible to define a new SRB for voice media but it has the drawback of needing RRC reconfiguration for CP solution which is not supported today.

Proposal 15	Reply to SA2 that it is possible to define a new SRB for voice media with RLC-UM, but it has the drawback of needing RRC reconfiguration for CP solution, which is not supported today, and the CP solution changes are larger compared to the UP solution.

# 4	References

RP-252473, “New WI: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4”, vivo, RAN#109, Sept. 2025

S4-251584, “LS on the RAN simulation assumptions for ULBC”, From: SA4, To: RAN1, RAN2, RAN4, SA2, CT1, cc:SA1, SA4#133e, Online July 18-25 2025

S2-2507578, “Reply LS on the RAN simulation assumptions for ULBC”, From: SA2; To: SA4; cc: RAN1, RAN2, RAN4, SA1, CT1; SA2#170, Gothenburg, Sweden, 25-29 August 2025

S2-2507636, “LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC”, From SA2; To: RAN2, SA4, CT1, SA3, SA1, RAN1; SA2#170, Gothenburg, Sweden, 25-29 August 2025

C1-255650, “Reply LS to SA4 on the RAN simulation assumptions for ULBC”, From: CT1; To: SA4; cc: SA2, SA1, RAN1, RAN2, RAN4; CT1#156, Gothenburg, Sweden, 25-29 August, 2025

R1-2506541, “Reply LS on the RAN simulation assumptions for ULBC”, From: RAN1, To: SA4; cc: RAN4, RAN2, SA2, CT1; RAN1#122; Bengaluru, India, Aug 25th – 29th, 2025

R4-2511782, “Response LS on the RAN simulation assumptions for ULBC”, From RAN4; To: SA4; cc: RAN2, RAN1, SA2; RAN4#116; Bengaluru, India, 25th – 29th August 2025

S4aA250232, “On assumptions for link budget analysis”, Ericsson LM, Dolby Laboratories Inc., SA4 audio SWG Ad-Hoc, Erlangen, Germany, 23-25 September 2025

TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, v16.2.0, 2023-03

# Appendix A

## A.1	Excerpt from 36.300

# 13	QoS

## 13.0	General

An EPS bearer/E-RAB is the level of granularity for bearer level QoS control in the EPC/E-UTRAN. That is, SDFs mapped to the same EPS bearer receive the same bearer level packet forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.), as specified in TS 23.401 [17].

One EPS bearer/E-RAB is established when the UE connects to a PDN, and that remains established throughout the lifetime of the PDN connection to provide the UE with always-on IP connectivity to that PDN. That bearer is referred to as the default bearer. Any additional EPS bearer/E-RAB that is established to the same PDN is referred to as a dedicated bearer. The initial bearer level QoS parameter values of the default bearer are assigned by the network, based on subscription data. The decision to establish or modify a dedicated bearer can only be taken by the EPC, and the bearer level QoS parameter values are always assigned by the EPC.

An EPS bearer/E-RAB is referred to as a GBR bearer if dedicated network resources related to a Guaranteed Bit Rate (GBR) value that is associated with the EPS bearer/E-RAB are permanently allocated (e.g. by an admission control function in the eNodeB) at bearer establishment/modification. Otherwise, an EPS bearer/E-RAB is referred to as a Non-GBR bearer. A dedicated bearer can either be a GBR or a Non-GBR bearer while a default bearer shall be a Non-GBR bearer.

## 13.1	Bearer service architecture

The EPS bearer service layered architecture is depicted in Figure 13.1-1 below, where:

-	An UL TFT in the UE binds an SDF to an EPS bearer in the uplink direction. Multiple SDFs can be multiplexed onto the same EPS bearer by including multiple uplink packet filters in the UL TFT.

-	A DL TFT in the PDN GW binds an SDF to an EPS bearer in the downlink direction. Multiple SDFs can be multiplexed onto the same EPS bearer by including multiple downlink packet filters in the DL TFT.

-	An E-RAB transports the packets of an EPS bearer between the UE and the EPC. When an E-RAB exists, there is a one-to-one mapping between this E-RAB and an EPS bearer.

-	A data radio bearer transports the packets of an EPS bearer between a UE and one or more eNB(s). When a data radio bearer exists, there is a one-to-one mapping between this data radio bearer and the EPS bearer/E-RAB.

-	An S1 bearer transports the packets of an E-RAB between an eNodeB and a Serving GW.

-	An S5/S8 bearer transports the packets of an EPS bearer between a Serving GW and a PDN GW.

-	A UE stores a mapping between an uplink packet filter and a data radio bearer to create the binding between an SDF and a data radio bearer in the uplink.

-	A PDN GW stores a mapping between a downlink packet filter and an S5/S8a bearer to create the binding between an SDF and an S5/S8a bearer in the downlink.

-	An eNB stores a one-to-one mapping between a data radio bearer and an S1 bearer to create the binding between a data radio bearer and an S1 bearer in both the uplink and downlink.

-	A Serving GW stores a one-to-one mapping between an S1 bearer and an S5/S8a bearer to create the binding between an S1 bearer and an S5/S8a bearer in both the uplink and downlink.

Figure 13.1-1: EPS Bearer Service Architecture

## 13.2	QoS parameters

The bearer level (i.e. per bearer or per bearer aggregate) QoS parameters are QCI, ARP, GBR, and AMBR, as specified in TS 23.401 [17]. Each EPS bearer/E-RAB (GBR and Non-GBR) is associated with the following bearer level QoS parameters:

-	QoS Class Identifier (QCI): scalar that is used as a reference to access node-specific parameters that control bearer level packet forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.), and that have been pre-configured by the operator owning the eNodeB. A one-to-one mapping of standardized QCI values to standardized characteristics is captured in TS 23.401 [17].

-	Allocation and Retention Priority (ARP): the primary purpose of ARP is to decide whether a bearer establishment / modification request can be accepted or needs to be rejected in case of resource limitations. In addition, the ARP can be used by the eNodeB to decide which bearer(s) to drop during exceptional resource limitations (e.g. at handover).

Each GBR bearer is additionally associated with the following bearer level QoS parameter:

-	Guaranteed Bit Rate (GBR): the bit rate that can be expected to be provided by a GBR bearer;

-	Maximum Bit Rate (MBR): the maximum bit rate that can be expected to be provided by a GBR bearer. MBR can be greater or equal to the GBR.

Each APN access, by a UE, is associated with the following QoS parameter:

-	per APN Aggregate Maximum Bit Rate (APN-AMBR).

Each UE in state EMM-REGISTERED is associated with the following bearer aggregate level QoS parameter:

-	per UE Aggregate Maximum Bit Rate (UE-AMBR).

The definitions of APN AMBR and UE-AMBR are captured in TS 23.401 [17].

The GBR and MBR denotes bit rate of traffic per bearer while UE-AMBR/APN-AMBR denote bit rate of traffic per group of bearers. Each of those QoS parameters has an uplink and a downlink component.

## 13.3	QoS support in Hybrid Cells

The following principles apply to serving non CSG members and CSG members of a Hybrid Cell:

NOTE:	The term "eNB" in this clause applies to HeNBs (as described in clause 4.6.1), as well as eNBs (as denoted in the basic E-UTRAN architecture in Figure 4-1).

-	When the UE connects to a Hybrid Cell, the MME shall inform the eNB serving this Hybrid Cell whether the UE is a member or not of the CSG associated with this Hybrid Cell;

-	Based on CSG membership, the offered QoS for UEs served by this Hybrid Cell may be modified as follows:

-	The eNB serving this Hybrid Cell may distinguish between a CSG member and non-member when determining whether to handover a UE, which GBR bearers to admit and which GBR bearers to deactivate;

-	The eNB serving this Hybrid Cell may distinguish between a CSG member and non-member for handover and packet scheduling on Uu interface (including reduced QoS) of non-GBR bearers.

## A.2	Excerpt from 36.401

## 5.1	General

The protocols over Uu and S1 interfaces are divided into two structures:

-	User plane protocols
These are the protocols implementing the actual E-RAB service, i.e. carrying user data through the access stratum.

-	Control plane protocols
These are the protocols for controlling the E-RABs and the connection between the UE and the network from different aspects (including requesting the service, controlling different transmission resources, handover etc.). Also a mechanism for transparent transfer of NAS messages is included.

## 5.2	User plane

The E-RAB service is offered from SAP to SAP by the Access Stratum. Figure 5.2-1 shows the protocols on the Uu and S1 interfaces that linked together provide this E-RAB service.

Note 1:	The radio interface protocols are defined in 3GPP TS 36.2xx and TS 36.3xx.

Note 2:	The S1 interface protocols are defined in 3GPP TS 36.41x.

Figure 5.2-1: S1 and Uu user plane

## 5.3	Control plane

Figure 5.3-1 shows the control plane (signalling) protocol stacks on S1 and Uu interfaces.

Note 1:	The radio interface protocols are defined in 3GPP TS 36.2xx and TS 36.3xx.

Note 2:	The protocol is defined in 3GPP TS 36.41x. (Description of S1 interface).

Note 3:	EMM, ESM: This exemplifies a set of NAS control protocols between UE and EPC. The evolution of the protocol architecture for these protocols is outside the scope of the present document.

Figure 5.3-1: S1 and Uu control plane

NOTE:	Both the Radio protocols and the S1 protocols contain a mechanism to transparently transfer NAS messages.

## A.3	Excerpt from 23.401

### 4.2.1	Non-roaming architecture

Figure 4.2.1-1: Non-roaming architecture for 3GPP accesses

Figure 4.2.1-2: Non-roaming architecture for 3GPP accesses. Single gateway configuration option

NOTE 1:	Also in this configuration option, S5 can be used between non collocated Serving Gateway and PDN Gateway.

NOTE 2:	Additional interfaces for 2G/3G access are shown in TS 23.060 [7].

### 4.2.2	Roaming architecture

Figure 4.2.2-1: Roaming architecture for 3GPP accesses. Home routed traffic

NOTE 1:	Additional interfaces/reference points for 2G/3G accesses are documented in TS 23.060 [7].

The figures 4.2.2-2 and 4.2.2-3 represent the Roaming with local breakout case with Application Function (AF) in the Home Network and in the Visited Network respectively. The concurrent use of AF's in the home network and AF's in the visited network is not excluded.

Figure 4.2.2-2: Roaming architecture for local breakout, with home operator's application functions only

NOTE 2:	See TS 23.203 [6] for the role of and functions related to Home and Visited PCRF and S9/Rx reference points.

NOTE 3:	In figure 4.2.2-2, the control plane signalling and the user plane for accessing to Home Operator's services traverse over the SGi reference point via the Visited Operator's PDN.

Figure 4.2.2-3: Roaming architecture for local breakout, with visited operator's application functions only

### 4.2.3	Reference points

S1-MME:	Reference point for the control plane protocol between E-UTRAN and MME.

S1-U:	Reference point between E-UTRAN and Serving GW for the per bearer user plane tunnelling and inter eNodeB path switching during handover. S1-U does not apply to the Control Plane CIoT EPS Optimisation.

S3:	It enables user and bearer information exchange for inter 3GPP access network mobility in idle and/or active state. This reference point can be used intra-PLMN or inter-PLMN (e.g. in the case of Inter-PLMN HO).

S4:	It provides related control and mobility support between GPRS Core and the 3GPP Anchor function of Serving GW. In addition, if Direct Tunnel is not established, it provides the user plane tunnelling.

S5:	It provides user plane tunnelling and tunnel management between Serving GW and PDN GW. It is used for Serving GW relocation due to UE mobility and if the Serving GW needs to connect to a non-collocated PDN GW for the required PDN connectivity.

S6a:	It enables transfer of subscription and authentication data for authenticating/authorizing user access to the evolved system (AAA interface) between MME and HSS.

Gx:	It provides transfer of (QoS) policy and charging rules from PCRF to Policy and Charging Enforcement Function (PCEF) in the PDN GW.

S8:	Inter-PLMN reference point providing user and control plane between the Serving GW in the VPLMN and the PDN GW in the HPLMN. S8 is the inter PLMN variant of S5.

S9:	It provides transfer of (QoS) policy and charging control information between the Home PCRF and the Visited PCRF in order to support local breakout function.

S10:	Reference point between MMEs for MME relocation and MME to MME information transfer. This reference point can be used intra-PLMN or inter-PLMN (e.g. in the case of Inter-PLMN HO).

S11:	Reference point providing control plane between MME and Serving GW. In addition, in order to support Control Plane CIoT EPS Optimisation, the S11-U reference point provides user plane between MME and Serving GW.

S12:	Reference point between UTRAN and Serving GW for user plane tunnelling when Direct Tunnel is established. It is based on the Iu-u/Gn-u reference point using the GTP-U protocol as defined between SGSN and UTRAN or respectively between SGSN and GGSN. Usage of S12 is an operator configuration option.

S13:	It enables UE identity check procedure between MME and EIR.

S17:	It enables procedures for RACS between MME and UCMF.

SGi:	It is the reference point between the PDN GW and the packet data network. Packet data network may be an operator external public or private packet data network or an intra operator packet data network, e.g. for provision of IMS services. This reference point corresponds to Gi for 3GPP accesses.

Rx:	The Rx reference point resides between the AF and the PCRF in the TS 23.203 [6].

NOTE 1:	Except where stated otherwise, this specification does not make an explicit assumption as to whether an interface is intra-PLMN or inter-PLMN.

When data forwarding is used as part of mobility procedures different user plane routes may be used based on the network configuration (e.g. direct or indirect data forwarding). These routes can be between eNodeB and RNC, eNodeB and SGSN, RNC and S-GW or between S-GW and SGSN. Explicit reference points are not defined for these routes. These user plane forwarding routes can cross inter-PLMN boundaries (e.g. in the case of Inter-PLMN HO).

Protocol assumption:

-	The S1-U is based on GTP-U protocol;

-	The S3 is based on GTP protocol;

-	The S4 and S11 are based on GTP protocol;

-	The S5 is based on GTP protocol. PMIP variant of S5 is described in TS 23.402 [2];

-	The S8 is based on GTP protocol. PMIP variant of S8 is described in TS 23.402 [2].

-	S3, S4, S5, S8, S10 and S11 interfaces are designed to manage EPS bearers as defined in clause 4.7.2.

NOTE 2:	Redundancy support on reference points S5 and S8 should be taken into account.

## A.4	Excerpt from 23.203

QoS class identifier (QCI): A scalar that is used as a reference to a specific packet forwarding behaviour (e.g. packet loss rate, packet delay budget) to be provided to a SDF. This may be implemented in the access network by the QCI referencing node specific parameters that control packet forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.), that have been pre-configured by the operator at a specific node(s) (e.g. eNodeB).

### 6.1.7	Standardized QoS characteristics

#### 6.1.7.1	General

The service level (i.e. per SDF or per SDF aggregate) QoS parameters are QCI, ARP, GBR and MBR.

Each Service Data Flow (SDF) is associated with one and only one QoS Class Identifier (QCI). For the same IP-CAN session multiple SDFs with the same QCI and ARP can be treated as a single traffic aggregate which is referred to as an SDF aggregate. An SDF is a special case of an SDF aggregate. The QCI is scalar that is used as a reference to node specific parameters that control packet forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.) and that have been pre-configured by the operator owning the node (e.g. eNodeB). When required by operator policy, the eNodeB can be configured to also use the ARP priority level in addition to the QCI to control the packet forwarding treatment in the eNodeB for SDFs having high priority ARPs.

#### 6.1.7.2	Standardized QCI characteristics

This clause specifies standardized characteristics associated with standardized QCI values. The characteristics describe the packet forwarding treatment that an SDF aggregate receives edge-to-edge between the UE and the PCEF (see figure 6.1.7-1) in terms of the following performance characteristics:

1	Resource Type (GBR or Non-GBR);

2	Priority;

3	Packet Delay Budget;

4	Packet Error Loss Rate;

5	Maximum Data Burst Volume (for some GBR QCIs);

6	Data Rate Averaging Window (for some GBR QCIs).

Figure 6.1.7-1: Scope of the Standardized QCI characteristics for client/server (upper figure) and peer/peer (lower figure) communication

The standardized characteristics are not signalled on any interface. They should be understood as guidelines for the pre-configuration of node specific parameters for each QCI. The goal of standardizing a QCI with corresponding characteristics is to ensure that applications / services mapped to that QCI receive the same minimum level of QoS in multi-vendor network deployments and in case of roaming. A standardized QCI and corresponding characteristics is independent of the UE's current access (3GPP or Non-3GPP).

The one-to-one mapping of standardized QCI values to standardized characteristics is captured in table 6.1.7-A and table 6.1.7-B. The main differences between the two parts are that, in contrast to Part A, Part B of Table 6.1.7 describes QCIs for which the Packet Error Loss Rate calculation includes those packets that are not delivered within the Packet Delay Budget; and, it provides additional information on the Data Rate Averaging Window as well as the Maximum Data Burst Volume that needs to be delivered within the Packet Delay Budget.

Table 6.1.7-A: Standardized QCI characteristics

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE 13) | Packet Error Loss Rate (NOTE 2) | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 (NOTE 3) |  | 2 | 100 ms (NOTE 1, NOTE 11) | 10-2 | Conversational Voice |
| 2 (NOTE 3) | GBR | 4 | 150 ms (NOTE 1, NOTE 11) | 10-3 | Conversational Video (Live Streaming) |
| 3 (NOTE 3, NOTE 14) |  | 3 | 50 ms (NOTE 1, NOTE 11) | 10-3 | Real Time Gaming, V2X messages Electricity distribution - medium voltage (e.g. clause 7.2.2 of TS 22.261 [51]) Process automation - monitoring (e.g. clause 7.2.2 of TS 22.261 [51]) |
| 4 (NOTE 3) |  | 5 | 300 ms (NOTE 1, NOTE 11) | 10-6 | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 3, NOTE 9, NOTE 12) |  | 0.7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 3, NOTE 12) |  | 2 | 100 ms (NOTE 1, NOTE 10) | 10-2 | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 3, NOTE 12) |  | 1.5 | 100 ms (NOTE 1, NOTE 10) | 10-3 | Mission Critical Video user plane |
| 75 (NOTE 14) |  | 2.5 | 50 ms (NOTE 1) | 10-2 | V2X messages |
| 71 |  | 5.6 | 150ms (NOTE 1, NOTE 16) | 10-6 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 72 |  | 5.6 | 300ms (NOTE 1, NOTE 16) | 10-4 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 73 |  | 5.6 | 300ms (NOTE 1, NOTE 16) | 10-8 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 74 |  | 5.6 | 500ms (NOTE 1, NOTE 16) | 10-8 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 76 |  | 5.6 | 500ms (NOTE 1, NOTE 16) | 10-4 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 5 (NOTE 3) |  | 1 | 100 ms (NOTE 1, NOTE 10) | 10-6 | IMS Signalling |
| 6 (NOTE 4) |  | 6 | 300 ms (NOTE 1, NOTE 10) | 10-6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 (NOTE 3) | Non-GBR | 7 | 100 ms (NOTE 1, NOTE 10) | 10-3 | Voice, Video (Live Streaming) Interactive Gaming |
| 8 (NOTE 5) |  | 8 | 300 ms (NOTE 1) | 10-6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file |
| 9 (NOTE 6) |  | 9 |  |  | sharing, progressive video, etc.) |
| 10 |  | 9 | 1100 ms (NOTE 1, NOTE 17, NOTE 18) | 10-6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access with these characteristics |
| 69 (NOTE 3, NOTE 9, NOTE 12) |  | 0.5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling, MC Video signalling) |
| 70 (NOTE 4, NOTE 12) |  | 5.5 | 200 ms (NOTE 7, NOTE 10) | 10-6 | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 (NOTE 14) |  | 6.5 | 50 ms (NOTE 1, NOTE 10) | 10-2 | V2X messages |
| 80 (NOTE 3) |  | 6.8 | 10 ms (NOTE 10, NOTE 15) | 10-6 | Low latency eMBB applications (TCP/UDP-based); Augmented Reality |
| NOTE 1: A delay of 20 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. This delay is the average between the case where the PCEF is located "close" to the radio base station (roughly 10 ms) and the case where the PCEF is located "far" from the radio base station, e.g. in case of roaming with home routed traffic (the one-way packet delay between Europe and the US west coast is roughly 50 ms). The average takes into account that roaming is a less typical scenario. It is expected that subtracting this average delay of 20 ms from a given PDB will lead to desired end-to-end performance in most typical cases. Also, note that the PDB defines an upper bound. Actual packet delays - in particular for GBR traffic - should typically be lower than the PDB specified for a QCI as long as the UE has sufficient radio channel quality. NOTE 2: The rate of non congestion related packet losses that may occur between a radio base station and a PCEF should be regarded to be negligible. A PELR value specified for a standardized QCI therefore applies completely to the radio interface between a UE and radio base station. NOTE 3: This QCI is typically associated with an operator controlled service, i.e. a service where the SDF aggregate's uplink / downlink packet filters are known at the point in time when the SDF aggregate is authorized. In case of E-UTRAN this is the point in time when a corresponding dedicated EPS bearer is established / modified. NOTE 4: If the network supports Multimedia Priority Services (MPS) then this QCI could be used for the prioritization of non real-time data (i.e. most typically TCP-based services/applications) of MPS subscribers. NOTE 5: This QCI could be used for a dedicated "premium bearer" (e.g. associated with premium content) for any subscriber / subscriber group. Also in this case, the SDF aggregate's uplink / downlink packet filters are known at the point in time when the SDF aggregate is authorized. Alternatively, this QCI could be used for the default bearer of a UE/PDN for "premium subscribers". NOTE 6: This QCI is typically used for the default bearer of a UE/PDN for non privileged subscribers. Note that AMBR can be used as a "tool" to provide subscriber differentiation between subscriber groups connected to the same PDN with the same QCI on the default bearer. NOTE 7: For Mission Critical services, it may be assumed that the PCEF is located "close" to the radio base station (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence delay of 10 ms for the delay between a PCEF and a radio base station should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these QCIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that QCI-65 and QCI-69 are used together to provide Mission Critical Push to Talk service (e.g. QCI-5 is not used for signalling for the bearer that utilizes QCI-65 as user plane bearer). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these QCIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these QCIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This QCI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this QCI value. NOTE 13: Packet delay budget is not applicable on NB-IoT or when Enhanced Coverage is used for WB-E-UTRAN (see TS 36.300 [19]). NOTE 14: This QCI could be used for transmission of V2X messages as defined in TS 23.285 [48]. NOTE 15: A delay of 2 ms for the delay between a PCEF and a radio base station should be subtracted from the given PDB to derive the packet delay budget that applies to the radio interface. NOTE 16: For "live" uplink streaming (see TS 26.238 [53]), guidelines for PDB values of the different QCIs correspond to the latency configurations defined in TR 26.939 [54]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PELR combinations are needed these configurations will have to use non-standardised QCIs. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270 ms, ~ 21 ms for LEO at 1200 km and ~13 ms for LEO at 600 km. When UE is accessing the network via satellite access supporting transparent payload, the UL scheduling delay that needs to be added is typically a two way propagation delay e.g. ~540 ms for GEO, ~42 ms for LEO at 1200 km and ~26 ms for LEO at 600 km. Based on that, the access network Packet Delay Budget is not applicable for QCIs that require access network PDB lower than the sum of these values when the specific types of satellite access are used (see TS 36.300 [19]). QCI-10 can accommodate the worst case PDB for GEO satellite type. NOTE 18: When UE is accessing the network via satellite access supporting regenerative payload, a static value of ~290 ms for GEO satellite, ~41 ms for LEO at 1200 km and ~33 ms for LEO at 600 km for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. |  |  |  |  |  |

Table 6.1.7-B: Standardized QCI characteristics

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE B1) | Packet Error Loss Rate (NOTE B2) | Maximum Data Burst Volume (NOTE B1) | Data Rate Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 82 (NOTE B6) | GBR | 1.9 | 10 ms (NOTE B4) | 10-4 (NOTE B3) | 255 bytes | 2000 ms | Discrete Automation (TS 22.278 [38], clause 8 bullet g and TS 22.261 [51], table 7.2.2-1, "small packets") |
| 83 (NOTE B6) |  | 2.2 | 10 ms (NOTE B4) | 10-4 (NOTE B3) | 1354 bytes (NOTE B5) | 2000 ms | Discrete Automation (TS 22.278 [38], clause 8 bullet g and TS 22.261 [51], table 7.2.2-1, "big packets") |
| 84 (NOTE B6) |  | 2.4 | 30 ms (NOTE B7) | 10-5 (NOTE B3) | 1354 bytes (NOTE B5) | 2000 ms | Intelligent Transport Systems (TS 22.278 [38], clause 8, bullet h and TS 22.261 [51], table 7.2.2). |
| 85 (NOTE B6) |  | 2.1 | 5 ms (NOTE B8) | 10-5 (NOTE B3) | 255 bytes | 2000 ms | Electricity Distribution- high voltage (TS 22.278 [38], clause 8, bullet i and TS 22.261 [51], table 7.2.2 and Annex D, clause D.4.2). |
| NOTE B1: The PDB applies to bursts that are not greater than Maximum Data Burst Volume. NOTE B2: This Packet Error Loss Rate includes packets that are not successfully delivered over the access network plus those packets that comply with the Maximum Data Burst Volume and GBR requirements but which are not delivered within the Packet Delay Budget. NOTE B3: Data rates above the GBR, or, bursts larger than the Maximum Data Burst Volume, are treated as best effort and, in order to serve other packets and meet the PELR, this can lead to them being discarded. NOTE B4: A delay of 1 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE B5: This Maximum Data Burst Volume value is set to 1354 bytes to avoid IP fragmentation on an IPv6 based, IPSec protected GTP tunnel to the eNB (the value is calculated as in Annex C of TS 23.060 [12] and further reduced by 4 bytes to allow for the usage of a GTP-U extension header). NOTE B6: This QCI is typically associated with a dedicated EPS bearer. NOTE B7: A delay of 5 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE B8: A delay of 2 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. |  |  |  |  |  |  |  |

The Resource Type determines if dedicated network resources related to a service or bearer level Guaranteed Bit Rate (GBR) value are permanently allocated (e.g. by an admission control function in a radio base station). GBR SDF aggregates are therefore typically authorized "on demand" which requires dynamic policy and charging control. A Non GBR SDF aggregate may be pre-authorized through static policy and charging control.

The Maximum Data Burst Volume, if defined for the QCI (see Table 6.1.7-B), is the amount of data which the RAN is expected to deliver within the part of the Packet Delay Budget allocated to the link between the UE and the radio base station as long as the data is within the GBR requirements. If more data is transmitted from the application, delivery within the Packet Delay Budget cannot be guaranteed for packets exceeding the Maximum Data Burst Volume or GBR requirements.

The Data Rate Averaging Window, if defined for the QCI (see Table 6.1.7-B), is the 'sliding window' duration over which the GBR and MBR for a GBR SDF aggregate shall be calculated (e.g. in the RAN, PDN-GW and UE).

The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the PCEF. For a certain QCI the value of the PDB is the same in uplink and downlink. The purpose of the PDB is to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). Except for QCIs 82 and 83, the PDB shall be interpreted as a maximum delay with a confidence level of 98 percent. For services using QCI 82 or 83, a packet delayed by more than the PDB is included in the calculation of the PELR if the packet is within the Maximum Data Burst Volume and GBR requirements.

NOTE 1:	The PDB denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded (e.g. by RLC in E-UTRAN). The discarding (dropping) of packets is expected to be controlled by a queue management function, e.g. based on pre-configured dropping thresholds.

The support for SRVCC requires QCI=1 only be used for IMS speech sessions in accordance to TS 23.216 [28].

NOTE 2:	Triggering SRVCC will cause service interruption and/or inconsistent service experience when using QCI=1 for non-IMS services.

NOTE 3:	Triggering SRVCC for WebRTC IMS session will cause service interruption and/or inconsistent service experience when using QCI=1. Operator policy (e.g. use of specific AF application identifier) can be used to avoid using QCI 1 for a voice service, e.g. WebRTC IMS session.

Services using a Non-GBR QCI should be prepared to experience congestion related packet drops and, except for QCI 80, 98 percent of the packets that have not been dropped due to congestion should not experience a delay exceeding the QCI's PDB. This may for example occur during traffic load peaks or when the UE becomes coverage limited. See Annex J for details. Packets that have not been dropped due to congestion may still be subject to non congestion related packet losses (see PELR below). Owing to its low latency objective, services using QCI 80 should anticipate that more than 2 percent of packets might exceed the PDB of QCI 80.

Except for services using QCI 82 or 83 services using a GBR QCI and sending at a rate smaller than or equal to GBR can in general assume that congestion related packet drops will not occur and 98 percent of the packets shall not experience a delay exceeding the QCI's PDB. Exceptions (e.g. transient link outages) can always occur in a radio access system which may then lead to congestion related packet drops even for services using a GBR QCI and sending at a rate smaller than or equal to GBR. Packets that have not been dropped due to congestion may still be subject to non congestion related packet losses (see PELR below). For services using QCI 82 or 83 a packet which is delayed by more than the PDB but is within the Maximum Data Burst Volume and GBR requirements, is counted as lost when calculating the PELR.

Every QCI (GBR and Non-GBR) is associated with a Priority level (see Table 6.1.7). The lowest Priority level value corresponds to the highest Priority. The Priority levels shall be used to differentiate between SDF aggregates of the same UE and it shall also be used to differentiate between SDF aggregates from different UEs. Via its QCI an SDF aggregate is associated with a Priority level and a PDB. Scheduling between different SDF aggregates shall primarily be based on the PDB. If the target set by the PDB can no longer be met for one or more SDF aggregate(s) across all UEs that have sufficient radio channel quality then the QCI Priority level shall be used as follows: in this case a scheduler shall meet the PDB of an SDF aggregate on QCI Priority level N in preference to meeting the PDB of SDF aggregates on next QCI Priority level greater than N, until the priority N SDF aggregate's GBR (in case of a GBR SDF aggregate) has been satisfied.

Other aspects related to the treatment of traffic exceeding an SDF aggregate's GBR are out of scope of this specification.

When required by operator policy, the eNodeB can be configured to use the ARP priority level in addition to the QCI priority level to determine the relative priority of the SDFs in meeting the PDB of an SDF aggregate. This configuration applies only for high priority ARPs as defined in clause 6.1.7.3.

NOTE 4:	The definition (or quantification) of "sufficient radio channel quality" is out of the scope of 3GPP specifications.

NOTE 5:	In case of E-UTRAN a QCI's Priority level and when required by operator policy, the ARP priority level may be used as the basis for assigning the uplink priority per Radio Bearer (see TS 36.300 [19] for details).

The Packet Error Loss Rate (PELR) defines an upper bound for the rate of SDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in E-UTRAN) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in E-UTRAN). Thus, the PELR defines an upper bound for a rate of non congestion related packet losses. The purpose of the PELR is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in E-UTRAN). For a certain QCI the value of the PELR is the same in uplink and downlink.

NOTE 6:	The characteristics PDB and PELR are specified only based on application / service level requirements, i.e. those characteristics should be regarded as being access agnostic, independent from the roaming scenario (roaming or non-roaming) and independent from operator policies.

#### 6.1.7.3	Allocation and Retention Priority characteristics

The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability. The priority level defines the relative importance of a resource request. This allows deciding whether a bearer establishment or modification request can be accepted or needs to be rejected in case of resource limitations (typically used for admission control of GBR traffic). It can also be used to decide which existing bearers to pre-empt during resource limitations.

NOTE 1:	The ARP priority level can be used in addition to the QCI to determine the transport level packet marking, e.g. to set the DiffServ Code Point of the associated EPS bearer, as described in TS 23.401 [17].

NOTE 2:	When required by operator policy, the eNodeB can be configured to use the ARP priority level in addition to QCI priority level to control the packet forwarding treatment for SDFs having high priority ARPs.

The range of the ARP priority level is 1 to 15 with 1 as the highest level of priority. The pre-emption capability information defines whether a service data flow can get resources that were already assigned to another service data flow with a lower priority level. The pre-emption vulnerability information defines whether a service data flow can lose the resources assigned to it in order to admit a service data flow with higher priority level. The pre-emption capability and the pre-emption vulnerability can be either set to 'yes' or 'no'.

The ARP priority levels 1-8 should only be assigned to resources for services that are authorized to receive prioritized treatment within an operator domain (i.e. that are authorized by the serving network). The ARP priority levels 9-15 may be assigned to resources that are authorized by the home network and thus applicable when a UE is roaming.

NOTE 3:	This ensures that future releases may use ARP priority level 1-8 to indicate e.g. emergency and other priority services within an operator domain in a backward compatible manner. This does not prevent the use of ARP priority level 1-8 in roaming situation in case appropriate roaming agreements exist that ensure a compatible use of these priority levels.

# Appendix B Draft LS to SA2 on support for UP solution

3GPP TSG-RAN2#131bis	R2-2507641

Prague, Czech Republic, 13th – 17th Oct. 2025

Title:	[DRAFT] LS on selection between CP and UP solutions for voice over NB-IoT-NTN

Response to:	-

Release:	Release 20

Work Item:	IoT_NTN_Ph4-Core

Source:	Ericsson (To be RAN2)

To:	SA2

Cc:	SA4

Contact person:

Name:	Robert S Karlsson

E-mail Address:	robert dot s dot karlsson AT ericsson DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

The RAN work item IoT_NTN_Ph4-Core aims at NB-IoT NTN support for voice over GSO.

RAN2 has carefully considered the implications of supporting NB-IoT NTN voice with Control Plane CIoT EPS optimization or User Plane CIoT EPS optimization and selected the UP solution.

# 2	Actions

To SA2:

ACTION: 	RAN2 respectfully asks SA2 to take above into account for their future work and reply if there are any major issues.

# 3	Dates of next TSG RAN WG2 meetings

RAN2#132	17th – 21st November 2025		Dallas, US

RAN2#133	9th – 13th February 2026			Göteborg, Sweden