3GPP TSG-RAN WG2 Meeting #131bis	R2-2507664

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RIL N085, S024, X250, A200

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This Tdoc discuss’ the RILs for NR NTN phase 3 (release-19 NR NTN).

# 2	Discussion

## 2.1	N085

The way smtc5list has been included in the specification may be confusing. We believe that smtc5list is a new list, and should be included in a new line in the table. Regarding the number of configurable SMTCs, currently it reads that the total number of configurable SMTCs across smtc4list and smtc5list is 6. However, during the CR discussions after the last RAN2 meeting, it has been mentioned that smtc4list always need to be included due to backward compatibility. However, if the intention of the smtc5list is to consider a delta configuration in relation to other entries in smtc4list, not all entries in smtc5list should be counted as a new one. We also pointed out during the review that if two neighbour cells can be measured with the same SMTC, the network will need to repeat the SMTC configuration in smtc5list because each reference location is associated to a SMTC.

Therefore, we propose that the total number of configurable SMTCs applies only to unique SMTCs. And a unique SMTC are the ones that share the same periodicity and offsets. This would solve the issues highlighted in the CR discussion.

Furthermore, a UE may not only support less SMTCs than signalled in smtc4list and smtc5list, but also only a single periodicity. Since smtc5list can also be used to configure 6 SMTCs with only a single periodicity, the UE should also downselect from 4+ to the supported number of SMTCs by UE implementation.

Proposal 1: <RIL N085> Update the specification text according to N085.

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports only a single periodicity or less SMTCs than what is included in smtc4list or smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable unique SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

As noted above, two neighbor cells with different reference locations may be measured with the same SMTC, but will require two entries in the SMTC5list. Therefore, we also propose the correction:

Proposal 2: Related to N085, update the specification text to mention unique SMTCs in section 5.5.2.10

#### 5.5.2.10 Reference signal measurement timing configuration << Text omitted >> If smtc5list is present, for cells indicated in the pci-List parameter in each unique SSB-MTC5 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity and offset parameter in each unique SSB-MTC5 configuration and use the duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition. << Text omitted >>

## 2.2	S024

This RIL propose to add “if available” to the UE procedure to include the reference location report in the RRCReconfigurationComplete message, although the agreement in RAN2 states that it is optional for the UE to report the location.

[Description]: RAN2 agreement is that - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations. The UE can report the N closest reference locations via the RRCReconfigurationComplete message. “UE can ..” in our understanding means UE optionally reports in RRCReconfigurationComplete message. If we implement in the current way below, it enforces UE always reports in RRCReconfigurationComplete message, which obviously is not aligned with the agreement! 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport;. [Proposed Change]: Add “if available” at the end. 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport, if available;

Similarly, as the above proposed change, RIL081 and 082 suggested adding notes to the spec stating that the UE may not be able to obtain the GNSS position i.e. due to bad satellite connection, user disabling GNSS, or other causes. These RILs received similar comments to S024, that the rapporteurs understanding is that a valid location must be obtained and maintained in order to initiate and continue NTN operation.

As seen below, since NR NTN operation is not mandated by periodic GNSS location acquisition in the spec, we need to ensure no ambiguity on the fact that location reporting for SMTC selection is likewise not mandating GNSS operations.

Observation 1: Location information for assisted SMTC selection should not imply mandating the UE to periodically acquire the GNSS location.

From 38.300 section 16.14.2.2 we see that a UE shall compute the RTT between UE and the RP based on the GNSS position, as well as the GNSS position must be valid before transmission. However, nowhere does it mandate the behaviour continuously receiving a GNSS position to support NTN operation.

#### 16.14.2.2 Timing Advance and Frequency Pre-compensation For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [38]), and autonomously pre-compensate the TTA for the RTT between the UE and the RP as illustrated in Figure 16.14.2.1-1 (see clause 4.3 of TS 38.211 [52]). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported. Figure 16.14.2.2-1: Void While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link and transponder frequency error is left to the network implementation.

Similarly, in 38.306 the GNSS operation is specified as required, but not mandated to be continuously updating.

| gnss-Location-r16 Indicates whether the UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON, MDT, and NTN related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE state. A UE shall set this field to supported if it indicates the support of nonTerrestrialNetwork-r17. | UE | CY | No | No |
| --- | --- | --- | --- | --- |

Lastly, in 38.331, the GNSS position is only mentioned in case of initiation of the transition to RRC connected mode but not during operation, other than what is specified in 38.300.

Observation 2: Currently, the specs does not mandate any requirements regarding the GNSS update periodicity, as long as the position is sufficient for TA and frequency compensation.

By using strict wordings, such as the current procedures in 38.331, it implicitly mandates the UE to periodically acquire the GNSS position and compare the current location with the neighbour cell locations with the goal of detecting a change in which to report as the nearest neighbours.

RAN2 also need to consider the fact that the user may disable GNSS or that the position is not available. However, as in legacy NR NTN operations, such as coarse location reporting, this should not preclude whether the NR NTN operation can continue. We believe that a smart UE implementation may be able to somehow compensate for the UE movements, or lack hereof, and thus understand whether the GNSS position is required to obtain in order to continue with the SMTC procedure as expected.

Observation 3: Multiple SMTC configuration is an optional feature considering an optimisation and should not set unnecessary requirements towards the UE.

Observation 4: It depends on the UE how long it continues to operate with a valid GNSS position.

Proposal 3: RAN2 to agree that in the end it is up to UE implementation how to maintain synchronisation towards the satellite, and the GNSS requirement just indicate the requirement to have GNSS available, in order to avoid unnecessary connection attempts.

Proposal 4: <RIL S024> RAN2 to agree to S024 and at least add “if available” to the inclusion of referenceLocationReport.

Proposal 5: RAN2 to agree that more than 4 SMTCs, or multiple periodicities, should not impact the UEs decision on whether to continue GNSS operation.

Proposal 6: RAN2 to further agree to and adopt the change in Annex A.

## 2.3	X250

RIL X250 suggests to broadcast a distance threshold together with the reference location, in order to accommodate for the different neighbour cell sizes and thus also the ambiguity of the reporting.

Our understanding of the issue is, that the UE should be allowed to not report a set of closest location(s), in case a certain distance threshold is not met. We have sympathy for the issue, but we do believe that distance thresholds for reporting is not easy to configure, and may even cause test case issues impact to RAN4, whom are already working heavily on the topic and may impact the double periodicity.

Observation 5: Allowing a UE not to report certain locations in the reference location reporting may have merit.

Observation 6: Distance thresholds may not be trivial to implement by gNB.

Proposal 7: <RIL X250> RAN2 not to agree on distance threshold.

Although the distance threshold may not be optimal, we think that there may be merit to allow the UE to select certain locations not to report, based on UE implementation.

Proposal 8: RAN2 to discuss whether the UE should be allowed to report fewer locations than the once indicated by network.

## 2.4	A200

This topic suggests associating smtc in SIB2 with a reference location in refLocList-r19.

[Description]: The current assumption from CR rapporteur is SMTC in SIB2 is associated with reference location in SIB19, and it is used for serving cell measurement. We think it is not always the case. Note that when NW provides smtc in SIB2, it should associate with the largest SSB periodicity of serving and neighbor cell. For example, if serving cell has a SSB periodicity of 160ms, and neighbor cell has a SSB periodicity of 20ms, the smtc in SIB2 has to be with 160ms periodicity. That is, it is actually for neighbor cell measurement. And smtc5list can provide another periodicity of 20ms for serving cell measurement. Therefore, we would like to clarify that there should be an explicit reference location associated with legacy smtc configured in SIB2. [Proposed Change]: Allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

In this issue, we do not agree with the proposed change. Here, we note that the smtc in SIB2 could be used by legacy UEs or by DL-CE UEs. In other words, UEs that do not support the DL-CE enhancements will also be able to read the smtc field. We agree that in the legacy case, the UE may expect that the SMTC periodicity is equal to or larger than the SSB periodicity, but the whole point of introducing the smtc5list is to support two different periodicities between smtc and smtc5list. As mentioned in the discussion, our understanding is that smtc is always used for the serving cell measurements. Currently, the specification allows the UE to measure less smtcs configured in smtc4list, if the number of configurations exceed the UE capability. If the understanding is that the neighbor cell smtc could be configured in smtc5list, there could be a situation in which a UE does not measure the serving cell, but measures the neighbor cells.

Proposal 9: <RIL A200>  Do not allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: <RIL N085> Update the specification text according to N085.

Proposal 2: Related to N085, update the specification text to mention unique SMTCs in section 5.5.2.10

Observation 1: Location information for assisted SMTC selection should not imply mandating the UE to periodically acquire the GNSS location.

Observation 2: Currently, the specs does not mandate any requirements regarding the GNSS update periodicity, as long as the position is sufficient for TA and frequency compensation.

Observation 3: Multiple SMTC configuration is an optional feature considering an optimisation and should not set unnecessary requirements towards the UE.

Observation 4: It depends on the UE how long it continues to operate with a valid GNSS position.

Proposal 3: RAN2 to agree that in the end it is up to UE implementation how to maintain synchronisation towards the satellite, and the GNSS requirement just indicate the requirement to have GNSS available, in order to avoid unnecessary connection attempts.

Proposal 4: <RIL S024> RAN2 to agree to S024 and at least add “if available” to the inclusion of referenceLocationReport.

Proposal 5: RAN2 to agree that more than 4 SMTCs, or multiple periodicities, should not impact the UEs decision on whether to continue GNSS operation.

Proposal 6: RAN2 to further agree to and adopt the change in Annex A.

Observation 5: Allowing a UE not to report certain locations in the reference location reporting may have merit.

Observation 6: Distance thresholds may not be trivial to implement by gNB.

Proposal 7: <RIL X250> RAN2 not to agree on distance threshold.

Although the distance threshold may not be optimal, we think that there may be merit to allow the UE to select certain locations not to report, based on UE implementation.

Proposal 8: RAN2 to discuss whether the UE should be allowed to report fewer locations than the once indicated by network.

Proposal 9: <RIL A200> Do not allow smtc in SIB2 to associate with a reference location in RefLocList-r19.

# Annex A

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxBW-PreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxMIMO-LayerPreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes minSchedulingOffsetPreferenceConfigExt:

4>	consider itself to be configured to provide its preference on the minimum scheduling offset for 480 kHz SCS and/or 960 kHz SCS for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report, SCGFailureInformation, successful handover report, and successful PSCell change or addition report (if received for the associated cell group);

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the assisted-SSB-MTC-Config:

2>	if the assisted-SSB-MTC-Config is set to setup:

3>	consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.7.4;

NOTE x:	The UE is requested to attempt to have valid detailed location information available whenever sending location information for assisted SMTC. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding how to determine the location information is up to UE implementation.

2>	else:

3>	consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state.

<< Text omitted >>

#### 5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information.

A UE capable of providing its preference on DRX parameters of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters and upon change of its preference on DRX parameters.

A UE capable of providing its preference on the maximum aggregated bandwidth of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum aggregated bandwidth preference and upon change of its maximum aggregated bandwidth preference.

A UE capable of providing its preference on the maximum number of secondary component carriers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of secondary component carriers preference and upon change of its maximum number of secondary component carriers preference.

A UE capable of providing its preference on the maximum number of MIMO layers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of MIMO layers preference and upon change of its maximum number of MIMO layers preference.

A UE capable of providing its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a minimum scheduling offset preference and upon change of its minimum scheduling offset preference.

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

A UE capable of providing configured grant assistance information for NR sidelink communication in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide traffic pattern information and upon change of traffic patterns.

A UE capable of providing an indication of its preference in being provisioned with reference time information may initiate the procedure upon being configured to provide this indication, or if it was configured to provide this indication and upon change of its preference.

A UE capable of providing an indication of its preference in FR2 UL gap may initiate the procedure if it was configured to do so, upon detecting the need of FR2 UL gap activation/deactivation.

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

A UE capable of providing MUSIM assistance information for gap priority preference and/or preference to keep the colliding MUSIM gaps may initiate the procedure if it was configured to do so, upon determining it has gap priority preference information and/or it has preference to keep the colliding MUSIM gaps.

A UE capable of providing MUSIM assistance information for leave indication may initiate the procedure if it was configured to do so upon determining that it needs to leave RRC_CONNECTED state.

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

A UE capable of relaxing its BFD measurements in serving cells of a cell group in RRC_CONNECTED shall initiate the procedure for providing an indication of its relaxation state for BFD measurements upon being configured to do so, and upon change of its relaxation state for BFD measurements in RRC_CONNECTED state.

A UE capable of SDT initiates this procedure when data and/or signalling mapped to radio bearers that are not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing).

A UE capable of providing its preference for SCG deactivation may initiate the procedure if it was configured to do so, upon determining that it prefers or does no more prefer the SCG to be deactivated.

A UE that has uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated shall initiate the procedure.

A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

A UE capable of providing service link propagation delay difference between serving cell and neighbour cell(s) shall initiate the procedure upon being configured to do so, and upon determining that service link propagation delay difference between serving cell and a neighbour cell has changed more than threshPropDelayDiff compared with the last reported value.

A UE capable of providing an indication of its preference on multi-Rx operation for FR2 may initiate the procedure if it was configured to do so, upon detecting having a preference on multi-Rx operation for FR2 and upon change of its preference on multi-Rx operation for FR2.

A UE capable of indicating the availability of flight path information may initiate the procedure, if it was configured to do so, upon determining that an initial or updated flight path information is available.

A UE capable of providing UL traffic information shall initiate the procedure when this information is available upon being configured to do so, and upon change of UL traffic information.

A UE capable of N3C remote UE operation initiates the procedure upon being configured to report relay UE information on the available non-3GPP connection(s), and upon change of its available non-3GPP connection(s).

A UE capable of providing configured grant assistance information including SL-PRS transmission periodicity, priority, bandwidth and delay budget for NR sidelink positioning in RRC_CONNECTED may initiate the procedure.

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so, and upon determining that the closest reference location(s) have changed compared with the last reported values.

NOTE x:	Further details, e.g. regarding how to determine the location information has changed is up to UE implementation since a UE may not be able to continuously obtain a valid GNSS location e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage.