3GPP TSG-RAN WG2 Meeting #132 	 R2-2508040

Dallas, USA, November 17th – 21st, 2025

Agenda item:	8.9.3

Source:	vivo

Title:	MAC Corrections regarding CB-Msg3

Document for:	Decision

# Introduction

In this contribution, we would like to propose two MAC corrections regarding CB-Msg3.

# Discussion

## 2.1	Correction on CB-Msg3-EDT grant reception

For CB-Msg3-EDT, grants are provided via CB-Msg3-ConfigSIB(-NB) in system information. It is a new and scenario-specific UL grant that is exclusively for CB-Msg3-EDT and not covered by existing UL grant definitions in the MAC spec. Thus, it is necessary to capture this kind of grant in sub-clause 5.4.1. For example,

### 5.4.1 UL Grant reception In order to transmit on the UL-SCH the MAC entity must have a valid uplink grant (except for non-adaptive HARQ retransmissions) which it may receive dynamically on the PDCCH or in a Random Access Response or which may be configured semi-persistently or preallocated by RRC or provided by RRC for transmission using PUR (see clause 5.4.7) or provided in system information for CB-Msg3 transmission (see clause 5.1a). To perform requested transmissions, the MAC layer receives HARQ information from lower layers. When the physical layer is configured for uplink spatial multiplexing, the MAC layer can receive up to two grants (one per HARQ process) for the same TTI from lower layers.

Observation 1: The existing definition of UL grants in the MAC spec fails to cover the UL grant that 9s exclusively for CB-Msg3-EDT.

Proposal 1: RAN2 to capture in the MAC spec that a valid uplink grant may be provided in system information for CB-Msg3 transmission.

## 2.2	Correction on CB-Msg3 resource selection

For CB-Msg3-EDT with DSA transmission, the network always configures cb-Msg3-TxWindow to enable the UE to determine a CB-Msg3 transmission window for replica resource selection, as shown in Figure 1.

Figure 1: CB-Msg3 Tx window

However, there is a critical ambiguity in the current specifications: the operational mechanism of the parameter windowSize is not defined. Specifically, neither the RRC spec nor the MAC spec provides any description of its usage scenarios or UE behavior implementation. In our understanding, it represents the validity duration of the CB-Msg3 PUSCH resources within the broader CB-Msg3 transmission window. In other words, even if the UE operates within the overall periodicity of CB-Msg3 transmission window, it shall not use the CB-Msg3 PUSCH resources if the selected time slot falls outside the time duration provided by windowSize.

To resolve this ambiguity and ensure the spec’s readability, it is necessary to explicitly clarify the operation role of windowSize in the MAC specification. Specifically, we recommend adding the following clarification to clause 5.1a.2.

cb-Msg3-TxWindow-r19 SEQUENCE { windowSize-r19 INTEGER {3..10}, windowPeriodicity-r19 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64, n96, n128, n160, n192, n224, n256, n288} } OPTIONAL, --Need OP cb-Msg3-TxWindow CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. For windowSize, value 3 corresponds to 3 PUSCH periods, 4 corresponds to 4 PUSCH periods and so on. For windowPeriodicity, value n1 corresponds to 10ms, n2 corresponds to 20ms and so on.

### 5.1a.2 CB-Msg3 transmission Before the CB-Msg3 transmission, the MAC entity shall select cb-Msg3-NumOfReplicas UL grants for CB-Msg3 transmission as follows: - select the next upcoming CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level; - randomly select cb-Msg3-NumOfReplicas PUSCH resources in the time domain permitted by the window size given by windowSize if configured within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by cb-Msg3-TimeResource associated with the selected enhanced coverage level; - randomly select a frequency domain PUSCH resource for each selected time domain resource provided by prb-AllocationInfoSet (npusch-SubCarrierSetList for NB-IoT) associated with the selected enhanced coverage level.

Observation 2: Neither the RRC specification nor the MAC specification provides any description of windowSize regarding its usage scenarios or the related UE behavior.

Proposal 2: RAN2 to capture in the MAC spec that UE randomly selects the CB-Msg3 PUSCH resources in the time domain permitted by the window size given by windowSize if configured.

# 3 Conclusion

In this contribution, we have discussed the MAC corrections regarding CB-Msg3. And we made the following observations and proposals,

2.1	Correction on CB-Msg3-EDT grant reception

Observation 1: The existing definition of UL grants in the MAC spec fails to cover the UL grant that 9s exclusively for CB-Msg3-EDT.

Proposal 1: RAN2 to capture in the MAC spec that a valid uplink grant may be provided in system information for CB-Msg3 transmission.

2.2	Correction on CB-Msg3 resource selection

Observation 2: Neither the RRC specification nor the MAC specification provides any description of windowSize regarding its usage scenarios or the related UE behavior.

Proposal 2: RAN2 to capture in the MAC spec that UE randomly selects the CB-Msg3 PUSCH resources in the time domain permitted by the window size given by windowSize if configured.