3GPP TSG-RAN WG2 Meeting #132 	 R2-2508042

Dallas, USA, November 17th – 21st, 2025

Agenda item:	8.17

Source:	vivo

Title:	Remaining Issues on IoT NTN TDD Operation

Document for:	Decision

# Introduction

In RAN2#129bis, it was agreed that the UL transmission, e.g., the SPS, PUR if in the unavailable UL subframe can be postponed to the next available UL transmission occasion.

4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

However, the UE behaviour is unclear if different UL transmissions are postponed to the same valid UL subframe. In this contribution, we would like to discuss the above open issues and give our considerations correspondingly.

# Discussion

RAN2 already agreed that the PUR or the SPS resource that does not align with the UL subframes can be postponed to the next valid UL subframe. Thus, the PUR and SRS may be postponed to the same next valid UL subframe. Further, upon network schedules the UL transmission, even if network can ensure the UL transmission always happens in the valid UL subframe up to network implementation, but due to the postponement of the PUR or SRS transmission, it is still possible that the PUR or SPS transmission conflicts with the scheduled UL transmission.

Figure 1: Conflicted UL transmission due to the postponement of the PUR/SRS transmission

Observation: The PUR or SPS may conflict with the scheduled UL transmission or SPS transmission or PUR transmission due to the postponement of the PUR and SPS transmission in the non-valid UL subframe to the next valid UL subframe.

However, according to the spec, there is no clarification on how to handle the issue. Upon the conflict happens, the UE behaviour is quite unclear. From our perspective, the dynamically scheduled UL transmission in general has higher priority, so it should be transmitted first. Further, as for the PUR and SPS transmission, no strong view on which should be prioritised; maybe we can just leave it for implementation.

Proposal 1: For conflict between UL SPS/UL PUR and the UL grant scheduled by PDCCH, the transmission of the UL grant scheduled by PDCCH is prioritised.

Proposal 2: For conflict between UL SPS and UL PUR, it is up to the UE implementation to decide which transmission is prioritised.

# 3 Conclusion

In this contribution, we have discussed the remaining issues on IoT NTN TDD operation. And we make the following observation and proposals,

Observation: The PUR or SPS may conflict with the scheduled UL transmission or SPS transmission or PUR transmission due to the postponement of the PUR and SPS transmission in the non-valid UL subframe to the next valid UL subframe.

Proposal 1: For conflict between UL SPS/UL PUR and the UL grant scheduled by PDCCH, the transmission of the UL grant scheduled by PDCCH is prioritised.

Proposal 2: For conflict between UL SPS and UL PUR, it is up to the UE implementation to decide which transmission is prioritised.

# 4 Reference

[1] 	R2-2508002, RAN2#131bis Meeting Report.