3GPP TSG-RAN WG2 Meeting #132 	 R2-2508045

Dallas, USA, Nov. 17th – 21st, 2025

Agenda item:	10.2.2

Source:	vivo

Title:	Consideration of TN and NTN Integration in 6GR

Document for:	Decision

- Introduction

To fulfil 6G’s vision of ubiquitous connectivity, TN-NTN integration, as explicitly stated in the 6G Radio (6GR) SID in [1], is identified as a foundational objective, aiming to eliminate coverage gaps across terrestrial, maritime, and aeronautical, also enabling emerging use cases such as disaster emergency communications.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In the 5G phase, the inherent characteristics of NTN (e.g., large round-trip time and fast satellite mobility) pose significant challenges to system design. Given that NR NTN was specified in a later NR release, and to ensure forward compatibility of legacy UE functions, there was no opportunity to achieve harmonization between TN and NTN in NR. This has resulted in a non-forward-compatible add-on approach for NR NTN, manifested in procedure patching (e.g., introducing NTN-specific signalling to extend parameter ranges) and redundant implementations (e.g., coexisting “TN" and "NTN" versions of the same function). What’s worse, certain fundamental bottlenecks requiring cross working group coordination remain intractable. A prominent example is that handover from an NR TN cell to an NR NTN cell is deemed unsupported due to the lack of RRM requirements.

In our understanding, a harmonized 6G Radio design for TN and NTN will help to eliminate redundant procedural text and address NTN bottleneck issues. In this contribution, we present our perspectives on TN and NTN integration, including baseline scenario/functionality reused from NR NTN, new technical requirements emerging, and TN-NTN integration design principles. The objective is to guide the definition of minimal essential NTN-related requirements and the subsequent unified 6G radio interface design.

- Discussion

  - 2.1	5G NTN overview: scenarios and functionalities

2.1.1 5G NTN scenarios

The basic platforms of 5G NTN operating with FDD include the GEO, MEO, and LEO, and the corresponding orbit characteristics for each platform are summarised in the following table.

Table 1: Characteristics of the 5G NTN platforms

| Platforms | Altitude range | Orbit | Typical beam footprint size |
| --- | --- | --- | --- |
| LEO | 300 – 1500 km | Circular around the earth | 100 – 1000 km |
| MEO | 7000 – 25000 km |  | 100 – 1000 km |
| GEO | 35 786 km | Notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | 200 – 3500 km |

For each NTN satellite, depending on whether the gNB is deployed in the satellite or on the ground, there are transparent payload mode and regeneration payload mode. For LEO orbit, it can provide lower latency (e.g., 12-50ms RTT) and higher data throughput, while it also introduces challenges such as frequent handover and increased Doppler effects due to rapid satellite movement. For MEO orbit, it can offer a balance between coverage area and latency, providing broader coverage than LEO and lower latency than GEO, but it faces moderate signal delays and more complex constellation management. For GEO orbit, it ensures stable coverage over large regions with simpler ground equipment, but the high altitude results in significant signal latency (e.g., 270-541 ms RTT).

Further, depending on whether the satellite beam is steerable or moving over the earth, the NTN cell can be one of the following types:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth's surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

We understand that these mature scenarios have been validated through theoretical analysis and potential prototype testing. Moreover, we expect them to remain typical deployment scenarios in 6G. Thus, they ought to be the primary focus in the design for 6G TN-NTN integration.

Proposal 1: For 6G NTN study, the following scenarios are taken as a starting point:

- Support GEO satellite, MEO satellite, and LEO satellite;

- Support transparent payload and regeneration payload w/wo ISL;

- Support (Quasi-)Earth-fixed cell and Earth-moving cell;

- FDD system is applicable.

2.1.2 5G NTN functionalities

5G NTN, standardized in Rel-17 and enhanced across Rel-18 and Rel-19, establishes a comprehensive technical framework that addresses core NTN challenges: large round-trip time, rapid satellite movement, and large cell coverage without obvious near-far effect. Based on Proposal 1, we have a critical observation that the core challenges of NTN remain prevalent in 6G, and the functionalities developed in 5G NTN have proven effective in addressing these issues. Therefore, it is necessary to review the key 5G NTN achievements before considering the minimal essential set of requirements for 6G NTN.

5G NTN’s key RAN2 features are summarised in the following:

- To adapt to the large round-trip time, the following enhancements are made:

- UE reporting of information related to pre-compensation TA, i.e., UE pre-compensates the TA based on the UE position and satellite ephemeris, and UE reports the pre-compensation TA to the network during RACH procedure or upon TA changes.

- Extended HARQ process number and configurable HARQ feedback, i.e., HARQ enabled or disabled for DL HARQ transmission or the HARQ mode A or HARQ mode B for UL HARQ transmission;

- MAC timer adjustment, i.e., delay the start time of the RAR Window and Contention Resolution Timer by UE-gNB RTT, adjust the value of HARQ RTT timer and the DRX active time calculation;

- Extension of the RLC/PDCP timer value range, e.g., t-Reassembly, discardTimer and t-Reordering.

- To adapt to the frequent feeder link switch and/or service link switch due to the rapid satellite movement, the following enhancements are made:

- Time-based cell reselection measurement, e.g., based on the time when the serving cell stops serving the area;

- Time-based trigger event for CHO, e.g., based on when the candidate cell serves the area;

- Satellite switch with resync, e.g., network to ensure there is unchanged PCI and configuration after the satellite switch, and UE just needs to reacquire the satellite assistance information via SIB and perform UL/DL sync;

- RACH-less HO, e.g., no need to perform RACH upon handover to accelerate the HO procedure;

- SMTC enhancement, e.g., multiple SMTC configurations, UE autonomous SMTC offset adjustment for IDLE/INACTIVE state, UE PDD reporting in CONNECTED state to assist the SMTC/Gap configuration;

- Enhanced tracking area management to support both soft TAC switch and hard TAC switch, e.g., one cell can broadcast more than one TAC per PLMN and UE does not perform TAU if one of the currently broadcast TAC belongs to UE’s registration area.

- To adapt to the large cell coverage and the inapparent near-far effect, the following location-based enhancements are made:

- Coarse UE location report;

- Location-based SMTC selection;

- Intended area-based service, e.g., broadcast MBS service, PWS;

- NTN-TN cell reselection enhancements, e.g., NTN cell broadcasts TN area information and TN cell measurement skipping;

- Location-based cell reselection measurement, e.g., based on the distance between UE and reference location of the serving cell;

- Location-based measurement report event, e.g., based on the distance between UE and the reference location of the serving cell and the neighbour cell;

- Location-based trigger event for CHO, e.g., based on the distance between UE and reference location of the serving cell and the CHO candidate cell;

- Satellite ephemeris provision and validity of the satellite ephemeris for location-based enhancement and the TA pre-compensation.

Among the listed features, most of them remain critical for 6G NTN, to address inherent core NTN communication challenges that persist in 6G. However, there are still some features (i.e., satellite switch with resync and MBS broadcast via intended service area) that can be excluded, with justifications detailed below.

Satellite switch resync for 5G NTN is limited to transparent payload mode and quasi-Earth fixed cell case. Since it is not a general feature for all NTN scenarios, we think it is not essential to consider it in 6G Day-1. Further, satellite switch with resync mechanism is designed to realize satellite switching without triggering the L3 HO procedure. Nevertheless, RRC reconfiguration is typically unavoidable. The key reason is that RRC Reconfiguration procedure is needed to update the SMTC and measurement gap configuration, in order to rematch the PDD between the target satellite and satellite of the adjacent cells. As such, the benefit of satellite switch with resync relative to L3 HO is still under doubt.

MBS broadcast within the intended service area is not a priority for 6G NTN. 6G NTN generally emphasises personalised and emergent communications, and MBS broadcast’s one-to-many media transmission model has limited use cases.

Based on the analysis above, it is proposed that,

Proposal 2: For 6G NTN study, at least the following are considered:

- TA report due to UE pre-compensation;

- HARQ disable/enable and HARQ mode A/B;

- UP timer value covering TN and NTN;

- HARQ process number;

- Multiple SMTC configurations;

- Multiple TAC management;

- RACH-less HO;

- Time-based mobility enhancement, e.g., time-based CHO;

- Location-based mobility enhancement.

  - 2.2	New requirements for 6G NTN

While 5G NTN provides a solid baseline, in the 6G era, emerging use cases demand new technical requirements.

VLEO Satellite Support (~200 km): it is critical for 6G’s low-latency NTN use cases (e.g., target RTT < 30ms) and high-bandwidth transmission. However, VLEO introduces new technical challenges. For example, VLEO Satellite has a higher orbital speed (e.g., 7.8 km/s) and shorter orbital period (e.g., 90-120 minutes) increases cell handover frequency (e.g., every 2-3 minutes per UE) and synchronization difficulties. Another thing is that VLEO constellations lead to coverage overlapping. We can consider seamless inter-satellite handover to avoid service interruption with advanced and optimised mobility management solutions.

GNSS Free Support: 5G NTN relies on UE GNSS positioning, but 6G’s target scenarios (e.g., GNSS jammed environments) have GNSS unavailability. That is 6G NTN requires reducing or eliminating UE’s dependency on GNSS, enabling NTN access for UE without valid GNSS information.

Seamless intra-RAT and inter-RAT TN-NTN/NTN-NTN Mobility: 5G NTN’s mobility mechanisms overlook intra-RAT TN-NTN handover and cell reselection, increasing latency. Further, 5G TN is already deployed, and the 5G NTN is going to be deployed by the satellite company and the operator. With the introduction of 6G NTN/6G TN, there is a scenario where the UE working in 5G TN/NTN needs to reselect/handover to the 6G NTN/TN cell. To enable the scenario and to facilitate seamless mobility, it is important to support the inter-RAT TN/NTN mobility.

Proposal 3: For 6G NTN study, RAN2 also considers the following new requirements:

- Support VLEO;

- GNSS information is unavailable/degraded/relaxed;

- Seamless intra-RAT and inter-RAT TN-NTN and NTN-NTN mobility.

  - 2.3 Design principles for 6G TN-NTN integration

In our understanding, the core technical objective of 6G TN-NTN integration lies in breaking the isolation between TN and NTN in 5G and developing a unified radio function that inherently accommodates NTN’s characteristics without NTN-specific "added-on" modules. For example, based on the unified radio function, the network can via configurable parameters to take care of NTN needs.

The next question that comes to us is how to realize the TN-NTN integration covering all the aforementioned requirements and functionalities. Herein, we would like to give our initial considerations.

- MAC layer TN-NTN integration:

- HARQ process handling: For the TN scenario with high-latency backhaul or the real-time service transmission, extending the HARQ process number and HARQ feedback enabled/disenabled or HARQ mode A/B is also beneficial to improve the data transmission efficiency and avoid unnecessary retransmission. To be compatible with the NTN or TN scenario with high-latency backhaul or the real-time service transmission scenario, the supported number of HARQ processes and the configuration of the HARQ feedback enabled/disabled for DL or the HARQ mode A/HARQ mode B for UL can be supported as a basic 6G Day-1 feature. Then, up to implementation, network can configure different HARQ process numbers and HARQ feedback configurations. For example, for TN case without long-latency backhaul or for delay-tolerant service transmission, the HARQ process number can be set to 16, and network configures the HARQ enabled for DL and HARQ mode B for UL. While for NTN case or TN scenario with high-latency backhaul or the real-time service transmission scenario, network can set the HARQ process number to 32, and set the HARQ is disabled for DL and HARQ mode A for UL.

- MAC timer adjustment: To achieve a unified handling for MAC timers without distinction between TN or NTN, one possible solution is to introduce a network configurable adjustment offset, i.e., regardless of TN/NTN, network provides an adjustment offset to UE, but setting different values in different cases, then UE adjusts the RACH timer start time and the HARQ RTT timer based on the adjustment offset.

- RLC/PDCP Layer TN-NTN integration:

- Due to the long RTT impact, the RLC t-Reassembly and discardTimer/t-Reordering need to be designed with consideration of both TN and NTN scenarios in 6G Day-1. Since these timers are all configured by NW, there is no UE autonomous behaviour to perform the adjustment. So one simple way to harmonize the TN/NTN operation is just to extend the value range of these timers, and up to network implementation to configure different values for different scenarios.

- Mobility aspects for TN-NTN integration:

- Considering the network deployment, 5G TN/NTN and 6G TN/NTN will be deployed simultaneously. To enable consistent and seamless UE mobility, the intra-RAT TN-NTN/NTN-NTN mobility and inter-RAT TN-NTN/NTN-NTN mobility should be considered. In general, the basic TN mobility procedure can be reused directly in the TN-NTN or NTN-NTN mobility scenario with some NTN assistance information. Moreover, cell on/off for network energy saving in TN scenario is consistent with the service time concept for NTN. Thus, the time information related mobility enhancement, i.e., time-based cell reselection, time-based CHO, is not only beneficial for NTN case but also for the network energy saving purpose in TN scenario.

- RACH-less HO is an important feature in 5G and it applies not only to TN but also NTN scenario. With the use of RACH-less HO, the handover latency can be reduced. Thus, we think that for 6G NTN, the RACH-less HO can also be investigated (in mobility session).

- Multiple SMTC/measurement gap configurations can be supported in 6G. It is beneficial to both NTN and network energy saving scenario. For example, for NTN case, it can adapt to the satellite movement and the diverse propagation delay difference between different neighbour cells and serving cell. For network energy saving case, it can adapt to the different cell DTX/DRX patterns and SSB periodicity.

Except the above features that can be harmonized between TN and NTN, there are also some NTN-specific add-on features that cannot be addressed via common functions. For example, the satellite ephemeris provision and validity, and the location-based functions. However, in 5G NTN, it is assumed that the GNSS is always available before Rel-20. While in R20 NR NTN, GNSS-resilience is studied because there are scenarios where the GNSS signal may not be available. Once there is no available GNSS, the satellite ephemeris information may also not be useful. Further, the location-based enhancement for 5G NTN may also not be used. Whether such features are essential for 6G Day-1 or can be introduced as further enhancement later should be further evaluated.

Proposal 4. RAN2 to study the 6G TN-NTN integration based on the following principles:

- Study in priority the following functionalities in 6G TN-NTN integration:

  - HARQ disable/enable and HARQ mode A/B;

  - UP timer value/HARQ process number;

  - Multiple SMTC configurations;

  - Multiple TAC management;

  - Time-based mobility enhancement;

- Support NTN access for UE without valid GNSS information;

- Ensure seamless service continuity across different networks, e.g., inter-RAT/intra-RAT TN-NTN and NTN-NTN mobility.

- Further evaluate the necessity of NTN-specific add-ons in 6G Day-1, e.g., satellite ephemeris signaling and the related location-based mobility enhancement.

3 Conclusion

In this contribution, we have provided technical insights into 6G TN-NTN integration, building on 5G NTN’s validated framework, addressing 6G-specific new technical requirements, and defining procedure-level design principles. All our proposals are summarized below,

Proposal 1: For 6G NTN study, the following scenarios are taken as a starting point:

- Support GEO satellite, MEO satellite, and LEO satellite;

- Support transparent payload and regeneration payload w/wo ISL;

- Support (Quasi-)Earth-fixed cell and Earth-moving cell;

- FDD system is applicable.

Proposal 2: For 6G NTN study, at least the following are considered:

- TA report due to UE pre-compensation;

- HARQ disable/enable and HARQ mode A/B;

- UP timer value covering TN and NTN;

- HARQ process number;

- Multiple SMTC configurations;

- Multiple TAC management;

- RACH-less HO;

- Time-based mobility enhancement, e.g., time-based CHO;

- Location-based mobility enhancement.

Proposal 3: For 6G NTN study, RAN2 also considers the following new requirements:

- Support VLEO;

- GNSS information is unavailable/degraded/relaxed;

- Seamless intra-RAT and inter-RAT TN-NTN and NTN-NTN mobility.

Proposal 4. RAN2 to study the 6G TN-NTN integration based on the following principles:

- Study in priority the following functionalities in 6G TN-NTN integration:

  - HARQ disable/enable and HARQ mode A/B;

  - UP timer value/HARQ process number;

  - Multiple SMTC configurations;

  - Multiple TAC management;

  - Time-based mobility enhancement;

- Support NTN access for UE without valid GNSS information;

- Ensure seamless service continuity across different networks, e.g., inter-RAT/intra-RAT TN-NTN and NTN-NTN mobility.

- Further evaluate the necessity of NTN-specific add-ons in 6G Day-1, e.g., satellite ephemeris signaling and the related location-based mobility enhancement.

4 Reference

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone.