3GPP TSG-RAN WG2 Meeting #132	R2-2508050

Dallas, USA, Nov. 17th-21st, 2025

Source:	vivo

Title:	6GR RRC Modelling and connection management

Agenda Item:	10.3.2.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#131-bis meeting, RAN2 had an initial discussion on RRC states and concluded to further study the following aspects:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In this contribution, we will further discuss the RRC state and connection management.

- Discussion

  - Potential functions for Inactive mode

    - Fast state transition

In the TR 38.914 [1] of the ongoing 6G SI "Study on 6G Scenarios and Requirements", the minimum requirement for control plane latency is 20 ms as follows.

### 5.1.8 Control plane latency Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). The minimum requirement for control plane latency is 20 ms.

In TR 37.910 [2], it is observed that NR Rel-15 evaluated the control plane latency requirement as the state transition time from RRC_INACTIVE to RRC_CONNECTED. To be specific, the latency is calculated from UE perspective, from the initiation of the connection resume procedure (e.g., transmission of the RACH Preamble) to the completion of the connection resume procedure (e.g., transmission of RRC Resume Complete message). In this sense, when it comes to 6GR study, RAN2 should further clarify how to calculate the state transition delay to align companies’ understanding. We think the definition in NR can be reused.

Figure 2.1-1: C-plane procedure (example for NR Rel-15)

Furthermore, it is observed that NR fulfills the control plane latency requirement of 20ms with the introduction of RRC_INACTIVE state in [2]. Besides, in the contribution [3] presented in RAN2#131-bis meeting, it was explained that the RRC_INACTIVE has not been widely enabled in 5G networks due to the following reasons from operator’s perspective:

1) Network planning complexity for RAN Notification Area (RNA);

2) Limited power saving gain compared to RRC_IDLE;

3) Lack of use cases requiring low state transition delay.

Among them, the key is the lack of use cases requiring low state transition delay in real-world applications. Therefore, for 6GR, there is no need to strive for a lower state transition delay on top of the NR connection resume procedure.

- RAN2 to clarify that the state transition delay is defined as the time from the initiation (e.g., the RACH Preamble transmission) to the completion of the connection resume procedure (e.g., RRCResumeComplete message transmission), i.e., equal to CP latency.

- The discussion on further enhancements of a faster state transition (on top of the NR connection resume procedure) should be deprioritized, due to:

- Lack of use cases requiring low state transition delay;

- The 6G CP latency requirement of 20ms can be fulfilled by introducing Inactive mode.

    - Data transfer in inactive mode

In NR, the SDT was introduced in Rel-17 with the following use cases, intentions, and mechanisms:

- Use case: Small and infrequent data traffic, including:

- Smartphone applications: traffic from Instant Messaging services; heartbeat/keep-alive traffic from IM/email clients and other apps; push notifications from various applications.

- non-smartphone applications: traffic from wearables (periodic positioning information, etc); sensors (Industrial Wireless Sensor Networks transmitting temperature, pressure readings periodically or in an event-triggered manner, etc); smart meters and smart meter networks sending periodic meter readings.

- Intention: To allow data and/or signalling transmission while the UE remains in RRC_INACTIVE (i.e., without transitioning to RRC_CONNECTED state) to reduce the power consumption and signalling overhead during state transition.

- Mechanism: Initiated with either a transmission over RACH (referred to as RA-SDT) or over Type 1 CG resources (referred to as CG-SDT).

However, the initiation of SDT is subject to significant restrictions:

-	less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, and;

-	the DL RSRP is above a configured threshold, and;

-	A valid SDT resource (either RACH or Configured grant) is available.

Furthermore, 6G will further study power saving mechanisms for connected mode. If the connected mode can also meet the requirements for small and infrequent data traffic under low power consumption, the necessity of studying data transfer during inactive mode would be significantly reduced. This can be revisited after the power saving mechanisms in connected mode are stable.

Therefore, we propose:

- RAN2 to deprioritize the discussion on data transfer during Inactive mode. And revisit the necessity after the power saving mechanisms in the Connected mode of 6G are stable.

    - RNA and paging

In NR, the last serving NG-RAN node should configure UE with an RNA when releasing UE to RRC_INACTIVE, the RNA can cover a single or multiple cells, and shall be contained within the CN registration area. A UE in the RRC_INACTIVE state can be configured with periodic RNA update, and is required to initiate RNA update procedure when it moves out of the configured RNA. When receiving RNA update request from the UE, the receiving gNB could trigger the Xn-based UE context retrieval procedure to get the UE context from the last serving gNB. Xn connectivity should be available within the RNA as only Xn-based UE context retrieval is supported in NR. On the other hand, if the last serving gNB receives DL data from the UPF or DL UE-associated signalling from the AMF while the UE is in RRC_INACTIVE, it pages the UE in the cells corresponding to the RNA and may send RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s).

During the above procedures, the last serving gNB will allocate a dedicated I-RNTI to the UE to identify the UE context in RRC_INACTIVE. The UE shall provide the I-RNTI in the RRC resume request message when it resumes from RRC_INACTIVE. And the I-RNTI will be included in the Paging message in the RAN Paging. The I-RNTI structure is described in the Annex of TS 38.300 as follows:

To support an NG-RAN node in resolving the Local NG-RAN ID of the NG-RAN node that allocated the UE context, the I-RNTI structure is as follows: - the first part (starting from the MSB of the I-RNTI) identifies the I-RNTI profile. It is 2 bits long for a Full I-RNTI or 1 bit long for a short I-RNTI, and it indicates the length of a Local NG-RAN Node Identifier of the NG-RAN Node that allocated the I-RNTI; - the second part, immediately following the first part, is a Local NG-RAN Node Identifier; - the third part, immediately following the second part, identifies the UE context stored in the NG-RAN node that allocated the I-RNTI.

As previously described, one key reason why RRC INACTIVE has not been enabled on the network side is that it requires additional effort for RNA management, which increases the OPEX for operators. Therefore, it is necessary for RAN2 to study how to minimize the complexity of supporting the Inactive mode.

One approach for further study is to support only CN paging, i.e., no RAN paging, thus no need to manage separate RNA. However, removing RAN paging raises several additional issues, including:

- Issue 1: Storage location of UE context;

- Issue 2: UE context retrieval;

- Issue 3: How the UE identification in the Paging message indicates the storage location of the UE context.

Regarding the above issues, we understand that the UE context can be stored in the last serving cell. If the UE initiates a RRC Resume procedure in the last serving cell, the UE context retrieval process is not required, which is highly beneficial for scenarios where UE has low mobility or remains stationary. For UE context retrieval, considering that the Xn interface between RAN nodes within a TA is not always available, e.g., if the UE moves over a large area during the Inactive mode, it becomes necessary to consider NG-based UE context retrieval. In such a case, the UE context should also be stored on the CN side to reduce the delay of UE context retrieval procedure. Furthermore, to indicate the storage location of the UE context, the UE identification in the Paging message should be further studied, e.g., identify the UE context and storage location based on the S-TMSI. The detailed procedure is illustrated in Figure 2.1-2.

Figure 2.1-2: State transition based on CN paging

Overall, this approach also impacts other groups, such as RAN3 and SA2. Therefore, coordination with these groups is necessary once RAN2 reaches a consensus.

- RAN2 to study how to minimize the complexity to support the Inactive mode in 6G. The following aspects could be considered:

- Paging is only initiated by CN, i.e., no RAN paging, thus no need to manage separate RNA.

- UE context can be stored at CN and/or RAN node (i.e., last serving cell).

- Support NG-based UE context retrieval.

- Study the content of UE identification in the paging message to identify the UE context and storage location.

Note: Coordinate with other WGs (e.g., SA2, RAN3) when RAN2 reaches a consensus.

  - Overall understanding of RRC state in 6G

In NR, support for the RRC_INACTIVE state is a mandatory feature for UE. However, the decision to release a UE to either RRC_IDLE or RRC_INACTIVE state is up to gNB implementation. Due to the previously mentioned reasons, operators may choose not to deploy this feature in the network. In such cases, the network can only release UEs to RRC_IDLE state rather than RRC_INACTIVE state. Consequently, although UEs are required to support the RRC_INACTIVE state, which increases implementation and testing complexity, operators may choose not to enable this feature.

Furthermore, even with enhancements to the inactive model, it cannot change the situation of lacking actual use cases requiring low state transition delay. As a result, the operator may still not widely deploy RRC_INACTIVE state.

- Even with enhancements to the Inactive model in 6G, it cannot change the situation of lacking actual use cases requiring low state transition delay.

Besides, the requirement for CP latency is only necessary for specific scenarios, such as low-latency communication use cases, and the RRC_INACTIVE state can be one alternative to fulfill the requirement.  For other cases, the network can release the UE to RRC_IDLE state for power saving purposes.

Overall, we believe that for 6G, RRC_CONNECTED and RRC_IDLE are still essential and should remain as mandatory RRC states, and the characteristics/functionalities of CONNECTED/IDLE in NR can be taken as the baseline. While for RRC_INACTIVE state, if it is supported in 6G, it should be defined as an optional feature.

- For the RRC state of 6G:

- The Idle and Connected should remain as mandatory RRC states. And the characteristics/ functionalities of RRC_CONNECTED/RRC_IDLE in NR can be taken as the baseline;

- The Inactive mode, if supported in 6G, should be defined as an optional feature for UE.

  - Connection management

    - RRC connection management procedures

Figure 2.3-1:	UE state machine and state transitions in NR

Figure 2.3-1 illustrates an overview of RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time. And the state transitions are supported via the following RRC procedures:

-	From RRC_IDLE to RRC_CONNECTED, via the "RRC connection establishment" procedure;

-	From RRC_CONNECTED to RRC_IDLE, via the "RRC connection release" procedure;

-	From RRC_CONNECTED to RRC_INACTIVE, via the "RRC connection release with suspend" procedure;

-	From RRC_INACTIVE to RRC_CONNECTED, via the "RRC connection resume" procedure;

-	From RRC_INACTIVE to RRC_IDLE, via the "RRC connection release" procedure.

We understand that the UE state machine and state transition procedures can be reused as the baseline in 6G. Therefore, we propose:

- The UE state machine and state transition procedures of NR can be reused as the baseline in 6G, including:

- From Idle to Connected mode, via the "RRC connection establishment" procedure;

- From Connected to Idle mode, via the "RRC connection release" procedure;

- From Connected to Inactive mode, via the "RRC connection release with suspend" procedure;

- From Inactive to Connected mode, via the "RRC connection resume" procedure;

- From Inactive to Idle mode, via the "RRC connection release" procedure.

    - UAC

To achieve a harmonized design for IoT devices and eMBB devices, there are some discussions in RAN1 on coverage target, bandwidth, unified initial access, etc. Per the discussion in RAN1, there is a large coverage gap (e.g., 10dB) for the eMBB and IoT coverage target, it is essential to study congestion control mechanisms that account for their disparate coverage targets. In NR, in addition to cell bar mechanism indicated by MIB and SIB1, UAC is also specified to achieve congestion control, which is based on access identity and access category. However, neither the NR UAC nor the NR cell bar mechanism has considered coverage level.

In contrast, UE category-based and CE level-based Cell Barring are introduced to improve access/load control in NB-IoT. Different from NB-IoT, which only considers coverage level-based cell bar, coverage level-based UAC for congestion control can be studied in 6G, which is more friendly compared with cell bar. For example, as mentioned in our company paper [4], extended coverage is regarded as one of the common features, and the eMBB device may utilize this feature when it is in poor coverage. If Barred is indicated for the corresponding coverage level of an eMBB UE in poor coverage, the eMBB UE cannot access this cell for a long time. In contrast, there is still a probability to access this cell via UAC mechanism, since the UAC mechanism is based on access probability and access prohibit timer.

- Study coverage level-based UAC for congestion control in 6G.

- Conclusion

In this paper, we discuss the RRC state and connection management of 6G radio. we have the following observations and proposals:

- Potential functions for Inactive mode

- RAN2 to clarify that the state transition delay is defined as the time from the initiation (e.g., the RACH Preamble transmission) to the completion of the connection resume procedure (e.g., RRCResumeComplete message transmission), i.e., equal to CP latency.

- The discussion on further enhancements of a faster state transition (on top of the NR connection resume procedure) should be deprioritized, due to:

- Lack of use cases requiring low state transition delay;

- The 6G CP latency requirement of 20ms can be fulfilled by introducing Inactive mode.

- RAN2 to deprioritize the discussion on data transfer during Inactive mode. And revisit the necessity after the power saving mechanisms in the Connected mode of 6G are stable.

- RAN2 to study how to minimize the complexity to support the Inactive mode in 6G. The following aspects could be considered:

- Paging is only initiated by CN, i.e., no RAN paging, thus no need to manage separate RNA.

- UE context can be stored at CN and/or RAN node (i.e., last serving cell).

- Support NG-based UE context retrieval.

- Study essential information in the UE identification in the paging message.

Note: Coordinate with other WGs (e.g., SA2, RAN3) when RAN2 reaches a consensus.

- Overall understanding of RRC state in 6G

- Even with enhancements to the Inactive model in 6G, it cannot change the situation of lacking actual use cases requiring low state transition delay.

- For the RRC state of 6G:

- The Idle and Connected should remain as mandatory RRC states. And the characteristics/ functionalities of RRC_CONNECTED/RRC_IDLE in NR can be taken as the baseline;

- The Inactive mode, if supported in 6G, should be defined as an optional feature for UE.

- Connection management

- The UE state machine and state transition procedures of NR can be reused as the baseline in 6G, including:

- From Idle to Connected mode, via the "RRC connection establishment" procedure;

- From Connected to Idle mode, via the "RRC connection release" procedure;

- From Connected to Inactive mode, via the "RRC connection release with suspend" procedure;

- From Inactive to Connected mode, via the "RRC connection resume" procedure;

- From Inactive to Idle mode, via the "RRC connection release" procedure.

- Study coverage level-based UAC for congestion control in 6G.

- Reference

- TR 38.914 (V0.2.0)		Study on 6G Scenarios and Requirements		3GPP

- TR 37.910		Study on self evaluation towards IMT-2020 submission		3GPP

- R2-2506900	Discussion on 6G control plane.	CMCC

- R2-2508046	6GR Design approaches and new services		vivo