3GPP TSG-RAN WG2 Meeting #132	R2-2508051

Dallas, USA, Nov. 17th – 21st, 2025

Source:	vivo

Title:	6GR RRC Structure and (re)configuration

Agenda Item:	10.3.2.2

Document for:	Discussion and Decision

- Introduction

At RAN2#131bis meeting, RAN2 has agreed the lists in below for RRC Structure and reconfiguration procedure [1]. In this contribution, we will discuss the detailed RRC configuration aspects, i.e. modular design, delta configuration and reconfiguration failure.

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

- Discussion

  - RRC configuration

    - Modular design

Motivations and issues

Last RAN2 meeting modular design has been discussed, however different companies may have different understanding regarding the term of RRC modular. RAN2 should firstly reach a consensus on the understanding of this term. Therefore, it is necessary to introduce the related background. According to ASN.1 specification (i.e., ITU-T Rec. X.680 and X.681), ASN.1 module is the fundamental unit of ASN.1. The sole purpose of an ASN.1 module is to name a collection of type definitions and/or value definitions (assignments) that constitute a data specification. There are two approaches to give modular design in existing 5G specification.

Approach 1: Use the “IMPORTS” keyword of ASN.1 to import the IEs defined in another module into current module, and the imported IEs are directly used in current module

An example of RRC specification defines the ASN.1 module NR-RRC-Definitions as below:

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

...

END

In the NR RRC specification [2], 6 ASN.1 modules are defined. The NR-RRC-Definitions module is the main module. The other 5 ASN.1 modules import some IEs from the NR-RRC-Definitions module, and also directly use the imported IEs in the 5 modules.

Figure 1: ASN.1 modules for RRC

Approach 2: The field is defined as the type of OCTET STRING in current module. If needed, further decode the field.

The SLPP specification [3] is regarded as having adopted a modular design. In the SLPP specification, 8 ASN.1 modules are defined. The SLPP-PDU-Definitions module defines the overall structures of SLPP messages, and the SLPP-PDU-CommonContents module defines the common IEs. 5 modules separately define the IEs related to a positioning method. The last NR-DiscoveryMessageMetaDataContents module defines the content specified by SA2.

Figure 2: ASN.1 modules for SLPP

The import relations between modules are shown in the following table:

Table 1: Import relations between modules

| ASN.1 modules | Import IEs from | Whether imported IEs are directly used or not? |
| --- | --- | --- |
| SLPP-PDU-Definitions | SLPP-PDU-CommonContents SLPP-PDU-CommonSL-PRS-MethodsContents SLPP-PDU-SL-AoA-Contents SLPP-PDU-SL-RTT-Contents SLPP-PDU-SL-TDOA-Contents SLPP-PDU-SL-TOA-Contents | No |
| SLPP-PDU-CommonContents | SLPP-PDU-Definitions | Yes |
| SLPP-PDU-CommonSL-PRS-MethodsContents | SLPP-PDU-Definitions SLPP-PDU-CommonContents | Yes |
| SLPP-PDU-SL-AoA-Contents | SLPP-PDU-Definitions SLPP-PDU-CommonContents | Yes |
| SLPP-PDU-SL-RTT-Contents | SLPP-PDU-Definitions SLPP-PDU-CommonContents | Yes |
| SLPP-PDU-SL-TDOA-Contents | SLPP-PDU-Definitions SLPP-PDU-CommonContents | Yes |
| SLPP-PDU-SL-TOA-Contents | SLPP-PDU-Definitions SLPP-PDU-CommonContents | Yes |
| NR-DiscoveryMessageMetaDataContents |  | N/A |

Although the SLPP-PDU-Definitions module imports the IEs from the other modules, but the imported IEs are not directly used. Instead, the corresponding fields are defined as the type of OCTET STRING. For example:

ProvideCapabilities-IEs ::= SEQUENCE {

commonIEsProvideCapabilities                  OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideCapabilities

commonSL-PRS-MethodsIEsProvideCapabilities    OCTET STRING    OPTIONAL, -- Containing CommonSL-PRS-MethodsIEsProvideCapabilities

sl-AoA-ProvideCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-AoA-ProvideCapabilities

sl-RTT-ProvideCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-RTT-ProvideCapabilities

sl-TDOA-ProvideCapabilities                   OCTET STRING    OPTIONAL, -- Containing SL-TDOA-ProvideCapabilities

sl-TOA-ProvideCapabilities                    OCTET STRING    OPTIONAL, -- Containing SL-TOA-ProvideCapabilities

lateNonCriticalExtension                      OCTET STRING    OPTIONAL,

nonCriticalExtension                          SEQUENCE {}     OPTIONAL

}

In fact, the import in the SLPP-PDU-Definitions module are unnecessary from the view of ASN.1 syntax, but it might be convenient for product developers. The above design in SLPP can let product developer easily extract the necessary ASN.1 codes and get rid of the unnecessary ASN.1 codes corresponding to the unsupported positioning method, i.e., if the UE does not support a positioning method, the corresponding module is not included and the corresponding import part from the module is removed from the SLPP-PDU-Definitions module. The following description are extracted from SLPP specification:

NOTE 1: An implementation needs to include only the supported "Method" PDUs. Not supported methods do not need to be included. For example, if SL-RTT is not supported by an implementation, the SLPP-PDU-SL-RTT-Contents PDU does not need to be included in the protocol. NOTE 2: An implementation supporting SL-RTT, SL-AoA, SL-TDOA, or SL-TOA must also support the SLPP-PDU-CommonSL-PRS-MethodsContents PDU.

To promote the progress of the discussion, RAN2 should firstly discuss the motivation for RRC modular, that is, the goals and benefits to be achieved. Different UE/device types may implement different features. If product developer can extract the necessary ASN.1 codes and eliminate the unnecessary ASN.1 codes corresponding to the unsupported features, it is conducive to the efficient implementation of memory size, especially for cost-limited devices, e.g., memory footprint related to ASN.1 can be smaller for IoT devices. Also, the UE complexity is decreased. Therefore, we think that a motivation of RRC modular is that product developer can easily extract the ASN.1 codes corresponding to the supported features by the UE, i.e., facilitate the ASN.1 codes trimming corresponding to the unsupported features. SLPP specification is an existing example for the motivation. For SLPP, the parameters related to each positioning method is put into a separate module. If the UE does not support a positioning method, the corresponding module is not included.

In addition, the existing RRC specification has already implemented the modular design for different pairs of communication objects, e.g., NR-RRC-Definitions module between UE and gNB, PC5-RRC-Definitions module between two UEs, and NR-InterNodeDefinitions module between two gNBs. We think that the motivation remains valid for 6G.

Another motivation may be the improvement of the readability of the specification, i.e., put the parameters related to a same purpose together. The NR-UE-Variables module, NR-Sidelink-Preconf module and NR-Sidelink-DiscoveryMessage module are the examples.

Observation 1: Motivations of RRC modular to be achieved at least support the following the goals and benefits:

- Motivation 1: Product developer can easily extract the ASN.1 codes corresponding to the supported features by the UE, i.e., facilitate the ASN.1 codes trimming corresponding to the unsupported features, which is conducive to the reduction of memory footprint, especially for cost-limited devices.

- Motivation 2: Different modules for different pairs of communication objects.

- Motivation 3: The improvement of the readability of the specification.

Based on the above background and motivation, we think that the modular discussed in RAN2 for 6G should fucus on the modular of ASN.1 codes in the RRC specification. In details, RAN2 should discuss the following two issues:

- Issue 1: the rules of how to divide ASN.1 codes into different modules?

- Issue 2: the principles of how to relate the modules (directly use the imported IEs from other modules, or the corresponding fields are defined as the type of OCTET STRING)?

Proposal 1: The modular discussed in RAN2 should fucus on the modular of ASN.1 codes in the RRC specification. RAN2 to discuss the following two issues:

- Issue 1: the rules of how to divide ASN.1 codes into different modules?

- Issue 2: the principles of how to relate the modules (directly use the imported IEs from other modules, or the corresponding fields are defined as the type of OCTET STRING)?

The rules of module division

For Issue 1, we think that how to divide ASN.1 codes into different modules mainly relates to the motivations of RRC modular. For Motivation 1, we understand that we can draw on the practices of SLPP. Among the 8 ASN.1 modules in SLPP specification, there are two basic/common/mandatory modules: the SLPP-PDU-Definitions module defines the overall structures of SLPP messages, and the SLPP-PDU-CommonContents module defines the common IEs. 5 additional optional modules separately define the IEs related to a positioning method. The last NR-DiscoveryMessageMetaDataContents module defines the content specified by SA2. Similarly, for RRC, the modules can be divided with feature dimension, i.e., one/several basic/common/mandatory modules + several additional optional modules. The additional optional modules may be vertical-specific/use-case-specific/device-type specific and can be discussed case by case. Note that whether an IE is put in common module or an optional module needs to consider the future-proof, which can reduce the IE duplication issue.

For Motivation 2, similar to the existing practices, the modules can be divided with the dimension of pair of communication objects.

For Motivation 3, besides the existing practices, we think that RAN2 can further consider the dimension of work group/protocol layer, e.g., physical configuration as a module. FFS for PDCP/RLC/MAC. Physical layer parameters are quite complex. RAN2 experts would spend a lot of time and efforts to sufficiently understand physical layer parameters. If physical configuration is designed as a module, it is desirable that RAN1 themselves maintain the parameters. RAN2 can discuss the possibility that physical configuration module is maintained by RAN1.

Proposal 2: study the possible rules for module division, including:

- Rule1: feature dimension, i.e., one/several basic/common/mandatory modules + several additional optional modules. The additional optional modules may be vertical-specific/use-case-specific/device-type specific and can be discussed case by case.

- Rule2: dimension of pair of communication objects.

- Rule3: dimension of different WG. Further consider work group/protocol layer division, e.g., physical configuration as a separate module. FFS for PDCP/RLC/MAC. Discuss the possibility that physical configuration module is maintained by RAN1.

The principles of module relation

For Issue2, there may be an association relationship between two modules. For example, a module needs to uses the IE defined in another module. As described in the above, there are two possible approaches:

- Approach 1: Use the “IMPORTS” keyword of ASN.1 to import the IEs defined in another module into current module, and the imported IEs are directly used in current module.

- Approach 2: The field is defined as the type of OCTET STRING in current module. If needed, further decode the field.

Approach 1 is simple, but introduces coupling between the two modules. Approach 2 avoids the coupling, but requires encoding/decoding twice and also increase a little additional signalling overhead. We think that the key point is how an implementation can easily extract only the relevant ASN.1 parts. The ideal scenario is only extracting the related modules. Therefore, we need to reduce the unnecessary coupling between models as much as possible. We think that RAN2 can consider the following principles:

- The IEs from a basic/common/mandatory module can be directly used by other modules after importing.

- The IEs from an optional module cannot be directly used by other modules. Instead, the corresponding field in other modules can be defined as the type of OCTET STRING. If needed, the UE further decodes the OCTET STRING.

As known, RRC specification uses the unaligned PER encoding rule. Generally, the field with the OCTET STRING type is quite long, even up to thousand bytes. The encoding and decoding efficiency for the OCTET STRING may be quite low in the vast majority of cases due to the large number of bit operations introduced by byte unalignment. If the OCTET STRING type is widely used, RAN2 can study the improvement of ASN.1 encoding and decoding efficiency for the OCTET STRING.

Proposal 3: RAN2 to study the improvement of ASN.1 encoding and decoding efficiency for OCTET STRING and consider the following principles:

- The IEs from a basic/common/mandatory module can be directly used by other modules after importing.

- The IEs from an optional module cannot be directly used by other modules. Instead, the corresponding field in other modules can be defined as the type of OCTET STRING. If needed, the UE further decodes the OCTET STRING.

    - Delta configuration

The primary intent of delta configuration is to reduce air interface signalling overhead and improve transmission efficiency. According to last RAN2#131bis meeting discussion, the current LTE/NR mechanism has several issues that limit its effective use in real networks and they are expected to be addressed in 6GR design. With all of those issues resolved, it’s deemed that delta configuration is still an effective method and thus will be inherited to 6GR RRC configuration framework. In this section, we will further discuss the potential use cases for delta configuration in 6GR design.

Use case#1: Supporting delta configuration for broadcast system information, e.g., SIB1

For 6GR, given that 6GR design aiming to provide initial access and its services to diverse device types in day-1, SIB1 size becomes a very critical issue and efficient system information delivery needs to be studied. To mitigate SIB1 size overhead, we should avoid two extreme approaches within a single cell i.e., either providing a single SIB1 type for all device types, or one SIB1 type per device type. Alternatively, a potential consideration is that full SIB1 contents broadcasted by the cell can be re-organized into Common configuration and Additional configuration, wherein the former one is for all device types (including eMBB and 6G IoT devices) with a large number of repetition transmissions, and the latter one is for eMBB device type only without repetition transmissions. Additional configuration uses delta signalling with reference to the Common configuration, which may include same information elements/parameters with different values or lists, and/or different information elements/parameters. From UE perspective, the 6G IoT devices simply rely on Common configuration to determine the full SIB1, while the 6G eMBB devices combine Common configuration and Additional configuration via delta configuration method to determine the full SIB1. This approach significantly reduces signalling redundancy and overhead for SIB1 design. Details can refer to another contribution in agenda 10.3.2.3 [4].

Observation 2: Delta configuration method is beneficial to reduce SIB1 overhead e.g., Common SIB1 configuration is applied for all device types (with a large number of repetition transmissions), and Additional SIB1 configuration for specific device type, use delta signalling based on Common SIB1 (with no repetition transmissions).

Use case#2: Supporting delta configuration between SIB1 and dedicated signalling

The current RRC connection control mechanism requires the network to transmit the full RRC configuration whenever a UE transits from RRC Idle to Connected state. This results in significant signalling overhead, as it redundantly retransmits information elements/parameters which are already broadcasted in SIB. This inefficiency is especially problematic when delivering configuration updates to a large number of UEs. A more efficient 6G approach should employ a delta configuration method. In scenarios where SIB parameters (e.g., L1 configurations including carrier frequency and BWP etc) remain applicable, only the modified information would be transmitted to a UE. For example, multiple configuration profiles are broadcasted within SIB. Upon RRC state transition, the UE receives a profile identifier to identify the basic configuration and just a small set of delta configuration for UE-specific adjustments. This method ensures optimal overhead reduction by signalling only the profile identifier and the necessary deviations.

Observation 3: Delta configuration method is beneficial in scenarios where the typical information elements/parameters’ value range acquired in SIB retains for a UE transiting to RRC Connected from RRC Idle.

Use case#3: Supporting delta configuration for dedicated signalling

At last RAN2#131bis meeting, there is an FFS issue regarding whether to introduce improvements to delta signalling or no delta signalling in inter-node RRC reconfiguration scenarios. In our understanding, the delta configuration method remains the most effective solution for mitigating signalling overhead. Rather than turning to the full configuration method, the focus should be on resolving the root causes of issues associated with delta signalling. For instance, as discussed in above Subclause 2.1.1, feature-based modular configuration framework could be studied. This would enable a target node to release features it does not support, which were previously configured by the source node to the UE. This approach would prevent the need for a full configuration, allowing delta configuration to be established as the baseline for inter-node reconfiguration.

Observation 4: Delta configuration is the most effective way to reduce signalling overhead and should be the baseline in inter-node reconfiguration scenarios instead of relying on full configuration.

- Proposal 4: Delta configuration study in 6GR considers to support the following use cases:

- Use case#1: Supporting delta configuration for broadcast system information, e.g, SIB1

- Use case#2: Supporting delta configuration between SIB1 and dedicated signalling

- Use case#3: Supporting delta configuration for dedicated signalling for inter-node reconfiguration

  - RRC reconfiguration failure

In NR, RRC reconfiguration procedure is used to modify an RRC connection. Although a UE is generally expected to apply the RRC Reconfiguration message immediately upon receipt, in some error cases (e.g., the new configuration exceeds the UE’s capability), the UE may be unable to comply with (part of) the received configuration. In such situations, the network and the UE may end up applying different configurations, which prevents subsequent communication. To handle these error cases, NR introduces a baseline solution: the UE triggers the RRC Connection Re-establishment procedure if its Access Stratum (AS) security is activated, or goes to RRC Idle if AS security is not yet activated.

Observation 5: Upon reception of an RRCReconfiguration message that cannot be applied, the UE takes the following baseline failure handling action for normal reconfiguration failure case in NR.

- If the UE's Access Stratum (AS) security has been activated, the RRC Connection Re-establishment procedure is triggered.

- Otherwise, the UE transits to RRC_IDLE.

We think the NR failure handling action could be reused in 6G to recover the link between UE and the network after RRC reconfiguration failure. However, because the RRC Connection Re-establishment procedure inevitably causes a communication interruption, recovery solution without triggering RRC Connection Re-establishment procedure should also be studied.

Proposal 5: For handling 6G RRC Reconfiguration failure, we can start with triggering the RRC Connection Re-establishment procedure. However, to minimize communication interruption, we should also investigate recovery solutions that avoid RRC Connection Re-establishment.

Several enhancements have been specified in NR to avoid it on a case-by-case basis. The specific involved cases are listed below：

Exceptional Case 1 in NR: UE cannot apply Cell Specific configuration fields in ServingCellConfigCommon

In NR, the System Information provides cell-specific configurations that apply to all UEs in the RRC_IDLE state, regardless of the UE's radio capabilities. However, when configuring a UE with an SCell addition, the network delivers these same cell-specific configurations for the SCell via dedicated signalling (e.g., using the ServingCellConfigCommon IE within an RRCReconfiguration message). These configurations may include fields that are not supported or not comprehended by the UE, meaning the network does not have to strictly respect the reported UE capabilities when generating the configuration. To prevent the triggering of a connection re-establishment, the NR protocol specifies a list of IEs(e.g. fields in ServingCellConfigCommon) that, if not understood by the UE, should simply be ignored by the UE.

Observation 6: The NR protocol lists certain cell-specific IEs (e.g., fields within ServingCellConfigCommon), and if an IE in the received RRCReconfiguration message UE failed to comprehend is in the list , UE disregards it without triggering a RRC Connection re-establishment procedure.

Exceptional Case 2 in NR: UE cannot apply SCG configuration received via SRB3

In NR, SRB3 can be utilized to convey an RRCReconfiguration message that exclusively contains SCG configuration. Consequently, the UE cannot apply the RRCReconfiguration message received via SRB3 will not lead to a configuration mismatch between the UE and the MCG. The UE can therefore report the SCG failure via the MCG, notifying the network of the reconfiguration error and avoiding the triggering of a re-establishment procedure.

Observation 7: In NR, when the UE is unable to apply an RRCReconfiguration message received via SRB3, it reports an SCG failure via the MCG, rather than triggering a RRC Connection re-establishment procedure.

In Exceptional Cases 1 and 2, a UE's inability to comprehend or apply the parameters within the RRC configuration will not result in a communication configuration mismatch between the UE and the MCG. Therefore, the RRC Connection Re-establishment procedure is unnecessary. It is a more efficient approach for the UE to either directly ignore the not comprehended IEs or trigger a report to the network indicating the configuration error. We therefore recommend that this mechanism be inherited in 6G design.

Proposal 6: During 6G RRC Reconfiguration, if UE's inability to comprehend or apply the parameters within a received RRC Reconfiguration message will not result in a communication configuration mismatch between the UE and the MCG, RRC Connection Re-establishment needs not to be triggered.

Exceptional Case 3 in NR: UE cannot apply configurations due to UE’s temporary capability restriction

In Multi-USIM scenario, the activity of a UE serving USIM1 may impose a temporary capability restriction on the UE serving USIM2 (hereinafter UE2). Since the network is unaware of these temporary restrictions on UE2, it may subsequently transmit an RRCReconfiguration message that exceeds UE2's current temporary capability.

In this case, according to the NR specification, the UE2 does not trigger RRC Connection Re-establishment; instead, the UE provides MUSIM assistance information regarding the temporary capability restriction via a UAI message and starts timer T348. The UE will stop T348 upon successfully receiving a subsequent reconfiguration that respects the temporary capability restriction if T348 is running. If T348 expires, the UE could apply the temporary UE capability restriction that was indicated in the last transmission of the UAI message.

Observation 8: In NR, when a UE receives an RRCReconfiguration message that cannot be applied due to the MUSIM temporary capability restriction, no matter the RRCReconfiguration message will cause configuration mismatch between the UE and MCG or not, the UE sends the temporary capability restriction information to the network, rather than triggering RRC Connection Re-establishment.

New Exceptional Cases can be considered in 6G

In addition to addressing configurations that exceed the MUSIM temporary capability restriction, many companies are proposing the introduction of dynamic UE capability change in 6G. The occurrence of issues where reconfigurations exceed temporary capability restrictions during the RRC Reconfiguration procedure is likely to increase.

What’s more, a more generic issue is the configurations in RRCReconfiguration exceed UE’s static capabilities. RAN2 is considering that modular design principle applies all RRC structures, modular RRC structure will give more robustness RRC reconfiguration, i.e., easily extract the ASN.1 codes corresponding to the supported features by the UE and exclude the ASN.1 codes corresponding to the unsupported features.

Hence, we recommend for 6G RRC Reconfiguration, solutions should be developed to handle the cases where configurations within the RRC Reconfiguration message that exceed UE capabilities, aims to further reduce the case where RRC Connection Re-establishment is mandatory to be applied. With existing NR mechanism for MUSIM should be utilized as the starting point for this development.

Proposal 7: Develop a solution for 6G RRC Reconfiguration failure that avoids RRC Connection Re-establishment, even when the failure causes a communication configuration mismatch between the UE and the MCG. The existing NR MUSIM mechanism can serve as the starting point for this development.

- Conclusion

In this paper, we discuss the detailed RRC configuration aspects, i.e. modular design, delta configuration and reconfiguration failure. And we have the following observations and proposals:

Modular design

Observation 1: Motivations of RRC modular to be achieved at least support the following the goals and benefits:

- Motivation 1: Product developer can easily extract the ASN.1 codes corresponding to the supported features by the UE, i.e., facilitate the ASN.1 codes trimming corresponding to the unsupported features, which is conducive to the reduction of memory footprint, especially for cost-limited devices.

- Motivation 2: Different modules for different pairs of communication objects.

- Motivation 3: The improvement of the readability of the specification.

Proposal 1: The modular discussed in RAN2 should fucus on the modular of ASN.1 codes in the RRC specification. RAN2 to discuss the following two issues:

- Issue 1: the rules of how to divide ASN.1 codes into different modules?

- Issue 2: the principles of how to relate the modules (directly use the imported IEs from other modules, or the corresponding fields are defined as the type of OCTET STRING)?

Proposal 2: study the possible rules for module division, including:

- Rule1: feature dimension, i.e., one/several basic/common/mandatory modules + several additional optional modules. The additional optional modules may be vertical-specific/use-case-specific/device-type specific and can be discussed case by case.

- Rule2: dimension of pair of communication objects.

- Rule3: dimension of different WG. Further consider work group/protocol layer division, e.g., physical configuration as a separate module. FFS for PDCP/RLC/MAC. Discuss the possibility that physical configuration module is maintained by RAN1.

Proposal 3: RAN2 to study the improvement of ASN.1 encoding and decoding efficiency for OCTET STRING and consider the following principles:

- The IEs from a basic/common/mandatory module can be directly used by other modules after importing.

- The IEs from an optional module cannot be directly used by other modules. Instead, the corresponding field in other modules can be defined as the type of OCTET STRING. If needed, the UE further decodes the OCTET STRING.

Delta configuration

Observation 2: Delta configuration method is beneficial to reduce SIB1 overhead e.g., Common SIB1 configuration is applied for all device types (with a large number of repetition transmissions), and Additional SIB1 configuration for specific device type, use delta signalling based on Common SIB1 (with no repetition transmissions).

Observation 3: Delta configuration method is beneficial in scenarios where the typical information elements/parameters’ value range acquired in SIB retains for a UE transiting to RRC Connected from RRC Idle.

Observation 4: Delta configuration is the most effective way to reduce signalling overhead and should be the baseline in inter-node reconfiguration scenarios instead of relying on full configuration.

- Proposal 4: Delta configuration study in 6GR considers to support the following use cases:

- Use case#1: Supporting delta configuration for broadcast system information, e.g, SIB1

- Use case#2: Supporting delta configuration between SIB1 and dedicated signalling

- Use case#3: Supporting delta configuration for dedicated signalling for inter-node reconfiguration

RRC reconfiguration failure

Observation 5: Upon reception of an RRCReconfiguration message that cannot be applied, the UE takes the following baseline failure handling action for normal reconfiguration failure case in NR:

- If the UE's Access Stratum (AS) security has been activated, the RRC Connection Re-establishment procedure is triggered.

- Otherwise, the UE transits to RRC_IDLE.

Proposal 5: For handling 6G RRC Reconfiguration failure, we can start with triggering the RRC Connection Re-establishment procedure. However, to minimize communication interruption, we should also investigate recovery solutions that avoid RRC Connection Re-establishment.

Observation 6: The NR protocol lists certain cell-specific IEs (e.g., fields within ServingCellConfigCommon), and if an IE in the received RRCReconfiguration message UE failed to comprehend is in the list , UE disregards it without triggering a RRC Connection re-establishment procedure.

Observation 7: In NR, when the UE is unable to apply an RRCReconfiguration message received via SRB3, it reports an SCG failure via the MCG, rather than triggering a RRC Connection re-establishment procedure.

Proposal 6: During 6G RRC Reconfiguration, if UE's inability to comprehend or apply the parameters within a received RRC Reconfiguration message will not result in a communication configuration mismatch between the UE and the MCG, RRC Connection Re-establishment needs not to be triggered.

Observation 8: In NR, when a UE receives an RRCReconfiguration message that cannot be applied due to the MUSIM temporary capability restriction, no matter the RRCReconfiguration message will cause configuration mismatch between the UE and MCG or not, the UE sends the temporary capability restriction information to the network, rather than triggering RRC Connection Re-establishment.

Proposal 7: Develop a solution for 6G RRC Reconfiguration failure that avoids RRC Connection Re-establishment, even when the failure causes a communication configuration mismatch between the UE and the MCG. The existing NR MUSIM mechanism can serve as the starting point for this development.

- Reference

- RAN2#131bis Meeting Chair Minutes.

- 3GPP TS 38.331 V19.0.0.

- 3GPP TS 38.355 V19.0.0.

- R2-2508052	Consideration on Common Signal/Channel Design for 6GR		vivo.