3GPP TSG-RAN WG2 Meeting #132	R2-2508053

Dallas, USA, Nov. 17th – 21st, 2025

Source:	vivo

Title:	6GR Data transfer, model transfer and AIML

Agenda Item:	10.3.3.1

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, the following agreement is captured regarding the data collection framework in 6G.

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

In this contribution, we will discuss the requirements of data transfer for various use cases (e.g., AI/ML, sensing, MDT, QoE), and further analyse the gap with the solutions based on the Radio Protocol Architecture of NR. This contribution will also discuss AI/ML functional framework and Unified UE AI/ML Processing Capability.

- Discussion

  - Use cases and requirements for data transfer

    - General

The 6G data collection framework includes both data collection and data transfer. In the last meeting, RAN2 lists several requirement aspects that need to be considered for data transfer.

- For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

On top of these, we believe there are additional aspects that should also be taken into account:

- For data transfer between UE and network, data transfer continuity should be considered for mobility scenarios.

- In the case of large volume data collection or transmission, data compression should be considered to reduce overhead of air interface.

As for UE related data transfer continuity, service continuity should be an important requirement. In the case that data transfer occurs between UE and network (e.g., RAN node, CN), service continuity should be guaranteed during mobility. That is, the unfinished data can be further transferred to the target node after handover. In addition, for some cases, the lossless transfer during handover should be ensured if the use case cannot tolerate any packet loss to damage the integrity of the entire dataset. These transmission attributes respectively correspond to the processing of AM and UM data during handover. The specific continuity requirements for each use case will be discussed in subsequent sections. In conclusion, data transfer continuity, including continuity during mobility and lossless transfer, should be considered in 6G data collection/transfer framework design.

Data compression is directly linked to data size requirements. Its core value lies in adapting to the needs of controlling data volume in different scenarios through algorithm optimization, laying an efficiency foundation for subsequent storage and transfer processes. Taking CSI prediction in AI/ML as an example, UE-sided model training data is based on channel estimation results of CSI-RS, i.e., frequency-domain channel impulse response. The channel estimation results based on CSI-RS are also potential sensing data. Therefore, we conducted a preliminary evaluation of compression gain using the channel estimation results of CSI-RS. As shown in Figure 2.1-1, adaptive quantization can be used to reduce the number of quantization bits according to the needs of data consumers, thereby achieving data compression. Additionally, entropy coding (such as Huffman coding) can be adopted to further compress data. In scenarios where the buffer size is fixed, data compression can significantly improve storage capacity. The volume of compressed data is reduced, allowing the same buffer space to accommodate more original data, and indirectly alleviating the pressure of storage resource occupation. Compression can also effectively reduce the load of data transfer, including the data volume of a single report or the total size of overall data. Therefore, when storage or available spectrum resources are limited, the value of on-demand data compression becomes particularly prominent. On-demand data compression can greatly reduce the consumption of resources such as buffers size and spectrum resources during data collection and transfer.

Figure 2.1-1: Evaluation results of data compression

Proposal 1: The requirements of data collection framework shall consider the following two additional aspects:

- Continuity and lossless transfer during mobility for data transfer between UE and network.

- Data compression, including downlink/uplink lossless and lossy data compression.

    - AI/ML

      - Data collection and dataset sharing for model training

In Rel-18 and Rel-19 of NR, the data collection and dataset sharing for model training was studied, and the alternatives have been captured in TR 38.843 [1]. The data collection includes network-side data collection for network-side model training and UE-side data collection for UE-side model training.

Currently, the NR only considers offline model training; therefore, there are no latency requirements in terms of QoS requirements, allowing for low-priority transmission. Additionally, supporting data transfer continuity during handover can avoid a decline in data collection efficiency due to discarding of collected data. However, if each data packet can be decoded independently, a certain amount of packet loss can be tolerated during handover, meaning lossless data transfer during handover is optional. As for security and privacy, given that some data does not has security risks, integrity and ciphering should be optional to reduce data processing complexity, while privacy could be guarded via user consent mechanism.

Figure 2.1-2: RAN node-centric and OAM-centric NW-sided data collection

For network-side data collection, both RAN node (Option 1) and OAM-centric (Option 2) data collection mechanisms are supported, i.e., data collection can be initiated by RAN node or OAM, as illustrated in Figure 2.1.2. For both Option 1 and Option 2, the UE is the data producer. The OAM is the consumer in Option 2, while the RAN node can also use the data for model training. Due to limited UE AS storage, the total data reported by a single UE cannot exceed 64 kB (minimized UE capability in NR), and each report should not exceed 9 kB.

Figure 2.1-3: Potential options for UE-sided data collection

The potential options of UE-sided data collection are shown in Figure 2.1-3, including:

- Option 1: UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected;

- Option 2: UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server;

- Option 3: UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

In all the above three options, the UE is the data producer, and the OTT server acts as the data consumer. However, the termination points of network-side are different: the data transfer is transparent to network in Option 1; data terminates at the CN in Option 2, and at the OAM (via RAN node) in Option 3. If the data is visible to the network, the CN, RAN node, or OAM can also reuse this data for model training. From the perspective of a single UE, the total data size to be transferred depends on the use case and the number of UEs involved in data collection.

Figure 2.1-4: Potential options for dataset/model parameters sharing

The potential options of dataset/model parameters sharing for two-sided model are shown in Figure 2.1-4, including:

-	Option 1: Non-Over-The-Air (non-OTA) approach, the data transfer path is BS -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server);

-	Option 2: Over-The-Air (OTA) approach, the data transfer path is BS -> NW dataset/model parameters collection entity (if needed) ->BS -> UE -> UE training entity (a server inside MNO or an OTT server).

Since Option 1 does not involve RAN2, the discussion on data transmission requirements for Option 1 can be postponed for now. In Option 2, the RAN node is the producer and the OTT server is the consumer. Taking CSI compression as an example, the total data size is about 225 MB.

Based on the above analysis, the data transfer requirements of AI/ML for model training are shown in Table 2.1-1.

Table 2.1-1: Data transfer requirements of AI/ML data: dataset for model training

| Data type | Applicable use case | End point pairs |  |  | Typical data size | QoS requirements | Continuity | Security and privacy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Producer | Termination point in NW | Consumer |  |  |  |  |
| AI/ML data: dataset for model training | UE side data collection for UE-sided model training | UE | Option 1: None (transparent to NW) Option 2: CN Option 3: OAM (Data can be utilized by BS) | OTT server | Depends on use case and number of involved UE | Relaxed latency, low priority | Continuity during mobility is required, lossless transfer is optional. | Security via optional Integrity and Ciphering, Privacy via user consent |
|  | NW side data collection for NW-sided model training | UE | Option 1: gNB Option 2: OAM (Data can be utilized by BS) | Option 1: BS Option 2: OAM | per report<9KB Total <64 KB |  |  |  |
|  | Dataset sharing for two-sided (UE part) model training (OTA solution) | BS | / | OTT server | Total: 225 MB |  |  |  |

      - Model transfer

The data transfer requirements for AI/ML models with model parameter sharing in the LS [2] are shown in Table 2.1-2. Note that the lossless transfer is required as AI/ML model cannot tolerate any packet loss to damage the integrity of the entire model. For potential new model transfer in 6G, the detailed description will be discussed in Section 2.3. The data transfer requirements for AI/ML models may be iteratively updated along with the discussions on 6G AI/ML.

Table 2.1-2: Data transfer requirements of AI/ML data: model transfer

| Data type | Applicable use case | End point pairs |  |  | Typical data size | QoS requirements | Continuity | Security and privacy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Producer | Termination point in NW | Consumer |  |  |  |  |
| AI/ML data: model transfer | Model parameter sharing (OTA solution) | BS/CN/OAM | / | UE/OTT server | Total: 11.6 MB | Relaxed latency, low priority | Continuity and lossless transfer during mobility are required | Security via Integrity and Ciphering |

    - Sensing

Twenty-four sensing use cases (UCs) are captured in the SA1 requirements TR22. 870 [3], such as low-altitude unmanned aerial vehicle (UAV) supervision, environment object reconstruction, sensing assisted communication in industrial park, etc. There are six potential sensing modes as shown in Figure 2.1-5.

Figure 2.1-5: Potential sensing modes

In TS 22.137 [4], the basic procedure of sensing has been described, including capability reporting, data collection, data transmission, data processing, and capability exposure. The data transfer requirements of sensing are shown in Table 2.1-3, and details on data size determination can be found in Appendix A.

Table 2.1-3: Data transfer requirements of sensing

| Data type | Applicable use case | End point pairs |  |  | Typical data size | QoS requirements | Continuity | Security and privacy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Producer | Termination point in NW | Consumer |  |  |  |  |
| Sensing raw data | - Object detection and tracking - Environment monitoring - Motion monitoring - Sensing assisted communication | UE | BS1 | BS | 134 Mbits | Relaxed requirement on latency，priority，and packet error rate, except for gestures recognition whose max sensing service latency is about 5~50ms4. | Continuity during mobility is required, lossless transfer is needed. | Security via optional Integrity and Ciphering, Privacy via user consent |
|  |  | UE | BS2/CN | CN | 134 Mbits |  |  |  |
|  |  | BS | BS/CN | BS/CN | 51.5Gbits3 |  |  |  |
| Per Path Results |  | UE | BS1 | BS | 112 Kbits |  |  | User consent, encryption and integrity for security, measurement results only transmit to third party authorized |
|  |  | UE | BS2/CN | CN | 112 Kbits |  |  |  |
|  |  | BS | BS/CN | BS/CN | 1.34 Mbits |  |  |  |
| Sensing Results |  | UE | BS1 | BS | 1920 bits |  |  |  |
|  |  | UE | BS2/CN | CN, OTT server | 1920 bits |  |  |  |
|  |  | BS | BS/CN | BS, CN, OTT server | 1920 bits |  |  |  |

Note 1：For sensing assisted communication use cases, three types of sensing data can be terminated at BS, and are used by BS to assist communication.

Note 2：Considering that the same sensing data can be simultaneously used for sensing-assisted communication and CN sensing services, the data should be reused by the BS and the CN to avoid repeated collection.

Note 3: The data size is calculated based on the assumption that 12 TRPs serve as sensing receiver.

Note 4: The refreshing rate for gestures recognition is 0.1s in [4], and the corresponding data rate for gestures recognition is from 19.2 kbps to 515 Gbps, where the data rate is equal to data size / refreshing rate.

    - MDT and QoE

MDT and QoE are typical data collection schemes in NR, designed for collecting radio-related data and application-layer data, respectively. Both support data collection in connected and non-connected states. We believe that the 6G data collection framework should support these two use cases. The current status for these use cases is summarized in table 2.1-4. It is worth noting that QoE and MDT only support on-demand reporting mechanism. That is, the network requests the data as needed after the UE indicates data availability. The UE continues to indicate in the report whether more data is available, and the network may further request accordingly until all data has been transmitted. During handover, if there is an ongoing data reporting procedure, lossless transmission is not supported. After handover, the data transfer will not resume automatically. Instead, the network must re-initiate a data transfer request. Therefore, we understand that data transfer continuity during mobility is not supported.

Table 2.1-4: Typical data collection mechanisms in NR

| Data type | Applicable use case | End point pairs |  |  | Typical data size | QoS requirements | Continuity | Security and privacy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Producer | Termination point in NW | Consumer |  |  |  |  |
| MDT | Logged MDT | UE | OAM (Data can be utilized by BS) | OAM | per report<9KB Buffer <64 KB | Relaxed latency, transfer via SRB2 | Continuity and lossless transfer during mobility is not supported. | Security via Integrity and Ciphering of RRC, Privacy via user consent |
|  | Immediate MDT | UE | OAM (Data can be utilized by BS) | OAM | per report<9KB | Relaxed latency, transfer via SRB1 |  |  |
| QoE | QoE in Connected mode | UE | OAM (Data can be utilized by BS) | OAM | per report<9KB Buffer <64 KB | Relaxed latency, transfer via SRB4/5 |  |  |
|  | QoE in Inactive mode | UE | OAM (Data can be utilized by BS) | OAM | per report<9KB Buffer <128 KB | Relaxed latency, transfer via SRB4/5 |  |  |

    - Summary of data transfer requirements

Based on the above analysis, the standardized data collection framework shall be designed to accommodate the full spectrum of data types and their transmission characteristics.

Proposal 2: The data transfer of the data collection framework shall support various data types with the following transmission requirements:

- Data types: AI/ML dataset, AI/ML model, sensing, MDT, and QoE.

- End point pairs: including UE-BS, UE-OAM, UE-CN, UE-OTT server, BS-BS, BS-CN, and BS-OTT server.

- Data size: including scalability for large-volume data (i.e., large single report or large total data size) and impact of data compression on data size.

- QoS requirements: including high data rate, relaxed latency, or flexible priority.

- On-demand continuity and/or lossless transfer during mobility.

- Security: optional integrity and ciphering.

- Privacy should be ensured via user consent.

  - Gap analysis for data transfer

Based on the 6G data framework requirements outlined in Section 2.1.5 and the Radio Protocol Architecture of NR, the gaps of the existing control plane (CP) and user plane (UP) are analysed in Table 2.2-1 and Table 2.2-2.

Table 2.2-1: Analysis of gaps for the existing CP

| Requirement aspect | Status in NR and corresponding Gaps |
| --- | --- |
| 1. Large data volume and data compression | Current status: - A single NR RRC message size cannot exceed 9KB. Thus, RRC or higher layer segmentation is needed to transfer data that cannot fit in a single RRC message. NR RRC message only supports up to 5 segments in DL and 16 segments in UL, which amounts to a maximum of 45 KB of downlink data size and 144 KB of uplink data size. - UDC is not supported for SRB. Gaps: - If data is transferred via RRC: The RRC segmentation in NR does not support downlink data exceeding 45 KB or uplink data exceeding 144 KB. - Data compression is not supported. |
| 2. QoS requirement | Current status: - NR RRC message can be transferred via SRBs with different QoS settings (e.g., SRB 1, 2, 4), while NAS messages are transmitted via SRB2 (after SRB2 has been established). The QoS settings (e.g., priority) of SRB2 is common, meaning it applies to all messages transmitted via SRB2. Gaps: - If data is transferred via NAS: Flexible settings (e.g., priority) is not supported, which will have an impact on the other signaling transmission. |
| 3. Flexible termination | Current status: - RRC operates between the UE and the gNB, and NAS operates between the UE and the CN entity. Gaps: No Gaps. |
| 4. Continuity and lossless transfer during mobility | Current status: - During mobility in RRC_CONNECTED, NR only supports UP data continuity with lossless transfer. Specifically, a data forwarding tunnel can be established between the source and target nodes. However, for CP transmission, if messages are not fully transferred before handover, the transmission will be stopped and new transmission can be restarted after mobility completion. Gaps: - Continuity and lossless transfer during mobility is not supported. |
| 5. On-demand security | Current status: - Integrity protection and integrity verification are mandatorily configured for signaling over SRB. Gaps: - On-demand security is not supported. |

Table 2.2-2: Analysis of gaps for the existing UP

| Requirement aspect | Current status and Gaps |
| --- | --- |
| 1. Large data volume and data compression | Current status: - The upper layer (e.g., IP) ensures the PDCP SDU does not exceed 9KB. - Only UL lossless compression (i.e., UDC) is supported in PDCP. Gaps: - DL Lossless data compression is not supported. - Lossy compression is not supported. |
| 2. QoS requirement | Current status: - Priority settings can be flexible among different DRBs. Gaps: No Gaps |
| 3. Flexible termination | Current status: - NR user plane is point-to-point transmission and is transparent to gNB. - PDU session terminated at gNB is not supported. Gaps: - Data transfer terminated at gNB is not supported. |
| 4. Continuity and lossless transfer during mobility | Current status: - During mobility in RRC_CONNECTED, a data forwarding tunnel can be established between the source and target nodes. Gaps: No Gaps |
| 5. On-demand security | Current status: - Integrity protection can be optionally configured per DRB. Gaps: No Gaps |

It can be observed in Table 2.2-1 and Table 2.2-2 that the data transfer schemes based on the existing NR Radio Protocol Architecture (including CP and UP) cannot fulfil the requirements of the 6G data framework.

Observation 1: The potential data transfer schemes based on the existing NR Radio Protocol Architecture (including CP and UP) cannot fulfil the requirements of the 6G data framework:

- For CP-based solution: RRC is not suitable for transmitting large-size data due to the limited segmentation number; Flexible priority setting for NAS transmission, data transfer continuity during mobility, data compression and on-demand security is not supported.

- For UP-based solution: Data transfer terminated at gNB is not supported; DL Lossless and lossy data compression is not supported.

Proposal 3: Study 6G Radio Protocol Architecture to address the data transfer requirements: address the identified gaps of NR CP/UP protocols, and explore the feasibility of introducing a new dedicated plane.

  - Requirements for model transfer

    - Case 1: Model download due to model unavailability at UE side

5G AI/ML has standardized at least three AI/ML features or use cases. Based on current discussions in RAN1 and potential AI/ML features from RAN2, 6G AI/ML is expected to introduce no fewer than three AI/ML features or use cases. From a practical deployment perspective, it is feasible to train and maintain multiple AI/ML models per feature or use case. However, due to limited UE storage memory, the UE may not be able to simultaneously store models for all or many supported AI/ML features.

RAN2#129bis 5G AI/ML Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

In the 5G AI/ML BM discussions, RAN2 agreed that the UE can report "inapplicable" for functionality applicability due to model inapplicability.

At the RAN2#131bis meeting, RAN2 reached the following 6G AI/ML LCM agreement:

RAN2#131bis Agreements on LCM: Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

If, after reporting inapplicability due to model unavailability, the UE can instantly download the model or the NW transfers the model to the UE, the UE would be able to apply the corresponding AI/ML feature(s). However, it may not be practical for the UE to directly download the model from a server, e.g., due to download delay. Therefore, it should be allowed for the NW to transfer the model to the UE when needed.

Observation 2: Due to limited UE storage and download latency, the UE may not be able to store/download all AI supported models in advance, thus on-demand model transfer from RAN/CN to the UE should be allowed.

    - Case 2: Site specific Finetuning

In 6G we find the more cases for model transfer requirements due to Site specific Finetuning. Both site-specific model and finetuning were considered during the 5G AI/ML discussions, as described in [1]. Based on current 6G AI study, potential use cases for site-specific finetuned/online finetuned model and can be categorized into the following ways:

- UE-sided model, e.g., tokenized CSI prediction, long-term channel information (LT-CI) aided CSI prediction/reconstruction, beam prediction, positioning;

- NW-sided model, e.g., positioning, interacted NW-UE side beam prediction

- UE/NW part model of two-sided cases, e.g., CSI feedback, RAN Digital Twin.

To further illustrate the above categories, we present three examples to demonstrate how these use cases work

- Take “joint source and channel coding/modulation (JSCC/JSCCM) for CSI feedback (for UE/NW part of two-sided model):

- A linear projection matrix is used as the UE-sided model, which can be transferred from the NW to the UE via model transfer.

- Alternatively, dataset transfer can be employed. UE finetunes its model using a dataset transferred from NW. Consider 10K samples of small finetuning datasets, and assuming eType-II-like format of a size 2000 bits for encoder input and 1000 bits for encoder output (as in R19 RAN1 agreements), the overhead of dataset transfer would be about 30M bits.

- Take “tokenized CSI prediction” as example for UE-sided model

- One approach uses a linear projection matrix pre-trained by the NW to enable CSI prediction, which is transferred to UE.

- Alternatively, the UE collects its own training data and trains the UE-sided model using the collected data.

For UE sided model and two-sided model, to further summarize based on above examples, as discussed in TR38.843, both on-device finetuning and model transfer/delivery can be used as solutions for site-specific finetuned models.

- On-device finetuning. An AI/ML training process where the model (used for inference) is (typically continuously) trained on the same device where inference occurs. he training dataset can be obtained via two methods:

- Dataset transfer. The counterpart entity (e.g., NW for UE-sided models) collects training data and transfers it to the device. The device then uses this dataset to finetune its model.

- On-device dataset collection. The device autonomously collects data (e.g., channel measurements) and finetunes its model using this locally gathered dataset.

- Model transfer. Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either as parameters of a model structure known at the receiving end or as a new model with parameters [1]. Model parameter update with known model structure is supported in two-sided CSI compression.

| Solution for finetuning | Pros | Cons |
| --- | --- | --- |
| On device dataset collection | Possible to train device specific models with collected data. OTA signaling overhead is low. | It would take minutes or even hours to collect enough data to finetune the model. It would require high computation power to finetune the model. |
| Dataset transfer | Possible to train device specific models with transferred data | Large OTA signaling overhead. High computation power/power consumption required for UE to finetune the model. |
| Model transfer | Low latency. Model transfer saves the time to collect the data and finetune the model. Low OTA signaling overhead (compared to dataset transfer). Compared with dataset transfer, the signaling overhead is low. Low power consumption | non-device specific. However, this should not be concern if the major use case is for projection matrix-based solution since this is already the most straightforward operation that can save UE from too much computation |

From above analysis, Model transfer-based finetuning is the preferred way for use cases with stringent low-latency requirements. We have the below observation.

Observation 3: Model transfer-based finetuning for site-specific model is the preferred way for use cases with stringent low-latency requirements.

Proposal 4: Study model transfer for UE sided and two-sided model for site-specific finetuning, e.g., “JSCC/JSCCM for CSI feedback for two-sided model and “tokenized CSI prediction” for UE-sided model.

Similar to collected data transfer, the size of an AI/ML model can vary significantly depending on the feature, e.g., ranging from a few MB to hundreds of MB. Therefore, the model transfer mechanism must support a wide range of model sizes.

The transferred model data shall be secured with integrity protection to prevent unauthorized or malicious models from being downloaded. Thus, both integrity verification and security of model transfer shall be ensured.

UE mobility may occur during model transfer. Events such as handover (HO) shall not interrupt the transfer process to the extent that the UE must restart the download. Otherwise, this would result in unnecessary resource waste and delays in model deployment. Therefore, service continuity of model transfer during UE mobility shall be guaranteed.

The MNO shall have full visibility and control over model transfers to the UE. At a minimum, this includes the ability to access, inspect, and understand the content of the transferred model data.

Therefore,

Proposal 5: The model transfer mechanism should satisfy the following requirements:

- Support various model sizes transfer

- Ensure service continuity of model transfer during UE mobility.

- Ensure integrity protection of model transfer.

- Provide the MNO with full visibility of model transfer, i.e., ability to access and understand the model data.

  - 6G AI/ML Framework

    - 6G AI/ML functional Framework

Based on the identified functions/procedures recognized in 5G, we try to further analyze the mutual interactions in between taking the functional framework in TR 38.843 as a start point.

Figure 2.4.1-1: 5G AI/ML functional framework

It is observed that some AI/ML LCM functions/procedures standardized in 5G are not directly reflected in the block diagram above, the interaction with other functions are also missed, which can cause some misunderstandings as an illuminating design guidance in 6G. To put it clearly, we identify the following improvements for a legible diagram:

- Most importantly, procedures like capability transfer and applicability determination are not indicated between the “management” function and the “inference” function in this diagram. However, capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical from the deployment entity’s perspective, especially for the case of UE-sided model.

Observation 4: UE capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical, especially for the case of UE-sided model.

- Regarding data collection, the training data and data used during inference and monitoring are treated and discussed separately in 5G, considering the different time-efficiency. To distinguish the stage at what types of data is used and the urgency of data requirements for a better discussion in future, we propose further dividing it into “non-real-time” for the “training” function and “real-time” for the “inference” function and the “management (e.g., monitoring)” functions.

Accordingly, we strongly suggest RAN2 revises the existing functional framework to incorporate the functions and procedures successfully specified during 5G phase as a starting point for 6G AI/ML framework. Figure 2.4.1-2 illustrates the fundamental enhancement from Figure 2.4.1-1.

Figure 2.4.1-2: suggested 6G AI/ML functional framework

Proposal 6: Capture 6G AI/ML functional framework in 6G TR taking the one in TR 38.843 as a starting point, with the following modifications:

- Indicate “capability/applicability reporting” procedure via an arrow from inference to management;

- Differentiate data collection with “real-time” for monitoring and inference, “non-real-time” for training.

    - Unified UE AI/ML Processing Capability

In the NR AI/ML-based CSI report framework for UE-side inference, the UE can indicate its AI/ML processing capability with the following components. The network is expected to configure appropriate inference request that does not exceed UE processing capability.

1. Number of APU pools.

2. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously in a CC.

3. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously across all CCs.

It is worth noting that the above capability is only for CSI-related AI/ML use cases, and similar capabilities were not introduced for non-CSI AI use cases, such as AI/ML-based positioning. This is because limited AI/ML use cases are supported in NR and only CSI-related AI/ML use cases require real-time inference.

Observation 5: Due to limited AI/ML use cases supported in NR and only CSI-related use cases require real-time inference, the UE AI/ML processing capability reporting is introduced for CSI-related use cases.

In 6G, it is expected that more use cases will be supported, some of which will also have real-time requirements for inference, such as AI/ML-based RS overhead reduction. In such cases, how to design the UE AI/ML processing capability should be discussed. If, similar to NR, UE AI/ML processing capability is defined per feature or per feature group, the implementation costs for the UE would be highly increased. Furthermore, since inference configurations (e.g., periodicity) for different use cases are up to network requirements, this may lead to low utilization efficiency of the APU corresponding to specific feature/feature group.

Figure 2.4-3: AI/ML processing capability for multiple use cases

Therefore, we believe that 6G should consider unified UE AI/ML processing capability. Specifically, the UE AI/ML processing capability should be shared for all the AI/ML-based use cases. With unified UE AI/ML processing capabilities, the network can set more efficient inference configuration based on the priorities of different functionalities.

Proposal 7: Investigate generic AI/ML processing capability reporting for all AI/ML use cases to enable the NW to efficiently activate the UE’s AI/ML functions based on its instantly available AI/ML processing capability.

- Conclusion

Use cases and requirements for data transfer

Observation 1: The potential data transfer schemes based on the existing NR Radio Protocol Architecture (including CP and UP) cannot fulfil the requirements of the 6G data framework:

- For CP-based solution: RRC is not suitable for transmitting large-size data due to the limited segmentation number; Flexible priority setting for NAS transmission, data transfer continuity during mobility, data compression and on-demand security is not supported.

- For UP-based solution: Data transfer terminated at gNB is not supported; DL Lossless and lossy data compression is not supported.

Proposal 1: The requirements of data collection framework shall consider the following two additional aspects:

- Continuity during mobility for data transfer between UE and network.

- Data compression, including downlink/uplink lossless and lossy data compression.

Proposal 2: The data transfer of the data collection framework shall support various data types with the following transmission requirements:

- Data types: AI/ML dataset, AI/ML model, sensing, MDT, and QoE.

- End point pairs: including UE-BS, UE-OAM, UE-CN, UE-OTT server, BS-BS, BS-CN, and BS-OTT server.

- Data size: including scalability for large-volume data (i.e., large single report or large total data size) and impact of data compression on data size.

- QoS requirements: including high data rate, relaxed latency, or flexible priority.

- On-demand continuity and/or lossless transfer during mobility.

- Security: optional integrity and ciphering.

- Privacy should be ensured via user consent.

Proposal 3: Study 6G Radio Protocol Architecture to address the data transfer requirements: address the identified gaps of NR CP/UP protocols, and explore the feasibility of introducing a new dedicated plane.

Requirements for model transfer

Observation 2: Due to limited UE storage and download latency, the UE may not be able to store/download all AI supported models in advance, thus on-demand model transfer from RAN/CN to the UE should be allowed.

Observation 3: Model transfer-based finetuning is the preferred way for use cases with stringent low-latency requirements.

Proposal 4: Study model transfer for UE sided and two-sided model for site-specific finetuning, e.g., “JSCC/JSCCM for CSI feedback for two-sided model and “tokenized CSI prediction” for UE-sided model.

Proposal 5: The model transfer mechanism should satisfy the following requirements:

- Support various model sizes transfer

- Ensure service continuity of model transfer during UE mobility.

- Ensure integrity protection of model transfer.

- Provide the MNO with full visibility of model transfer, i.e., ability to access and understand the model data.

6G AI/ML framework

Observation 4: UE capability transfer and applicability determination are primary procedures to determine whether an AI/ML model/functionality can be viewed as practical, especially for the case of UE-sided model.

Observation 5: Due to limited AI/ML use cases supported in NR and only CSI-related use cases require real-time inference, the UE AI/ML processing capability reporting is introduced for CSI-related use cases.

Proposal 6: Capture 6G AI/ML functional framework in 6G TR taking the one in TR 38.843 as a starting point, with the following modifications:

- Indicate “capability/applicability reporting” procedure via an arrow from inference to management;

- Differentiate data collection with “real-time” for monitoring and inference, “non-real-time” for training.

Proposal 7: Investigate generic AI/ML processing capability reporting for all AI/ML use cases to enable the NW to efficiently activate the UE’s AI/ML functions based on its instantly available AI/ML processing capability.

- Reference

- TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2500015	LS on signalling feasibility of dataset and parameter sharing (R1-2410922; contact: Qualcomm)	RAN1	LS in	Rel-19	FS_NR_AIML_air	To:RAN2	Cc:SA2, SA3, SA5

- S1-253370, 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements, September 2025.

- 3GPP TS 22.137, Service requirements for Integrated Sensing and Communication, March 2024.

- Appendix A: Sensing Data Size

Sensing measurements can be divided into the following levels according to the sensing processing procedure.

- Level 1, Raw data

Sensing raw data consists of subcarrier-domain samples of original received signals from each antenna port.

- Level 2, Per Path Results

Per path results consist of detection information for each detected path for a given time stamp. There are two options on associated detection information for each detected path.

o	Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp

Where one path is associated with one delay, one Doppler, one power and one or multiple angles

o	Option 2: Position, velocity, and power per detected path for a given time stamp

Where one path is associated with one velocity, one power and one or multiple positions.

To simplify calculation, Option 1 is adopted in the following data size determination.

- Level 3, Sensing results

Sensing results are the final output of sensing signal processing. A sensing result is described by sensing target’s attributes, like its location, velocity and target ID (optional). Compared with Level 1 and Level 2, it has the lowest requirement on data rate.

In this section, we would like give some preliminary data size evaluations for different levels based on the following assumptions.

- Number of symbols used for sensing: Nsymbol=128

- Sub-carrier space: SCS=30kHz

- Bandwidth: B=100MHz

- Number of TRPs: NTRP=1 or 12

- Number of Rx branches/channels: Nchannel=128,   at TRP side4,   at UE side

- Number of paths1: NPath=500

- Number of sensing targets: Ntar=15

- Associated information dimensions (i.e., delay, doppler, power, AoA*2 and ZoA*2) per path2: Ndim= 7

Note1: Number of detected paths is around 500 assuming 15 sensing targets

Note2: Number of associated angles per path is 2, and each angle consists an AoA and a ZoA.

Level 1: Raw data, CIR I/Q sample

If the single precision floating point number (32 bits) is used to describe the CIR I/Q samples, the data size would be about 51.5 Gbits based on the following calculation when 12 TRPs serve as sensing receivers (Mode 1, 3 and 5).

2×32×Nsymbol×Nsample×NTRP×Nchannel
=2×32×128×4096×12×128=51.5×109bits

The data size would be about 4.3 Gbits based on the following calculation when one TRP serves as sensing receiver (Mode 1, 3 and 5).

2×32×Nsymbol×Nsample×NTRP×Nchannel
=2×32×128×4096×1×128=4.3×109bits

The data size would be about 134 Mbits based on the following calculation when one UE serves as sensing receiver (Mode 2, 4 and 6).

2×32×Nsymbol×Nsample×Nchannel
=2×32×128×4096×4=134.2×106bits

Level 2: Per Path Results

If the single precision floating point number (32 bits) is used, the data size would be about 1.34 Mbits based on the following calculation when 12 TRPs serve as sensing receivers (Mode 1, 3 and 5)

32×NPath×NTRP×Ndim
=32×500×12×7=1.34×106bits

If the single precision floating point number is used, the data size would be about 112 Kbits based on the following calculation when one TRP serves as sensing receiver (Mode 1, 3 and 5).

32×NPath×NTRP×Ndim
=32×500×1×7=112000bits

The data size would be about 112 Kbits based on the following calculation when one UE serves as sensing receiver (Mode 2, 4 and 6).

32×NPath×Ndim
=32×500×7=112000bits

Level 3: Sensing Results

Since the sensing results are the final output of sensing signal processing, which is described by target’s attributes, there is no differential on data size for different sensing modes.

Suppose that target’s attributes for each trajectory point have 4 dimensions of statement: {X-Y-Z coordinates, and velocity}. If the single precision floating point number is used, the data size of the trajectory point for 15 sensing targets would be

32×4×Ntar=32×4×15=1920 bits