3GPP TSG-RAN WG2 Meeting #132	R2-2508055

Dallas, USA, Nov.17th– 21st, 2025

Source:	vivo

Title:	Further Discussion on 6G security

Agenda Item:	10.3.3.3

Document for:	Discussion and Decision

- Introduction

In RAN#109, the SID “Study on 6G Radio” was agreed[1]. Main RAN2 leading items regarding control plane are listed in below.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

Further RAN2#131bis reached the following agreements on Access Stratum security aspects[2]:

Send LS to SA3 to indicate the existing 5G MAC CE information and that some of these control information may be carried over in 6G L2. Ask them what information would require security. Explain RAN2 concerns of overhead (size and mobility security context exchange) and processing. Please identify only critical information that needs to be secure and what type of security (i.e. integrity, ciphering). Indicate that if there are information that critical to be protected RAN2 and SA3 should work jointly to develop a solution.

This contribution further discusses AS security aspects as follows:

- System Information

- Paging message

- Quantum-safe alghorithms

- Discussion

Access Stratum (AS) security, as the cornerstone of communication technology, will continue to drive the development of 6G radio protocol design. The AS security is expected to comprehensively address security challenges and provide a significantly higher level of security. Based on insights from 5G systems and inputs from SA1 6G study on 6G Use Cases and Service Requirements, we provide an analysis of existing security threats and initial thoughts on potential new security requirements as follows:

  - System Information

The 5G study on security enhancements against False Base Stations (FBS), as documented in SA3 TR 33.809, identified several threats related to a UE in RRC_IDLE state camping on a rogue cell, including the following:

- UE camping on a fake base station: Attackers can spoof essential SIs (e.g., MIB, SIB1-5) to force UEs onto their malicious networks.

- Public safety threats: Fake SIBs (e.g., SIB 6-8) can be used to transmit false public safety alerts, such as ETWS and CMAS messages, causing social panic.

- Service attacks: SIBs related to commercial services like MBMS SIB20/24, V2X SIB12/13/14, ProSe SIB12, and NTN SIB19/25 etc. can be manipulated to disrupt those services.

Malicious people can deploy portable fake base stations are real examples that caused significant damage to cellular networks and their users. These security threats highlight the need for security protection for System Information to authenticate a cell. However, implementing security protection for System Information to authenticate a cell would have required significant changes to the fundamental security architecture of NR Release (Rel-15). For this reason, the relevant study was closed without conclusions due to compatibility issues. Notably, this security requirement has been reintroduced in the 6G study. As in SA1 TR 22.870, it is captured that “[PR 5.5.3.2-1] Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station.”. Based on the observations above, now is an opportune time to initiate a thorough study on security protection mechanisms for System Information to address the False Base Station (FBS) issue comprehensively.

As noted above, securing System Information represents a significant evolution from existing communication technologies, potentially impacting the hardware of both the UE and BS. From a deployment perspective, introducing these security measures as mandatory features in early 6G design would be simpler, as retrofitting enhancements later would be impractical—a lesson learned from the 5G-A experience mentioned above. According to the approved LS in RP-252891 at the RAN#109 meeting, parallel discussions between RAN2 and SA3 are assumed to be underway to advance the AS security topic, with a checkpoint in place to ensure early alignment between WGs. Given this, we recommend prioritizing the analysis of potential security requirements at the AS layer to reach consensus and identify corresponding issues and impacts in RAN2. For example, to facilitate RAN2 in conducting a more effective analysis for solution evaluation, as documented in the LS R2-2208985 during the 5G FBS study, RAN2 held discussions and reached conclusions on the identified potential issues/impacts that require SA3 inputs to support security protection of System Information. These are abstracted from the LS as below:

- Size of the security information or feasible ranges for the size

- Latency requirements for the delivery of the security information

- How often and for how long the new information is expected to be sent

- Whether all System Information or some part needs to be protected

- Whether the security information should be updated whenever any of the SIB contents change

- RAN2 has previously requested SA3 to provide the following information on the requirements of the security information to be broadcast so that RAN2 can make a better analysis for this feature [3]:

- Size of the security information or feasible ranges for the size

- Latency requirements for the delivery of the security information

- How often and for how long the new information is expected to be sent

- Whether all System Information or some part needs to be protected

- Whether the security information should be updated whenever any of the SIB contents change

To secure that UE detects the False Base Stations, at least integrity protection of system information can be considered. Integrity protection ensures the integrity of system information and prevents it from being tampered with. Thus, integrity protection should at least apply to critical system information, such as MIB and SIB1-5 as illustrated above. Whether integrity protection should apply to all system information can be further checked with SA3. However, the motivation to support encryption of critical system information is not well justified yet, which could bring additional delay and complexity for both the Network and UE due to message encryption/decrption. Therefore, encryption of system information should be further studied.

- Integrity protection should apply to at least, critical system information. FFS whether to support encription of System Information.

- Send LS to ask SA3 to confirm RAN2 assumption on integrity protection for critical System Information, ask what System Information is critical and also whether System Information encription is needed.

  - Paging message

In addition to securing System Information, protecting Paging messages is also considered highly effective in preventing tampered/spoofed sensitive information, such as the Paging cause value. This security gap in 5G Paging has hindered the adoption of features that could enhance performance. One example is the EPS fallback feature. “a new EPS fallback indicator included in paging/SIB” was considered for a promising solution, which can accelerate a UE’s fallback from 5G to 4G for voice service. However, according to the LS (S3-221162), this solution was ultimately excluded because these indicators lacked security protection.

Similar to System Information, Paging messages are also delivered as broadcast signaling for UEs in RRC_IDLE state, their security procedure and fundamental frameworks are similar. Therefore, it would be beneficial to study them together as potential enhancements for AS security in RRC_IDLE state.

- Similar to System Information, integrity protection should also apply to Paging message, at least for Paging messages containing sensitive information, e.g., paging cause.

- Send LS to SA3 to confirm RAN2 assumption on integrity protection for Paging message, at least for Paging messages containing sensitive information e.g., paging cause.

  - Quantum-safe algorithm

Traditional encryption (such as RSA and ECC) relies on mathematical problems that are hard for classical computers to solve (e.g., factoring large primes). However, quantum computing technology poses significant threats to some classical cryptography, especially to widely-used cryptographic algorithms (including both symmetric and asymmetric cryptography) that secure much of today's data, rendering electronic secrets vulnerable to discovery [4].  Quantum-safe algorithms are essential because they can resist potential attacks from quantum computing. Thus, for more rubust security algorithm, quantum-safe algorithms are under study in 6G system security design [5] [6].

- Quantum-safe algorithms are essential in resisting potential attacks from quantum computing.

NIST Post-Quantum Cryptography (PQC) standardization process aims to replace public-key cryptography (e.g., RSA and ECC) that is vulnerable to attacks by large-scale quantum computers. some security categories with different levels. Security Level 5 (equivalent to AES-256) provides the highest security and is intended for applications where maximum security is the top priority, such as protecting highly sensitive information. The implementation of PQC scheme may require significantly higher bit lengths than traditional security algorithms, such as 256 bits or 512 bits. The challenges to existing technical solutions are analyzed as follows:

On one hand, to accommodate quantum-safe algorithms with longer bit lengths, the MAC-I (Message Authentication Code - Integrity) length output by security protection operations based on these algorithms may exceed 128 bits. Consequently, the length of the short message integrity verification code transmitted over the wireless air interface, such as MAC-I  included in PDCP PDUs, which is derived from truncating the computed MAC-I, may also exceed the current length of 32 bits (for example, increasing to 64 bits). This will definitively impact the 6G User Plane (UP) design.

On the other hand, RRCReestablishmentRequest and RRCResumeRequest messages that carry shortMAC-I/resumeMAC-I (fixed 16bit) are transmitted over the Common Control Channel (CCCH). Similarily, length of shortMAC-I/resumeMAC-I  may also be extended. However, the uplink (UL) message size on CCCH is limited, currently supporting only fixed lengths of 48 bits (for RRCReestablishmentRequest and RRCResumeRequest) and 64 bits (for RRCResumeRequest1). Due to these size constraints, the current CCCH design cannot support the transmission of longer shortMAC-I/resumeMAC-I. This will have impact on required RRC message size  over CCCH.

- 6G system security design framework will support quantum-safe algorithms and RAN2 should study quantum-safe algorithms impact on RAN protocol as follows:

- Longer length of MAC-I impact on 6GR UP design e.g., PDCP PDU format.

- Longer length of shortMAC-I/resumeMAC-I impact on required CCCH size in MSG3.

- Conclusion

In this paper, we discuss 6G radio control plane. we have the following observations and proposals:

System information

- RAN2 has previously requested SA3 to provide the following information on the requirements of the security information to be broadcast so that RAN2 can make a better analysis for this feature [3]:

- Size of the security information or feasible ranges for the size

- Latency requirements for the delivery of the security information

- How often and for how long the new information is expected to be sent

- Whether all System Information or some part needs to be protected

- Whether the security information should be updated whenever any of the SIB contents change

- Integrity protection should apply to at least, critical system information. FFS whether to support encription of System Information.

- Send LS to ask SA3 to confirm RAN2 assumption on integrity protection for critical System Information, ask what System Information is critical and also whether System Information encription is needed.

Paging messages

- Similar to System Information, integrity protection should also apply to Paging message, at least for Paging messages containing sensitive information, e.g., paging cause.

- Send LS to SA3 to confirm RAN2 assumption on integrity protection for Paging message, at least for Paging messages containing sensitive information e.g., paging cause.

Qauntum-safe algorithm

- Quantum-safe algorithms are essential in resisting potential attacks from quantum computing.

- 6G system security design framework will support quantum-safe algorithms and RAN2 should study quantum-safe algorithms impact on RAN protocol as follows:

- Longer length of MAC-I impact on 6GR UP design e.g., PDCP PDU format.

- Longer length of shortMAC-I/resumeMAC-I impact on required CCCH size in MSG3.

- Reference

- RP-252912	Revised SID: Study on 6G Radio.

- R2_131bis_ChairNotes_25-10-17_17-00_final

- R2-2208985_Reply LS on authenticity and replay protection of system information

- 3GPP TR 22.870	Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20)

- 3GPP TR 33.703	Study on Transitioning to Post Quantum Cryptography (PQC) in 3GPP (Release 20)

- 3GPP TR 33.771	Study on supporting AEAD algorithms (Release 20)