3GPP TSG-RAN WG2 Meeting #132	R2-2508058

Dallas, USA, Nov. 17th - 21st, 2025

Source: 	Xiaomi

Title:  	Discussion on RRM measurement event prediction

Agenda Item:	9.3.3

Document for:	Discussion and Decision

Introduction

A Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR [1] has been approved. In the WID, one of the objectives is to specify support for RRM measurement prediction and measurement event prediction for UE sided models, as shown below.

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported.

In this contribution, we provide our understandings on RRM measurement event prediction.

Discussion

## 2.1 General

Based on the “Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions” in the WID [1]. In this contribution, we provide our views for indirect event prediction.

In the WID, there are three types of indirect prediction as following:

- Temporal domain prediction case A

- Temporal domain prediction case B

- Frequency domain prediction for co-located cases

In SI phase, RAN2 reached the following agreement [2]:

RAN2#131 Agreement: [2] For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

The goal of temporal domain prediction case B and frequency domain prediction is to reduce measurement. If the UE is configured with temporal domain prediction case B and frequency domain prediction, the UE may not always have actual measurement results.

For temporal domain prediction case B, the predicted result and the actual result can be used for the event evaluation. It is possible to predict a fulfilled event in advance based on case B, but it is not always achievable and the gain is not significant. So, the optimization method shall not be considered. For frequency domain prediction, the UE can only use predicted results to evaluate events.

Given the above, when temporal domain case B or frequency domain prediction is activated, it is reasonable to only determine whether an event has occurred at the current time instance. Apart from the way to obtain measurement results, i.e., prediction or measurement, everything else is the same as the legacy event-based measurement reporting procedure. Therefore, no additional specification impact to support event prediction based on temporal domain prediction case B or frequency domain prediction compared to RRM measurement prediction.

Proposal 1: RAN2 can confirm no additional specification impact to support event prediction based on temporal domain prediction case B or frequency domain prediction compared to RRM measurement prediction.

The goal of temporal domain prediction case A is to improve the handover performance. Based on predicted results of case A, the UE can predict the event occurs in future and two handover models (option 1 and option 2) are captured in the TR 38.744 [3] to improve the handover performance. Some mechanisms for the measurement event prediction based on intra-frequency temporal domain case A need to be discussed. Following discussions are only applicable to measurement event prediction based on case A.

## 2.2 Inference configuration

In the TR 38.744 [3], for inference configuration of UE sided RRM measurement prediction, there is the following description:

### 6.1.2 RRM measurement prediction #### 6.1.2.1 UE-sided model ##### 6.1.2.1.2 Inference configuration and report Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. When a full inference configuration is received, UE shall maintain it until it is released by network explicitly. Following inference parameters can be configured to UE for inference and assessing applicability: - PW length for intra-frequency temporal domain case A; - Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction; - Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement); - Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available); - Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction; - Optional associated ID.

Same as the RRM measurement prediction based on case A, for the inference configuration of case A based event prediction, existing RRM measurement configuration in RRC layer is baseline for inference configuration and the additional inference configurations: PW length, list of cells and associated ID can be included.

Proposal 2: For event prediction based on case A, the existing RRM measurement configuration in RRC layer is baseline for inference configuration, and following additional inference parameters can be configured:

- PW length;

- Optional list of cells for which network expects inference report (if available);

- Optional associated ID.

For the measurement prediction, the network can configure UE to perform prediction for a specific cell or cell list. In addition, event types also need to be considered for the event prediction. To support the indirect event A3, A5 and A6 prediction, the UE needs to perform prediction on both serving and candidate cells, and both serving cell and neighbour cell side additional condition need to be considered for the model identification. For indirect event A1, A2 and A4 prediction, only the serving cell side or the neighbour cell side additional condition needs to be considered. Hence, the inference is specific to cell/cell list and the event type.

Measurement events are configured via the measurement report configuration, and neighbour cells to be measured are configured via the measurement object configuration. Each measurement ID links one measurement object with one reporting configuration. Therefore, the measurement ID can be used to identify RRM event prediction.

Proposal 3: The measurement ID can be used to identify the event prediction based on case A.

## 2.3 Inference control

In the SI phase, RAN2 has discussed the inference control and agreed the following agreement:

RAN2#129bis:[4] The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

One open issue is when the UE can perform inference. One method based on the dynamic signalling has been excluded.

The measurement event prediction based on the case A is to enhance the handover performance and used to predicted the future event. It seems to be more useful when actual measurement results are close to the event threshold. In this case, it is more likely to predict that the event will occur within the prediction window. To avoid unnecessary inference, the UE doesn’t perform the inference of applicable functionality if the radio condition is not met. The radio condition is associated with the radio of serving and/or neighbour cell. The radio condition can be similar as A1~A6 alike and configured by the network, whether TTT and hysteresis are needed can be discussed.

Proposal 4: For event prediction based on case A, the network can configure the radio condition for inference. And the UE is not required to perform inference if the radio condition is not met.

## 2.4 Inference report

As the description in TR 38.744 [3], for inference configuration of UE sided RRM measurement prediction, existing RRM measurement reporting framework in RRC layer is baseline for inference report. Like the RRM measurement prediction, following the existing RRM framework is also the simple and straightforward method for the inference report of event prediction, and RAN2 can support it.

Compared with the legacy event-based MR, in addition to include RRM measurement results, the event prediction report can also include predicted events and the time-related information about the predicted event (e.g., the time/window of occurrence).

Proposal 5: Existing RRM measurement reporting framework is used to report predicted results of event prediction based on case A, and the event prediction report can include following information:

- RRM measurement results;

- Predicted event together with the time-related information about the predicted event (e.g., the time/window of occurrence).

One open issue is what types of RRM measurements are included in the event prediction report. In the legacy event-based MR, only the ground-truth measurement result is included in the MR. In the event prediction report, apart from the ground-truth measurement result, the predicted measurement results can also be included. There is the same issue for the inference report of RRM measurement prediction case A. In TR 38.744 [3], it is considered to report one or more instances of predicted measurement result(s) in PW for intra-frequency temporal domain case A. For the event prediction report, the reported RRM measurement results can follow the MR of intra-frequency temporal domain case A, and to report the predicted RRM measurement results within PW based on the network configuration. Some details can be discussed based on the MR of intra-frequency temporal domain case A.

Proposal 6: For the case A based event prediction report, the reported RRM measurement results can follow the MR of intra-frequency temporal domain case A, and to report the predicted RRM measurement results within PW based on the network configuration.

The UE tiggers the predicted event report based on the predicted measurement results of case A to indicated the event will occurs in the future. Considering the potential prediction errors, the UE may have different prediction results on the same timing instance in future as time goes. For example, there may be two cases:

- Case 1: the UE has predicted one event occurs in T1 in the 1st prediction, but in the 2nd prediction, the UE may predict the event doesn’t occur in the PW.

- Case 2: the UE has predicted one event occurs in T1 in the 1st prediction, but in the 2nd prediction, the UE may predict the event occurs in T2. There is timing difference between T1 and T2.

Considering the closer the predicted time is to the actual time, the higher the prediction accuracy will be, for the same event prediction, the latest prediction might be the most accurate. Therefore, for both case 1 and case 2, the network needs to change the predicted results based on the 2nd prediction. In other words, for case 1 and case 2, the UE needs to cancel or update the previously reported prediction to ensure that the prediction results provided to the network side are always the latest and relatively accurate.

Proposal 7: The UE is allowed to cancel or update the previously reported prediction.

In TR 38.744 [3], for measurement event prediction based on case A, one handover model is relying on the predicted measurement event, as shown below:

Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time, i.e. t0, measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

For the handover model (option 2), the network transmits the HO command at t2, and t2 is the time when entry condition of the predicted measurement event is met based on actual measurement results. Hence, the network needs to know T2. But in the legacy event-based MR, the UE shall not trigger the report upon entering condition met. For option 2, it is needed. Based on the network configuration, after the UE triggered the event prediction report (at T0), the UE triggers report upon entering condition met based on actual measurement result.

Proposal 8: The UE can trigger report when entry condition of the predicted measurement event is met based on actual measurement result after event prediction report.

Conclusions

According to the analysis given above, we have the following proposals:

2.1 General

Proposal 1: RAN2 can confirm no additional specification impact to support event prediction based on temporal domain prediction case B or frequency domain prediction compared to RRM measurement prediction.

2.2 Inference configuration

Proposal 2: For event prediction based on case A, the existing RRM measurement configuration in RRC layer is baseline for inference configuration, and following additional inference parameters can be configured:

- PW length;

- Optional list of cells for which network expects inference report (if available);

- Optional associated ID.

Proposal 3: The measurement ID can be used to identify the event prediction based on case A.

2.3 Inference control

Proposal 4: For event prediction based on case A, the network can configure the radio condition for inference. And the UE is not required to perform inference if the radio condition is not met.

2.4 Inference report

Proposal 5: Existing RRM measurement reporting framework is used to report predicted results of event prediction based on case A, and the event prediction report can include following information:

- RRM measurement results;

- Predicted event together with the time-related information about the predicted event (e.g., the time/window of occurrence).

Proposal 6: For the case A based event prediction report, the reported RRM measurement results can follow the MR of intra-frequency temporal domain case A, and to report the predicted RRM measurement results within PW based on the network configuration.

Proposal 7: The UE is allowed to cancel or update the previously reported prediction.

Proposal 8: The UE can trigger report when entry condition of the predicted measurement event is met based on actual measurement result after event prediction report.

Reference

- RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, OPPO, Interdigital

- R2-2503302, “Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China”

- 3GPP TR 38.744 V19.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR”

- R2-2506702, “Report of 3GPP TSG RAN WG2 meeting #131, Bengaluru, India”