3GPP TSG-RAN WG2 Meeting #132		R2-2508062

Dallas, USA, Nov. 17th - 21st

Source:	vivo

Title:	Discussion on RRM measurement prediction

Agenda Item:	9.3.2

Document for:	Discussion and Decision

# Introduction

In the Rel-20 AI/ML mobility WID[1], the objectives regarding RRM measurement prediction of the UE-sided model are as follows:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we will further discuss the detailed aspects related to RRM measurement prediction.

# Discussion

## 2.1 UE capability

In Rel-19 AI/ML PHY, the following UE features of AI/ML beam management were introduced (extracted from R1-2507979 [1]):

Table 2.1-1: UE features list for Rel-19 AI/ML for beam management

| Feature group | Components |
| --- | --- |
| UE-side beam prediction for BM Case1 for inference | <omitted as Case 1 is Spatial-domain prediction> |
| UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types for Set B 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 15. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance 21. supported number of occupied CPU 22. supported number of occupied CPU,2/CPU,3 23. supported value of d for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 24. supported value of d’ for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 25. Occupied resource pool between CPU,2 and CPU,3 |

As can be observed from the above Table 2.1-1, AI/ML beam management categorizes feature groups according to different use cases, such as spatial domain prediction (Case 1) and temporal domain prediction (Case 2). For temporal domain prediction, it includes multiple components related to inference configuration, such as RS type, report type, prediction instance, etc.

For AI/ML-based mobility, the scenarios expected to be supported include: temporal domain case A, temporal domain case B, and inter-frequency prediction. And different scenarios may require different specific inference configuration parameters, e.g., PW length is needed specifically for intra-frequency domain case A, skipping pattern/MRRT is needed specifically for intra-frequency domain case B, and predicted frequency carrier information is needed specifically for inter-frequency domain.

Taking the AI/ML beam management as a reference, the UE capability for RRM measurement prediction should categorize feature groups based on scenarios, since different scenarios require different inference configurations. The detailed capability components for each scenario should be discussed after determining the detailed inference configuration.

- For RRM measurement prediction, the UE capability should categorize feature groups based on scenarios (e.g., temporal domain case A, temporal domain case B, inter-frequency prediction). The detailed capability components for each scenario should be discussed after the detailed inference configuration parameters are concluded.

## 2.2 Applicability reporting

During the SI phase, the applicability reporting in AI/ML beam management was taken as a reference. The following description was captured in the TR regarding applicability reporting.

UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 in a RRCReconfiguration message. Upon transition to RRC_IDLE or RRC_INACTIVE state or upon RLF, UE follows existing behaviour defined in [2] on whether to release or keep an inference configuration. Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message. If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release it . When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported. For an inapplicable full inference configuration, UE continues to perform the inference. It is up to network implementation what to do with reported result after UE indicates the corresponding full inference configuration is inapplicable. If a full inference configuration is applicable, UE applies it. No dynamic lower layer signalling is needed to active a full inference configuration. Applicability can be updated via UAI. A flag in OtherConfig is introduced to indicate whether applicability reporting via UAI is enabled or not. When an inference configuration becomes inapplicable UE shall report its inapplicability via UAI. No prohibit timer needs to be introduced for prohibiting applicability reporting.

We understand that the above applicability reporting mechanisms are still valid except for the highlighted part. Specifically, at RAN2#131bis meeting, companies thought that the network always needs to know if applicability has changed. Then RAN2 agreed to remove the flag in the AI PHY WI:

[E042] Applicability reporting via UAI is always enabled for option A and B. Remove flag from RRC CR and update procedural text to indicate that it is configured if option A or B are configured. Wording offline.

Therefore, we propose that RAN2 confirms that applicability reporting via UAI is always enabled for options A and B for AI/ML mobility. No explicit flag is needed.

- Confirm that the description of the applicability reporting procedure in the TR is valid, except that UAI for applicability update is always enabled (i.e., no explicit flag is needed in Otherconfig).

AI-based BM takes CSI framework as a baseline, which only involves prediction related to the RS of the serving cell. The inference configuration of AI-based BM is relatively simple, making the applicability determination fairly clear. For AI-based mobility, the current framework is based on RRM measurement configuration/reporting. RRM measurement configuration can include different granularities, such as frequency, cell, and beam. At RAN2#131 meeting, RAN2 agreed that inference configuration can include the list of predicted target cells for the temporal domain case A:

Agreements 2. For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration. This list is optional.

In such cases, for a single inference configuration, it is possible that only part of the configuration is applicable. For example, part predicted target cells are applicable, while the other target cells are inapplicable. Under these circumstances, the UE should be able to report which part of the inference configuration is applicable.

- UE could indicate partial applicability in the applicability reporting, e.g., only part of the cells in the inference configuration are predictable.

Additionally, for inter-frequency prediction, simulation results indicate that the performance of the AI model meets requirements only under specific frequencies as input and output. Therefore, for a specific frequency as output, the UE should indicate the required measurements of the specific frequency as input to the network, meaning the network needs to configure the corresponding measurement object. The UE can indicate this information via applicability reporting, indicating the associated conditions under which the inference configuration is applicable, such as the need for the network to configure the corresponding measurement object to acquire input. If the UE does not indicate the specific frequency and the network blindly configures the as-input frequency (e.g., as part of inference configuration), the efficiency is quite low.

- UE could indicate the associated conditions under which the inference configuration is applicable, e.g., the required measurements of specific frequency as input for inter-frequency prediction.

## 2.3 Inference configuration/reporting

At RAN2#129bis meeting, RAN2 agreed that the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework:

Agreements 1 The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

We think that the above agreement is conducive to reducing the complexity of the specification. RRM measurement prediction is mainly related to obtaining the measurement result. Based on the logic, we think that how to perform the measurement (e.g., measurement reduction) should be configured in the MeasObjectNR IE, i.e., no new object (e.g., prediction object) needs to be introduced. And how to report the predicted measurement results (e.g., instance number and interval for temporal domain Case A) is configured in the ReportConfigNR IE.

- For the inference configuration of RRM measurement prediction:

- How to perform the measurement (e.g., measurement reduction) is configured in the MeasObjectNR IE.

- How to report the predicted measurement results (e.g., instance number and interval for temporal domain Case A) is configured in the ReportConfigNR IE.

In NR, the measurement reference signal can be SSB or CSI-RS. In the AI/ML-based beam management, the two types of reference signals are supported. However, for AI mobility, we understand that at least SSB-based measurement prediction should be supported in Rel-19/20 since the mobility is typically based on SSB. Besides, all the simulation evaluations in the study phase are based on SSB. We think that only SSB type should be supported in Release-20.

- Only support SSB as the RS type in AI/ML-based mobility, e.g., the RS type can only be set as SSB in inference configuration.

At RAN2#129bis meeting, RAN2 concluded that the associated ID should be optionally configurable for training and inference, the details are FFS and should be discussed in WI phase:

Agreements 2 Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

For AI/ML-based beam management, the associated ID was introduced to ensure the consistency between model training and inference configuration without exposure of NW implementation of Tx beam. The UE may assume the similar properties of a Tx beam or beam set/list associated with the same associated ID. We think that the underlying logic of RRM measurement prediction and beam management is the same, i.e., prediction accuracy relies on the consistency of DL Tx beam characteristics between training and inference. Furthermore, as the Tx beam characteristics have an impact on the L1 RSRP, L3 beam-level and L3 cell-level RSRP, the associated ID can be applicable for all the sub-use cases of RRM measurement prediction.

In the AI/ML-based beam management, if the network does not provide the associated ID, it is up to UE implementation how to determine the applicability. We think this is still valid for AI/ML-based mobility.

As to the granularity, since only L3 Cell-level prediction is supported in this release, we think the associated ID can be per cell as a baseline.

- Associated ID is supported in AI/ML-based mobility, with:

- Associated ID is utilized to indicate the properties of network without exposure of NW implementation.

- Associated ID is applicable for all scenarios/use cases.

- If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

- Associated ID can be configured per cell.

Similar to traditional RRM measurement, the predicted measurement results should be reported in Measurement Report message. One difference is that there may be multiple instances reported for a cell. We propose to support reporting the difference of measurement results for multiple instances to save the signalling overhead.

- For RRM measurement prediction, the predicted measurement results are reported in the Measurement Report message. For the report of multiple instances, support differential reporting between instances to reduce the signalling overhead.

## 2.4 Inference triggering

At the RAN2#129bis meeting, RAN2 agreed that the UE applies inference configuration for the applicable RRM measurement prediction upon receiving configuration via RRC, and it is FFS when the UE can perform inference.

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

From our understanding, for different goals, the time of performing inference should be different. In general:

- For measurement reduction, the simulation results of SLS is based on low UE mobility (30km/h). Therefore, similar to relaxed measurement, the UE should perform the inference when the UE is with low mobility.

- For mobility performance improvement, to reduce the power consumption of inference, the UE should perform the inference at the edge of the serving cell.

- Support event-triggered inference for RRM measurement prediction:

- For measurement reduction, similar to relaxed measurement, the inference can be triggered when UE is with low mobility.

- For HO performance improvement, to reduce power consumption, the inference can be triggered when UE is at the cell edge.

# Conclusion

In the contribution, we have the following observations and proposals:

UE capability

- For RRM measurement prediction, the UE capability should categorize feature groups based on scenarios (e.g., temporal domain case A, temporal domain case B, inter-frequency prediction). The detailed capability components for each scenario should be discussed after the detailed inference configuration parameters are concluded.

Applicability reporting

- Confirm that the description of the applicability reporting procedure in the TR is valid, except that UAI for applicability update is always enabled (i.e., no explicit flag is needed in Otherconfig).

- UE could indicate partial applicability in the applicability reporting, e.g., only part of the cells in the inference configuration are predictable.

- UE could indicate the associated conditions under which the inference configuration is applicable, e.g., the required measurements of specific frequency as input for inter-frequency prediction.

Inference configuration/reporting

- For the inference configuration of RRM measurement prediction:

- How to perform the measurement (e.g., measurement reduction) is configured in the MeasObjectNR IE.

- How to report the predicted measurement results (e.g., instance number and interval for temporal domain Case A) is configured in the ReportConfigNR IE.

- Only support SSB as the RS type in AI/ML-based mobility, e.g., the RS type can only be set as SSB in inference configuration.

- Associated ID is supported in AI/ML-based mobility, with:

- Associated ID is utilized to indicate the properties of network without exposure of NW implementation.

- Associated ID is applicable for all scenarios/use cases.

- If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

- Associated ID can be configured per cell.

- For RRM measurement prediction, the predicted measurement results are reported in the Measurement Report message. For the report of multiple instances, support differential reporting between instances to reduce the signalling overhead.

- Support event-triggered inference for RRM measurement prediction:

- For measurement reduction, similar to relaxed measurement, the inference can be triggered when UE is with low mobility.

- For HO performance improvement, to reduce power consumption, the inference can be triggered when UE is at the cell edge.

# Reference

- RP-252899		Revision of WID on AI mobility

- R1-2507979	Updated RAN1 UE features list for Rel-19 NR after RAN1 #122bis

- TR 38.744		Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR