3GPP TSG RAN WG2 #132               	                                                             R2-2508067

Dallas, USA, Nov. 17th-21st , 2025

Source:	CATT

Title:	Discussion on 6GR RRC Modelling and Connection Management

Agenda Item:	10.3.2.1

Document for:	Discussion and Decision

- Introduction

Lasting meeting, for 6GR it was agreed to support RRC idle mode, connected mode, and inactive or sub-state as following shows:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

This contribution will further discuss the functions of the modeling of inactive mode or sub-state and the connection management.

- Discussion

  - Inactive or sub-state

In last meeting,  it was agreed at least the following functions should be supported for inactive or sub-state: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW. This section will further discuss the supported functions. And give the suggested modeling.

- Function

- UE context and identification:

In order to achieve the aim of fast transition compared with UE in RRC idle state, both UE and the RAN node should keep the UE’s AS UE context when the UE in this state (inactive or sub-state). Generally, UE is identified by the UE ID allocated by CN e.g. S-TMSI when UE in idle state, and identified by the UE ID allocated by RAN node e.g. C-RNTI when UE in connected state. But the CN UE ID is a global UE ID which needs more overhead to transmission. On the other hand, the cell UE ID is a cell specific UE ID which is only applicable in the serving RAN node. Hence the cell UE ID is not applicable when UE moves out of the coverage of the anchor RAN node. Hence when UE in this state(inactive or sub-state), a specific UE ID should be allocated e.g. I-RNTI used in 5G, based on this specific UE ID, the target node could identify the UE’s AS context or the anchor RAN node where the UE’s context is stored.

Observation 1: A specific UE ID can achieve lower signaling overhead compared to a CN UE ID, and it is applicable across a wider coverage area than the UE ID used in the connected state.

- RNA( RAN-based notification area) management

Some companies expect to simplify the RNA management e.g. unified with TA area used for idle state. However if the unified area configured with smaller coverage (e.g., under one CU) is used for both TA and RNA, it will result in frequent TAU procedures. On the other hand, if this area is configured with larger coverage (e.g., including all cells under the CN node), the fact that these RAN nodes belonging to the same CN may not have direct connection with each other will introduce complexity in UE context sharing among them. Additionally, the latency reduction benefit when resuming the connection will not be obvious. Furthermore the TA is used to track the UE for CN paging, while the RNA is used for RAN paging. Hence it is suggested to configure the RNA as a separate area from TA. This approach provides greater flexibility in RNA management, e.g. allowing operators to configure cells under a RAN node as RNA or designate one cell as RNA. In 5G NR, both timer based RNAU and event triggered RNAU (the UE triggers the RNA update procedure upon moving out of the RNA) are supported. RNAU procedure should be studied in 6G.

Observation 2: By decoupling RNA from TA, the network achieves higher flexibility in mobility management, as it avoids the trade-offs between frequent TAU procedures (caused by small unified areas) and UE context sharing complexity (caused by large unified areas)‌.

- UE based mobility

It was agreed to support UE based mobility e.g. cell (re)selection, measurement should be performed to select suitable cell last meeting. The measurement should be based on the NW configuration as 5G (e.g., the frequency configuration via system information or the priority configuration configured via dedicated signaling). Importantly, there is no need to trigger measurement report in order to reduce power consumption.

Observation 3: NW configuration based measurement without measurement reports can reduce UE power consumption by eliminating unnecessary signaling overhead.

- Data transfer

MO-SDT (Mobile Originated Small Data Transmission) was introduced in NR Rel-17, while MT-SDT (Mobile Terminated Small Data Transmission) was introduced in NR Rel-18. SDT is initiated with either a transmission over RACH or over CG-SDT. It allows data and/or signaling transmission while the UE remains in inactive (i.e., without transitioning to connected). It can not only reduce signaling overhead and power consumption by avoiding unnecessary transition to connected state, but also reduce latency by enabling fast transmission of small, infrequent packets. Small data and/or signaling can be sent together with the first RRC Resume Request message.

Observation 4: SDT enables concurrent reduction in signaling overhead and power consumption while achieving fast data transmission.

- Paging/wake-up

In NR, inactive UEs without any ongoing SDT procedure must monitor paging channels for both RAN-initiated paging and CN-initiated paging. For inactive UEs, different paging DRX cycles may be used for RAN-initiated paging and CN-initiated paging. It results in inactive UEs possibly using different, non-overlapping POs to monitor RAN-initiated paging and CN-initiated paging, which introduces additional UE behaviors and power consumption. But the issue has been resolved since NR Rel-17. Inactive UEs monitor only one PO set, i.e., inactive UEs use the same i_s as for idle, despite paging DRX cycles are different for RAN-initiated paging and CN-initiated paging.

Observation 5: In NR, inactive UEs can only monitor one PO set since Rel-17 even if they need to monitor both RAN-initiated paging and CN-initiated paging, preventing additional power consumption introduced.

Hence we suggest:

Proposal 1: The Inactive state or sub-state shall support the following functionalities:

  - UE identification: allocation of a specific UE identifier (e.g., I-RNTI-like ID) for UE context identification purpose.

  - RAN-specific Notification Area management‌: implementation of a distinct RAN-based notification area management mechanism, independent of the Core Network area management.

  - UE based mobility: support of measurements without reporting, configured by the network.

  - Data transfer‌: support of transferring unicast data and/or signaling to/from the UE for small data.

  - Paging: unified PO set monitoring mechanism that simultaneously supports both RAN-initiated and CN-initiated paging procedures.

- modelling

Based on above discussion of the supported functions in inactive state or sub-state, we should further consider the modeling of this state, i.e., to define one separate RRC inactive state or one sub-state of RRC connected or RRC idle state.  The comparison of this state with the other two states listed in the following table.

Table 1: comparison with RRC connected and idle state

|  | Inactive or sub-state | RRC_Connected | RRC_Idle |
| --- | --- | --- | --- |
| Mobility | - UE based mobility e.g. Cell (re)selection - without notification to NW - without measurement report | - Network based handover - UE based handover with notification to the NW - with measurement report | - UE based mobility e.g.Cell (re)selection - without notification to NW - without measurement report |
| Applicable area | - Specific area e.g. RNA | - Serving cell | - TA |
| UE context management | - Store AS UE context - Store NAS UE context | - Store AS UE context - Store NAS UE context | - Store NAS UE context |
| UE identification | - Separate UE ID e.g. I-RNTI | - cell specific UE ID e.g. C-RNTI | - CN allocated UE ID e.g. S-TMSI |
| Data transfer | - support direct data/signaling transfer for small data | - support direct data /signaling transfer | - not support direct data/signaling transfer |
| Paging monitor | - needs to monitor | - not need to monitor | - need to monitor |

As shown in the table above, while a UE in this state (inactive or sub-state) stores the same UE AS context as in  RRC connected state, there are different mechanisms or requirement between these two states for key functions, such as mobility, paging, measurement, data transfer, applicable area management, and UE identification. Compared with RRC idle state, it shares common procedures for mobility, but the two states have different applicable area management, different UE context management, different UE ID, and different data transfer requirement.

Based on the above comparison, since there are significant differences in procedures and requirements between this state and the other two states, it is suggested to define a separate RRC state (Inactive) rather than as either a sub-state of RRC Connected or RRC Idle.

Observation 6: The Inactive state (or sub-state) is fundamentally distinct from both RRC connected and RRC idle states.

Proposal 2: Define separate RRC inactive state rather than a sub-state of RRC connected or RRC idle state.

  - RRC management

- Baseline procedure

The RRC connection management should take the three states into consideration. Hence the RRC procedure should support the RRC state transition as following which is the same as 5G.

Besides the RRC state transition, not only the RRC reestablishment procedure should also be supported to achieve fast recovery RRC connection, but also RRC reconfiguration message should be supported to (re-)configure RB configuration and radio resource, etc.

Due to above procedure are supported in 5G for the same intention, we suggest reusing 5G procedure as baseline for 6GR.

Proposal 3: The 5G NR procedures for the following scenarios can generally be adopted as a baseline:

- RRC state transition

- RRC Reconfiguration

- Reestablishment

- Fast transition and improvements to UE identification

In 5G NR, the S-TMSI (48bits) is used to identify the idle UE, and the I-RNTI (40bits) is used to identify the inactive UE. The transmission of large UE ID makes the UL coverage restriction. In order to transmit the message successfully, repetition may be performed which may consequently increase access latency. In addition, for access procedures (RRC setup/RRC resume/ RRC reestablishment) which involve CBRA procedure, the UE could be scheduled only after contention resolution. Specifically for RRC setup procedure, the connection between the RAN node and CN node could be setup only after the RAN node receives the RRCSetupComplete message which includes part of S-TMSI and the registered AMF. Further study on improvements to UE identification to reduce the access latency is needed in 6GR.

Observation 7: In NR, large UE ID restricts the UL coverage and increases access latency.

In order to improve the UL coverage and further reduce the access latency, to achieve fast transition, RAN2 consider to introduce unique UE ID mechanism. Based on the unique UE ID, the RAN node within the configured applicable area of the unique UE ID can identify the UE regardless of its RRC state. Since the applicable area is smaller than the TA, and the unique UE ID only identifies the correct UE within its applicable area, it can be shorter than the legacy UE ID e.g. S-TMIS and I-RNTI. The shorter UE ID could improve the UL coverage.

For a UE which has registered in the CN and assigned a unique UE ID by the network, when the UE (re)selects to one cell within the configured applicable area and expects to establish/resume the RRC connection, the UE could initiate a fast transition procedure which provides the unique UE ID in the Message 3/Message B e.g., via RRC signaling or MAC CE or scrambled with the UL data.  After the transmission of the Message 3/Message B, the UE monitors the PDCCH identified by the unique UE ID.

When the RAN node receives the unique UE ID via Message 3/Message B, it could identify the UE. Specifically, for idle UE the RAN node could identify UE’s full CN UE ID e.g. S-TMSI, and the registered CN. For inactive UE the RAN node could identify the UE’s AS context or its I-RNTI and then further identify its anchor RAN node. Hence, the RAN node could initiate the connection establishment with the registered CN for the UE if needed or perform data/signaling transmission. The scheduling information of the subsequent message/data is addressed by the unique UE ID.

In order to further reduce the latency, the anchor RAN node can provide the unique UE ID and its related UE AS context to other RAN nodes among the applicable area when the anchor RAN node establishes the UE context for the UE, or before/when the RAN node release the UE to the inactive state. This approach may reduce the context fetch time when the target RAN node receives the unique UE ID. But it could leave to network implementation. If the anchor RAN node doesn’t provide the UE context to the target RAN node, the target RAN node should fetch the UE context from the anchor node upon receiving the unique UE ID if the UE is in inactive state.

When the UE receives the PDCCH addressed by the unique UE ID, the UE considers the fast transition procedure is successful.

Observation 8: Shorter unique UE ID may improve UL coverage and further reduce the access latency.

For simple, for UE in inactive, the applicable area for the unique UE ID is the RNA.

If the UE is not allocated with such a unique UE ID, or if the selected cell is not in the configured applicable area, the UE should perform the baseline procedure e.g. using legacy UE ID e.g. S-TMSI, I-RNTI to access the network. For UE in connected, the RAN node among the applicable area use the unique ID for the UE .i.e. if cell switch is triggered, the unique UE ID is used in the target cell too. Considering the unique UE ID could be used to address the PDCCH as the C-RNTI, for simple, the same length as C-RNTI e.g. 16 bits could be used for this unique UE ID.

Proposal 4: In addition to the baseline RRC connection procedure, RAN2 considers to introduce a unique UE identification (same length as C-RNTI) which identifies the UE across RAN node(s) within a configurable valid area, applicable in any UE state (idle/inactive/connected) to support fast transition.

- Conclusion

This contribution has discusses the functions supported by inactive state and the RRC management and propose:

Inactive or sub-state

Observation 1: A specific UE ID can achieve lower signaling overhead compared to a CN UE ID, and it is applicable across a wider coverage area than the UE ID used in the connected state.

Observation 2: By decoupling RNA from TA, the network achieves higher flexibility in mobility management, as it avoids the trade-offs between frequent TAU procedures (caused by small unified areas) and UE context sharing complexity (caused by large unified areas)‌.

Observation 3: NW configuration based measurement without measurement reports can reduce UE power consumption by eliminating unnecessary signaling overhead.

Observation 4: SDT enables concurrent reduction in signaling overhead and power consumption while achieving fast data transmission.

Observation 5: In NR, inactive UEs can only monitor one PO set since Rel-17 even if they need to monitor both RAN-initiated paging and CN-initiated paging, preventing additional power consumption introduced.

Proposal 1: The Inactive state or sub-state shall support the following functionalities:

  - UE identification: allocation of a specific UE identifier (e.g., I-RNTI-like ID) for UE context identification purpose.

  - RAN-specific Notification Area management‌: implementation of a distinct RAN-based notification area management mechanism, independent of the Core Network area management.

  - UE based mobility: support of measurements without reporting, configured by the network.

  - Data transfer‌: support of transferring unicast data and/or signaling to/from the UE for small data.

  - Paging: unified PO set monitoring mechanism that simultaneously supports both RAN-initiated and CN-initiated paging procedures.

Observation 6: The Inactive state (or sub-state) is fundamentally distinct from both RRC connected and RRC idle states.

Proposal 2: Define separate RRC inactive state rather than a sub-state of RRC connected or RRC idle state.

RRC management

Proposal 3: The 5G NR procedures for the following scenarios can generally be adopted as a baseline:

- RRC state transition

- RRC Reconfiguration

- Reestablishment

Observation 7: In NR, large UE ID restricts the UL coverage and increases access latency.

Observation 8: Shorter unique UE ID may improve UL coverage and further reduce the access latency.

Proposal 4: In addition to the baseline RRC connection procedure, RAN2 considers to introduce a unique UE identification (same length as C-RNTI) which identifies the UE across RAN node(s) within a configurable valid area, applicable in any UE state (idle/inactive/connected) to support fast transition.

- References

[1] RAN2_131bis_MeetingReport, RAN2#131bis