3GPP TSG RAN WG2 #132          	                                                                  R2-2508069

Dallas, USA,  Nov. 17th - 21st

Source:	CATT

Title:	Discussion on Energy Efficiency

Agenda Item:	10.3.3.2

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, we had an initial discussion on energy efficiency with the following conclusions.

Design goal - Strive to develop solutions that consider network energy efficiency and UE power saving For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, considerations on energy efficiency in 6GR, including both time domain and frequency domain, are provided.

- Discussion

The table 1 summarizes NR UE power saving techniques and network energy saving techniques which have RAN2 impacts. Considering energy efficiency aspects related to system information and paging discussed in CP AI, the following energy efficiency aspects, including time domain and frequency domain, are discussed.

Table 1 NR energy saving techniques which have RAN2 impacts

| Rel-15 | Rel-16 | Rel-17 | Rel-18 | Rel-19 |
| --- | --- | --- | --- | --- |
| 1)C-DRX 2)Intra-band SSB-less SCell 3)SCell (De)activation 4) BWP adjustment | 1)DCP 2)SCell dormancy 3)RRM measurement relaxation | 1)PEI 2)assistance TRS 3)RLM/BFD relxation 4)Further RRM relaxation for redcap UE | 1)Cell DTX/DRX 2)inter-band SSB-less SCell 3)spatial&power domain adaptation | 1)on-demand SSB SCell 2)on-demand SIB1 3)adaptation of common channel(SSB, PRACH, paging) 4)LP-WUS in idle/inactive 5)RRM relaxation with LP-WUS in idle/inactive 6)LP-WUS(WUR) with PDCCH monitoring in connected |

  - Time domain

C-DRX/DCP/LP-WUS(WUR)

In NR Rel-15, in order to minimize unnecessary PDCCH monitoring and reduce UE power consumption from time domain, C-DRX was introduced. In Rel-16, to enable further power saving, DCI with CRC scrambled by PS-RNTI (DCP) was specified, i.e., the UE can be indicated to monitor the PDCCH during the next occurrence of the drx-onDurationTimer by a DCP. In Rel-19, the low-power wake-up receiver (LP-WUR) for UE was introduced to receive low-power wake-up signals (LP-WUS) and low-power synchronization signals (LP-SS) from network to trigger Main Radio (MR) of UE wake-up when needed. By low power receiver of UE in 5G, UE could achieve significant power saving gain in TR 38.869 [1].

In 6GR, the design target of the PDCCH monitoring adaptation should be perfectly match the different types of traffic patterns, such as immersive traffic, IM, web browsing, video streaming, gaming etc. That means the PDCCH monitoring behavior could be adaptively adjusted based on the characteristics of the traffic patterns. DRX with/without DCP is more suitable for periodic traffic, but may lead to power saving gain decrease or the latency increase for the aperiodic traffic. In Rel-19 LP-WUS, Option 1-1 and Option 1-2 were introduced. For Option 1-1, the UE starts drx-onDurationTimer after LP-WUS reception, while for Option 1-2, UE starts lpwus-PDCCH-MonitoringTimer after LP-WUS reception which does not follow DRX cycle. Option 1-2 of LP-WUS can well adapt to the uncertainty of the traffic arrival with guarantee of acceptable power saving gain. Therefore, LP-WUS which enables both lower transmission latency and acceptable power saving gain can be supported in 6G Day1. However, considering details on LP-WUS design and frequency range is still under RAN1 discussion, RAN2 can study power saving taking RAN1 progress into account.

In terms of function, DCP and LP-WUS has similar procedure, i.e. indicating whether the UE to monitoring the PDCCH for the upcoming time. And it was targeted to identify what UE/NW power saving features avoiding specifying multiple features for the same purpose in RAN2. Therefore, it is suggested support DL WUS/WUR operation for UE as a candidate technology in the 6GR SI.

Observation 1: DCP and LP-WUS/LP-WUR in NR has redundant procedure/function for UE power saving.

Observation 2: LP-WUS/LP-WUR shows outstanding performance on energy efficiency and latency reduction especially for traffic arriving irregularly.

Proposal 1: Consider DL WUS/WUR operation for UE as a candidate technology for PDCCH monitoring in RRC_CONNECTED, while excluding DCP from 6GR.

Cell DTX/DRX

In order to increase “sleep time” for gNB to save network energy consumption, Cell Discontinuous Transmission (DTX)/ Cell Discontinuous Reception (DRX) was introduced in Rel-18 NR. When cell DTX is configured and activated for the concerned cell, the UE may not monitor PDCCH in selected cases or does not monitor SPS occasions during cell DTX non-active duration. When cell DRX is configured and activated for the concerned cell, the UE does not transmit on CG resources or does not transmit a SR during cell DRX non-active duration. But to avoid impact on legacy UEs, this feature is only applicable to UEs in RRC_CONNECTED state and it does not affect Random Access procedure, SSB transmission, paging, and system information broadcasting. The network energy saving gain is limited as gNB has to perform SSB transmission, paging transmission, system information broadcasting, and monitor random access procedure.

To further improve network energy saving, Cell DTX/DRX could be extended to cover non-connected UEs. During the non-active period of Cell DTX/DRX, less frequent or sparser SSB and/or common signal transmissions could be jointly considered. Thereby further network energy saving can be achieved via increasing additional sleep opportunities in RRC non-connected state(s).

Observation 3: Cell DTX/DRX in NR can reduce network energy consumption. However this feature is only applicable to UEs in RRC_CONNECTED state but not applicable to transmission/reception of common channels/signals.

Proposal 2: Support Cell DTX/DRX in 6GR, extending Cell DTX/DRX to UEs in RRC non-connected state(s).

Measurement relaxation

In NR Rel-16, neighbour cells RRM measurement relaxation in RRC_IDLE/INACTIVE state based on not at cell edge criterion and/or low mobility criterion was introduced to reduce UE power consumption. And in NR Rel-17 further relaxed neighbour cells RRM measurement relaxation in RRC_IDLE/INACTIVE state for a stationary (e)RedCap UE was introduced.

In NR Rel-17, relaxation of UE measurements for RLM and/or BFD in RRC_CONNECTED state based on good serving cell quality criterion and low mobility criterion was introduced for UE power saving.

In NR Rel-19, power saving can also be achieved by allowing UEs supporting LP-WUS to relax serving cell measurements on MR, further relax neighbour cell measurements on MR and/or offload serving cell measurements from MR to LR.

In 6GR, measurement relaxation can be considered to reduce UE power consumption. Measurement relaxation mechanisms in NR in RRC_IDLE/INACTIVE state can be considered as a starting point. In addition, simplification and/or enhancements can be considered, e.g. whether and how to reduce the UE power consumption on measurement in RRC_CONNCTED state, whether and how to determine UE mobility state, and so on.

Observation 4: To achieve UE power saving, the following measurement relaxation mechanisms were introduced in NR:

-	Neighbor cell RRM measurement relaxation for UEs in RRC_IDLE/INACTIVE state,

-	Relaxed neighbor cell RRM measurements for stationary (e)RedCap UEs in RRC_IDLE/INACTIVE state

-	Relaxation of UE measurements for RLM and/or BFD in RRC_CONNECTED state

-	Relaxing serving cell and neighbour cell measurements on the MR, including offloading serving cell measurements from MR to LR with LP-WUS

Proposal 3: Measurement relaxation mechanisms in NR in RRC_IDLE/INACTIVE state, including neighbor cell RRM measurement relaxation and measurement relaxation/offloading with LP-WUS, are considered as a starting point for 6GR.

RACH adaptation

In NR Rel-18, RACH adaptation in time domain was introduced. It can dynamically adjust random access occasion in time domain, e.g. between sparse and dense random-access occasions, which can facilitate network energy saving.

In 6GR, for the purpose of network energy saving, RACH adaptation can be inherited from 5G.

Observation 5: In NR Rel-18, RACH adaptation in time domain was introduced for network energy saving.

Proposal 4: RACH adaptation in time domain can be inherited from 5G in 6GR.

  - Frequency domain

From frequency domain perspective, the following techniques were introduced for power saving, SCell (de)activation, BWP adjustment, SCell dormancy, etc.

CA

To enable reasonable UE battery consumption when CA is configured, an activation/deactivation mechanism of Cells is supported in5G. MAC CE is used to activate/deactivate the SCell. When an SCell is deactivated, the UE:

- does not need to receive PDCCH for or on the SCell;

- does not need to receive PDSCH on the SCell;

- cannot transmit RACH, PUSCH, SRS on the SCell;

- cannot report CSI for the SCell;

- is required to perform CQI measurements.

Furthermore, in order to accommodate diverse data arrival together with fast SCell activation when CA is configured, one dormant BWP can be configured for an SCell. A DCI is used to control entering/leaving the dormant BWP for one or more SCell(s) or one or more SCell group(s). If the active BWP of the activated SCell is a dormant BWP, the UE:

- stops monitoring PDCCH on the SCell;

- stops transmitting SRS/PUSCH/PUCCH on the SCell;

- continues performing CSI measurements, AGC and beam management, if configured.

It can be seen that SCell (de-)activation and dormant BWP have similar UE behaviour. In 6GR, a unified and energy-efficient design for SCell should be further studied to minimize signaling overhead and simplified UE behavior in CA operations.

In Rel-19 NES, in order to further minimize the power consumption of NW, on-demand SSB-based SCell operations are supported for UEs in RRC_CONNECTED. NW can send OD-SSB transmission activation/deactivation command to the UE to activate/deactivate OD-SSB transmissions. Considering CSI reporting on the SCell depends on SSB transmission, the SCell (de-)activation MAC CE and the OD-SSB transmission (de-)activation command have duplicated functions.

Observation 6: The similar UE behaviour for SCell (de-)activation, dormant BWP and on demand SSB-based SCell in NR introduces duplicated signalling and procedures.

Proposal 5: A unified and energy efficient design for SCell activation/deactivation, dormant BWP and on demand SSB-based SCell should be further studied for CA aiming to minimize redundant signaling and optimize power consumption in 6GR.

BWP

For the frequency domain resource adaptation in NR, the BWP adaptation based on traffic load was considered, which can achieve 16%–45% UE power saving gain according to the study in TR 38.840[2]. This gain is obtained by switching between BWPs with different bandwidths. The current BWP switching mechanism enables the gNB to adjust UE bandwidth according to traffic demand: when traffic is high, the UE can be instructed to operate on a wide bandwidth for data transmission, while during low traffic periods, the UE can be restricted to a narrow bandwidth, thereby saving energy by reducing RF and baseband processing. From the power saving gain perspective, if the BWP concept is still adopted in 6GR, BWP switching could remain a potential solution for frequency-domain resource adaptation.

Observation 7: BWP switching is beneficial on power saving to adapt various bandwidths in NR.

Proposal 6: If the BWP concept is still adopted in 6GR, BWP switching can be inherited for frequency-domain resource adaptation.

6G Spectrum

In the 6G system, UEs are expected to support a wider range of bandwidths for flexible transmission and it might support larger bandwidths than NR, as well as wider spectrum aggregation to improve transmission efficiency. However, from UE power saving’s perspective, UE does not always need the whole frequency-domain resources for traffic transmission. Furthermore, unlike NR, 6G is expected to be allocated increasingly fragmented spectrum resources with narrow bandwidths in the future. For UE allocated frequency-domain resources with a wide bandwidth or non-contiguous spectrum resources with large bandwidth for burst traffic transmission, the UE may not need to transmit or receive over the entire frequency-domain resources all the time. Consequently, frequency-domain resource adaptation for 6G, particularly in scenarios involving wide bandwidth or discontinuous spectrum resources should be further studied, with focus on both network and UE power efficiency.

Proposal 7: RAN2 should take energy efficiency as a key consideration in the study of new solutions for 6G spectrum management.

- Conclusion

In this contribution, we provide our views on the 6GR energy efficiency aspects, including time domain and frequency domain:

Time domain

C-DRX/DCP/LP-WUS (WUR)

Observation 1: DCP and LP-WUS/LP-WUR in NR has redundant procedure/function for UE power saving.

Observation 2: LP-WUS/LP-WUR shows outstanding performance on energy efficiency and latency reduction especially for traffic arriving irregularly.

Proposal 1: Consider DL WUS/WUR operation for UE as a candidate technology for PDCCH monitoring in RRC_CONNECTED, while excluding DCP from 6GR.

Cell DTX/DRX

Observation 3: Cell DTX/DRX in NR can reduce network energy consumption. However this feature is only applicable to UEs in RRC_CONNECTED state but not applicable to transmission/reception of common channels/signals.

Proposal 2: Support Cell DTX/DRX in 6GR, extending Cell DTX/DRX to UEs in RRC non-connected state(s).

Measurement relaxation

Observation 4: To achieve UE power saving, the following measurement relaxation mechanisms were introduced in NR:

-	Neighbor cell RRM measurement relaxation for UEs in RRC_IDLE/INACTIVE state,

-	Relaxed neighbor cell RRM measurements for stationary (e)RedCap UEs in RRC_IDLE/INACTIVE state

-	Relaxation of UE measurements for RLM and/or BFD in RRC_CONNECTED state

-	Relaxing serving cell and neighbour cell measurements on the MR, including offloading serving cell measurements from MR to LR with LP-WUS

Proposal 3: Measurement relaxation mechanisms in NR in RRC_IDLE/INACTIVE state, including neighbor cell RRM measurement relaxation and measurement relaxation/offloading with LP-WUS, are considered as a starting point for 6GR.

RACH adaptation

Observation 5: In NR Rel-18, RACH adaptation in time domain was introduced for network energy saving.

Proposal 4: RACH adaptation in time domain can be inherited from 5G in 6GR.

Frequency domain

CA

Observation 6: The similar UE behaviour for SCell (de-)activation, dormant BWP and on demand SSB-based SCell in NR introduces duplicated signalling and procedures.

Proposal 5: A unified and energy efficient design for SCell activation/deactivation, dormant BWP and on demand SSB-based SCell should be further studied for CA aiming to minimize redundant signaling and optimize power consumption in 6GR

BWP

Observation 7: BWP switching is beneficial on power saving to adapt various bandwidths in NR.

Proposal 6: If the BWP concept is still adopted in 6GR, BWP switching can be inherited for frequency-domain resource adaptation.

6G Spectrum

Proposal 7: RAN2 should take energy efficiency as a key consideration in the study of new solutions for 6G spectrum management.

- References

- TR 38.869 Study on low-power Wake-up Signal and Receiver for NR, V18.0.0 (2024-01)

- TR 38.840 Study on User Equipment (UE) power saving in NR, V16.0.0 (2019-06)