3GPP TSG RAN WG2 Meeting #132         	                             R2-2508082

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.3.1

Source:	Xiaomi

Title:	Consideration on 6GR data transfer and AI/ML framework

Document for:	Discussion and Decision

# Introduction

During RAN2 #131bis meeting, RAN2 identified several aspects for 6G data transfer study, including identifying requirement and protocol(s) of data collection framework for different types of data, studying termination points, applicable use cases, data size, frequency of data reporting, and QoS requirement, etc.

Furthermore, RAN2 also agreed basic functionalities to be supported by 6G AI/ML LCM framework, including capability reporting, applicability reporting, inference, monitoring, and activation/deactivation.

In this contribution, for 6G data transfer, we analyse requirements of transferring different types of data and then discuss the potential transport protocol/tunnel based on same termination points. Additionally, we also discuss some further considerations on AI/ML LCM framework and how to discuss AI/ML use cases in RAN2 for 6G evaluation.

# Data Collection and Transfer

## 6GR Data Collection Requirements

During RAN2 #131bis meeting, RAN2 has discussed requirements for AI/ML data within standardized data collection framework with following agreements:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

For UE-involved data collection in NW-side use cases, following data collection requirement should also be considered:

Efficient UE Participation

Besides minimizing impact to UE battery, processing and memory utilization, when UE is involved in data collection for NW-side use cases, it is also important to avoid UE spending extra effort (e.g., measurement) to serve the purpose of NW-side data collection. In 5G, there are mainly three NW-side data collection use cases, e.g., SON/MDT, QoE, and AI/ML NW-side data collection.

For SON/MDT, data collection is mainly used by network management for optimization (e.g., coverage, resource scheduling, mobility, etc). On the other hand, data collection related to service quality (e.g., throughput, packet delay, etc) can also be beneficial to UE, as network should at least guarantee the QoS requirement for different types of services. Though we understand those data are useful for network optimization and network awareness of service quality, SON/MDT data collection should only collect UE available data whenever possible, instead of requesting UE to perform additional measurement, which may cost further extra power consumption at the UE side. In 5G, this has been achieved by e.g., reusing RRM measurement for immediate MDT, reporting the measurement results whenever available, etc.

In Rel-19, network-side data collection for AI/ML NW-side model was introduced, where UE can optionally perform logging for NW-side data collection, to reach a balance between UE memory cost and power consumption caused by frequent measurement reporting.

Observation 1: Consideration of UE efficient involvement for NW-side use cases is already supported in 5GNR, e.g., UE reports SON/MDT reports whenever available, UE optionally logging for NW-side data collection, etc.

However, as observed in 5G, a UE needs to support almost a minimum 256kB memory if the UE wants to support multiple features for NW-side use cases till Rel-19, including:

- Logged MDT (a minimum 64kB memory);

- QoE (a minimum 64kB memory for CONNECTED, another minimum 64kB for IDLE/INACTIVE);

- AI/ML network-side data collection (a minimum 64kB memory).

This minimum memory requirement from multiple NW-side use cases significantly increases the cost at UE device when supporting multiple features at the same time. Additionally, whenever a new NW-side use case being supported in future releases, another memory size needs to be added by the UE, which is not a forward-compatible design.

On the other hand, depends on different UE implementations, the UE should be allowed to support larger memory size as an optional feature. Within the supported memory size, it is up to UE implementation about how to use the memory for different purposes.

Observation 2: In 5GNR, UE has been requested to support multiple minimum memory sizes for different use cases, which increases the cost at UE side. How UE utilizes its memory should be up to UE implementation.

In 5G, to support different purposes of data collection, e.g., MDT, AI/ML training, inference and monitoring, RRM, etc, the UE is configured by network with many duplicated configurations for similar measurement and reporting:

- L3 measurement:

  - Logged MDT configuration via LoggedMeasurmentConfiguration and report via UEInformationResponse;

  - Immediate MDT configuration via RRCReconfiguration (reusing RRM measurement framework) and report via MeasResults;

  - Mobility measurement configuration via MeasConfig and report via MeasResults.

- CSI measurement:

  - CSI feedback configuration via CSI-MeasConfig and report via CSI framework;

  - AI/ML inference configuration via CSI-ReportConfig with specific reportQuantity and report via CSI framework;

  - AI/ML network-side data collection configuration via CSI-LoggedMeasurementConfig and report via UEInformationResponse;

  - AI/ML UE-side data collection configuration via CSI-ReportConfig with specific reportQuantity for UE-side data collection, and report via CSI framework.

Moreover, Quality of Experience (QoE) was introduced to collect data for optimizing service quality based on users’ subjective experience from application layer, including video, gaming, etc. In LTE, QoE is only visible in application layer and 3GPP network (including RAN) is only used as the data transfer tunnel. In 5GNR, RAN visible QoE (RVQoE) is introduced to allow RAN to optimize radio resources based on application layer service quality. However, the UE needs to report the same QoE value(s) to both application layer (as container which terminated at RAN node first) and Uu interface (as separate parameters, e.g., appLayerBufferLevel, playoutDelayForMediaStartup).

Of course, if the data consumer is different, it would be difficult to use the same configuration and reporting signalling. However, as long as the data transfer tunnel (termination of data transfer) are the same, the duplicated measurement and reporting not only leads to a waste of air interface resources, but also increases UE complexity and power consumption. In 6GR, such duplicate measurement and reporting should be avoided from Day 1.

Observation 3: Even with the same termination, UE has been requested to send duplicate reports over the air interface for the same measurement in 5GNR.

Respect UE Preference

During RAN2 #131bis meeting, it was discussed whether UE should be able to reject requests for network use cases based on UE internal consideration.

In 5G AI/ML NW-side data collection, low power indication was introduced, indicating UE may not have sufficient power to perform data collection for NW-side model. Upon receiving this indication, NW is expected to release NW-side data collection configuration. However, due to undefined network behavior, there’s a big risk that UE may be continuously configured to perform NW-side data collection regardless UE’s low power state.

Moreover, UE may also prefer not to perform NW-side data collection due to other reasons (e.g., CPU occupancy, overheating, etc). It was discussed in [7], to guarantee UE performance, it is expected NW to respect UE preference for stopping data collection for NW-side use cases.

Observation 4: In 5G, even UE indicates its low power state, due to lack of defined network behaviour, UE may still suffer from bad user experience due to continuous data collection for NW-side use case.

Requirement applicability to sensing data type

Last but not least, it is FFS whether RAN2 can have common requirements across different use cases. Recalling AI/ML UE-side data collection requirement discussion on MNO controllability and visibility in 5G, RAN1/2 first discussed AI/ML use cases and further identified the need of UE-side data collection for UE-sided model training. Based on the potential use cases (e.g., AI/ML based beam management, AI/ML CSI prediction, etc), RAN2 further discussed different UE-side data collection options by considering different termination points.

Observation 5: In 5G, RAN2 first identified AI/ML use cases and termination points for UE-side data collection before studying visibility and controllability requirement of AI/ML UE-side data collection.

Compared with data types supported in 5G, sensing data is the only new data type that needs to be additionally supported in 6G. Depends on whether 3GPP network entities can provide sensing data processing/calculation or not, the above requirements may or may not be applicable to sensing data.

For the case where sensing result is calculated inside of 3GPP entities, e.g., sensing function (SF), it is clear that the data will be collected/transferred/processed inside of 3GPP. For such cases, all requirements above can be applicable to sensing data. However, due to user privacy, there’s another case that sensing data processing and sensing result calculation are only performed at UE or UE-related application layer (as discussed in Section 2.2.2). For such scenario, similar as UE-based positioning, data processing and result calculation are up to UE implementation, i.e., no need of MNO involvement.

Therefore, RAN2 needs to first discuss and understand sensing use cases and corresponding sensing data. Based on this, RAN2 can further discuss whether MNO controllability and visibility for sensing data is needed or not case by case.

Proposal 1: On requirements for standardized data collection framework:

- Efficient UE participation of data collection for NW-side use cases

  - Only collect UE available data whenever possible without additional UE measurement;

  - Optionally support data logging with a shared memory across all NW-side use cases;

  - Avoid duplicate measurements and reporting for same producer/consumer;

- Network to respect UE preference for starting/stopping data collection for NW-side use cases with a high-level specified network behaviour that the network should respect/follow the received UAI for network configuration;

- RAN2 to first conclude the study on sensing use cases and its required sensing data type(s), then discuss whether MNO controllability and visibility requirements is needed for sensing data or not case by case.

## Data Types

With the diverse service growth, explosive data growth brings the requirement for network to collect and transfer data supporting various services in 6GR. Services which need data collection and transfer includes the following cases:

- Data for network operation and management optimization (e.g., SON/MDT);

- Data for user experience improvement (e.g., QoE);

- Data for enabling new service(s) (e.g., AI/ML, sensing, etc).

It was agreed in RAN2 #131bis meeting to consider following aspects for diverse data types:

- 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). - For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

Before discussing the data types, several point to clarity:

During RAN2 #131bis meeting discussion, there were some discussions on whether there’s any ambiguity between measurements and data to be discussed under data collection framework. In our understanding, all data generated by 3GPP air interface are either measurement data or measurement-variant data. Measurement data refers to any data generated based on measuring reference signals. For example, RRM measurement, L1-RSRP, CSI feedback, etc. Measurement-variant data means the data after post-processing of measurement data. This may include logging of measurement data, statistic based on measurement data (e.g., RA report, HOF report, etc), etc. It is straightforward that measurement framework and/or CSI framework can be used for measurement data configuration and reporting (which is discussed in the companion contribution [8]). For measurement-variant data, the data collection framework may be needed to support data transfer between different terminations and different use cases.

Observation 6: 3GPP generated data can be categorized into two types:

1) Measurement data (i.e., data generated based on measurement of reference signals) via measurement framework and/or CSI framework;

2) Measurement-variant data (i.e., data after post-processing (e.g., logging, statistic, etc)) via potential new data collection framework.

Another key question is whether there’s a need to define QoS requirement for different types of data. In 5G, QoS characteristics are defined for traffic transferred via DRB, while only priority is defined for SRB. For 6G data transfer, whether to define QoS characteristics or only priority level depending on the data type and its use case.

As defined by SA2, following QoS characteristics are considered [x]:

1 Resource type (Non-GBR, GBR, Delay-critical GBR); 2 Priority Level; 3 Packet Delay Budget (including Core Network Packet Delay Budget); 4 Packet Error Rate; 5 Averaging window (for GBR and Delay-critical GBR resource type only); 6 Maximum Data Burst Volume (for Delay-critical GBR resource type only).

Compared with SRB priority level, QoS characteristics also provides other traffic transfer requirement, such as PDB, PER, MDBV, etc. Therefore, depending on whether there’s different delay/error/throughput requirements for different types of data, QoS requirement may be needed.

Observation 7: QoS characteristics need to be defined if different data types have different delay/error/throughput requirements.

In the below sections, we mainly focus on the new service data types (e.g., AI/ML related data, sensing data).

### AI/ML Data Types

During 5GNR study on AI/ML air interface, data collection for UE-side model, NW-side model and two-sided model has been studied in RAN1/2. Though AI/ML based mobility didn’t have any practice on size analysis for training/inference/monitoring data, the similar assumption can be made, as the requirement for AI/ML data with different purposes are common across all use cases.

Table 1 summarizes the key data types and their corresponding producer and consumer based on RAN1 LS in 5GA [1][2][3] and R20 RAN1 discussion on CSI compression two-sided model.

As it comes to 6GR, 5GA AI/ML use cases over the air interface can be considered at least to be supported in Day 1, including AI/ML beam management, CSI compression and CSI prediction, though the final decision is up to RAN1.

Based on this assumption, 5GNR study on data size/latency/frequency/producer-consumer for training/inference/monitoring data can be considered as baseline for the study of 6GR AI/ML related data.

Proposal 2: For AI/ML data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR AI/ML data transfer. RAN2 considers Table 1 as starting point for AI/ML data types study, including:

- For NW-side training data collection and transfer, data collection framework is considered as baseline and consider the following characteristics:

  - For logging data: low priority, delay tolerant/relaxed latency, low PER, large MDBV;

  - For non-logging data: medium priority, medium delay/near-real-time, low PER, medium MDBV;

- For inference data collection and transfer, at least for AI/ML based BM/CSI/Mobility, measurement framework and/or CSI framework is considered as baseline. New use cases need to be discussed case by case;

- For monitoring data collection and transfer, whether measurement framework/CSI framework or data collection framework needs to be discussed case by case and consider the following characteristics:

  - Medium priority, medium delay/near-real-time, low PER, medium MDBV.

Table 1. Typical Data Types of AI/ML air use cases

| Data Collection Purpose | Use Case(s) | Producer | Consumer | Typical data size (per data sample) for Transfer over air interface | Total Size | Typical latency requirement | Measurement framework or Data collection framework? | Need of QoS Characteristics? |  |  |  | Impacted WGs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Priority | PDB | PER | MDBV |  |
| Training | UE-sided model | UE | UE-side OTT server | Example in AI/ML BM: Up to 1.5Mbits assuming float 32 and 10 CSI-RS observation instances; | The total size is up to UE implementation how to collect/store the collected data. | Relaxed | Data collection framework | Low | Delay tolerant | Low | Large | RAN2 (led), RAN1, RAN3, SA2/SA5 |
|  | NW-sided model | UE | Training entity for NW-side model (e.g., gNB/OAM) | Example in AI/ML BM: Up to 1.5Mbits assuming float 32 and 10 CSI-RS observation instances; | Depends on 1) whether logging is supported 2) the size of UE logging 3) when NW retrieves logged data from UE | Non-logging: near-real-time Logging: Relaxed |  | Non-Logging |  |  |  |  |
|  |  |  |  |  |  |  |  | Medium | Medium | Low | Medium |  |
|  |  |  |  |  |  |  |  | Logging |  |  |  |  |
|  | Two-sided model | UE | gNB | ~ 9K bits |  |  |  | Low | Delay tolerant | Low | Large |  |
|  |  | gNB | UE-side OTT server | Example of dataset/model parameter transfer in CSI feedback: Option 4-1: 225MB Option 3a-1 without target CSI: (36kB – 52MB) |  | N/A |  | N/A |  |  |  | RAN3 (led), RAN1, RAN2, SA2/SA5 |
| Inference | UE-sided model | UE | UE | N/A |  | N/A | N/A | N/A |  |  |  | N/A |
|  | NW-sided model | UE | gNB | Size of L1/L3 measurement reports, CSI feedback (~ a few Kbits) |  | Time-critical | Measurement framework | No |  |  |  | RAN1 led use case(s): RAN1 (led), RAN2; RAN2 led use case(s): RAN2 (led) |
|  | Two-sided model | UE | gNB |  |  |  |  |  |  |  |  |  |
| Monitoring | UE-sided model | UE | gNB | Ground-truth measurement results and inference output; or performance results. Example 1: In BM, same as CSI feedback size (~ a few 1000 bits) Example 2: In AI/ML mobility, similar as L3 measurement reports | Depends on reporting periodicity and/or number of samples | Near-real-time | Depends on use case(s), measurement framework or data collection framework | Using measurement framework |  |  |  |  |
|  |  |  |  |  |  |  |  | N/A |  |  |  |  |
|  |  |  |  |  |  |  |  | Using data collection framework |  |  |  |  |
|  |  |  |  |  |  |  |  | Medium | Medium delay | Low | Medium |  |
|  | NW-sided model | UE | gNB | Size of L1/L3 measurement reports |  |  |  |  |  |  |  |  |
|  | Two-sided model | UE | gNB | L3 signaling as agreed by RAN1 in R20 CSI compression |  |  |  |  |  |  |  |  |

### Sensing Data Types

TRP monostatic

TRP-TRP bistatic

UE-TRP bistatic

UE monostatic

TRP-UE bistatic

UE-UE bistatic

Figure 1. Sensing Modes

During RAN #109 meeting, it has been agreed that 6GR should study all sensing modes [5].

- 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. - 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs.

Furthermore, it is also agreed that at least sensing use case ‘detection and/or tracking of passive objects’ need to be supported in 6GR within 6G RAN architecture [5].

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

In this section, we analyse different sensing data collection and transfer scenarios based on different sensing modes.

Depends on the use cases, the collected sensing data can further be requested by the following consumers:

Consumer Candidate 1: Non-UE-related Application layer (trusted 3rd party)

For the use case ‘detection and tracking of passive objects’ or ‘environment monitoring’, the collected sensing data can be consumed by the application layer which requests the sensing service.

Consumer Candidate 2: UE or UE-related Application layer (trusted 3rd party)

For use cases such as ‘intelligent driving’ and ‘motion detection’, the sensing data is consumed by the UE itself. For this scenario, considering the data security aspects, it is most likely the UE will request itself as the sensing data producer and consumer. Hence, there’s no need to perform data transfer over the air interface.

Even the UE itself has a low data processing capability and needs further processing on its sensing server which is located in application layer (i.e., UE-owned application layer), similar to UE-based positioning, such sensing data transfer is transparent to 3GPP network.

On the other hand, due to lack of sensing capability at the UE device, UE’s application layer may also request sensing data from the network.

Consumer Candidate 3: Network (wireless sensing service)

Network itself can consume the collected sensing data for ‘sensing for network optimization’ use cases, e.g., sensing assisted beam management, sensing assisted mobility, etc.

On the other hand, in our understanding, the most important thing for sensing data transfer discussion is the entity which is responsible for sensing data processing and sensing result calculation. This entity should be the termination point of measured and collected sensing data, instead of sensing result consumer. If the sensing data processing and sensing result calculation is inside of 3GPP, the sensing receiver may first transmit 3GPP sensing data to sensing processing entity (SF in UE/RAN/CN), where the sensing results can be further consumed by wireless sensing service or trusted 3rd party [TS 22.137]. Otherwise, the sensing data can be directly transfer to 3rd party for further processing and calculation without 3GPP impact (same as UE-based positioning).

Observation 8: Instead of sensing result consumer, sensing data transfer termination should consider the entity responsible for sensing data processing and result calculation.

Figure 2. Sensing Data Transfer

For 3GPP sensing data, the following measurement metrics were proposed and discussed during 5GNR ISAC RAN1 study [4], which may need to be transferred between entities:

  - Type 1 Raw channel data (e.g., channel matrix) [Data size: kB-GB];

  - Type 2 Partial raw data: Amplitude and phase profile of delay, and/or Doppler, and/or angle [Data size: hundreds to thousands of kB];

  - Type 3 Per path (associated with one delay) sensing results: Delay, Doppler, Angle, and Power per detected path for a given time stamp [Data size: tens to hundreds of bits];

  - Type 4 Per point sensing results: (Position, Velocity, and Power) or (Range, Velocity, Angle, and Power) per detected point for a given time stamp [Data size: tens to hundreds of bits];

  - Type 5 Final sensing results: Position and velocity for a detected object/target for a given time stamp, e.g., {position, velocity (, target ID)} [Data size: tens of bits]

    - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target

    - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is (not) reported

However, we understand that RAN1 will continue discuss which sensing metrics based on performance evaluation, including sensing data size for each sensing data type and its reporting frequency.

Observation 9: RAN1 will discuss sensing data metrics (5 types of sensing data) and its data size, reporting frequency in Rel-20.

On the other hand, no matter which types of sensing data is transmitted, sensing performance requirements for different use case(s) needs to be satisfied, which further raise the desire to define QoS requirement for transmitting different types of sensing data. Sensing performance requirements of different sensing use cases are defined in TS 22.137 [6], the detailed performance metrics can be found in Annex A. We observe that those performance requirements can further be mapped to QoS requirement for sensing data transmission in the following aspects:

| Sensing Performance Requirements | QoS Characteristics |
| --- | --- |
| Max sensing service latency | Priority Level |
| Max sensing service latency | Packet Delay Budget |
| Accuracy of positioning/velocity estimated by sensing; Sensing Resolution Missed detection False alarm | Packet Error Rate |
| Accuracy of positioning/velocity estimated by sensing; Sensing Resolution Refreshing rate | Maximum Data Burst Volume |

It is then straightforward that QoS characteristics of sensing data should be defined based on different sensing use cases. It is also observed that even within the same sensing scenario (e.g., object detection and tracking), there are four sensing service categories with different performance requirements defined. Therefore, based on different sensing service categories, different performance requirements and QoS characteristics may be further needed.

Observation 10: Different QoS characteristics of sensing data are needed for different sensing service categories/use cases, which can be defined based on sensing performance requirements.

Based on above discussion, we further summarize Table 2 for sensing data transfer and Table 3 for sensing data QoS characteristics with below proposal:

Proposal 3: For Sensing Data:

- Data metrics, data size, and reporting frequency, RAN2 needs to wait for further input from RAN1 evaluation on measurement metrics;

- RAN2 considers Table 2 as starting point for sensing data collection and transfer study, and focuses on the data transfer framework where sensing data producer is UE, entity for sensing data processing/result calculation is SF (either in RAN or in CN);

- RAN2 considers QoS characteristics in Table 3 as starting point for different sensing service use cases/categories, which are defined based on sensing performance requirements captured in Table 6.2.-1 TS 22.137.

Table 2. Sensing Data Transfer

| Sensing Modes | Producer | Entity for Sensing Data Processing/Result Calculation | Location of SF | Sensing data Transfer Path | Impacted WGs | Consumer | Sensing result Transfer Path | Sensing result Transfer impacted WGs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TRP monostatic TRP-TRP bistatic UE-TRP bistatic | TRP | SF | RAN | N/A | RAN3, RAN1 | Application layer | RAN -> Application layer | RAN3, SA2, RAN1 |
|  |  |  |  |  |  | Network | RAN -> CN or N/A |  |
|  |  |  | CN | RAN -> CN |  | Application layer | CN -> Application layer |  |
|  |  |  |  |  |  | Network | CN -> RAN or N/A |  |
|  |  | Application layer | N/A | RAN -> Application layer | RAN3, SA2, RAN1 | Application layer | N/A |  |
| UE monostatic TRP-UE bistatic UE-UE bistatic | UE | SF | RAN | UE -> RAN | RAN2 (led), RAN1, RAN3 | Application layer | RAN -> Application layer |  |
|  |  |  |  |  |  | Network | RAN -> CN or N/A |  |
|  |  |  | CN | UE -> CN |  | Application layer | CN -> Application layer |  |
|  |  |  |  |  |  | Network | CN -> RAN or N/A |  |
|  |  | Application layer | N/A | N/A | N/A | Application layer | N/A | N/A |

Table 3. Sensing Data QoS Characteristics

| Sensing Use Cases | Data Metrics/ Size/ Reporting Frequency | Sensing Service Category [6] | Sensing Performance Requirements | Need of QoS Characteristics? |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Priority | PDB | PER | MDBV |
| Detection and/or tracking of passive objects | Wait for RAN1 | 1 | As defined in Table 6.2-1 TS 22.137 [6] | Medium; | Low | Low | Large |
|  |  | 2 |  | Medium | Low | Low | Large |
|  |  | 3 |  | High/Medium | Low | Low | Large |
|  |  | 4 |  | High/Medium | Low | Low | Large |
| Environment monitoring |  | 5 |  | Low | High | Low | Large |
| Motion monitoring |  | 6 |  | Low | High | Low | Medium |
|  |  | 7 |  | Extreme High | Low | Low | Large |
| Sensing for network optimization |  | N/A | N/A | Low | Low | Low | Small |

## Data Collection Framework

Based on the above discussion, it is obvious that different types of data may terminates at different network entities. Depends on the producer and consumer of each use case, the data transfer tunnel could be different. But of course, if the producer and consumer are the same, the same solution of data transfer tunnel between same producer/consumer should be considered.

Observation 11: There’s no single solution/tunnel is perfect for different types of data, considering different pairs of producer/consumer, use case scenario, etc. But it is still possible to use the same tunnel for data transfer with the same termination points (i.e., same producer and consumer).

As summarized in above sections, data collection framework is a cross-WG topic which impacting RAN1, RAN2, RAN3, SA2 and SA5. Before addressing the detailed solution, it is important to understand what is the RAN2 focused area(s) for data collection framework.

First of all, as discussed earlier, measurement framework and data collection framework serve for different purposes. For data collection framework, it should focus on the procedure for measurement-variant data (i.e., data after post-processing (e.g., logging, statistic, etc)), including how logging is performed (optionally), how data is transferred from producer to consumer (e.g., transport protocol, bearer, format, etc) and how this transfer path is being configured by network. How data is measured is within remit of measurement framework.

Another aspect is the responsible WGs for defining QoS characteristics for different types of data transfer. Before 6G, traffic mainly comes from application layer, where SA2 will define QoS characteristics and its mapping to 5QI based on different requirements from applications. As analysed above, the key driving force for 6G data transfer is AI/ML and sensing data. On one hand, those data are generated internally inside RAN and may be further consumed by RAN, which needs RAN deeply involved in QoS characteristic definition. On the other hand, CN may also consume same types of data as in RAN, where SA2 can be responsible.

There are two possible way-forwards:

- For data only consumed by RAN, e.g., AI/ML NW-side training data collection, two-sided model monitoring, since those data are mainly related to air interface, RAN2 can define the QoS characteristics based on the knowledge of the concerned use case(s) led by RAN;

- For data that can be consumed by both RAN and CN, RAN2 informs SA2 on the need of defining different QoS characteristics for the concerned data type(s).

Another key aspect to clarify is the meaning of ‘protocol’. In our understanding, from data collection framework point of view, ‘protocol’ here means the transport layer protocol used for data transfer. Functionality protocol, based on different data types, such as data processing (e.g., segmentation, assembly, etc), functionality management should be discussed separately in AI/ML and sensing agenda. Based on this assumption, functionality protocol used for AI/ML and/or Sensing can leverage the same transport layer protocol for data transfer if termination points are the same.

Observation 12: Transport layer protocol of data collection framework is used for carrying data defined by AI/ML protocol or sensing protocol.

Proposal 4: Separate discussion on procedure/functionality management of the services (e.g., AI, sensing), and the data collection framework of the service:

- Data Collection Framework focuses on the procedure of measurement-variant data (i.e., data after post-processing (e.g., logging, statistic, etc)) and transport layer used for data transfer, including the following aspects:

  - Procedure(s) for measurement-variant data reporting, logging (optionally), etc;

  - Transport layer protocol(s)/radio bearer(s) for data reporting/transfer from producer to consumer;

  - On QoS characteristics:

  - RAN2 to discuss whether and how to define QoS characteristics of data consumed by RAN, including AI/ML NW-side training data collection, two-sided model monitoring, sensing data for network optimizations;

  - For data that can be consumed by both RAN and CN, RAN2 informs SA2 on the need of defining different QoS characteristics for the concerned data type(s), including UE-side data collection, sensing data terminated/consumed by SF (in either RAN or CN).

- Functionality protocol(s) for AI/ML and sensing (including data processing (e.g., segmentation, assembly, etc), management, etc) to be discussed in AI/ML air and sensing specific agenda, respectively.

Based on use cases and data producer/consumer summarized in above sections, the following transport layer protocol options can be considered for different termination points.

Option 1: Data producer: UE; Data consumer: CN/AF

Suitable data types include:

- AI/ML UE-side model data collection in Table 1.

- Sensing data when sensing data processing and result calculation is either SF in CN or application layer in Table 2.

Table 4. Transport Layer Protocol for UE-CN/AF Data Transfer

| E2E Data Transfer | Transport Layer Protocol between UE and CN | Tunnel Termination |
| --- | --- | --- |
|  | Option 1: Over NAS | CN |
|  | Option 2: Over IP/non-IP and UP-L2 NOTE: Including SUPL |  |
|  | Option 3: NAS over UP |  |

Option 2: Data producer: RAN, Data consumer: CN/OAM/AF

Suitable data types:

- Two-sided model dataset/model parameter in Table 1.

- Sensing data when TRP is the sensing data producer and further transferred to SF in CN or application layer in Table 2.

However, for this option, there’s no RAN2 impact on radio protocol, as data is transferred from RAN to CN/OAM/AF without air interface impact.

Option 3: Data producer: UE; Data consumer: RAN

Suitable data types:

- NW-side training data collection and monitoring data collection in Table 1.

- Sensing data when UE is the sensing data producer and further transferred to SF in RAN in Table 2.

Table 5. Transport Layer Protocol for UE-RAN Data Transfer

| E2E Data Transfer |  |  |
| --- | --- | --- |
| Transport Layer Protocol between UE and RAN |  |  |
| Option 1: Reuse RRC | Option 2: Over RRC | Option 3: Over CP-L2 |
| Option 4: Over UP-L2 | Option 5: Over IP and UP-L2 | Option 6: Over non-IP and UP-L2 |
| NOTE: The details of L2 protocol (e.g., SDAP, PDCP, RLC, MAC) is up to user plane discussion |  |  |

Based on the requirements of data transfer for different types of data as analysed in Table 1, Table 2 and Table 3, RAN2 can further analyse the pros/cons of different transport layer protocol options with below principles:

  - Forward compatibility to support various data sizes;

  - Whether support differentiation across different QoS/priority requirement for different data types;

  - Whether can maximize reuse of air interface radio protocols between traffic transfer and data transfer;

  - Whether can maximize reuse of air interface radio protocols between UE<->CN and UE<->RAN;

  - Specification effort.

Proposal 5: Transport layer protocol used for data transfer supports the following options:

- For ‘data produced by UE and consumed by CN/AF’, RAN2 to study two data transfer transport layer protocol options as shown in Table 4: Option 1) Over NAS, Option 2) Over IP/non-IP and UP-L2;

- For ‘data produced by RAN and consumed by CN/OAM/AF’, no RAN2 impact on transport layer protocol;

- For ‘data produced by UE and consumed by RAN’, RAN2 to study six data transfer transport layer protocol options as shown in Table 5: Option 1) Reuse RRC, Option 2) Over RRC, Option 3) Over CP-L2, Option 4) Over UP-L2, Option 5) Over IP and UP-L2, Option 6) Over non-IP and UP-L2;

- RAN2 to analysis pros/cons of transport layer protocol options based on the following aspects:

  - Forward compatibility to support various data sizes;

  - Whether support differentiation across different QoS/priority requirement for different data types;

  - Whether can maximize reuse of air interface radio protocols between traffic transfer and data transfer;

  - Whether can maximize reuse of air interface radio protocols between UE<->CN and UE<->RAN;

  - Specification effort.

# AI/ML Framework

## Life Cycle Management and Framework

Online training

On-device model training requires dedicated UE hardware and software support. Furthermore, it demands substantial computational resources due to numerous iterative operations and consumes significant storage for training data and intermediate parameters. These factors substantially increase UE complexity and cost. Furthermore, whether to support online training should be carefully evaluated case by case and only be supported with significant performance gain compared to offline training. Otherwise, the pains of having online training will be much larger than gains that 3GPP receives.

Observation 13: Online training significantly increases UE complexity, which needs a significant performance gain compared to offline training for justification.

AI/ML Protocol

Since AI/ML has been introduced in the late release of 5GNR, for backward compatibility, supporting AI/ML for air interface/NG-RAN as a RAN functionality didn’t introduce any impact to network architecture/protocol/radio bearer design, etc. By integrating AI/ML function as a functionality integrated inside of RAN, it requires every RAN node introducing AI/ML functions to update its hardware, e.g., integrating GPU inside of RAN node. This increases CPEX for each RAN node and further increases challenges towards market succeed to support native AI/ML in RAN.

Observation 14: 5GNR introduces AI/ML functionalities in late releases and the AI/ML architecture design is limited by existing NG-RAN architecture and protocol, which further constrains market deployment due to the cost.

Starting from 6GR, native AI/ML RAN architecture with flexible deployment options need to be studied. An AI/ML dedicated function entity can be considered for flexible deployment, either inside RAN or outside of RAN. However, supporting such flexible deployment at network side should be transparently to UE, to avoid duplicated configuration/reporting over the air interface for the same functionality.

Hence, from RAN2 point of view, in order to support such flexibly deployment with UE transparency, a separate AI/ML protocol needs to be studied on how to handle all AI/ML related functionalities, including LCM, data collection and transfer, etc.

Table 6. AI/ML Architecture and Protocol Design

| AI/ML Function Entity Inside of RAN | AI/ML Function Entity Outside of RAN |
| --- | --- |
| Unified Protocol Design over the air interface |  |

UE-autonomous decision on activation/deactivation/fallback/switching

During RAN2 #131bis meeting, following agreements were reached for 6G AI/ML LCM:

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

For functionality (de-)activation, fallback/switching, only ‘NW decision NW-initiated’ AI/ML management is supported in 5G. For 6G, with more and more attention on user experience, UE autonomous decision used to assist on AI/ML air interface use case(s) becomes more and more important. This includes ‘UE decision, event-triggered’ and ‘UE autonomous and decision reported to network’, which were being studied in Rel-18.

Proposal 6: For 6G AI/ML framework:

- Avoid on-device training;

- Study protocol design for AI/ML functionalities to support UE-transparent flexible network deployment of AI/ML functionality, e.g., define AI/ML protocol for AI/ML LCM;

- Support UE-autonomous decision (e.g., UE decision event triggered, UE autonomous decision and decision reported to network) in functionality (de-)activation/fallback/switching.

## AI/ML use cases

5GA has studied the feasibility of enabling AI/ML for multiple use cases, including RAN1-led AI/ML beam management, CSI feedback, CSI prediction, positioning accuracy enhancement, and RAN2-led AI/ML mobility optimization. During the study in 5G, simulation assumptions and evaluations were discussed to prove the feasibility and benefit of AI/ML.

We observed that RAN1 already started to study AI/ML use cases since RAN1 #122 meeting, and further agreed on how to select/evaluate among diverse AI/ML use cases for 6G with following agreements:

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

In general, we agree that most of the above information is also essential for RAN2 AI/ML use case evaluation, except collaboration/interaction between UE and NW and specification impact.

Regarding collaboration/interaction between UE and NW, based on observation from RAN1 summarized table of different use cases in RAN1 #122bis meeting, this bullet doesn’t provide any additional information compared to ‘model location for inference’.

Example from RAN1 #122bis meeting Chair Notes [Chair notes RAN1#122b eom3]: Sub-use case; Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain; Sub-case B: Superimposed pilot; Sub-case C: DMRS free. Model location for inference; UE-sided model for DL or NW-sided model for UL; UE-sided model for DL NW-sided model for UL Two-sided model5; UE-sided model for DL1 NW-sided model for UL3,4 Two-sided model2,5. Collaboration/interaction between UE and NW; Similar to UE-sided or NW-sided model as NR; Similar to UE-sided or NW-sided model as NR Similar to two-sided model as NR; Similar to UE-sided model as NR Similar to NW-sided model as NR Similar to two-sided model as NR.

Moreover, due to lack knowledge of how basic system design works, specification impact can be discussed in later phase.

In the end, as for simulation results, in 5G-A, RAN2 has done a number of simulations for AI/ML based mobility, including the accuracy and generalization evaluation. We understand that, if AI/ML based mobility is agreed to be supported in 6G, there’s no need to repeat the evaluation work to justify its feasibility in 6G. AI/ML mobility can be smoothly studied in 6G without further simulation.

Therefore, we suggest to study RAN2-specific AI/ML use cases with following information as inputs for RAN2 6GR AI/ML use cases identification/categorization:

Proposal 7: For RAN2 6GR AI/ML use cases identification/categorization, RAN2 to study based on the following aspects:

- Definition of each (sub-)use case, including at least AI/ML model input/output;

- For new RAN2 AI/ML use cases (i.e., except AI/ML based mobility studied in 5G) - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results, including label construction (if applicable), whether/how to obtain label data for model training, etc;

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model;

- Specification impact can be discussed in later phase of 6G SI.

# Conclusion

6G Data Collection Requirement

Proposal 1: On requirements for standardized data collection framework:

- Efficient UE participation of data collection for NW-side use cases

  - Only collect UE available data whenever possible without additional UE measurement;

  - Optionally support data logging with a shared memory across all NW-side use cases;

  - Avoid duplicate measurements and reporting for same producer/consumer;

- Network to respect UE preference for starting/stopping data collection for NW-side use cases with a high-level specified network behaviour that the network should respect/follow the received UAI for network configuration;

- RAN2 to first conclude the study on sensing use cases and its required sensing data type(s), then discuss whether MNO controllability and visibility requirements is needed for sensing data or not case by case.

General aspects of data types

Observation: 3GPP generated data can be categorized into two types:

1) Measurement data (i.e., data generated based on measurement of reference signals) via measurement framework and/or CSI framework;

2) Measurement-variant data (i.e., data after post-processing (e.g., logging, statistic, etc)) via potential new data collection framework.

Observation: QoS characteristics need to be defined if different data types have different delay/error/throughput requirements.

AI/ML Data Type

Proposal 2: For AI/ML data, assumption on data size/latency/frequency used in 5GNR can be considered as baseline for 6GR AI/ML data transfer. RAN2 considers Table 1 as starting point for AI/ML data types study, including:

- For NW-side training data collection and transfer, data collection framework is considered as baseline and consider the following characteristics:

  - For logging data: low priority, delay tolerant/relaxed latency, low PER, large MDBV;

  - For non-logging data: medium priority, medium delay/near-real-time, low PER, medium MDBV;

- For inference data collection and transfer, at least for AI/ML based BM/CSI/Mobility, measurement framework and/or CSI framework is considered as baseline. New use cases need to be discussed case by case;

- For monitoring data collection and transfer, whether measurement framework/CSI framework or data collection framework needs to be discussed case by case and consider the following characteristics:

  - Medium priority, medium delay/near-real-time, low PER, medium MDBV.

Sensing Data Type

Observation: Instead of sensing result consumer, sensing data transfer termination should consider the entity responsible for sensing data processing and result calculation.

Proposal 3: For Sensing Data:

- Data metrics, data size, and reporting frequency, RAN2 needs to wait for further input from RAN1 evaluation on measurement metrics;

- RAN2 considers Table 2 as starting point for sensing data collection and transfer study, and focuses on the data transfer framework where sensing data producer is UE, entity for sensing data processing/result calculation is SF (either in RAN or in CN);

- RAN2 considers QoS characteristics in Table 3 as starting point for different sensing service use cases/categories, which are defined based on sensing performance requirements captured in Table 6.2.-1 TS 22.137.

Data Collection Framework

Observation: Transport layer protocol of data collection framework is used for carrying data defined by AI/ML protocol or sensing protocol.

Proposal 4: Separate discussion on procedure/functionality management of the services (e.g., AI, sensing), and the data collection framework of the service:

- Data Collection Framework focuses on the procedure of measurement-variant data (i.e., data after post-processing (e.g., logging, statistic, etc)) and transport layer used for data transfer, including the following aspects:

  - Procedure(s) for measurement-variant data reporting, logging (optionally), etc;

  - Transport layer protocol(s)/radio bearer(s) for data reporting/transfer from producer to consumer;

  - On QoS characteristics:

  - RAN2 to discuss whether and how to define QoS characteristics of data consumed by RAN, including AI/ML NW-side training data collection, two-sided model monitoring, sensing data for network optimizations;

  - For data that can be consumed by both RAN and CN, RAN2 informs SA2 on the need of defining different QoS characteristics for the concerned data type(s), including UE-side data collection, sensing data terminated/consumed by SF (in either RAN or CN).

- Functionality protocol(s) for AI/ML and sensing (including data processing (e.g., segmentation, assembly, etc), management, etc) to be discussed in AI/ML air and sensing specific agenda, respectively.

Proposal 5: Transport layer protocol used for data transfer supports the following options:

- For ‘data produced by UE and consumed by CN/AF’, RAN2 to study two data transfer transport layer protocol options as shown in Table 4: Option 1) Over NAS, Option 2) Over IP/non-IP and UP-L2;

- For ‘data produced by RAN and consumed by CN/OAM/AF’, no RAN2 impact on transport layer protocol;

- For ‘data produced by UE and consumed by RAN’, RAN2 to study six data transfer transport layer protocol options as shown in Table 5: Option 1) Reuse RRC, Option 2) Over RRC, Option 3) Over CP-L2, Option 4) Over UP-L2, Option 5) Over IP and UP-L2, Option 6) Over non-IP and UP-L2;

- RAN2 to analysis pros/cons of transport layer protocol options based on the following aspects:

  - Forward compatibility to support various data sizes;

  - Whether support differentiation across different QoS/priority requirement for different data types;

  - Whether can maximize reuse of air interface radio protocols between traffic transfer and data transfer;

  - Whether can maximize reuse of air interface radio protocols between UE<->CN and UE<->RAN;

  - Specification effort.

AI/ML LCM Framework

Observation: Online training significantly increases UE complexity, which needs a significant performance gain compared to offline training for justification.

Proposal 6: For 6G AI/ML framework:

- Avoid on-device training;

- Study protocol design for AI/ML functionalities to support UE-transparent flexible network deployment of AI/ML functionality, e.g., define AI/ML protocol for AI/ML LCM;

- Support UE-autonomous decision (e.g., UE decision event triggered, UE autonomous decision and decision reported to network) in functionality (de-)activation/fallback/switching.

AI/ML use case

Proposal 7: For RAN2 6GR AI/ML use cases identification/categorization, RAN2 to study based on the following aspects:

- Definition of each (sub-)use case, including at least AI/ML model input/output;

- For new RAN2 AI/ML use cases (i.e., except AI/ML based mobility studied in 5G) - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results, including label construction (if applicable), whether/how to obtain label data for model training, etc;

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model;

- Specification impact can be discussed in later phase of 6G SI.

# Reference

[1] R1-2308730	Reply LS on Data Collection Requirements and Assumptions

[2] R1-2310681	Reply LS on Data Collection Requirements and Assumptions

[3] R1-2410922	LS on signalling feasibility of dataset and parameter sharing

[4] R1-2507427	Summary #5 on evaluations for NR ISAC

[5] RP-252960	pCR for 38.914 on Requirements of new and existing services for 6G

[6] TS 22.137	Service requirements for Integrated Sensing and Communication

[7] R2-2508076	UE Capability pain points and considerations	Xiaomi

[8] R2-2508085	Discussion on 6G mobility		Xiaomi

# Annex A – TS 22.137 Sensing Performance Requirement

# 6 5G wireless sensing performance requirements ## 6.1 Description 5G wireless Sensing service is required to fulfil different performance requirement (e.g., accuracy, resolution, latency, etc.) based on the characteristics of one or multiple target object(s) and/or the environment to be sensed in a target sensing service area. The following set of key performance requirements is used and shown for each service scenario in Table 6.2-1. - Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude. - Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity. - Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy. - Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude. - Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results. NOTE 1: An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive. - False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results. NOTE 2: An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative. - Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface. - Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results. 5G wireless sensing performance requirements are applied to 3GPP sensing data and sensing results. ## 6.2 Requirements The 5G system shall be able to provide sensing results with the performance requirements in Table 6.2-1. Table 6.2-1: Performance requirements for 5G Wireless sensing Scenario; Sensing service category; Confidence level [%]; Accuracy of positioning estimate by sensing (for a target confidence level); Accuracy of velocity estimate by sensing (for a target confidence level); Sensing resolution; Max sensing service latency [ms]; Refreshing rate [s]; Missed detection [%]; False alarm [%]; Sensing service description in a target sensing service area. Horizontal [m]; Vertical [m]; Horizontal [m/s]; Vertical [m/s]; Range resolution [m]; Velocity resolution (horizontal/ vertical) [m/s x m/s]. Object detection and tracking; 1; 95; 10; 10; N/A; N/A; 10 [3]; 5 [3]; 1000; 1; 5; 2; Indoor/outdoor (e.g., detection of human, UAV). 2; 95; 2; 5; 1; N/A; 1; 1; 1000; 0.2; 0.1 to 5; 5; Outdoor (e.g., detection of human, UAV). 3; 95; 1; 1; 1 [3], [4]; 1; 1 [3], [4]; 1 x 1 [3]; 100 [2], or 1000 (NOTE 3); 5000 for detection in highway; 0.05 to 1; 2; 2; Indoor/outdoor (e.g., detection and tracking of human, animal, UAV). 4; 99 for public safety, otherwise, 95; 0.5; 0.5; 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1; 1.5 for pedestrian; 0.5; 0.5 x 0.5 for factories; 100 to 5000; 0.1; 1; 3; Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle). Environment monitoring; 5; 95; 10; 0.2 (NOTE 4); N/A; N/A; N/A; N/A; 60000; 60 to 600; 0.1 to 5; 3; Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring). Motion monitoring; 6; 95; N/A; N/A; N/A; N/A; N/A; N/A; 60000; 60; 5; 5; Human motions and activities obtained by sensing (NOTE 5). 7; 95; 0.2; 0.2; 0.1; 0.1; 0.375; 0.3; 5 to 50; 0.1; 5; 5; Human hand gestures obtained by sensing (NOTE 6). NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources..