3GPP TSG-RAN2 #132	R2-2508094

Dallas USA, Nov 17th – 21st, 2025

Agenda Item:	10.3.1.3

Source: 	Xiaomi

Title:  	Consideration on 6G UP Scheduling and retransmission

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [1] was approved with the following scope highlighted in yellow regarding to 6G user plane:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In last RAN2 #131bis meeting, it was agreed that:

=> Support at least the following scheduling schemes: dynamic grant and configured grant. Further study configured grant. => Study need for scheduling enhancements to address the SR/BSR/DSR latency => Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we provide our views on 6G UP scheduling and retransmission enhancement.

# Discussion

## LCP enhancement

In LTE and NR, uplink resource acquisition relies on Buffer Status Reports (BSR), Scheduling Requests (SR), and Random Access Channel (RACH) procedures. Concurrently, the Logical Channel Prioritization (LCP) function in the MAC layer multiplexes data from different logical channels with different QoS requirements(e.g., high-priority control packets vs. best-effort data) into a single MAC PDU One important functionality of LCP is to allocate uplink resources to different logical channels based the logical channel priority, PBR and BSD, which is to meet each LCH's QoS and at the same time ensure a certain level of fairness among LCHs. In 6G, this basic LCP functionality would still be needed.

One particular area we can look into in 6G is whether LCP requires architectural modifications to improve implementation flexibility and user experience. In 5G NR, the UE selects logical channels and allocates data volumes during LCP based on static priority parameters. Specifically, the UE maintains the LCP priority for each logical channel by NW's configurations. Hence, complete or partial starvation still occurs when high-priority LCHs monopolize resources, starving lower-priority channels. Rel-18 XR enhancements partially address this by incorporating deadline awareness (e.g., using Delay Status Reports to let gNB aware low priority but delay critical data, and thereby allowing gNB allocating more resources to cover both high priority data and low priority but delay critical data). And in rel-19, priority boosting was introduced to increase the priority of LCHs with delay critical packets. However, priority boosting applies only at the entire LCH level and there is no support for intra-LCH prioritization (e.g., differentiating urgent control packets from regular data within the same channel). And boosting mechanisms activate solely based on packet deadlines and ignore other critical QoS dimensions.

Moreover, both Prioritized Bit Rate (PBR) and Bucket Size Duration (BSD) remain statically configured by the network. This rigidity prevents bucket size (Bj) adjustments from adapting to dynamic network conditions such as congestion deterioration. For such case, fixed PBR continues token accumulation despite reduced channel capacity, the consequence is  all UEs accumulate inflated Bj values and schedulers allocate excessive resources to 'token-rich' UEs based on virtual Bj metrics. This wastes air interface resources and starves other UEs.
In addition, application requirements are inherently dynamic, fluctuating with user activity, network load, and service characteristics. Static LCP configurations cannot adapt to these shifts, resulting in suboptimal resource allocation. This is especially problematic for delay-sensitive services (XR, cloud gaming, video streaming), where stringent latency bounds demand precise rate enforcement. 6G would further acknowledge these limitations, highlighting the need for an evolved LCP framework with finer-grained control, such as per-QoS-flow scheduling within a DRB.

In summary, 6G stands to benefit significantly from reduced uplink latency and enhanced support for diverse traffic. These goals can be achieved through QoS-aware, adaptive enhancements that improve coordination with upper layers for traffic awareness. For instance:

- Dynamic Priority: enforce priority adaptation by the content more dynamically;

- Dynamic Bj: enforce adaptation for the bucket size adjustment.

Observation 1: The legacy LCP procedure leads to LCH starvation, resource inefficiency, and inflexibility in handling dynamic traffic requirements—particularly for delay-sensitive applications.

Proposal 1: Study enhanced LCP mechanism allowing dynamic adjusting of LCP parameters.

## SR/BSR/DSR scheduling latency

Low user plane latency is critical both for low latency services, but also for other services. For services such as eMBB, it allows fast transport layer feedback, which also translates into higher achievable data rate. The dynamic UL scheduling requires the NW to have timely information on buffered data at the UE, which is obtained with BSR and/or DSR. When UL resources are already available, e.g. during a data transfer, BSR and/or DSR can be provided by the UE without delay. However, for the initial UL resource acquisition, when no UL resources are available, it is not the case. This makes it the weak point of the UL scheduling latency.

In 6G, it is expected that more low latency services such as XR will be supported. Moreover, new transport layers protocol such as QUIC gain more and more attraction and would benefit from reduced latency. It appears essential to ensure that the initial UL resource acquisition does not become the bottleneck of the UL scheduling latency.

Regarding scheduling enhancements to address the SR/BSR/DSR latency, several directions could be considered. Some of those directions were already studied in the past, for instance in the LTE Latency Reduction WI.

Keep SR/BSR, while optimizing SR:

One of the main contributors to initial UL scheduling is the SR waiting time. The other factors are actually quite low. The TTI lengths and processing time are much lower compared to typical SR periodicity, and might even lower in 6G. The RTT can also be neglected (33us for 5km cell), which means that multi-step approach per-se may not be a problem for latency.

Obviously, one issue here is the resource overhead of allocating SR with short periodicity to connected UEs. However, it could be investigated whether any enhancements could help to configure more frequent D-SR to help reduce latency.

Observation 2: Enhancements allowing more frequent SR can help to reduce scheduling latency.

Enhanced SR (to avoid BSR transmission step):

Instead of transmitting 1 bit in SR, more information could enable the NW to directly schedule an UL grant.

In our view, this direction seems quite challenging as SR is a dedicated resource, and it seems already difficult for the NW to accommodate that single bit with low periodicity. Another issue is that the enhanced SR would likely have a lot coarser granularity than the BSR, which would increase quantization errors for the TBS size, leading to bad spectrum efficiency for the first initial grant.

Observation 3: Enhancing SR to skip BSR may be challenging due to increased dedicated resources reservation.

Shared CG/CB-CG for BSR/DSR transmission (avoiding SR step):

This approach is attractive, as it would allow to send to the NW the full picture of UE buffer status directly. It is a natural direction to go in order to avoid the overhead of reserving dedicated resources.

In our understanding, this would be already feasible in NR. CG Type 1 can be configured as a contention-free shared resource to multiple UEs (discriminated by different DMRS sequences), and it could be used to transmit the BSR. Hence, one potential baseline for shared CG could be the NR CG Type 1.

An alternative or complement would be to consider contention-based CG (CB-CG), which can provide better capacity compared to shared CG. And compared to 2-step RA, the benefit of CB CG is that it doesn't require PRACH resources, which is kind of beneficial if UE is in connected mode and UL is already synchronized.

One main issue here is the collision. If collision happens once, it will result in the same latency as SR. If collision happens multiple times, it will actually result in longer latency. So, it is critical to keep the collision rate low. In REL-19 IOT NTN CB-Msg3 simulation discussion, the target collision rate is set to 1% [1], which also means that the resource utilization ratio is 1%. So, CB-CG based BSR/DSR is at the price of low resource utilization, compared with DG scheduled BSR/DSR. The larger and denser the CB resources, the worse the system resource efficiency. And as CB-CG resources require to transmit way more bits than SR, it is likely that CB-CG will consume far more resources than SR, although SR is dedicated resource.

Observation 4: CB-CG for BSR/DSR achieves low latency at the price of low resource efficiency compared with DG scheduled BSR/DSR.

Above 3 directions involve SR design or shared CG design. They should be discussed in coordination with RAN1. In addition, some other directions are not related to RAN1.

NW pre-scheduling enhancements:

There are situations where the NW might predict UL data transmission, and send UL grant proactively. This is made easier by allowing the UE to skip transmission when there is no data to transmit, which was one of the output of the LTE Latency Reduction WI. We think that this mechanism should still be possible, and it can be also useful to investigate whether any enhancements could be done in this area.

UE pre-requesting enhancements:

Similarly, the UE may in some cases predict UL data arrival and e.g. trigger BSR in advance. This was for instance implemented for IAB with “Pre-emptive Buffer Status Reporting” MAC CE. In our view there could be some possible ways to implement this kind of enhancements in 6G.

Proposal 2: Study mechanisms to allow fast/early transmission of buffer/delay status with a reasonable resource efficiency compared to 5G SR/BSR/DSR transmission.

## BSR/DSR unification

In NR, scheduling information reporting is based on the BSR reporting early in Rel-15 which conveys UL data volume per LCG without the delay status of the buffered data. In Rel-18 and Rel-19 XR, in order to fulfil the tight delay budget for delay sensitive service (e.g. XR), single entry Delay Status Report (DSR) for reporting at most single pair of buffer size/remaining time per LCG and multiple entry DSR for reporting multiple pairs of buffer size/remaining times per LCG were introduced to support delay-aware handling at gNB. The current BSR reporting mechanism reports the total uplink data volume in the terminal, while DSR exclusively reports urgent uplink data requiring low latency. Their reporting triggers operate under completely independent triggering conditions.

We analyze the triggering mechanisms of BSR and DSR to identify issues arising from their independent design:

- Independent triggering conditions causing conflicts/redundancy:

A Scenario is a logical channel simultaneously meets both BSR and DSR triggering conditions (e.g., new latency-critical data arrives in an empty buffer, triggering BSR; while its discardTimer falls below remainingTimeThreshold, triggering DSR). For such case:

- Signaling redundancy: The UE must generate and potentially transmit two separate MAC CEs (BSR and DSR), increasing uplink control signaling overhead.

- SR trigger conflict: Without available UL resources, both processes may independently trigger Scheduling Requests (SR). If BSR and DSR are triggered by different LCH, separate SR may be send if different LCHs are configured with different SR resources.

- Independent cancellation mechanisms causing redundancy:

The BSR cancellation condition is the UL Grant accommodates all pending data but insufficient for BSR CE plus the subheader, or if a sufficiently new BSR CE is included. DSR cancellation condition is the associated PDCP SDU is discarded, included in a DSR CE, or transmitted in a MAC PDU. There would be potential redundancy:

- If a UL Grant transmits data without DSR CE (due to resource limits or UE implementation choices), DSR remains uncanceled (since the SDU was sent but not reported via DSR CE). The UE may continue reporting stale latency states.

- When a UL Grant guarantees the DSR MAC CE be priority transmission over BSR MAC CE, all pending BSRs still not be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE, leading potentially triggering unnecessary SRs.

In Rel-19 XR, we discussed those issues but considering the time limit, they were not resolved.

- Degraded network scheduling from incomplete state information:

The gNB receives BSR and DSR reports at different times, e.g., when a DSR MAC CE is reported but forcing it to manually reconstruct the full logical channel state (e.g., data volume, latency urgency). This increases scheduler complexity and latency. If BSR reports lag behind DSR, schedulers may make suboptimal decisions based on incomplete data lacking the picture of the whole buffer status of UE leading resource insufficiency and fragmented transmission and added latency . On the other hand, if BSR arrives first, scheduler allocates large resources without urgency awareness leading to resources wasted on non-urgent data. While separate reporting is protocol-compliant, the time-sensitive coordination between buffer volume and delay context makes joint reporting fundamentally more efficient in dynamic radio conditions. Hence, the NW can configure that UE report both when UE has such informations.

Observation 5: Current BSR and DSR signaling are triggered and canceled separately leading to inefficiency of design:

- independent triggering conditions causing conflicts;

- Independent cancellation mechanisms causing redundancy;

- Degraded network scheduling from incomplete state information.

In 6G, the existing BSR/DSR procedure in 5G should be considered as a starting point. And For 6GR, there is an opportunity to jointly consider these similar BSR/DSR functions in 6G Day-1. A unified buffer status reporting with UL data volume information while adding delay status to enable truly delay-aware scheduling would be considered.

Proposal 3: 6G should study a unified buffer and delay status reporting procedure.

## Fast ARQ retransmission

In 5G, ARQ retransmission relies on RLC status report, which is triggered either by t-Reassembly expiry or Polling. t-Reassembly configuration is generally large, wait for its expiry introduces a lot of latency. Polling in theory can reduce the latency of RLC status report, but it comes at the price of significant signalling overhead of frequent RLC status report. In 6G, we should consider how to enable fast ARQ retransmission. One possible way to do this is based on HARQ transmission status.

In 5G, for uplink transmission, the HARQ ACK/NACK is not feeded back to UE by gNB. Instead, network will schedule a retransmission if decoding fails. After reaching a maximum retransmission times, network will give up the retransmission and may schedule a new transmission instead. From UE point of view, UE has no idea whether the uplink HARQ transmission finally succeeds or not. If network can provide this HARQ failure information to UE, UE can use this information to trigger ARQ retransmission.

For downlink transmission, UE sends ACK/NACK feedback to network. However, ACK/NACK may flip. If that happens, network will stop HARQ retransmission and rely on ARQ retransmission to resolve it. In this case, UE will receive a new transmission scheduling following a NACK indication. If UE can tell this situation to network, it will facilitate network fast ARQ retransmission.

Observation 6: 5G ARQ retransmission relies on RLC status report, which is not in a timely manner.

Proposal 4: RAN2 considers to study the following areas for improving ARQ/HARQ efficiency:

- Study mechanisms to allow UE to aware the UL HARQ transmission status (e.g. succuss or not);

- Study mechanisms to allow the UL HARQ ACK/NACK flip issue for downlink transmission.

# Conclusions

Observation 1: The legacy LCP procedure leads to LCH starvation, resource inefficiency, and inflexibility in handling dynamic traffic requirements—particularly for delay-sensitive applications.

Proposal 1: Study enhanced LCP mechanism allowing dynamic adjusting of LCP parameters.

Observation 2: Enhancements allowing more frequent SR can help to reduce scheduling latency.

Observation 3: Enhancing SR to skip BSR may be challenging due to increased dedicated resources reservation.

Observation 4: CB-CG for BSR/DSR achieves low latency at the price of low resource efficiency compared with DG scheduled BSR/DSR.

Proposal 2: Study mechanisms to allow fast/early transmission of buffer/delay status with a reasonable resource efficiency compared to 5G SR/BSR/DSR transmission.

Observation 5: Current BSR and DSR signaling are triggered and canceled separately leading to inefficiency of design:

- independent triggering conditions causing conflicts;

- Independent cancellation mechanisms causing redundancy;

- Degraded network scheduling from incomplete state information.

Proposal 3: 6G should study a unified buffer and delay status reporting procedure.

Observation 6: 5G ARQ retransmission relies on RLC status report, which is not in a timely manner.

Proposal 4: RAN2 considers to study the following areas for improving ARQ/HARQ efficiency:

- Study mechanisms to allow UE to aware the UL HARQ transmission status (e.g. succuss or not);

- Study mechanisms to allow the UL HARQ ACK/NACK flip issue for downlink transmission.

# References

[1] R2-2407502 Discussion on DSA and CRDSA Performance, ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot.