3GPP TSG-RAN WG2 Meeting #132	                                                             R2-2508097

Dallas, USA, Nov. 17th-21st, 2025

Source:	CATT

Title:	Considerations on 6GR UE Capability

Agenda Item:	10.2.1

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, the following agreements on UE capability were made for further study:

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

In this contribution, we further discuss key challenges in the 5G capability framework, considerations for the 6G UE capability framework, and dynamic capability reporting.

- Discussion

  - Critical problems with UE capability framework

5G adopted a flexible UE capability framework to accommodate diverse features and use cases. It causes the signaling complexity and excessive message size. The critical problems we observed include:

Complex signaling structures and band combinations

All UE capabilities are explicitly reported in UECapabilityInformation message. However, the message size grows exponentially due to complex signaling structures and continuously expansion of band combinations.

Redundant capabilities across features/BCs

Redundant UE capabilities exist across features, such as CA and UL Tx Switching capabilities are reported in different BC lists. All UE capabilities per BC must be reported separately. Even if most or all of them are identical across band combinations, they must be reported independently for each BC.

Inadequate filtering mechanism

Current UE capability filtering, i.e. RAT/frequency bands filtering, still requires reporting all supported feature-related capabilities, leading to suboptimal filtering efficiency.

Observation 1: Excessive message size is the critical problem for 5G UE capability information due to:

- Complex signalling structures;

- Proliferation of band combinations;

- Redundant capabilities across multiple features/BCs;

- Inadequate filtering (RAT/frequency band only).

To mitigate the challenges described above, the following solutions can be considered and discussed in RAN2:

Device-type/category based capability signaling structure

The device type concept is being discussed in RAN plenary. Regardless of the final definition of device type, RAN2 can evaluate whether to use a device type/category-based capability framework as a baseline.

Enhanced feature reuse and reduced redundancy

To address the low reuse rate and redundant capabilities across features/BCs, RAN2 should collaborate with RAN1 and RAN4 to optimize PHY/RF capabilities to reducing redundancy, e.g., to investigate the potential redundancy reduction of the PHY/RF capabilities across BC/bands.

Feature based capability filtering

To improve filtering efficiency, a feature-based capability filtering mechanism can be introduced, moving beyond the current RAT/frequency-based approach.

UE capability message compression

Regardless of the adopted framework, RRC compression techniques can be used to further reduce UE capability signaling overhead. For instance, a UE capability ID can serve as a dictionary indicator, allowing the UE to compress its capability information message by referencing the corresponding capability ID. This approach minimizes signaling overhead without requiring complex delta signaling design.

Proposal 1: RAN2 to further study the following ways to reduce the overhead of capability signalling:

- Feature based capability filtering;

- UE capability message compression within UE capability ID as compression reference.

- Device-type/category based capability signaling structure, depending on further conclusion in RAN/RAN1;

Proposal 2: Coordination with RAN1 and RAN4 to investigate the potential redundancy reduction of the PHY/RF capabilities across BC/bands.

  - Dynamic capability change

In 5G, UE can report its temporary restricted capabilities (e.g. overheating, power saving, IDT, MUSIM) to network via UAI (UE assistance information) message. However, the network's response must carefully consider KPI impacts since these scenarios typically trigger resource reconfigurations - including MIMO layer/Rank adjustments, BWP switching, and SRS port reconfiguration - which introduce significant network-side complexity. Without proper management, frequent capability updates may further degrade network performance. A similar challenge arises with dynamic capability changes in foldable UEs. These devices exhibit different operational capabilities between folded/unfolded states (e.g., varying MIMO layer). When such capability changes occur, the UE proactively notify the gNB to enable appropriate resource allocation. To effectively balance dynamic capability update requirements with network performance optimization, Dynamic capability update should be under network control. For example, the network configures the conditions or restrictions for UE-initiated capability changes.

Proposal 3: Frequent capability updates should be avoided as much as possible.

Proposal 4: Dynamic capability update, if supported, should be under network control. How to respond to capability update is up to network implementation.

- Conclusion

In this contribution, we provide our views on 6G UE capability including below observations and proposals:

Observation 1: Excessive message size is the critical problem for 5G UE capability information due to:

- Complex signalling structures;

- Proliferation of band combinations;

- Redundant capabilities across multiple features/BCs;

- Inadequate filtering (RAT/frequency band only).

Proposal 1: RAN2 to further study the following ways to reduce the overhead of capability signalling:

- Feature based capability filtering;

- UE capability message compression within UE capability ID as compression reference.

- Device-type/category based capability signaling structure, depending on further conclusion in RAN/RAN1;

Proposal 2: Coordination with RAN1 and RAN4 to investigate the potential redundancy reduction of the PHY/RF capabilities across BC/bands.

Proposal 3: Frequent capability updates should be avoided as much as possible.

Proposal 4: Dynamic capability update, if supported, should be under network control. How to respond to capability update is up to network implementation.