3GPP TSG-RAN WG2#132		                                                      R2-2508098

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT

Title:	Considerations on RRC Structure and (re)configuration

Agenda Item:	10.3.2.2

Document for:	Discussion and Decision

- Introduction

In last meeting [1], the issues about RRC Signalling Design and Reconfigurations have been discussed, and the agreements below have been achieved:

Agreements

1.	ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

2.	Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

3.	Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

4.	RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

In this contribution, we share our views on the 6G RRC Structure design, delta configuration, and failure of re-configuration.

- Discussion

2.1 RRC Structure

In 5G, the ASN.1 structure design is based on a multi-level hierarchy. All RRC parameter and structure content in ASN.1 can be regarded as a single comprehensive module with one initial IMPORT statement without detailed partitioning - i.e., it is contained within a set of sections marked by "BEGIN" and "END" delimiters:

- 6.2: RRC messages, including the top level of Control Channel (BCCH, CCCH, DCCH, PCCH, MCCH…), and the second level of message definitions;

- 6.3: RRC information elements, including all IEs used in the other;

- 6.4: RRC multiplicity and type constraint values

Observation 1: 5G RRC structure can be regarded as a single comprehensive module without detailed partitioning.

To make RRC signaling structure more rational, improve readability, and prevent degradation in readability caused by subsequent feature enhancements, it is agreed to study the modular design of the 6G RRC structure.

For the RRC modular, some companies propose to consider SLPP protocol as an example. We analyzed the SLPP protocol, which consists of a total of 7 different levels of modules:

- One general module ( SLPP-PDU-Definitions). The module defines the following:

  - General message structure

    - SLPP-Message (containing identification parameters such as transactionID, and messageBody field)

    - SLPP-MessageBody (message catalogs)

  - Message body IE definitions (this level contains no detailed parameters, only child IEs/modules, and NCE/lateNCE for extension)

    - Six Message body-level IEs such as RequestCapabilities/ProvideCapabilities/RequestAssistanceData/ProvideAssistanceData/RequestLocationInformation/ProvideLocationInformation, each containing only the subIE/module structures defined in other modules

    - Abort/Error message

  - SLPP information elements (containing multiple common parameters callable by other modules)

    - Physical layer IEs, e.g. ARFCN-ValueNR/NCGI

    - UE capability-related IEs of ScheduledLocationTimeSupportPerMode

  - Constant definitions (containing multiple constants callable by other modules)

- Two common modules (SLPP-PDU-CommonContents and SLPP-PDU-CommonSL-PRS-MethodsContents). The module defines the following:

  - Six common IEs such as CommonIEsRequestCapabilities for six messages such as RequestCapabilities

  - Six common SL-PRS IEs such as CommonSL-PRS-MethodsIEsRequestCapabilities, for six messages

(These common IEs are referenced in the preceding Message body but are not referenced elsewhere.)

- Four positioning method specific modules. Each module defines the parameters and structures only involve the specific positioning method.

Among the modules, SLPP-PDU-Definitions could schedule the IEs in all other six modules, if the IEs are declared in the IMPORT part at the beginning of the module; SLPP-PDU-CommonContents could schedule the IEs in SLPP-PDU-Definitions; and the four positioning method specific modules could schedule the IEs in both SLPP-PDU-Definitions and SLPP-PDU-CommonContents.

By studying the SLPP protocol structure as a starting point, we can attempt to partition all existing RRC content. Regarding the partitioning of basic functionalities, functions with weak cross-coupling can be separated into individual modules, such as measurement, mobility, and physical layer configuration, while the rest can be grouped into a single, large general module. This approach not only completes the module partitioning but also significantly reduces the extensive cross-calls between modules.

For other features/verticals, separate modules can be established to encompass them, simplifying the process of adding future enhancement content while improving readability and ease of understanding.

In summary, we propose to partition the ASN.1 content of RRC protocol as following:

- Basic RRC module: One general module + several common modules which have less/weak  cross-coupling with other common modules, e.g. mobility, measurement;

- Feature modules: Separate modules for specific features, e.g. NTN.

Figure 1: Partitioning of RRC modules

Proposal 1: To partition the ASN.1 content of RRC protocol as following:

- Basic RRC modules: One general module + several common modules which have less/weak cross-coupling with other common modules, e.g. mobility, measurement;

- Feature modules: Separate modules for specific features, e.g. NTN.

After the first release of specification is frozen, the enhancements by subsequent release can be added based on it:

- For enhancement of basic function, e.g. mobility enhancement, the involved common module can be improved;

- For enhancement of new features, new independent modules can be established in parallel with other feature based modules;

- For enhancement of existing features, the improvement can be made only in the relevant feature based modules, without impacting on other feature modules and the basic modules.

Proposal 2: General principle: For future enhancements to any feature module, aim at minimal impact on other feature modules and the basic RRC modules.

Furthermore, the introduction of a large number of physical layer and RF parameters has increased the complexity of the 5G RRC protocol and reduced its readability. Therefore, after the partitioning method for the RRC module is finalized, it needs to be communicated to RAN1 and RAN4. Additionally, in the future, RAN1/RAN4 should better ensure that the feature list parameters they send to RAN2 in the LS are also partitioned according to the RRC modular design specified by RAN2.

Proposal 3: Once RAN2 reaches a decision on RRC modular structure, we should inform RAN1 and RAN4 to follow such structure in their work.

2.2 Delta Configuration for RRC signalling

The RRC protocol uses ‌delta configuration‌ to save transmission overhead, but this approach may cause different interpretation in UE, and the Need Code approach introduced as a result may lead to confusion in protocol implementation understanding.

To avoid ambiguity and implementation errors caused by delta configuration mechanism as much as possible, RAN2 should study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small. There are some options can be considered:

- Option 1: Only use full configuration.

  - ‌Pros:‌ Simple to understand, low implementation complexity;

  - ‌Cons:‌ Unacceptable signaling overhead.

- ‌Option 2:‌ Define reference configurations. After the initial configuration, each subsequent delta configuration is based on this reference configuration, avoiding "change on change."

  - ‌Pros:‌ Significantly reduces misinterpretation and also avoids increased overhead;

  - ‌Challenges: Requires discussion on how to define the general reference configuration, and it may lead to difficulties in scalability.

- ‌Option 3: Keep the existing Need Code mechanism, with textual enhancements in the field descriptions (e.g., adding "functionality mandatory" clarifications or introducing fixed writing templates to improve interpretability)

  - ‌Pros:‌ Minimal changes compared to 5G, but further refines understanding and reduces interpretation errors;

  - ‌ Cons: Inability to fully decode configurations via machine processing; human intervention is still required for handling configuration interpretations.

In 5G implementation, the introduction of delta configuration means that most unchanged parameters do not need to be transmitted over the Uu interface. Full configuration in Option 1 would introduce significant signaling overhead which is not acceptable. For Option 2, the reference configuration mechanism is already used in 5G for e.g. LTM, subsequent CPAC. Such mechanism can continue to be adopted in 6G together with delta configuration mechanism.

Observation 2: Delta configuration can significantly reduce air interface signaling which should not be removed from 6G.

To address issues such as potential ambiguity or poor readability caused by the Need code, Option 3 can be considered. For example, to clarify which parameter is "functionality mandatory", i.e. the parameter is mandatorily used in UE side, and should be configured at least in the first configuration message to the UE; or to define a “template sentence” to help with the comprehension of the Need Code, the “template sentence” has only one clear interpretation and will not be misunderstood as another operational action.

Proposal 4: Delta configuration and Need Code mechanism continue to be used for RRC signaling in 6G, with textual enhancements in the field descriptions (e.g., adding "mandatory functionality" clarifications or introducing writing templates to improve interpretability).

2.3 Failure of re-configuration

In last meeting, it is suggested studying the reason of the re-configuration failure issue:

For next meeting, can study the reasons why these failures happens and understand the root cause of the problem.

In existing 5G specifications, if the UE fails to comply with part of the reconfiguration message content, it will be unable to fully apply all configurations, ultimately triggering RRC re-establishment with failure cause “reconfigurationFailure”. The root cause cannot be inferred from a single cause value.

In our observations, the overall probability of UE re-establishment in the field is approximately 2% to 3%. However, the majority of the causes  are set to “otherFailure”. This cause, as defined in the protocol, primarily includes RLF, integrity protection failure, etc.

Observation 3: The overall probability of UE re-establishment in the field is approximately 2% to 3%, and re-establishment caused by the UE's failure to comply with partial reconfiguration messages account for only a small portion of the total re-establishment.

Therefore, since introducing an enhanced mechanism would increase processing complexity for both the UE and the network, we recommend first studying the failure probability caused by the UE's inability to comply with the configuration, and then determining whether a partial success/failure mechanism is required.

Proposal 5: RAN2 first studies the failure probability caused by the UE's inability to comply with the configuration, and then determining whether a partial success/failure mechanism is required.

- Conclusion

In this contribution, we present our views on the 6G RRC Structure design, delta configuration, and failure of re-configuration. The following observations and proposals are provided:

RRC Structure

Observation 1: 5G RRC structure can be regarded as a single comprehensive module without detailed partitioning.

Proposal 1: To partition the ASN.1 content of RRC protocol as following:

- Basic RRC modules: One general module + several common modules which have less/weak cross-coupling with other common modules, e.g. mobility, measurement;

- Feature modules: Separate modules for specific features, e.g. NTN.

Proposal 2: General principle: For future enhancements to any feature module, aim at minimal impact on other feature modules and the basic RRC modules.

Proposal 3: Once RAN2 reaches a decision on RRC modular structure, we should inform RAN1 and RAN4 to follow such structure in their work.

Delta Configuration for RRC signalling

Observation 2: Delta configuration can significantly reduce air interface signaling which should not be removed from 6G.

Proposal 4: Delta configuration and Need Code mechanism continue to be used for RRC signaling in 6G, with textual enhancements in the field descriptions (e.g., adding "mandatory functionality" clarifications or introducing writing templates to improve interpretability).

Failure of re-configuration

Observation 3: The overall probability of UE re-establishment in the field is approximately 2% to 3%, and re-establishment caused by the UE's failure to comply with partial reconfiguration messages account for only a small portion of the total re-establishment.

Proposal 5: RAN2 first studies the failure probability caused by the UE's inability to comply with the configuration, and then determining whether a partial success/failure mechanism is required.

- References

- R2-2508002, RAN2#131bis Meeting Report