3GPP TSG-RAN WG2 #132		                                                      R2-2508099

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT

Title:	Considerations on 6G data transfer and AI framework

Agenda Item:	10.3.3.1

Document for:	Discussion and Decision

- Introduction

The study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility was started in 3GPP Rel-19. Several AI/ML use cases such as RRM measurement prediction and measurement event prediction have demonstrated performance gains over non-AI solutions. It is expected that AI/ML will play an important role in more scenarios in 6G for tackling fundamental challenges and achieving improvements in mobility robustness, energy efficiency, and other critical KPIs.

In last meeting [1], the AI/ML framework and the data collection framework have been discussed, and the agreements below have been achieved:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this contribution, we intend to discuss the characteristics for AI data, the data transfer design, AI/ML framework study for 6G air-interface.

- Discussion

2.1 Data/model transfer in 6G

- 2.1.1 Data transfer design

Background

AI/ML function was introduced in the mid-stage of 5G evolution without structure modification. For the large size AI/ML involved data transfer in both UL and DL, multiple solutions have been provided by companies without convergence in 5G.

For data collection of network side model, MDT report like mechanism was used. But for data collection of UE side model, multiple solutions were provided:

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

RAN2 has analyzed that aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved for data collection, MNO controllability and visibility over the collected data, etc. But down selection among these solutions was still difficult.

In 5G discussions, for various candidate uplink AI/ML data collection solution, none of the schemes can converge. There are some limitations based on legacy CP/UP solution for large volumes of data. Therefore it is necessary to study new mechanism to avoid the issues that arise in 5G.

Observation 1: There are some limitations based on legacy CP/UP solution for large volumes of data in 5G AI/ML, to study new mechanism to avoid the issues that arise in 5G is necessary.

Characteristics for AI data of diverse types

By last RAN2 meeting, it is propose to first discuss the diverse types of data and its services/use case scenarios in this meeting:

For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

For these existing 5G use cases and potential 6G sensing use case, we list a table as below, to compare the similarities and differences of  diverse types of data:

Table 1: Comparison of characteristics across diverse types of data

|  | Applicable use case(s) | End point pairs | Data size (in single reporting) | Data size (in total data size) | Frequency of data reporting | QoS requirements |
| --- | --- | --- | --- | --- | --- | --- |
| AI/ML related data | AI for PHY, AI based Mobility, AI for NG-RAN | UE and RAN, UE and CN, UE and OAM, RAN and CN, RAN and OAM | Limit by the PDCP SDU size of 9Kbytes in 5G | Training: depend on UE memory, may up to several MB or even hundreds of MB; Inference/ Monitoring: Hundreds of bits | Training: regular / sometimes; Inference: frequent / continuous Monitoring: regular / sometimes | Training: latency requirement is relaxed; Priority of SRB6 is lower than SRB1; Inference: time-critical for most use cases; May have high priority as SRB1; Monitoring: Near-real-time for most use cases; Lower priority than inference data; |
| sensing data | Object detection and tracking | UE and RAN, UE and CN, RAN and CN |  | Depend on measurement type, may up to several MB | Object Detection: regular / sometimes; Object Tracking: frequent / continuous | Time-critical for some use cases; Low priority; |
| MDT | Logged MDT | UE and OAM (include UE and RAN, RAN and OAM) |  | depend on UE memory, may up to several MB or even hundreds of MB; | May up to 5M; | Latency requirement is relaxed; Priority of SRB2 is lower than SRB1; |
|  | Immediate MDT |  |  | Hundreds of bits | Hundreds of bits |  |
| QoE | QoE | Similar as MDT |  | Up to configuration, may similar as MDT | Up to configuration, may similar as MDT | Latency requirement is relaxed; Priority of SRB4/5 is lower than SRB1; |
|  | RAN visible QoE | UE and RAN |  |  |  |  |

For the diverse data of the use cases listed in the table, all the data should be classified as ‌non-GBR‌ since none of it requires ‌Guaranteed Bit Rate (GBR)‌ service. And the packet error rate should be low. For other characteristics, we summarized as below:

- Data of diverse use cases or data for different usage may have various starting and endpoints;

- Some data has large data size, e.g. up to hundreds of MB;

- Frequency of data reporting may be various for different use cases or different usage;

- Data of diverse use cases or data for different usage may have diverse QoS requirements, e.g. latency, priority;

Observation 2: Data of various use cases or data for different usage may have various starting and endpoints, large data size, various frequencies of data reporting or diverse QoS requirements.

Among all the data, some 5G existing mechanism is already used, e.g. logged/immediate MDT report and NW-side AI/ML data report via RRC signaling in the air-interface, and the QoS requirement can be ensured. And for some other data, e.g. UE-side AI/ML data report in Release 18/19 AI/ML item, it has been generally concerned that the current CP or UP cannot efficiently or reliably transmit the data after multiple times of discussions in 5G. For the concerned data transmission, it is necessary to collect and analyze the characteristics to find an effective mechanism to address the transmission issue. Furthermore, for data whose transmission mechanisms have already been defined in 5G, new mechanism may potentially be leveraged to enhance transmission efficiency and reliability, thereby optimizing data transfer.

Data transfer design

In 6G Day1, we propose to introduce a data transfer plane other than legacy CP and UP. Compared with 5G, 6G will introduce diverse services/use cases, with more complex scenarios/requirements. Transmission characteristics include large amount of data‌, ‌high-frequency transmission‌, as well as ‌diverse start/end nodes‌ with ‌varying QoS requirements.

Below are some analysis of Advantages and Disadvantages of introducing new data transfer mechanism in 6G network:

      - Qos requirement

5G QoS can be configured by mapping to different 5QI values over the air interface RB, with existing radio bearer types including SRB and DRB. 5G protocol explicitly defines high-priority settings only for SRB0, SRB1, and SRB3, while the settings for SRB2, SRB4, SRB5 and various DRB are not clearly defined. Their priority levels can be configured by the network based on actual bearer content.

Facing the numerous service requirements that 6G may introduce, data can be carried by the following three methods:

- Option 1: Add new SRBs;

- Option 2: Use existing DRBs, or expand the number of DRBs for bearing;

- Option 3: Add new types of RBs.

SRBs are dedicated to transmitting control signaling between the UE and the NB, which have strict latency and reliability requirements to ensure smooth network control. There are a limited number of standard SRB types with fixed functions. Adding new SRB should be based on plenty discussion on the fixed function, applied scenario, QoS parameters. As the number of 6G AI/ML services increases, such discussions seem redundant if new SRBs are used for new services.

DRBs are used to carry user plane data, such as voice, video, and application data, which have diverse QoS profiles tailored to user services. DRBs are more flexible in quantity—multiple DRBs can be established for different services of a single UE, each with unique QoS configurations.

For AI/ML data in different scenarios/applications, there may be significant differences in QoS requirements, e.g., training data is not sensitive to latency and allows packet loss; model data does not allow packet loss and has a large data volume, etc. The introduction of a new plane in 6G can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

Observation 3: From QoS requirement aspect, introduction of a new plane can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

      - Routing and delay

The CP tunnel can only be established between UE and NB, and the UP tunnel can only be established between UE and UPF, the fixed start/terminate node may not fulfill the data transmission requirements of diverse AI/ML services. By introducing data plane with flexible start/terminate nodes, this issue can be resolved, settings can be made by flexibly configuring different start and terminate points of data transmission on the network side, e.g. data between UE and RAN node / CN / OAM, data between RAN nodes / relay(s), and data routing via RAN nodes. Take BM data collection as an example, UEs need to first report the data to gNBs, which then forward it to training nodes. In 6G, using the potential data plane for transmission can avoid multi-node forwarding. The gNB can directly perform transparent transmission of data, saving the time required for repackaging.

Especially, the AS data carried in AS bearer without intervention of NAS or back-and-forth delivery between RAN and CN become possible. This is benefit for the data transmission only involving air-interface, e.g. BM data collection.

Observation 4: From data generation and routing aspect, compared with legacy SRB/DRB, introduction of a new plane can more flexibly configure different start and terminate points of data transmission on the network side, and the delay due to data forwarding via intermediate node can be avoided.

      - Data size

Currently, for air interface content related to AI, most signaling can reuse existing procedures such as RRC/NAS signaling. The potentially added data plane only has effective solutions for large-volume data content, e.g. training data collection/report.

From the perspective of data volume, regardless of which plane a large amount of data is transmitted through, it occupies air interface resources. Therefore, the total amount of data transmitted over the air interface will not change due to the introduction of potential data plane. Moreover, limited by the size of PDCP SDUs, the upper limit of individual PDCP SDUs is fixed irrespective of the RB type. This value is 9000 bytes in 5G, but may be expanded in 6G.

For data collection in existing standardized use cases, only the BM use case involves UEs collecting large amounts of data. In the Positioning use case, the existing LPP/NRPPa protocols are reused, and the reporting to LMFs is basically enhanced using existing signaling. For the unified processing of multiple use cases in the future, the potential data plane can terminate at RAN nodes or other nodes, e.g. NWDAF, LMF, and OAM, etc. Unified design for data transmission of multiple use cases, and future-proof for use cases added in later releases.

Observation 5: From use case scalability aspect, introduction of a new plane can unify design for large size data transmission of multiple use cases, and future-proof for use cases added in later releases.

Based on the analyses above, we propose to introduce a RAN data plane to support the large volumes of AI data collection and transmission, especially for model training to enables flexible configuration of QoS and flexible start/end points for data transmission, and reduces the forwarding latency of data at intermediate nodes.

Proposal 1: RAN2 to support the data transfer via a RAN data plane in 6G to fulfill the various QoS requirements, flexible routing and use case scalability of diverse types of data.

- 2.1.2 Model transfer requirement

Background

For model transfer/delivery, the sharing of dataset/model parameters from NW-side to UE or UE-side training entity was discussed by RAN1 for two-sided models, and two alternatives for the sharing of dataset/model parameters are provided by RAN1. And how the data (including e.g. the dataset/model parameters) are transferred between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

- Non-Over-The-Air (non-OTA) approach:

  - gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)

- Over-The-Air (OTA) approach:

  - gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

In 5G discussions, whether it is uplink AI/ML data collection solution or downlink AI model transfer/delivery solution, none of the schemes can converge. There are also limitations based on legacy CP/UP solution for large volumes of model or model related parameters in DL. Therefore it is necessary to study new mechanism to avoid the issues that arise in 5G.

Observation 6: Besides data transfer in UL, there are also limitations based on legacy CP/UP solution for large volumes of model or model related parameters in DL.

Model transfer requirement

Since both involve ‌high-volume information transmission over the air interface‌, we have referenced the ‌data transfer requirements‌ and outlined the potential needs for ‌OTA-based model transfer‌ as follows:

- For data size: Based on the candidate model transfer solutions (Direction A#3a-1/4-1, Direction C) in R20 AI-PHY WID [2] and RAN1 LS [3], the data size  of model or model related parameters/dataset may up to hundreds of MB;

- For End point pairs: For UE-side model, the endpoint is UE, the start point can be server/RAN-node/CN-node/OAM; and for NW-side model, the start point can be any of the network node responsible for executing training;

- For frequency of transmission: The frequency of model transfer can be low for offline training, and may be more frequent based on online training if any;

- For QoS requirement: The transmission latency can be relaxed, and the priority may be similar as the UL data collection.

Since the DL OTA-based model transfer also complies with the new data type characteristics such as large data volume‌, ‌various starting and endpoints, and various frequencies of transmission similar as UL data transfer‌‌, it is recommended to ‌unify the framework for both data transfer and model transfer‌.

Observation 7: OTA-based model transfer in DL also involves new data type characteristics such as large data volume, various starting and endpoints, and various frequencies of transmission similar as UL data transfer.

Proposal 2: To design ‌unified framework for both UL data transfer and DL model transfer‌.

2.2 AI/ML framework for 6G

In NR, the following use cases were discussed, including

- AI/ML based BM: utilizing intra-cell downlink beam prediction of the serving cell to reduce measurement/RS overhead and to improve the accuracy of beam selection which consists of spatial-domain and temporal-domain prediction.

- AI/ML-based CSI prediction: utilizing intra-cell downlink temporal-domain beam prediction of the serving cell to reduce measurement/RS overhead and to improve the accuracy of beam selection for UE-side model without inference related parameters configuration.

- AI/ML-based CSI compression: utilizing intra-cell downlink spatial-frequency domain CSI compression using two-sided AI model to reduce reporting overhead.

- AI/ML-based Positioning: making use of the downlink signals received from multiple TPs to determine AI/ML model input, at the UE. The UE performs inference using AI/ML model with assistance data received from the positioning server, and other configuration information to locate the UE.

- AI/ML-based mobility: including RRM measurement prediction and measurement event prediction to reduce measurement efforts in temporal, spatial or frequency domain and improve the handover performance

For each AI/ML use case, the LCM procedure was discussed separately. Thus, RAN2 has repetitive discussions on LCM procedure of different AI/ML use case. For example, LCM on BM was discussed and the general LCM procedure was agreed. When it comes to AI/ML based positioning and AI/ML-based mobility, the similar LCM was discussed and agreed in principle. It can be seen that RAN2 spent redundant time on the LCM procedure discussion. For 6G, more use cases may be studied and introduced and the more efficient way is to define one unified LCM procedure so that no redundant discussion on LCM procedure of newly introduced use cases can be avoided unless critical issues are identified.

Proposal 3: RAN2 to study one unified LCM procedure across different use cases for AI/ML in 6G.

And it has been agreed in RAN2#131bis for 6G that:

Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality

It can be seen that the 6G AI/ML LCM framework follows that in NR. We think the at least applicability procedure in 6G can also follow the steps agreed in NR which is illustrated in Figure 1.

Figure 1: Example of Unified AI applicability reporting procedure in 6G

Proposal 4: The LCM procedure agreed in 5G for UE-sided model can be agreed as starting point which consists of the following steps:

- Step 1: NW enquiry for capability;

- Step 2: UE indicating capability;

- Step 3: NW indicating inference configuration;

- Step 4: UE determines functionality applicability;

- Step 5: UE reporting functionality applicability;

- Step 6: NW/UE functionality (de-)activation.

In NR, the LCM procedure mainly focuses on UE-sided model. And two-sided model, i.e., a paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network [4], is under discussion for CSI compression use case.

Regarding two-sided model, the following issues can be further considered in RAN2:

- CSI feedback enhancement, e.g., Model pairing procedure and applicability reporting, inference aspects, etc.

- Inter-vendor training collaboration for two-sided AI/ML model.

Two-sided model which is one kind of inference model brings benefits on reporting overhead reduction, it can be studied together with UE-sided model. RAN2 can study the LCM impact of two-sided model in 6G Day1, and the detailed discussion can be started after sufficient progress made in R20 AI-PHY WI…

Proposal 5: RAN2 to study the LCM impact of two-sided model in 6G Day1, and the detailed discussion can be started after sufficient progress made in R20 AI-PHY WI.

- Conclusion

In this contribution, we present our views on data transfer design, AI/ML framework study for 6G air interface. The following observations and proposals are provided:

Data/model transfer in 6G

Observation 1: There are some limitations based on legacy CP/UP solution for large volumes of data in 5G AI/ML, to study new mechanism to avoid the issues that arise in 5G is necessary.

Observation 2: Data of various use cases or data for different usage may have various starting and endpoints, large data size, various frequencies of data reporting or diverse QoS requirements.

Observation 3: From QoS requirement aspect, introduction of a new plane can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

Observation 4: From data generation and routing aspect, compared with legacy SRB/DRB, introduction of a new plane can more flexibly configure different start and terminate points of data transmission on the network side, and the delay due to data forwarding via intermediate node can be avoided.

Observation 5: From use case scalability aspect, introduction of a new plane can unify design for large size data transmission of multiple use cases, and future-proof for use cases added in later releases.

Proposal 1: RAN2 to support the data transfer via a RAN data plane in 6G to fulfill the various QoS requirements, flexible routing and use case scalability of diverse types of data.

Observation 6: Besides data transfer in UL, there are also limitations based on legacy CP/UP solution for large volumes of model or model related parameters in DL.

Observation 7: OTA-based model transfer in DL also involves new data type characteristics such as large data volume, various starting and endpoints, and various frequencies of transmission similar as UL data transfer.

Proposal 2: To design ‌unified framework for both UL data transfer and DL model transfer‌.

AI/ML framework for 6G

Proposal 3: RAN2 to study one unified LCM procedure across different use cases for AI/ML in 6G.

Proposal 4: The LCM procedure agreed in 5G for UE-sided model can be agreed as starting point which consists of the following steps:

- Step 1: NW enquiry for capability;

- Step 2: UE indicating capability;

- Step 3: NW indicating inference configuration;

- Step 4: UE determines functionality applicability;

- Step 5: UE reporting functionality applicability;

- Step 6: NW/UE functionality (de-)activation.

Proposal 5: RAN2 to study the LCM impact of two-sided model in 6G Day1, and the detailed discussion can be started after sufficient progress made in R20 AI-PHY WI.

- References

- R2-2508002, RAN2#131bis Meeting Report

- RP-251870 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

- R1-2410922 LS on signalling feasibility of dataset and parameter sharing, RAN1

- TR38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, V19.0.0