3GPP TSG-RAN WG2 Meeting #132                                                                    R2-2508102

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT, Turkcell, CBN

Title:	Functionality management for RRM measurement event prediction

Agenda Item:	9.3.3

Document for:	Discussion and Decision

- Introduction

In Rel-19, we had studied RRM measurement prediction and RRM measurement event prediction use cases in SI FS_NR_AIML_Mob. In Rel-20, we start the normative work for the two use cases in WI NR_AIML_Mob, and the objective of the WID is as follow [1].

… The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. …

In this contribution, based on the WID, we will analyze the spec impact of RRM measurement event prediction from the following aspects for UE-sided model:

- UE capability request and response procedure;

- Applicability report for both full and partial configuration approach;

- Inference configuration and inference report.

- Discussion

In this section, we discuss the spec impact for RRM measurement event prediction for UE-sided model. Both indirect measurement event prediction and direct measurement event prediction are considered.

In Rel-20 AI/ML mobility WID, it indicates that temporal domain prediction case A and case B, and frequency domain prediction for co-located cases are supported, spatial domain prediction is not supported for UE-side model.

During SI phase, for indirect measurement event prediction, the temporal domain prediction case A and case B, and frequency domain prediction are defined as follow, which can be the baseline to start the spec impact analysis.

Agreements 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

  - UE capability request and response procedure

In Rel-19 AI/ML PHY WI, for functionality-based LCM for UE-sided model for beam management use case, RAN2 has studied and worked on the signalling procedure of applicable functionality reporting, and RAN2 further has made following signalling procedure figure (see the attached Figure 1 below) on applicable functionality reporting for beam management UE-sided model.

Figure 1: signalling procedure on applicability reporting for BM UE-sided model

For Step 1 and Step 2 in the Figure 1, which is about the UE capability, the following agreements were achieved for beam management UE-sided model:

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

In Rel-20 AI/ML mobility WI, UECapabilityEnquiry/UECapabilityInformation message can be used also for UE to report UE capability for RRM measurement event prediction, i.e., network sends UECapabilityEnquiry to initiate the procedure to a UE reporting its supported AI/ML based RRM measurement event prediction capability, UE sends UECapabilityInformation message to network, containing supported AI/ML based RRM measurement event prediction capability at the UE side.

Proposal 1: For AI/ML based RRM measurement event prediction, the following UE capability signaling can be used:

- UECapabilityEnquiry: network sends UECapabilityEnquiry to initiate the procedure to a UE reporting its supported AI/ML based RRM measurement event prediction capability;

- UECapabilityInformation: UE sends UECapabilityInformation message to network, containing supported AI/ML based RRM measurement event prediction capability at the UE side.

For the detailed UE capability for AI/ML based RRM measurement event prediction, it can be further discussed after more agreements are achieved for AI/ML based RRM measurement event prediction use case.

  - Applicability report for both full and partial configuration approach

- Indirect measurement event prediction

For RRM measurement event prediction use case, for all the 3 indirect measurement event prediction, considering that the RRM measurement prediction (temporal domain case A, temporal domain case B, frequency domain) model is used for indirect measurement event prediction, the applicability reporting of indirect measurement event prediction is same as RRM measurement prediction.

Proposal 2: For indirect RRM measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

- Direct measurement event prediction

In TR 38.744 [2], indirect/direct measurement event prediction is descripted as follow:

It is up to UE’s implementation to decide on choice between indirect and direct event prediction methodology. On top of inference parameters captured in section 6.1.2.1.2, event-related parameters e.g. event type are part of inference configuration of measurement event prediction. UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact. For indirect event prediction, RSRP difference can be used as performance monitoring metric. NOTE 1: The spec impact captured in section 6.1.2.1 is applicable for measurement event prediction unless otherwise described explicitly in this section. NOTE 2: A single framework is aimed for direct and indirect event prediction. NOTE 3: There is no consensus on the feasibility of performance monitoring of direct event prediction. it should be concluded before proceeding with normative work.

Based on the TR (highlighted in green), during SI phase, we prefer to design a single framework for direct and indirect measurement event prediction, and it is up to UE’s implementation to decide on choice between indirect and direct measurement event prediction methodology. We understand that it means the full inference configuration/partly inference configuration from network for applicability reporting for direct and indirect measurement event prediction are same, i.e., the network does not need to provide different configuration for direct measurement event prediction and indirect measurement event prediction separately, the network even does not get to know whether direct or indirect measurement event prediction is used for inference eventually. Therefore, for direct RRM measurement event prediction, the applicability reporting is the same as that for indirect RRM measurement event prediction.

Proposal 3: For direct RRM measurement event prediction, the applicability reporting is the same as that for indirect RRM measurement event prediction.

- Both direct and indirect measurement event prediction

Based on the TR (highlighted in blue), on top of parameters which are considered for applicability reporting in RRM measurement prediction use case, for both direct and indirect measurement event prediction for applicability reporting, event-related parameters are included, e.g. event type, in full inference configuration, and in order to design a single and unified framework, event-related parameters is included in also partly inference configuration for applicability reporting for both direct and indirect measurement event prediction. FFS whether other parameter is needed.

Proposal 4: For RRM measurement event prediction for applicability reporting, on top of parameters which are considered for applicability reporting in RRM measurement prediction, event-related parameters are included in full inference configuration and partly inference configuration, if configured. FFS other parameter.

  - Inference configuration and inference report

- Indirect measurement event prediction

As mentioned in section 2.2, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the inference configuration for indirect measurement event prediction should be same as RRM measurement prediction. Besides, event-related parameters, e.g., event type, should be included in inference configuration for indirect measurement event prediction. FFS whether other parameter is needed.

Proposal 5: For indirect RRM measurement event prediction, the inference configuration is the same as that for RRM measurement prediction. Besides, event-related parameters should be included in the inference configuration. FFS other parameter.

In [2], the following content is captured about inference report, and can be confirmed in WI phase.

UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact.

Proposal 6: For indirect RRM measurement event prediction temporal domain case A, UE can report measurement results, predicted measurement event together with its timing related information.

Proposal 7: For indirect RRM measurement event prediction temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message without specification impact.

- Direct measurement event prediction

As mentioned in section 2.2, a single framework is aimed for direct and indirect event prediction. Therefore, for direct RRM measurement event prediction, the inference configuration is the same as that for indirect RRM measurement event prediction.

For inference report for direct measurement event prediction, RAN2 needs to discuss whether the report content is same as indirect measurement event prediction. From our point of view, we prefer to unify the direct and indirect procedure; the network does not need to know whether direct or indirect model is used by UE. So we prefer to report the same content for direct and indirect measurement event prediction.

Proposal 8: For direct RRM measurement event prediction, the inference configuration and inference report are the same as that for indirect RRM measurement event prediction.

- Conclusion

In this contribution, we provide our views on the specification impact for RRM measurement event prediction, and the proposals are summarized as follows:

UE capability request and response procedure

Proposal 1: For AI/ML based RRM measurement event prediction, the following UE capability signaling can be used:

- UECapabilityEnquiry: network sends UECapabilityEnquiry to initiate the procedure to a UE reporting its supported AI/ML based RRM measurement event prediction capability;

- UECapabilityInformation: UE sends UECapabilityInformation message to network, containing supported AI/ML based RRM measurement event prediction capability at the UE side.

Applicability report for both full and partial configuration approach

Proposal 2: For indirect RRM measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

Proposal 3: For direct RRM measurement event prediction, the applicability reporting is the same as that for indirect RRM measurement event prediction.

Proposal 4: For RRM measurement event prediction for applicability reporting, on top of parameters which are considered for applicability reporting in RRM measurement prediction, event-related parameters are included in full inference configuration and partly inference configuration, if configured. FFS other parameter.

Inference configuration and inference report

Proposal 5: For indirect RRM measurement event prediction, the inference configuration is the same as that for RRM measurement prediction. Besides, event-related parameters should be included in the inference configuration. FFS other parameter.

Proposal 6: For indirect RRM measurement event prediction temporal domain case A, UE can report measurement results, predicted measurement event together with its timing related information.

Proposal 7: For indirect RRM measurement event prediction temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message without specification impact.

Proposal 8: For direct RRM measurement event prediction, the inference configuration and inference report are the same as that for indirect RRM measurement event prediction.

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 38.744-j00