3GPP TSG-RAN WG2 Meeting #132	R2-2508105

Dallas, USA, Nov. 17th - 21th, 2025

Agenda Item:	10.3.1.1

Source:	OPPO

Title:	Discussion on the 6G user plane features

Document for:	Discussion, Decision

# Introduction

According to the discussion in the RAN2#132 meeting, RAN2 made the following agreements [1] on the design principles of the RAN2 user plane features

- RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements. - UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

In this contribution, we provide our understandings on the redundant function in both 5G PDCP and 5G RLC, and the 6G L2 protocol design after merging the duplicated functions.

# Discussion

## Merging redundant functions of PDCP and RLC

Figure 1: Redundant functions in 5G PDCP and 5G RLC

According to the 5G PDCP specification [2] and the 5G RLC specification [3], the redundant functions in 5G PDCP and 5G RLC are highlighted in the above figure. In the following table, we provide our understandings on how the redundant functions in both PDCP and RLC are merged in a single layer.

Table 1: Merging 5G two-layer redundant functions of PDCP+RLC to 6G single-layer

| 5G PDCP and 5G RLC | 6G single-layer merging PDCP and RLC |
| --- | --- |
| - Two-layer SN(s) for ARQ(s) in both PDCP and RLC - In PDCP, according to the PDCP status report, the PDCP SDU not received correctly is re-transmitted according to PDCP SN. - In RLC, according to the RLC STATUS PDU, the RLC SDU/segment not received correctly is re-transmitted according to RLC SN and segment offset. | - Single SN for ARQ - According to the STATUS PDU, the SDU/segment not received correctly is re-transmitted according to SN and segment offset. |
| - Two-layer SN(s) for segmentation/reassembly in RLC - In PDCP, the PDCP SN is added for each PDCP PDU. - In RLC, the RLC SN is added for each RLC SDU (i.e. PDCP PDU) with segmentation. | - Single SN for segmentation/reassembly - The PDCP SN is added for each PDU. |
| - Two-layer SN(s) for protocol error detection in AM RLC - In PDCP, the PDCP SN is added for each PDCP PDU. - In RLC, the RLC SN is added for each RLC AM SDU (i.e. PDCP PDU) for protocol error detection. | - Single SN for protocol error detection - The SN is added for each PDU for protocol error detection. |
| - Two-layer SN(s) for duplicate detection(s) in both PDCP and AM RLC: - In PDCP, the PDCP SN is added for each PDCP PDU, for discarding the duplicated PDCP PDU. - In RLC, the RLC SN is added for each RLC AM SDU (i.e. PDCP PDU), for discarding the duplicated RLC AM SDU/segment. | - Single SN for duplicate detection: - The SN is added for each SDU, for discarding the duplicated SDU/segment. |

Observation 1.1: In 5G, ARQ is supported in both PDCP and AM RLC.

Observation 1.2: In 5G, duplicate detection is supported in both PDCP and AM RLC.

Observation 1.3: In 5G, RLC SN and PDCP SN are duplicated for ARQ, segmentation/reassembly, protocol error detection and duplicate detection.

Figure 2: Single ARQ with single SN in 6G

According to the observations and the analysis given above, we think that the 6G user plane protocol should merge PDCP and RLC to a single layer (as illustrated above), so that the duplicated functions and SN(s) can be removed. By merging the functions of both PDCP and RLC, the implementation of both the UE and the network can be simplified, and the extra signalling overheads due to redundant SN can be removed.

Proposal 1: PDCP and RLC is merged to a single layer to support the following functions:

- Single SN for ARQ (for SDU and segment)

- Single SN for segmentation/reassembly

- Single SN for protocol error detection (if configured)

- Single SN for duplicate detection (for SDU and segment)

## SDU concatenation for MAC-I calculation

3GPP TS 24.501: #### 9.11.4.7 Integrity protection maximum data rate .. 8; 7; 6; 5; 4; 3; 2; 1. Integrity protection maximum data rate IEI; octet 1. Maximum data rate per UE for user-plane integrity protection for uplink; octet 2. Maximum data rate per UE for user-plane integrity protection for downlink; octet 3. Figure 9.11.4.7.1: Integrity protection maximum data rate information element

According to the 3GPP TS 38.323 [5], when DRB integrity protection is configured, each PDCP SDU with a separate SN creates a separate MAC-I, which causes heavy processing load at the UE and lots of signalling overhead. According to the 3GPP TS 24.501 [8], an extra UE capability indication (i.e. “Integrity protection maximum data rate”) is introduced to differentiate the UE processing capability for DRB integrity protection at different data rate (i.e. one for 64 kbps and one for full data rate). The capability as quoted above means that when the DRB integrity protection is configured, some low-cost UE implementation may only be able to support up-to 64 kbps data rate. In 6G, if multiple PDCP SDUs are concatenated to a single PDCP PDU with single SN and single MAC-I, the UE processing load for calculating MAC-I can be reduced, and the signalling overhead of separate SN and MAC-I for each PDCP SDU is also reduced.

Observation 2: In 5G, the MAC-I generated for each PDCP SDU causes extra signalling overheads and UE processing capability (i.e. impacting the UE’s transmission bit rate).

Proposal 2: Multiple PDCP SDUs can be concatenated to a single PDCP PDU.

# Conclusion

According to the study item description [4] of 6G, the 6G user plane should be more agile to various new services and more efficient of avoiding duplicated functions. To remove the redundant functions and reduce the processing load in L2 protocols, we have the following observations and proposals.

Merging PDCP and RLC

Observation 1.1: In 5G, ARQ is supported in both PDCP and AM RLC.

Observation 1.2: In 5G, duplicate detection is supported in both PDCP and AM RLC.

Observation 1.3: In 5G, RLC SN and PDCP SN are duplicated for ARQ, segmentation/reassembly, protocol error detection and duplicate detection.

Proposal 1: PDCP and RLC is merged to a single layer to support the following functions:

- Single SN for ARQ (for SDU and segment)

- Single SN for segmentation/reassembly

- Single SN for protocol error detection (if configured)

- Single SN for duplicate detection (for SDU and segment)

PDCP SDU concatenation

Observation 2: In 5G, the MAC-I generated for each PDCP SDU causes extra signalling overheads and UE processing capability (i.e. impacting the UE’s transmission bit rate).

Proposal 2: Multiple PDCP SDUs can be concatenated to a single PDCP PDU.

# Reference

[1] 3GPP RAN2#131bis meeting minutes

[2] 3GPP TS 38.323, “NR; Packet Data Convergence Protocol (PDCP) specification”

[3] 3GPP TS 38.322, “NR; Radio Link Control (RLC) protocol specification”

[4] RP-252912, “New SID: Study on 6G Radio”