3GPP TSG-RAN WG2 Meeting#132	R2-2508109

Dallas, U.S.A., 17th – 21st Nov. 2025

Source:	vivo, Huawei, HiSilicon, CMCC

Title:	Discussion on CT1 LS about the device behavior of service collision

Agenda Item:	8.2.1

Document for:	Discussion and Decision

- Introduction

CT1 has sent RAN2 an LS regarding how the device handles the scenario that it is in process of an A-IoT service procedure while running into another service. However, CT1 saw the issue in terms of attack risk based on what we RAN2 has achieved and asked RAN2’s view.

CT1 notices that in R2-2504935 (LS on parallel service request), RAN2 has agreed on the following on support of parallel service requests: Agreements on parallel service request - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.. Based on above RAN2 agreement, CT1 has discussed the following two scenarios: a) Collision between inventory procedure and inventory procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the inventory procedure b) Collision between inventory procedure and command procedure: The AIoT NAS layer receives a new inventory from lower layer when the AIoT NAS layer is performing the command procedure CT1 concluded that there can be an attack scenario of false AIoT pagings that would abort any ongoing valid paging or command, and thereby temporarily or continuously block valid inventory procedure and command procedure execution by the AIoT devices. Hence, CT1 kindly ask RAN2 to provide their view on whether it is possible for the AIoT device to respond to the new AIoT paging after completion of an ongoing inventory or command when collision happens.

In this contribution, we try to analyse AS and NAS impacts on both approaches about how UE treats the service collision scenario.

- Discussion

  - Always responding to the currently received paging

During the early stage of R19 WI, RAN2 has discussed parallel services from device perspective, and has come to the conclusion that R19 device can count on NW implementation to not receive parallel service request. The main argument lies in the indoor inventory/command for R19 A-IoT service, where multi-reader overlapping scenario is not likely to be deployed in the private networks. In this sense, we can rely on some reader/NW implementation to ensure that the device would only receive one service at one time to simplify the device processing complexity. And the current Stage 2 describes this as below [2].

NOTE: In this release, the A-IoT device is not expected to process parallel service requests indicated by A-IoT paging messages, and relies on network implementation to address the issue of parallel service requests caused by A-IoT reader overlapping scenario.

Observation 1a.	RAN2 deems that the device only responds to the currently received paging, primarily assuming overlapping-reader scenarios are improbable and avoided by network implementation under R19 indoor A-IoT deployments.

With this prerequisite, the device can assume that, every time it receives a paging message addressed to it, it could consider that this is intended by the network on purpose, even if the former procedure is not successful yet. Considering the device processing complexity, it highly reduces the device memory to maintain certain “status” for the corresponding on-going service, in turn reduce the device power consumption.

Observation 1b.	If the device always responds to the currently received paging, it would ease the burden of R19 low-capable device’s AS processing complexity, memory and power consumption.

  - Only responding to paging after completion of an ongoing procedure

In CT1’s consideration, the normal service handling in the A-IoT NAS layer should be finished without interruption of any other operations request from the AS layer. The current A-IoT NAS Specification describes the device’s behavior on collision of A-IoT paging as an abnormal case, which is of the contradictory handling with the AS layer [3].

### 5.2.5 Abnormal cases in the AIoT device The following abnormal cases can be identified: a) Failure of authentication. If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message. b) Collision of AIoT paging. If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

Seeing the listed two cases in LS, we draw figures to show how A-IoT NAS serial operation handling would suffer from such collided services. Note that we try to split the definition of service completion from the perspective of NAS and AS, respectively. That is to say, from the NAS perspective, service completion can be seen as to generate and deliver the service response to the AS layer.

a)	Collision between inventory procedure and inventory procedure: The A-IoT NAS layer receives a new inventory from lower layer when the A-IoT NAS layer is performing the inventory procedure.

Figure 2.2-1 depicts that when performing paging ID check for Service1, the device’s A-IoT NAS layer receives another request for paging ID check of Service2 from its A-IoT MAC layer.

However, according to RAN1 agreement made in RAN1#122bis meeting [4], before MSG1 transmission, the device is not required to monitor any further R2D message. In this sense, there would be no problem for A-IoT NAS to finish paging ID check for one service, i.e., CT1’s concern is not valid as long as the new paging message is received before MSG1 transmission.

Agreement A device is not required to monitor R2D during TR→D for preparing D2R transmission.

Figure 2.2-1: collision in paging ID check procedure with another inventory

Figure 2.2-2 depicts that after confirming itself to be selected, when performing the access procedure for Service1, the device’s AS layer receives another paging message of Service2. According to the current A-IoT MAC specification, upon reception of this new paging message, the device releases AS ID if any and starts to determine whether and how to respond to this paging. However, without implementation restriction on the procedure timeline, as long as the NAS layer can generate and pass the INVENTORY REPORT of Service1 to AS layer right before AS layer’s Service2 reception, there would be no problem of NAS handling on these two services. This operation requires the AS layer has certain buffer to temporarily store the pending MSG3 content, and to release it upon new paging reception. However, when it comes to the false attack, the timing is not guaranteed.

Figure 2.2-2: collision in access procedure with another inventory

b)	Collision between inventory procedure and command procedure: The A-IoT NAS layer receives a new inventory from lower layer when the A-IoT NAS layer is performing the command procedure.

Figure 2.2-3: collision in command procedure with another inventory

Similar to collision in paging ID check procedure with another inventory, collision in command procedure with another inventory is illustrated in Figure 2.2-3. It even worsens the situation for the WRITE service. For instance, the device is required to write 8-byte content for its memory. Passing the security check for the command operation itself, its NAS layer begins to write the memory bit by bit. Upon writing the third byte, the A-IoT NAS entity receives another paging ID check request from its MAC entity, it has to abort the writing, leaving the first 3 bytes written and last 5 bytes unchanged. This is not something the A-IoT client would like to see. To avoid this problem with no change to the current agreement, additional operation should be introduced to remove the written memory once the device starts another service with the former one unfinished. Another example can be the disable command, it is not fully discussed and defined that whether the device should or should not respond to another paging while its ongoing command service is to disable itself. In this sense, some kind of reverse operation should be defined by CT1 for NAS memory.

Above all, regarding the false attack issue, any solution of AS layer cannot totally resolve it, which should be of SA3 realm to find a way out.

Observation 2a.	CT1 considers that the device responds to the new paging after completion of an ongoing inventory or command service i.e., the device’s NAS layer would not expect to leave the service unfinished, primarily because:

- system is vulnerable to false attack;

- especially for the write or disable command, partially written memory and partially disabling issue needs to be discussed solved by some reversal operation.

Observation 2b.	If the device does not respond to the new paging until completion of the former service, it would at least ensure the device to complete the ongoing procedure, and simplify the device NAS operation.

Observation 3.	In regard to the false attack issue, since the device cannot verify the network, the both current solutions cannot fully address it.

  - Contrastive analysis

Based on the above observations, a comparison of these two different UE behaviours about A-IoT NAS and MAC impacts, and the corresponding pros and cons, respectively, in Table 2.3-1.

Table 2.3-1: contrastive analysis on two different UE behaviors

| Device behaviour | Always responding to the currently received paging | Only responding to paging after completion of an ongoing procedure |
| --- | --- | --- |
| MAC impacts | N/A | - Definition on completion of an ongoing procedure from device’s perspective, or; introduction of dedicated R2D msg to let the reader/NW release the device from one service procedure - Device to ignore received paging msgs until completion of an ongoing service |
| NAS impacts | Abortion/reversion of the ongoing service upon receiving paging ID check request from lower layer | N/A |
| Pros | - No complex MAC logical judgement - Fit for R19 deployment | - At least allowing device to finish each service (i.e., except for security check failure case) - forward compatible to future deployment - Suitable for the serial service handling - Mitigate the security attack |
| Cons | - Risky to complete any normal service, still not completely address false attack issue - Additional NAS operation | - huge RAN2 work at the end of R19 with possible NBC change - still not completely address false attack issue |

We hope RAN2 could broadly discuss and determine which way to go on the basis of the above analysis, and then provide feedback to CT1 accordingly.

In our point of view, the problem from CT1 mainly regarding the false attack scenario. However, their solution cannot fully address the issue due to no verification at the device side, and the scenario may not exist in R19 A-IoT which may be only in a private network, the fake reader is physically isolated. Overall, we would not want a fallback to the previous RAN2 discussion and still come to an incomplete solution, but rather count on the network deployment and wait for a more complete security solution in future. In this understanding, we provide a draft reply LS to CT1 in Annex.

- Regarding the CT1 raised issue, no RAN2 Spec change is needed, i.e., RAN2 does not consider any parallel service scenarios for R19 A-IoT indoor private network deployment. RAN2 feedbacks CT1 that the R19 device always responds to the new service indicated by the received paging message applicable for the device.

- Conclusion

On a basis of above analysis, we make the following observations and proposals regarding the device behavior upon the new paging reception with another ongoing procedure:

Observation 1a.	RAN2 deems that the device only responds to the currently received paging, primarily assuming overlapping-reader scenarios are improbable and avoided by network implementation under R19 indoor A-IoT deployments.

Observation 1b.	If the device always responds to the currently received paging, it would ease the burden of R19 low-capable device’s AS processing complexity, memory and power consumption.

Observation 2a.	CT1 considers that the device responds to the new paging after completion of an ongoing inventory or command service i.e., the device’s NAS layer would not expect to leave the service un-finished, primarily because:

- system is vulnerable to false attack;

- especially for the write or disable command, partially written memory and partially disabling issue needs to be discussed solved by some reversal operation.

Observation 2b.	If the device does not respond to the new paging until completion of the former service, it would at least ensure the device to complete the ongoing procedure, and simplify the device NAS operation.

Observation 3.	In regard to the false attack issue, since the device cannot verify the network, the both current solutions cannot fully address it.

- Regarding the CT1 raised issue, no RAN2 Spec change is needed, i.e., RAN2 does not consider any parallel service scenarios for R19 A-IoT indoor private network deployment. RAN2 feedbacks CT1 that the R19 device always responds to the new service indicated by the received paging message applicable for the device.

- Reference

- C1-256624	LS on handling of inventory and command collision

- R2-2506490	Introduction of Ambient IoT	CMCC

- TS 24.369	Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS)

- Chair’s note RAN1#122bis

- Annex

3GPP TSG-RAN WG2 Meeting #132	R2-xxxxxx

Dallas, U.S.A., 17th – 21st Nov. 2025

Title:	[DRAFT] Reply LS on handling of inventory and command collision

Response to:	R2-2508006 (C1-256624)

Release:	Release 19

Work Item:	AmbientIoT-CT

Source:	vivo [TO BE RAN WG2]

To:	CT WG1

Cc:	SA WG3

Contact Person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 thanks CT1 for the thorough thoughts over the device behaviour in the service collision scenario.

RAN2 acknowledges that based on the current RAN2 agreement, that the device always responds to the current paging message. Regarding the CT1’s question on false attacks, RAN2 understands based on R19 A-IoT deployment of the indoor isolated private network, the attack may not exist. Besides, since R19 authentication is verified at the network, the device cannot verify oppositely according to SA3. The attack issue cannot be actually resolved in a way that the device does not respond to the new paging message until the ongoing service completion, or at least there is no suitable AS solution.

In conclusion, RAN2 retains the previous agreement that The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device.

2. Actions:

To CT1 group

ACTION: 	RAN2 respectfully asks CT1 to take RAN2’s understanding into account at least to support R19 Ambient IoT.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #133	February 9 – 13, 2026	Gothenburg, SE

TSG RAN WG2 Meeting #133bis	April 14 – 17, 2026	Malta, MT