3GPP TSG-RAN WG2 Meeting #132	R2-2508113

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.2.1

Source:	OPPO

Title:	Discussion on 6G UE Capability

Document for:	Discussion and Decision

Introduction

In RAN2#131bis, R2 reached the following conclusion

Agreements on UE capabilities for next meeting For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them. Identify what actions to be triggered with other WGs. Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities. Understand if there is impact to other WGs. Companies can focus on the practical IODT problems in the field. Possible ways to address them.

This contribution is to discuss the remaining issues accordingly.

Discussion

  - UE Capability Signalling Overhead Reduction

Since 4G when CA was introduced, UE capability message size grows rapidly. Considering the capability can be categorized as per-band and per-BC (which can be further refined as per-FS, per-FSPC and etc.), more supported bands would lead to the increase of per-band capability in a linear manner, but the increase of per-BC (and per-FS, per-FSPC capability) in an exponential manner.

The following is one example calculation of the signalling overhead for a capability message, for NR SA case, which shows that the main source of signalling overhead is the BC list and/or the FS/FSC list. And it can be understood that larger number of supported band combination and/or the number of supported FSC entries, would lead to higher ratio of overhead.

Table 4 Two examples of UE capability signalling overhead (for NR SA)

| Source of Capability IE/Fields | Case-1 |  | Case-2 |  |
| --- | --- | --- | --- | --- |
|  | Ratio (of the whole capability message) | More details | Ratio (of the whole capability message) | More details |
| supportedBandListNR | 12% | ~20 Bands | 4% | ~10 Band entries |
| supportedBandCombinationList | 50% | ~180 BC:s | 13% | ~80 BC:s |
| featureSetCombinations + featureSet | 35% | ~110 FSC:s (containing ~660 FSC entries) ~30 DL FS:s ~20 UL FS:s | 75% | ~80 FSC:s (containing ~2100 FSC entries) ~35 DL FS:s ~30 UL FS:s |

The same issue is expected for 6G system, due to the support of band combination, i.e., for spectrum aggregation.

Although UE capability is theoretically reported only upon initial registration, there are scenarios when network further enquires UE capability after initial registration, e.g., due to the different capability of network node (like different supported band(s)). It means the signalling overhead issue for capability is not one-shot, but would consume resources in the network consistently, and thus signalling overhead reduction for capability message is always a key issue.

- In 5G, UE capability message size grows quickly with the number of supported band-combinations / FS(C) entries, which leads to high signaling overhead.

To reduce the signaling overhead, in 5G, 3GPP design the FS/FSC-based method, i.e., to produce a ‘dictionary’ for the entries that potentially to be repeated multiple times, and thus replace them with an index, which points to the entries defined in the dictionary. In this way, the repetition of entries is replaced by repetition of index, and thus the related signaling overhead is expected to be reduced. Essentially, the key point here is to look for redundancy in the message and to compress it without information loss.

On the other hand, this method did not solve the issue ultimately, and then in later releases 3GPP enables segmentation of capability messages, to the maximum of 16 segmentations (which is later reduced to 6 segmentations, and the allowed number of segmentations is further changed to configurable).

While the question is, in the end, whether there is still non-trivial remaining redundancy in the capability message (after adopting the FS/FSC-based method). If so, further compression is helpful / meaningful. If not, unless major change of the signaling framework (which probably ends up with restricted UE/chipset implementation flexibility), there is no much potential gain expected by the exercise of capability signaling overhead reduction.

To answer this question, we impose some typical compression methods on top of a capability message directly, in order to understand the level of remaining redundancy (if any) after adopting the FS/FSC-based method.

Table 5 Potential capability signalling compression ratio (NR SA)

| Compression Method | Case-1 |  | Case-2 |  |
| --- | --- | --- | --- | --- |
|  | Compressed message size (compared to original size) | Further analysis of the contribution | Compressed message size (compared to original size) | Further analysis of the contribution |
| Deflate | 46% | 40% from LZ77 | 51% | 47% from LZ77 |
|  |  | 6% from Huffman Coding |  | 4% from Huffman Coding |
| LZMA | 50% |  | 53% |  |
| PPMD | 49% |  | 56% |  |

The result showed that the ASN.1 grammar-based compression (i.e., FS/FSC-based method) did not fully explore / remove the redundancy within the message, which then shows the potential to further explore method to reduce capability message size.

- Even though it has been identified that one source of the capability message overhead is per-FS/FSPC capability and thus the FS/FSC based dictionary + index method has been defined to reduce redundancy, the capability overhead is still large with remaining redundancy.

If take a step back, thinking about the root cause of this issue, i.e., the redundancy of SRB messages, it can be understood as that the lack of source-encoding (as adopted for DRB, at APP layer) caused this issue. And in 5G spec, the segmentation of SRB message hints the concern of large message size, covering

- – DLDedicatedMessageSegment The DLDedicatedMessageSegmentmessage is used to transfer one segment of the RRCResume or RRCReconfigurationmessages. - – ULDedicatedMessageSegment The ULDedicatedMessageSegment message is used to transfer segments of the UECapabilityInformationor MeasurementReportAppLayer message. SRB1 is used at transfer of segments of UECapabilityInformationand SRB4 or SRB5 is used at transfer of segments of MeasurementReportAppLayer.

And also, the segmentation-based method has been adopted to SIB7/8/12/17/17bis/23.

If any compression method to be adopted, it is preferred to be adopted/applied to different SRB messages, rather than design message-specific optimization method.

- Different from DRB, SRB has no source-encoding to reduce redundancy, which is the root cause of the redundancy and thus high signalling overhead of capability message.

Besides, there are also other solution candidates on the table, E.g.,

ASN.1 Syntax-based method: i.e., to further reduce the signaling overhead by further optimizing the ASN.1 syntax (on top of FSC/FS framework). Examples of this method includes

- To reduce the per-band/per-band-combination reporting to per-band-group/per-band-group-combination reporting. And of course, this relies on whether the band-group concept is agreed (e.g., by R4) or not.

- To merge the BC list for normal CA and Tx-Switching;

- To reduce the redundancy of FSC/FS, by defining common index and delta index separately. As shown in Table 1, redundancy remains in the FSC dictionary because even a single different field will lead to one distinct index. By introducing separate definitions for a common index and a delta index, the shared portion can be represented by the common index, while the delta index captures only the differences. This approach can further reduce redundancy.

- Further optimization of ASN.1 syntax can be considered to reduce capability signaling overhead, focusing on overhead reduction of BC list and/or FSC/FS list.

RACS-like method: i.e., to represent the capability via (a set of) ID(s). This idea has been developed during 5G, yet was not commercialized. In our view, the reason is not it was introduced late, but is more about the difficulty of defining such ID, since this requires coordination by vendors and MNOs, which may lead to the difficulty in reality.

Nevertheless, even if RACS-like method is adopted, there is a need for RRC-based capability delivery, when the ID is failed to be interpreted by AMF. I.e., the usage of RACS-based method is not exclusive to the effort of optimizing RRC-based capability reporting procedure.

- For 6G UE capability reporting, R2 study methods to reduce signaling redundancy / overhead, including but not limited to 1) Compression-based method, 2) Syntax-based method, and 3) RACS-based method, considering redundancy / overhead reduction gain and commercialization complexity.

  - UE Capability Update during RRC_CONNECTED

Based on the current 5G design, the UE capability can be updated during RRC_IDLE

<From 23.501> - 5.4.4.1 UE radio capability information storage in the AMF If the trigger to change the UE's NG-RAN or E‑UTRAN UE Radio Capability information happens when the UE is in CM-CONNECTED state, the UE shall first enter CM-IDLE state and then perform the Registration procedure with the Registration type set to Mobility Registration Update and it also includes "UE Radio Capability Update". <From 23.502> - 4.2.2.2 Registration procedures 4.2.2.2.1 General A UE needs to register with the network to get authorized to receive services, to enable mobility tracking and to enable reachability. The UE initiates the Registration procedure using one of the following Registration types: - Initial Registration to the 5GS; - Mobility Registration Update upon changing to a new Tracking Area (TA) outside the UE's Registration Area in both CM-CONNECTED and CM-IDLE state, or when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure with or without changing to a new TA, or a change in the UE's Preferred Network Behaviour that would create an incompatibility with the Supported Network Behaviour provided by the serving AMF, [...]

Due to the difficulty to do that in RRC_CONNECTED:

1) not only about how to do that on-the-fly considering the on-going R1/R2 procedure,

2) but also about how to update the RAN entity and CN entity on-the-fly considering it is a joint RAN and CN procedure.

<From 23.501> The RAN stores the UE Radio Capability information, received in the N2 message or obtained from the UE, for the duration of the UE staying in RRC_CONNECTED or RRC_INACTIVE state. Before any 5G SRVCC handover attempt from NG-RAN to UTRAN, the RAN retrieves the UE's UTRA UE Radio Capabilities from the UE. (For specific requirements for a UE operating in dual-registration mode see clause 5.17.2.1).

- In 4G/5G, UE capability update can be performed during RRC_IDLE, via TAU/MRU procedure.

On the other hand, there are scenarios / reasons that UE would like to do this during RRC_CONNECTED state, like overheating, power saving and MUSIM reasons. The way to handle that in 5G was via UAI messages, which is essentially a delta part, on top of the static UE capability, and each RAN node can combine both, to derive the applicable UE capability.

Looking into the content of UAI message, the degraded UE capability is expressed in a simplified manner

- For overheating: Reduced maximum CC number, Bandwidth, MIMO layer, which is per-CC preference

- For MUSIM: CC(s) to release, SCG release preference, and reduced MIMO layer and bandwidth (per-CC preference) and reduced MIMO layer and bandwidth (Per-FS preference)

There seems some concern raised regarding the optional reaction from network perspective, upon reception of UAI. On the other hand, there seems no logical reason that network ignores such UAI report, given that ignorance would lead to failure / interruption in the end, and hurt network performance anyway.

Anyway, it is worthwhile to re-screen the procedure to solve the concern expressed above, e.g., to compare the pros and cons between the delta-based (via UAI message reporting to revise the capability) method and directly allowing dynamic capability change during RRC_CONNECTED.

- During 5G RRC_CONNECTED, UE capability temporary down-gradation can be performed via UAI procedure, for the reasons like overheating, and MUSIM reasons.

For 6G, it would be beneficial to re-screen the procedure, not only for the ignorance issue above, but also to check the impact due to new scenario to handle, e.g., foldable UE.

For foldable UE, the difference is that the impact is limited to MIMO layer (and it is not for sure to happen, if all antennas are deployed at a same side of the UE), but not related to the degradation of support CC/CG/Bandwidth.

Given this difference, one way to handle this rely on a lite method, e.g., via L1 report, by reflecting that to be degraded MIMO performance, e.g., higher coherence level between antennas/channels.

Table 6 Comparison between different use cases of capability update

|  | Overheating | MUSIM | Foldable UE | Method for the report |
| --- | --- | --- | --- | --- |
| CC Number, CG Number | Y | Y | N | Capability report |
| Bandwidth | Y | Y | N | Capability report |
| MIMO layer | Y | Y | Y | Capability report + L1 report (RI, CSI) |

- For 6G UE capability reporting, foldable UE only relates to MIMO layer, which can be handled by either capability update or dynamic L1 report as coherence/channel-quality degradation.

Given all the aspects above, it is helpful to compare the pros/cons of different solutions

Table 7 Comparison between different capability update schemes

| Solution | Pros | Cons |
| --- | --- | --- |
| Capability updates only during RRC_IDLE (legacy solution) | No spec effort compared to 5G CN Impact is avoided | Interruption due to the RRC state transition, and lack of effective tools for UE triggered RRC state transition |
| Capability updates during RRC_CONNECTED, via reporting a delta-part/different-version (which is stored/forwarded at RAN side only) | No interruption due to RRC state transition CN Impact is avoided | But the ignorance of the report should be avoided |
| Capability updates during RRC_CONNECTED, via reporting a delta-part/different-version (which is stored/forwarded at RAN side and CN side) | No interruption due to RRC state transition Ignorance of the report can be avoided | CN Impact (relying on dynamic RAN-CN interaction during CM_ CONNECTED) |
| Capability updates during RRC_CONNECTED, by reporting multiple-versions upon initial registration, and indicate the applicable-version dynamically. | No interruption due to RRC state transition Ignorance of the report can be avoided | CN Impact (but no need for dynamic RAN-CN interaction during CM_ CONNECTED) There might be too many combinations to be pre-reported, and it is questionable whether the UE can predict all the degraded capability in advance. Even if it is feasible, it may further increase the capability report size upon initial registration, unnecessarily (since not all degraded capability would happen for sure) |
| Capability updates only during RRC_IDLE, + dynamic L1 report to reflect MIMO layer change | No spec effort compared to 5G CN Impact is avoided No interruption due to RRC state transition | Only applicable to cases with MIMO layer impact only (e.g., foldable UE) |

- For 6G UE capability update, R2 firstly identify related use cases, at least including MUSIM.

- For 6G capability update, R2 identify the affected UE capability(ies) for each identified use case respectively, to understand the similarity/difference in-between.

- For 6G UE capability update, R2 identify solution candidates for the use cases which have similar capability impact (at least including MUSIM), by evaluating the cross-WG/TSG impact, and signalling overhead increase, taking 5G solution (i.e., a delta capability stored/forwarded at RAN side) as start point.

  - UE Capability IoDT Issue

In legacy system, IoDT is an essential step before real deployment, which can be divided into two phases:

- Lab IoDT: which happens between chipset vendor and infra-vendors, mainly to identify functionality issue(s);

- Field IoDT: which involves not only chipset vendor and infra-vendors, but also UE and operators, mainly to identify performance issue(s);

Normally, before the release of a new UE or a major feature of released UEs, these IoDT has to be done sufficiently, to ensure the stability, before selling the device to market by UE vendor, and before deployment of the UE/infrastructure by MNO.

- The sufficient IoDT test (including lab testing and field testing) for chip-set and infrastructure is critical and necessary before release of product by UE-vendor and deployment by MNO.

Based on our observation, the legacy IoDT practice is able to identify and solve the issues, before product release / deployment. And it is rarely observed that there is still major remaining issue, after the IoDT has been done. I.e., it is unclear what the typical scenario of the configuration failure is, after IoDT (Lab and Field Test) has been done as in legacy, since IoDT intends to solve the issues beforehand.

- In real field, it is rarely observed that an IoDT issue still happens after IoDT test (Lab Test and Field Test) has been properly done.

Otherwise, it seems the logic of the proponent is:

Considering the IoDT has to be done for at least two vendors, one may claim this issue happens for specific network vendor, and thus it is the target that a standardized solution aim at. However, this vendor-specific IoDT issue will have severe consequence to

- Operator: since it would cause consistent failure between the concerned UE/chip-set and network in the network, which would cause major degradation of all KPIs of the network. And it would affect the operator decision on the selection of network vendor (!)

- UE: since it would cause wide feedback from consumers which hold the phone and operate in the concerned network, which means a major accident for the sold phone, and major effort for the correction. And it is quite hard to explain (and would even cause business problem rather than technical problem) if a phone that the consumer bought can enable a feature for one network vendor but not the other.

That explains the reason that operator and UE expect the product verified sufficiently, based on the IoDT test, instead of an insufficiently test product + a standardized scheme for failure handling/recover.

- If an implemented feature does not work for a specific network + chipset vendor combination, it would cause consequence for operator for collaboration with the network/chipset vendor.

- If an implemented feature (by a chip-set) does not work for a specific network + chipset vendor combination, it would cause business issue to consumer who bought the phone and access to the infrastructure of the concerned network vendor.

From this perspective, it is highly risky if 3GPP encourage a solution to reduce IoDT effort, while attempt to solve the issues, which should have been identified by IoDT, via a standardized solution, but end up with degraded performance (compared to the performance if the concerned feature is enabled).

- For 6G UE capability IoDT, R2 encourage sufficient IoDT to identify functionality and performance issue before deployment, rather than encourage standardized solution for handle the configuration failure issue due to insufficient IoDT.

- For 6G UE capability IoDT, R2 de-prioritize mechanism that leads to network/chipset vendor specific feature/capability enabling/disabling.

  - UE Capability vs. Device-Type

To achieve the “scalable and forward compatible design for diverse device types” as required in 6GR SID, a scalable feature can be adopted for 6GR air interface, and discussed at RP. A low-cost and energy-efficient “Basic feature set” can be defined for 6GR air interface, which provides “global” mandatory feature for various device types. For a specific device type, a device-type specific feature set can be defined which “grows up” from the Basic feature set.

- Beyond the Basic feature set, device type specific feature sets can be defined for 6G MBB (Smart phone, Wearable IoT), low-end IoT and other verticals.

- The Basic feature set is a lean “global” mandatory feature list for all device types, where each feature in the list shall be well justified to be truly mandatory, and provides basic energy-efficient features for various device types.

- R109 discussed the feasibility of defining a mandatory feature set across device types.

Based on R109 discussion, the follow conclusion was reached

Proposal 3: To investigate further: Motivations/justifications behind the proposed diverse device types, which should be a limited set Whether/how to have one or more device types for eMBB or 6G IoT Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. Whether/how to explicitly standardize device types Ensuring forward compatibility Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement.

While RP is to further study the related aspects, it would be helpful to speed up the progress at RAN2 level, by initiating some discussion regarding the two bullets above.

- RP has set the scope for further discussion on the diverse device types.

In our view, the two aspects above are coupled with each other, e.g., the benefit (if any) from specifying the device-type in specification.

The lesson we learned from 5G is: when 5G is standardized, there is always a long debate for UE-vendor, chipset-vendor, network-vendor and operator to decide whether/why a specific feature is mandatory or optional, which relates to all WGs and Plenary. Essentially, the difficulty comes from the fact that different capability may be required / not-required for different device-types. From network-vendor / operator perspective, it is safer to mandate one feature if it is required for at least one device-type, while from UE-vendor / chipset-vendor perspective, it is not preferred to mandate a feature if it is not for the concerned device-type.

- In 5G Day-1, the large number of UE capability leads to complexity to both vendors and operators, where one root reason is that different UE capability may be required / not-required for various UE types.

From this perspective, introducing the device-type would be very helpful to guide the mandatory / optional UE capability discussion in different WGs and Plenary. And this benefit comes without affecting signalling aspects, i.e., the capability bits (for IoDT bits and for optional capabilities) can be still reported. While the device-type specific conditionally-mandatory requirement (as adopted in legacy already) simply act as a reference for vendors and operators to decide on associated capability / features. So, there should be less impact to the capability report procedure.

- The impact of diverse-device-type on capability signalling is unclear.

- For 6G UE capability design, R2 trigger WG-level discussion (e.g., impact to capability reporting due to device-type concept) after further progress from RAN Plenary and RAN1 (e.g., clarifying the device-type concept/definition) .

Conclusion

Based on the discussion in section2, we have following proposals:

Observation 1	In 5G, UE capability message size grows quickly with the number of supported band-combinations / FS(C) entries, which leads to high signaling overhead.

Observation 2	Even though it has been identified that one source of the capability message overhead is per-FS/FSPC capability and thus the FS/FSC based dictionary + index method has been defined to reduce redundancy, the capability overhead is still large with remaining redundancy.

Observation 3	Different from DRB, SRB has no source-encoding to reduce redundancy, which is the root cause of the redundancy and thus high signalling overhead of capability message.

Observation 4	Further optimization of ASN.1 syntax can be considered to reduce capability signaling overhead, focusing on overhead reduction of BC list and/or FSC/FS list.

Observation 5	In 4G/5G, UE capability update can be performed during RRC_IDLE, via TAU/MRU procedure.

Observation 6	During 5G RRC_CONNECTED, UE capability temporary down-gradation can be performed via UAI procedure, for the reasons like overheating, and MUSIM reasons.

Observation 7	For 6G UE capability reporting, foldable UE only relates to MIMO layer, which can be handled by either capability update or dynamic L1 report as coherence/channel-quality degradation.

Observation 8	The sufficient IoDT test (including lab testing and field testing) for chip-set and infrastructure is critical and necessary before release of product by UE-vendor and deployment by MNO.

Observation 9	In real field, it is rarely observed that an IoDT issue still happens after IoDT test (Lab Test and Field Test) has been properly done.

Observation 10	If an implemented feature does not work for a specific network + chipset vendor combination, it would cause consequence for operator for collaboration with the network/chipset vendor.

Observation 11	If an implemented feature (by a chip-set) does not work for a specific network + chipset vendor combination, it would cause business issue to consumer who bought the phone and access to the infrastructure of the concerned network vendor.

Observation 12	R109 discussed the feasibility of defining a mandatory feature set across device types.

Observation 13	RP has set the scope for further discussion on the diverse device types.

Observation 14	In 5G Day-1, the large number of UE capability leads to complexity to both vendors and operators, where one root reason is that different UE capability may be required / not-required for various UE types.

Observation 15	The impact of diverse-device-type on capability signalling is unclear.

Proposal 1	For 6G UE capability reporting, R2 study methods to reduce signaling redundancy / overhead, including but not limited to 1) Compression-based method, 2) Syntax-based method, and 3) RACS-based method, considering redundancy / overhead reduction gain and commercialization complexity.

Proposal 2	For 6G UE capability update, R2 firstly identify related use cases, at least including MUSIM.

Proposal 3	For 6G capability update, R2 identify the affected UE capability(ies) for each identified use case respectively, to understand the similarity/difference in-between.

Proposal 4	For 6G UE capability update, R2 identify solution candidates for the use cases which have similar capability impact (at least including MUSIM), by evaluating the cross-WG/TSG impact, and signalling overhead increase, taking 5G solution (i.e., a delta capability stored/forwarded at RAN side) as start point.

Proposal 5	For 6G UE capability IoDT, R2 encourage sufficient IoDT to identify functionality and performance issue before deployment, rather than encourage standardized solution for handle the configuration failure issue due to insufficient IoDT.

Proposal 6	For 6G UE capability IoDT, R2 de-prioritize mechanism that leads to network/chipset vendor specific feature/capability enabling/disabling.

Proposal 7	For 6G UE capability design, R2 trigger WG-level discussion (e.g., impact to capability reporting due to device-type concept) after further progress from RAN Plenary and RAN1 (e.g., clarifying the device-type concept/definition) .