3GPP TSG-RAN WG2 Meeting #132	R2-2508115

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.2.2

Source: 	OPPO

Title:  	Discussion on 6G RRC ASN.1 Encoding

Document for:	Discussion and Decision

Introduction

This contribution is to discuss the key issues for 6G RRC ASN.1 encoding.

Discussion

Condition

For the condition of ASN.1, what has been defined in TS 36.331 is as follows

Table 1 Condition definition in 4G

| Cond conditionTag (Used in downlink only) | Conditionally present A field for which the need is specified by means of conditions. For each conditionTag, the need is specified in a tabular form following the ASN.1 segment. In case, according to the conditions, a field is not present, the UE takes no action and where applicable shall continue to use the existing value (and/ or the associated functionality) unless explicitly stated otherwise (e.g., in the conditional presence table or in the description of the field itself). |
| --- | --- |

On the contrary, which has been defined in TS 38.331 is as follows

Table 2 Condition definition in 5G

| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |
| --- | --- |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |

Where it can be observed that the usage of Cond has been proved to be useful.

- Cond is a very useful tool, to restrict the parameter combinations that UE has to tackle with.

However, for CondC and CondM, although it was used to differentiate between the two cases:

for which the following types can be distinguished:

-	Message Contents related constraints e.g., that a field B is mandatory present if the same message includes field A and when it is set value X.

-	Configuration Constraints e.g., that a field D can only be signalled if field C is configured and set to value Y. (i.e., regardless of whether field C is present in the same message or previously configured).

It has not been used so far although defined, as clarified

NOTE:	In this version of the specification, the condition tags CondC and CondM are not used.

- CondC and CondM, although defined in NR initially, have not been used in the end, i.e., NR ASN.1 fully relies on Cond.

Considering the status, it seems Cond is sufficient.

- For 6G ASN.1, R2 study to simplify the usage of condition, e.g., keep the cond but avoid defining CondC and CondM.

Need Code

Then compare the need code definition in TS 36.331 (upper)

Table 3 Need code in 4G

| Need OP (Used in downlink only) | Optionally present A field that is optional to signal. For downlink messages, the UE is not required to take any special action on absence of the field beyond what is specified in the procedural text or the field description table following the ASN.1 segment. The UE behaviour on absence should be captured either in the procedural text or in the field description. |
| --- | --- |
| Need ON (Used in downlink only) | Optionally present, No action A field that is optional to signal. If the message is received by the UE, and in case the field is absent, the UE takes no action and where applicable shall continue to use the existing value (and/ or the associated functionality). |
| Need OR (Used in downlink only) | Optionally present, Release A field that is optional to signal. If the message is received by the UE, and in case the field is absent, the UE shall discontinue/ stop using/ delete any existing value (and/ or the associated functionality). |

and TS 38.331 (lower).

Table 4 Need code in 5G

| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behaviour performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behaviour when field is not configured). |
| --- | --- |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e., not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e., not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

I.e., the legacy Need-ON in 4G is further split into need-M and need-N in 5G. In other words, the identified cases are the same between 4G and 5G, while the only difference is whether to handle it jointly or separately.

1) Need-OP/S are similar to each other, i.e., it has to rely on spec to handle the absence case anyway, i.e., it is hard to rely on a parameterized type to handle this type.

2) Need-N does not need any action from UE, so there is no need to spend spec effort on it specifically

- Need-OP/Need-S relies on specified text, it is hard to be handled by pre-defined parameterized type. And Need-N does not require further indication.

3) The two remaining cases, i.e., the Need-OR/Need-R and Need-ON/Need-M, there are two ways on the table: one is to follow legacy way, i.e., to define it in the specification which consumes no additional bit and thus low overhead, the other is to adopt a new method, i.e., to indicate it via a newly defined parameterized type, and to indicate either of the two cases via 1-bit, which saves effort for implementation to remember the need-code, but consumes additional bit / overhead.

- Need-OR/Need-R and Need-ON/Need-M operation is clear, and there are two methods to handle the operation, i.e., via need-code or via a pre-defined parameterized type.

According to 131bis conclusion

Agreements

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g., improvements to delta signaling or no delta signaling).

Between the two, the benefit from avoiding remembering need-code is unclear, considering the need-S and need-N may require pre-storing of the need-code anyway, while the pain is clear, i.e., the additional bit for overhead is huge, since 1-bit is to be consumed for each concerned field.

- Considering that need-code would be anyway needed at least for need-S, there seems no big reason to waste bit (1-bit per field) to explicitly indicate the need-M and need-R for each related field.

Considering this point, we tend to keep the current way, i.e., to avoid additional signalling overhead.

- For 6G ASN.1, reuse need-code definition in 5G, including Need-S, need-N, need-M and need-R.

Modular ASN.1

According to 131bis conclusion

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g., based on features, functions, verticals, etc.

I.e., one topic is which dimension is to be used for the modularization of ASN.1.

To have an answer on this question, one has to understand the original reason for the modularization of ASN.1. In our view, it is to simplify the implementation, i.e., if for device-A, the implementation of ASN.1 can be, to a large extent, decoupled from the implementation for device-B, then the two cases can be used as a dimension of ASN.1 modularization, i.e., the separation of the ASN.1 of the two devices helps, in the sense that one implementation does not have to concern the implementation of the other.

- One key motivation for Modular ASN.1 design is mainly to avoid device-type-A implementing ASN.1 for device-type-B unnecessarily.

Then the issue here is to look into which dimension to use for the modularization, given the options listed in 131bis.

Yet here at this time point, it is hard to predict which features / functions /verticals are to be developed for 6G, not only in Day-1, but also in Day-2. In order to shed a light for this, one way is to look back at 5G, i.e., which features were developed via normative work.

Table 5 Topics for 5G normative work

| Type-A | Type-B |
| --- | --- |
| NTN Redcap/Wearable M-IoT (NB/A-IoT and etc.) SL/V2X UAV HRLLC IAB NCR MBMS | AIML Mobility MIMO Coverage Enh Energy Efficiency New Spectrum (e.g., 71GHz) Multi-Carrier Multi-Connectivity DSS/MRSS Slicing |

In our view, the features can be categorized as two types, where

- Type-A is more for verticals, or device-types, which to a large extent has a different form factor, for which it is easier to claim the benefit of modular ASN.1, i.e., the implementation for these verticals / device-types are sort of independent;

- Type-B is more for functions / features, which is not exclusive to each other (not only features within type-B, but also the verticals within type-A). E.g., one cannot ensure one device implementing AIML will not implement Mobility Enh, and also one cannot ensure one device implementing MIMO will not implementing NTN.

So that the two types should make a difference in terms of the ASN.1 modularization.

- Looking back to normative work during 5G, they can be categorized into two categories: 
1) Category-A: features for a verticals / device type that potentially has specific form factor, e.g., NTN, SL/V2x, A/NB/M-IoT, UAV, Redcap, IAB, NCR, MBMS; 
2) Category-B: features / functions that can be supported by various device types: AIML, Mobility Enh, MIMO, Coverage, Power, DSS, 2-step RACH, SDT, MUSIM, 71GHz, XR, Multi-Carrier/Connectivity, Slicing.

- Features for category-A has specific form-factor, and thus motivate a separate track of ASN.1.

For the category-B

- On the one hand, there could be some motivation to adopt the modular ASN.1 design, if there is one feature which is not mandatory to all device-types, so that the implementation of the feature can be independent / decoupled from the core functions;

- On the other hand, considering that R21 would only focus on critical features that are essential for Day-1 release of 6GR, so it is hard to claim which feature(s) are optional for a specific device type (which may become clearer though during normative phase) at the current early stage. Furthermore, sometimes it is hard to differentiate which feature a specific parameter belongs to, i.e., it may be necessary for multiple features.

So, it seems even though some modularization is applied based on category-B, the decoupling extent / level should be smaller / lower than category-A.

- Features for category-B motivates more a core-function + additional function type design, yet it is highly dependent on the normative phase for 6GR to understand, e.g., which feature(s) are to be included as core (or additional/optional) functionality.

And thus, it is straightforward that type-A should be the base of ASN.1 modularization.

- For 6G ASN.1 design, R2 study to adopt modular ASN.1 design based on verticals / device-types of different form factor, to decouple the RRC design. FFS on device-types (pending progress of RP and other RAN WGs).

- For 6G ASN.1 design, R2 study to adopt modular ASN.1 design based on features / functions, for a minimum / core function set + additional / optional function set. While the decision has to be postponed until it is clearer which features are to be covered for 6GR Day-1 and thus it is clearer whether there is a way to categorize those features.

Then the next step is how to implement the modularized ASN.1, which can be in different forms

- (A set of ) IE level: Within the same message, independent IE threads for the different cases;

- RRC Message level: Within the same RRC modular in the same specification, to define separate RRC messages (and thus separate procedures) for the different cases;

- ASN.1 module level: Within the same specification, to define different RRC modular, for different cases;

- Specification level: The extreme way is to separate the specification for different cases, so that helps the industry from the very beginning.

And if both category-A and B are adopted, likely different level of modularization is to be applied accordingly.

- For 6G ASN.1 modularization, R2 study to which level the modularization is to be done, e.g., IE structure level, RRC message level, ASN.1 module level or specification level.

Conclusion

Based on the discussion in section2, we have following proposals:

Observation 1	Cond is a very useful tool, to restrict the parameter combinations that UE has to tackle with.

Observation 2	CondC and CondM, although defined in NR initially, have not been used in the end, i.e., NR ASN.1 fully relies on Cond.

Observation 3	Need-OP/Need-S relies on specified text, it is hard to be handled by pre-defined parameterized type. And Need-N does not require further indication.

Observation 4	Need-OR/Need-R and Need-ON/Need-M operation is clear, and there are two methods to handle the operation, i.e., via need-code or via a pre-defined parameterized type.

Observation 5	Considering that need-code would be anyway needed at least for need-S, there seems no big reason to waste bit (1-bit per field) to explicitly indicate the need-M and need-R for each related field.

Observation 6	One key motivation for Modular ASN.1 design is mainly to avoid device-type-A implementing ASN.1 for device-type-B unnecessarily.

Observation 7	Looking back to normative work during 5G, they can be categorized into two categories:  1) Category-A: features for a verticals / device type that potentially has specific form factor, e.g., NTN, SL/V2x, A/NB/M-IoT, UAV, Redcap, IAB, NCR, MBMS;  2) Category-B: features / functions that can be supported by various device types: AIML, Mobility Enh, MIMO, Coverage, Power, DSS, 2-step RACH, SDT, MUSIM, 71GHz, XR, Multi-Carrier/Connectivity, Slicing.

Observation 8	Features for category-A has specific form-factor, and thus motivate a separate track of ASN.1.

Observation 9	Features for category-B motivates more a core-function + additional function type design, yet it is highly dependent on the normative phase for 6GR to understand, e.g., which feature(s) are to be included as core (or additional/optional) functionality.

Proposal 1	For 6G ASN.1, R2 study to simplify the usage of condition, e.g., keep the cond but avoid defining CondC and CondM.

Proposal 2	For 6G ASN.1, reuse need-code definition in 5G, including Need-S, need-N, need-M and need-R.

Proposal 3	For 6G ASN.1 design, R2 study to adopt modular ASN.1 design based on verticals / device-types of different form factor, to decouple the RRC design. FFS on device-types (pending progress of RP and other RAN WGs).

Proposal 4	For 6G ASN.1 design, R2 study to adopt modular ASN.1 design based on features / functions, for a minimum / core function set + additional / optional function set. While the decision has to be postponed until it is clearer which features are to be covered for 6GR Day-1 and thus it is clearer whether there is a way to categorize those features.

Proposal 5	For 6G ASN.1 modularization, R2 study to which level the modularization is to be done, e.g., IE structure level, RRC message level, ASN.1 module level or specification level.