3GPP TSG-RAN WG2 Meeting #132	R2-2508130

# Dallas, US, 17 – 21 Nov 2025

Agenda item:		10.2.2

Source:	T-Mobile USA

Title:	Introduction of explicit Network Type Indicator for 6G

Document for:		Discussion and Decision

# Introduction

In RAN#108 meeting, following TN and NTN objective was introduced [1]:

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In this contribution, we propose to introduce an explicit network type indicator (TN or NTN) for 6G network from the beginning of its design.

# Discussion

## Background

NR NTN was first introduced in R17 to enable global and remote area coverage where terrestrial networks are unavailable or impractical. To prevent legacy devices from attempting to access NTN systems, a MIB-based barring mechanism (cellBarred) was introduced; meanwhile,  NTN-capable devices are allowed to ignore this MIB barring while cellBarredNTN is included in SIB1.

Per TS 38.331

cellBarred

Value barred means that the cell is barred, as defined in TS 38.304 [2]. This field is ignored by IAB-MT and NCR-MT. This field is ignored for connectivity to NTN or ATG.

Per TS 38.304:

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

Originally, a barred cell meant no UE access under any condition. However, to accommodate later-added NTN functionalities, multiple exceptions were introduced (e.g., barring in MIB but unbarring in SIB1).

Observation 1: In 5G, the lack of network type indicator required exceptions to MIB barring.

As 6G is being designed from the ground up, there is an opportunity to eliminate these inconsistencies and design a more streamlined and robust system architecture from the start.

An explicit Network Type Indicator would eliminate the need for post-hoc exceptions like cellBarredNTN, simplifying UE behavior and ensuring robust forward compatibility for TN and NTN integration.

We would assume that the 6G MIB message would be similar to today’s 5G MIB; we would propose to introduce Network Type Indicator in MIB message, which can explicitly indicate the current network is TN or NTN network.

Proposal 1: RAN2 to consider including Network Type Indicator in MIB message for 6G system from the very beginning.

# Conclusion

This contribution highlights the opportunity to simplify system design by introducing an explicit network type indicator at 6G network:

Observation 1: In 5G, the lack of network type indicator required exceptions to MIB barring.

Proposal 1: RAN2 to consider including Network Type Indicator in MIB message for 6G system from the very beginning.

# References

[1] RP-251881, NTT Docomo (Moderator), "Study on 6G Radio"

[2] 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state