3GPP TSG-RAN WG2 Meeting #132															R2- 2508135

Dallas, USA, 12-16 Nov., 2025

Agenda item:	8.7.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues for RLC in R19 XR

Document for:	Discussion and Decision

- Introduction

During RAN2#131bis, the issue [RLC-H02] has been discussed based on the following paper

R2-2507056	Discussion on remaining issues for RLC for R19 XR	Huawei, HiSilicon	discussion	NR_XR_Ph3-Core

After the discussion, the following agreement has been reached

- (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

We discuss on the remaining issues for RLC spec for R19 XR

- Discussion

In section 5.3.4 of RLC specifications, the STATUS PDU construction is common regardless of the STATUS PDU trigger. However, when t-RxDiscard expires, RX_Next is updated to RX_Next_Discard_Trigger and after the update, RX_Next might be larger than RX_Highest_Status. An example has been given during the meeting in [R2-2507903].

Hence, the following condition is wrong as it implies that RX_Next is always lower than RX_Highest_Status.

- for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

When the status report is triggered by t-RxDiscard, because the RX_Next is updated to the position of RX_Next_Discard_Trigger, all the RLC SDUs below RX_Next_Discard_Trigger will be discarded. Hence, a straightforward solution would be just to set the ACK_SN to the updated RX_Next. Then, the SDUs before RX_Next would be indicated as ACK.

During the meeting, some companies have argued that we could update RX_Highest_Status to RX_Next_Discard_Trigger. However, this solution is still problematic, because:

- When RX_Highest_Status is updated to RX_Next_Discard_Trigger, RX_Highest_Stauts=RX_Next. However, the assumption in the condition of the procedure is that RX_Next< RX_Highest_Status as mentioned above. So, the condition in the legacy text is still wrong in this case.

- RX_Highest_Status is coupled with the variable RX_Next_Status_Trigger, which keeps track of the value to which the RX_Highest_Status should be updated when t-Reassembly expires. The use of RX_Highest_Status and RX_Next_Status_Trigger go hand in hand such that the RX side does not repeatedly wait for a certain part of the reception window. Deviating from the previous procedure by updating the RX_Highest_Status when t-RxDiscard expires will result in un-predicted UE behaviors and offsets the previous intended behavior for UE RX side reception.

Based on the discussion above, we believe that the best solution is to specify the content of the RLC STATUS PDU triggered by the t-RxDiscard separately and indicate that ACK_SN is set to RX_Next for this case. The corresponding TP can be as follows:

When constructing a STATUS PDU not triggered by the expiry of t-RxDiscard, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: =====omitted=== When constructing a STATUS PDU triggered by the expiry of t-RxDiscard, the AM RLC entity shall: - set the ACK_SN as the updated RX_Next

Hence, we propose the following:

Proposal 1:	Specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate paragraph where ACK_SN is set to RX_Next.

- Conclusion

This contribution makes the following proposals:

Proposal 1:	Specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate paragraph where ACK_SN is set to RX_Next.