3GPP TSG-RAN WG2 Meeting #132	R2-2508139

Dallas, USA,  Nov. 17th - 21st

Agenda Item		:	10.3.2.2

Source		:	LG Electronics Inc.

Title		:	Considerations on RRC (re)configuration structure

Document for		:	Discussion and Decision

1. Introduction

In RAN2#131bis meeting, following agreements are agreed.

RAN2#131bis Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

During the meeting, companies shared their views on NR RRC structure. It has been identified that too-deep nested structure of RRC ASN.1 definition causes excessive UE parsing overhead especially for delta configuration. To this end, RAN2 agreed to study how to modularize RRC.

In this contribution, we provide our views on how to address too-deep nested structured RRC ASN.1 definition.

2. Discussion

In the early release of NR, hierarchical structuring principle is used to define ASN.1 structure. Hierarchy principle has clear benefit that it is straightforward to define signalling structure among configurations having conceptual superior-subordinate relations. For example, physicalCellGroupConfig, mac-CellGroupConfig, rlc-BearerConfig and RadioBearerConfig are included within CellGroupConfig. Each IE includes lower-layer specific configurations of PHY, MAC, RLC and PDCP for cell group, respectively. In addition, cell specific configurations (spCellConfig and sCellConfig) are also included within CellGroupConfig. Each IE includes serving cell specific configurations, e.g. downlink/uplink configuration, respectively. The legacy IEs are already modularized as layer configuration and cell specific configuration. In our understanding, the current RRC ASN.1 structure is already defined as module basis.

Observation 1. 	Current ASN.1 structure is already modularized as layer configuration and RRC functionality configuration, in which they are defined as individual IE, respectively.

Among modules, cell specific configuration is one of the most deepest module that has a maximum depth of 9 as illustrated in Figure 1. As BWP is introduced in NR, the configuration for BWP is defined as the series of IEs, i.e. BWP, BWP-Downlink, BWP-DownlinkCommon, BWP-DownlinkDedicated, BWP-Uplink, BWP-UplinkCommon and BWP-UplinkDedicated. They are defined to be included under cell specific IEs according to hierarchical structuring principle. However, the cell specific configurations are already defined as depth 3, the depth of BWP configurations cannot help to being signalled with significant depth.

Observation 2. 	Hierarchical structure of BWP configuration is too-deep, resulting in excessive parsing overhead for delta configuration.

Proposal 1. 	RAN2 to identify which module has too-deep nested structure resulting in excessive UE parsing overhead for delta configuration. BWP modules can be considered how to relax the depth as a starting point.

Figure 1. Hierarchy of CellGroupConfig focused on BWP specific configuration for SpCell.

As a potential solution to reduce the depth of BWP configuration, one can consider to define BWP configuration module  outside the cell specific configuration as a list of BWP instances and cell specific configuration is associated with one or more BWP instances via BWP instance ID as shown in Figure 2. Specifically, BWP configuration having a list of BWP instances is defined as depth-1 IE, where each BWP instance is assigned a BWP-Id. Then, when reconfiguration of BWP instance is needed, network only needs to update the BWP configuration module by reconfiguring the BWP instance with the BWP-Id. Note that network does not need to signal anything related to CellGroupConfig module in this flat structure.

Figure 2. Flatten BWP configuration structure to resolve too-deep nested structure for BWP configuration.

Proposal 2. 	RAN2 to focus on flattening signalling structure, by defining modules as e.g., depth-1.

- Instance of a module has an identifier.

- One module can have configuration of another module as its sub-configuration, where the instance of the calling module is associated with the instance of the callee module via ID of the module instance.

3. Conclusion

In this contribution, we made following observations:

Observation 1. 	Current ASN.1 structure is already modularized as layer configuration and RRC functionality configuration, in which they are defined as individual IE, respectively.

Observation 2. 	Hierarchical structure of BWP configuration is too-deep, resulting in excessive parsing overhead for delta configuration.

Finally, we propose following statements:

Proposal 1. 	RAN2 to identify which module has too-deep nested structure resulting in excessive UE parsing overhead for delta configuration. BWP modules can be considered how to relax the depth as a starting point.

Proposal 2. 	RAN2 to focus on flattening signalling structure, by defining modules as e.g., depth-1.

- Instance of a module has an identifier.

- One module can have configuration of another module as its sub-configuration, where the instance of the calling module is associated with the instance of the callee module via ID of the module instance.