3GPP TSG-RAN WG2 Meeting #132	R2-2508152

Dallas, USA, Nov. 17th - 21st

Source:	CATT

Title:	Consideration on LO determination in TS 38.304

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

RAN1 sent an LS on the LO determination based on lpwus-LoFrameOffsetList for LP-WUS operation in IDLE/INACTIVE mode to RAN2 [1]. In this contribution, we would like to confirm to capture corresponding agreements in TS 38.304 with a corresponding TP in Annex.

Discussion

The following is the description in the LS[1].

In RAN1#122bis, the following clarification was made regarding the LO determination based on lpwus-LoFrameOffsetList: Agreement The LO determination based on lpwus-LoFrameOffsetList is clarified as follows: - If the number of POs associated with a LO is less than Ns (the number of POs per PF), the UE uses the (i_sNPOWO+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (i_sNPOWO+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList. - For the case when both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured, - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay; - Otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay; - Otherwise, the UE monitors legacy PO. - For the case when only one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is configured, - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay (whichever is configured) and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO; - Otherwise, the UE monitors legacy PO. RAN1 respectfully asks RAN2 to capture the above agreement in TS 38.304.

How to determine the value of frame level offset within the configured list

According to the LS, it can be seen that RAN1 defines how to determine the value of frame level offset based on lpwus-LoFrameOffsetList, which is not captured in current RAN1 and RAN2 specification. Hence,  RAN2 needs to confirm whether and how to determine the value of frame level offset according to the received lpwus-LoFrameOffsetList, as highlighted in green above.

For PEI, the similar clarification is added in the field description in TS 38.331 as follows.

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

Similarly, the procedure on determining the value of frame level offset based on lpwus-LoFrameOffsetList should be captured in the spec.

Regarding on which spec to capture the agreement, we can follow RAN1 LS to capture this in TS 38.304. Hence, in TS 38.304, we need to clarify: If lpwus-PoNumPerLo (the number of POs associated with a LO) is less than Ns (the number of POs per PF), the UE uses the (floor(i_s/lpwus-PoNumPerLo)+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (floor(i_s/lpwus-PoNumPerLo)+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList. Furthermore, similar as PEI, we need to clarify: If lpwus-PoNumPerLo is not less than Ns, the UE use the first configured value in offsetForLongerWakeUpDelay (if configured) and offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList.

Proposal 1: Clarify in TS 38.304 how to determine the value of frame level offset according to the received lpwus-LoFrameOffsetList according to the RAN1 LS. Furthermore, clarify that the UE use the first configured value in offsetForLongerWakeUpDelay (if configured) and offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList if lpwus-PoNumPerLo is not less than Ns.

Whether to monitoring with LP-WUS and which frame level offset is applied

For the second part in the LS highlighted with cyan, i.e., how the UE determines whether to monitor with LP-WUS and which LO is monitored, RAN2 can update these in TS 38.304 according to the LS directly to make the UE behavior more clear.

Proposal 2: Update how the UE determines whether to monitors with LP-WUS and which LO is monitored in TS 38.304 according to the RAN1 LS.

Conclusion

According to section 2, the following and proposals are provided:

Proposal 1: Clarify in TS 38.304 how to determine the value of frame level offset according to the received lpwus-LoFrameOffsetList according to the RAN1 LS. Furthermore, clarify that the UE use the first configured value in offsetForLongerWakeUpDelay (if configured) and offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList if lpwus-PoNumPerLo is not less than Ns.

Proposal 2: Update how the UE determines whether to monitors with LP-WUS and which LO is monitored in TS 38.304 according to the RAN1 LS.

The corresponding TP is provided in Annex.

Reference

- R2-2508020, LS on the LO determination based on lpwus-LoFrameOffsetList for LP-WUS operation in IDLE/INACTIVE mode (R1-2508170; contact: Apple)

Annex

- 7.5.0	General

The UE supporting LP-WUS may monitor LP-WUS in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If LP-WUS configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting LP-WUS (except for the UEs expecting MBS group notification) may start LP-WUS monitoring using LP-WUS parameters in system information according to the procedure described below if the entry condition in clause 7.5.1 is fulfilled. The UE may stop LP-WUS monitoring and follows the paging monitoring procedure as described in clause 7.1 or 7.2 if the exit condition in clause 7.5.1 is fulfilled.

If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.4C in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2 if PEI is supported and PEI configuration is provided in system information, which is up to UE implementation. If the UE does not detect an LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.4C in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1.

The UE monitors one LO per DRX cycle. An LO is a set of LP-WUS monitoring occasions (LP-WUS MOs). In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams for which LP-WUS is configured and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation.

The time location of an LO for UE’s PO is determined by a reference PF and the configured frame-level offset:

-	The reference PF is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is configured), associated with the LO. The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) *T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4C in TS 38.213 [4], T, Ns, and N are determined in clause 7.1.

-	The frame-level offset between the LO and the reference PF is provided by lpwus-LoFrameOffsetList in SIB1. If lpwus-PoNumPerLo is less than Ns, the UE uses the (floor(i_s/lpwus-PoNumPerLo)+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (floor(i_s/lpwus-PoNumPerLo)+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList. Otherwise, the UE use the first configured value in offsetForLongerWakeUpDelay (if configured) and offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList.

When only one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is configured in lpwus-LoFrameOffsetList:

-	If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay (whichever is configured) and the start of the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset;

-	Otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

When both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured in lpwus-LoFrameOffsetList:

-	If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay;

-	Otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay;

-	Otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

If the UE in RRC_IDLE and RRC_INACTIVE is unable to monitor the LP-WUS occasion in all MO corresponding to its PO, the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

In RRC_INACTIVE state, when the UE uses the same i_s as for RRC_IDLE state as specified in clause 7.1, the UE shall use the same iPO for LP-WUS as for RRC_IDLE state. Otherwise, the UE determines the iPO for LP-WUS based on the i_s for RRC_INACTIVE state.

For a UE supporting paging adaptation and LP-WUS, if lpwus-LoFrameOffsetListForPagingAdapt is signaled in system information, the UE in RRC_IDLE or RRC_INACTIVE state can monitor the LP-WUS occasion according to lpwus-LoFrameOffsetListForPagingAdapt.