3GPP TSG-RAN WG2 Meeting #132	R2-2508153

Dallas, USA,  Nov. 17th - 21st

Source:	CATT

Title:	Discussion on Reply LS to SA2 on PO-to-LO configuration

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In SA2#171 meeting, SA2 replied LS to RAN3 and RAN2 [1] with following questions.

SA2 discussed the issue and would like to receive some further information from RAN3 and RAN2: a) What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose. b) Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less? c) Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213. SA2 would appreciate replies to the above questions. 2. Actions: To RAN3, RAN2 ACTION: SA2 kindly asks RAN3 to provide answers for a, c) and RAN2 to provide answers for b).

As highlighted in yellow, RAN2 needs to answer whether the paging PO-to-LO association configuration will be change often, e.g., to save base station energy when the paging load is less.

In this document, we discuss this issue.

Discussion

According to current RRC specification, the paging PO-to-LO association configuration, i.e., lpwus-PoNumPerLo, indicates the number of PO(s) associated with one LO for LP-WUS in the cell for RRC IDLE/INACTIVE, with 3 candidate values including 1, 2 and 4.

The paging rate per PO was considered as one of the key aspects when RAN1 defined the values of lpwus-PoNumPerLo. During the first stage, RAN1 agreed that at least one-to-one mapping between LO and PO is supported. However, some companies thought the effective paging rate per PO is relatively low, e.g. about 10% based on the assumption used in RAN1 simulation. If one-to-one mapping between LO and PO is used, it means that for 90% of the time there is no LP-WUS transmission in one LO. This would result in much resource wasting for LP-WUS. Therefore, one-to-many mapping between LO and PO was agreed in RAN1. NW can configure suitable values of lpwus-PoNumPerLo for different paging rate which also facilitates balancing base station energy and the paging load.

From RAN2 perspective, the network configures lpwus-PoNumPerLo according to the statistical analysis on the served UEs with large time scale. Therefore, the paging rate per PO doesn’t change dynamically. Hence, RAN2 can reply SA2 that the paging PO-to-LO association configuration will NOT be changed often.

Proposal: RAN2 confirms and replies to SA2 that the paging PO-to-LO association configuration will NOT be changed often.

Conclusion

In this document, we analyze the issue whether the paging PO-to-LO association configuration will be changed often, and the following proposal is provided:

Proposal: RAN2 confirms and replies to SA2 that the paging PO-to-LO association configuration will NOT be changed often.

Reference

- R2-2508030, Reply LS on allocation of CN assigned subgroup ID for LP-WUS.