3GPP TSG-RAN WG2 Meeting #132	R2-2508158

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	8.9.3

Source: 	CATT, Google, Huawei

Title: 	Discussion on cell reselection enhancement based on the S&F monitoring list

Document for: 	Discussion and Decision

Introduction

In RAN2#131, we discussed whether to use the S&F monitoring list during the cell reselection procedure to assist UE in selecting a satellite included in the S&F monitoring list, which is beneficial for UE to save power and to determine which satellites to receive MT data/signalling [1]. Companies had divergent views on this issue. For compromise, it is proposed to forward the satellite ID information to NAS layer and it is NAS’s responsibility for using this information. We sent an LS to CT1 [2] asking whether the satellite ID information broadcast in SIB can be useful in some NAS procedures. We received reply from CT1 [3] with the following answer.

1. Overall Description: CT1 thanks RAN2 for their LS informing about the recent RAN2 agreement in relation to S&F Satellite Operation and asking whether the Satellite ID information broadcast in SIB can be useful in some NAS procedures. CT1 would like to inform that in the context of S&F Satellite Operation, Satellite ID information read from SIB is used by NAS as already specified in TS 24.301 V19.4.0 (2025-09). 2. Actions: To RAN2 group. ACTION: CT1 would like to ask RAN2 to take the above feedback into consideration.

After checking the spec TS 24.301 V19.4.0 [4], we found the satellite ID is used by the NAS layer only to decide whether to initiate NAS procedure when the S&F wait timer is running, which is differs from our intention for using the S&F monitoring list during the cell reselection.

a) if the S&F wait timer duration field is present in the S&F satellite operation parameters IE and its value does not indicate zero, the UE shall stop timer T3451, if running and start timer T3451 with the S&F wait timer duration value provided in the S&F satellite operation parameters IE. While timer T3451 is running, the UE shall not initiate NAS procedures over S&F satellite E-UTRA cells in the TAI list, unless the S&F satellite E-UTRA cell broadcasts the same satellite ID broadcasted by the S&F satellite E-UTRA cell from which the SERVICE ACCEPT message was received; [Unrelated part is omitted] NOTE 7: The estimated S&F uplink delivery time duration is associated only with the satellite ID of the satellite which provides the estimated S&F uplink delivery time duration.

We propose that RAN2 revisit this issue and make a final decision. This contribution presents the necessary cell reselection enhancements based on the S&F Monitoring list, including:

- Section 2 provides the technical issue analysis, which can help you review the background of this issue.

- Section 3 provides detailed solutions and corresponding TPs to justify the Spec impacts needed.

Technical Issue Analysis

As illustrated by the coversheet and the changes of the agreed CR [5], SA2 agreed that the S&F Monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling. According to the latest TS 23.401 [6] which already implemented the agreed SA2 CR in [5], although it says that how the UE behaves upon reception of this S&F Monitoring list is up to UE implementation, it was also captured that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Citation from coversheet part [5] Reason for change:; As described in the discussion paper (S2-2500780), this contribution removes this EN: Editor’s Note: How to handle the specification of the “List of satelliteID(s)” and “S&F Monitoring List” as per conclusions in TR 23.700-29 is FFS. The satellite list is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources. While how UE behaves when receiving the satellite list from MME is UE implementation. It is difficult to define the UE behaviour to only follow the received satellite list to access the network. This contribution proposes MME sends S&F Monitoring list to UE and UE behaviour is based on UE implementation and it may use it to determine which satellites to access. For example, if the UE determines a satellite is operating in S&F Mode, the UE may follow the S&F Monitoring List to ensure a successful access; if the UE determines a satellite is not operating in S&F Mode, the UE can access to this satellite even it is not in the S&F Monitoring List..

Citation from TS 23.401, 4.13.9.1 [6] […] - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with a satellite of the same PLMN that is operating in S&F Mode. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. […]

Observation 1: SA2 agreed that the S&F Monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, SA2 intends to leave some room for the UE to decide which S&F satellite to access, with the consequence also specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

However, based on the current cell (re)selection mechanism in TS 36.304 [7], the UE shall select the cell relying on frequency priority (if supported) and RSRP/RSRQ, as specified in the following excerption from TS 36.304. This means that UE has to camp on/reselect to the corresponding cells meeting the RSRP/RSRQ conditions, irrespective of whether the cells are provided by satellites included in the S&F Monitoring list. That means, current cell reselection procedure leaves no room for UE implementation to further consider the S&F Monitoring list as specified in SA2 procedure. The consequence, as what is also highlighted above in TS 23.401, is the increasing chance for UE access failure, if the current cell reselection procedure is followed without considering the S&F Monitoring list.

#### 5.2.4 Cell Reselection evaluation process ##### 5.2.4.5 E-UTRAN Inter-frequency and inter-RAT Cell Reselection criteria [...] In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in clause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows: - If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6; - If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the S&F Monitoring list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempt to a satellite not in the S&F Monitoring list).

Observation 3 above corresponds to a possible case for UE access failure due to the existing cell reselection procedure in the UE AS, and thus this is an essential technical issue that should be well settled by RAN2.

Discussion on Solutions

To align with the design of SA2 Spec, especially to avoid the potential UE access failure as highlighted by SA2, we propose to leave some room for the UE to consider the S&F Monitoring list during cell reselection procedure.

Based on contributions from companies [8-10] and companies' views expressed during online discussion in the past meetings, we propose to enhance the definition of suitable cell and related operations by taking into account the S&F Monitoring list. Particularly, the UE may simply consider a detected cell as unsuitable and does not treat it as a candidate for reselection, if the UE is configured with a S&F Monitoring list and the detected cell is not provided by the satellites included in the S&F Monitoring list. This can be simply specified in 4.3 of TS 36.304, as follows.

## 4.3 Service types in Idle Mode [...] suitable cell: A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements. - The cell is part of either: - the selected PLMN, or: - the registered PLMN, or: - a PLMN of the Equivalent PLMN list - For a CSG cell, the cell is a CSG member cell for the UE; According to the latest information provided by NAS: - The cell is not barred, see subclause 5.3.1; - The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" [4], which belongs to a PLMN that fulfils the first bullet above; - The cell selection criteria are fulfilled, see subclause 5.2.3.2; - Except for NB-IoT, if the UE supports authorization of coverage enhancements and upper layers indicated that use of coverage enhancements is not authorized for the selected PLMN: - the cell selection criterion S in normal coverage shall be fulfilled; - If the UE supports CE mode B and upper layers indicated that CE mode B is restricted: - the cell selection criterion S in normal coverage based on values Qrxlevmin and Qqualmin or in enhanced coverage based on values Qrxlevmin_CE and Qqualmin_CE shall be fulfilled. If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell. For a S&F Satellite operation capable UE, if a S&F Monitoring list is configured by upper layers, the UE may not consider a cell operating in S&F mode as suitable cell if it is not a cell provided by the satellites configured in that list. [...] #### 5.2.4.4 Cells with cell reservations, access restrictions or unsuitable for normal camping For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1. If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes. If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed. If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed. If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection. For a S&F Satellite operation capable UE, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell operating in S&F mode which is not suitable due to not belonging to a satellite in the S&F Monitoring list configured by upper layers, the UE may not consider this cell as candidate for reselection.

Based on the above analyses, we propose that RAN2 agrees to enhance the suitable cell definition and related operation during cell reselection procedure, in the case that S&F Monitoring list is provided to a S&F capable UE.

Proposal 1: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the S&F Monitoring list.

The complete TP to TS 36.304 for above Proposal 1 is summarized in the Annex A. We propose that RAN2 adopts the TP in Annex A for the TS 36.304.

Proposal 1a: RAN2 adopts the TP provided in the Annex A.

Besides the intended UE behaviours in above proposal 1, another aspect that needs to be confirmed is whether any new assistance information needs to be introduced in system information. Specifically, it can be considered whether a mapping between satellite ID and {frequency(ies), PCI(s)} can be provided by the serving cell's system information, for the UE to know the satellite actually associated with the highest ranked cell/best cell. However, one may also wonder whether it is possible to rely on the UE to read the SIB31 and acquire the satellite ID of the highest ranked cell/best cell by itself, in order to avoid introducing a mapping configuration.

To this end, we propose that RAN2 further discusses whether any assistance information needs to be introduced to support the S&F Monitoring list, pending the conclusion on Proposal 1.

Proposal 2: RAN2 discusses whether any new assistance information needs to be introduced into system information for the S&F operation (e.g. mapping between satellite ID(s) and {frequency(ies), PCI(s)}), if Proposal 1 is agreeable.

Conclusion

In this contribution, the cell reselection enhancements for S&F operation are discussed and the corresponding observations and proposals listed as follows:

Observation 1: SA2 agreed that the S&F Monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, SA2 intends to leave some room for the UE to decide which S&F satellite to access, with the consequence also specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the S&F Monitoring list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempt to a satellite not in the S&F Monitoring list).

Proposal 1: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the S&F Monitoring list.

Proposal 1a: RAN2 adopts the TP provided in the Annex A.

Proposal 2: RAN2 discusses whether any new assistance information needs to be introduced into system information for the S&F operation (e.g. mapping between satellite ID(s) and {frequency(ies), PCI(s)}), if Proposal 1 is agreeable.

Reference

- RAN2#131 ChairNotes.

- R2-2506285	LS on Satellite ID for S&F	Sateliot	LS out	Rel-19	IoT_NTN_Ph3-Core To:CT1	Cc:SA2

- C1-256622	Reply LS on S&F Operation	LS out	Rel-19	5GSAT_Ph3_ARCH To:RAN2	Cc:SA2

- 3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS)" (V19.4.0).

- S2-2502513	Support of S&F Monitoring List in S&F Mode.

- 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access" (V19.2.0).

- 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

- R2-2502053	Discussion on RAN2 impacts for the support of S&F operation	CATT	discussion	Rel-19.

- R2-2502101	On the support for the S&F operation	Google	discussion	Rel-19	IoT_NTN_Ph3-Core.

- R2-2504617	Discussion on “S&F Monitoring List” and “S&F Wait Timer” and potential RAN2 impacts	Sateliot, Thales, Novamint	discussion.

- Annex A: TP for cell reselection enhancements

## 4.3	Service types in Idle Mode

This clause defines the level of service that may be provided by the network to a UE in Idle mode.

The action of camping on a cell is necessary to get access to some services. Three levels of services are defined for UE:

-	Limited service (emergency calls, ETWS and CMAS on an acceptable cell). It is not applicable to RRC_INACTIVE state.

-	Normal service (for public use on a suitable cell)

-	Operator service (for operators only on a reserved cell)

Furthermore, the cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications), and it is not applicable to RRC_INACTIVE state. Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in a E-UTRAN network:

-	The cell is not barred, see clause 5.3.1;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements.

-	The cell is part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

-	For a CSG cell, the cell is a CSG member cell for the UE;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" TS 22.011 [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

-	Except for NB-IoT, if the UE supports authorization of coverage enhancements and upper layers indicated that use of coverage enhancements is not authorized for the selected PLMN:

-	the cell selection criterion S in normal coverage shall be fulfilled;

-	If the UE supports CE mode B and upper layers indicated that CE mode B is restricted:

-	the cell selection criterion S in normal coverage based on values Qrxlevmin and Qqualmin or in enhanced coverage based on values Qrxlevmin_CE and Qqualmin_CE shall be fulfilled.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

For a S&F Satellite operation capable UE, if a S&F Monitoring list is configured by upper layers, the UE may not consider a cell operating in S&F mode as suitable cell if it is not a cell provided by the satellites configured in that list.

barred cell:

A cell is barred if it is so indicated in the system information TS 36.331 [3].

reserved cell:

A cell is reserved if it is so indicated in system information TS 36.331 [3].

Following exceptions to these definitions are applicable for UEs:

-	camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [5], TS 24.301 [16]) is suitable but provides only limited service.

-	as an outcome of the manual CSG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and inform NAS that access is possible (for location registration procedure).

NOTE:	UE is not required to support manual search and selection of PLMN or CSGs while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

-	if a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for the duration of the emergency call.

-	if the UE in RRC_IDLE fulfils the conditions to support sidelink communication or PS related sidelink discovery in limited service state as specified in TS 23.303 [29], clause 4.5.6, the UE may perform sidelink communication or PS-related sidelink discovery.

-	if the UE in RRC_IDLE fulfils the conditions to support V2X sidelink communication or NR sidelink communication in limited service state as specified in TS23.285 [36], clause 4.4.8 and TS 23.287 [40], clause, 5.7, the UE may perform V2X sidelink communication or NR sidelink communication.

For E-UTRA the cell categorization defined above is per CN type. In this specification, when the term suitable/acceptable cell is used without specifying the CN type, it means the cell is suitable/acceptable for any of the CN type(s) supported by the UE.

NOTE:	The selected CN Type is not considered during cell selection and reselection procedure.

[...]

#### 5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

For a S&F Satellite operation capable UE, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell operating in S&F mode which is not suitable due to not belonging to a satellite in the S&F Monitoring list configured by upper layers, the UE may not consider this cell as candidate for reselection.