3GPP TSG-RAN WG2 Meeting #132   									R2-2508160

Dallas, US, Nov. 17th – Nov. 21th

Agenda Item:	8.17

Source: 	CATT

Title:  	Discussion on SPS collision issues for IoT NTN TDD

Document for:	Discussion and Decision

Introduction

In RAN2#129bis meeting [1], RAN2 discuss the issue of UL SPS overlaps with non-U NB-IoT subframe in IoT NTN TDD mode and agree that UE postpones the UL SPS resource to the next valid UL subframe.

RAN2#129bis Agreement - When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

In the last meeting, the SPS collision issue due to SPS postponement was discussed but not concluded. The issue was supposed to be further discussed in this meeting [2]:

RAN2#131bis Agreement - Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

In this contribution, we further analyse the SPS collision issue and propose potential solutions.

Discussion

From our perspective, there is ambiguity about the interpretation of “next valid UL subframe”. Below gives our understandings of the agreement and further discusses whether and how to solve the SPS collision issue due to SPS postponement.

- 4.4 Frame structure type 1 for IoT NTN TDD Frame structure type 1 is applicable to IoT NTN TDD in band 249. Each radio frame is Tf=307200Ts=10 ms long and consists of 10 subframes of length 30720Ts=1 ms, numbered from 0 to 9. Subframe i in frame nf has an absolute subframe number nsfabs=10nf+i where nf is the system frame number. The frame structure for IoT NTN TDD, at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [4] consists of D=8 consecutive downlink subframes, followed by 50 consecutive guard period subframes, followed by U=8 consecutive uplink subframes, followed by 24 consecutive guard period subframes in each 90 ms interval.

In 36.211[3], the frame structure for IoT NTN TDD includes 8 downlink subframes, 50 guard period subframes, 8 uplink subframes and 24 guard period subframes in 90ms interval. Based on the frame structure for IoT TDD, UE postpones the UL SPS resource to the next valid UL subframe, which can be understood as two options:

- Option a): the next valid UL subframe is the next UL subframe, even if it does not match the UE's semiPersistSchedIntervalUL resource. As illustrated in Figure 1, the SPS UL resources of UE1 and UE2 overlap with guard period subframes and are postponed to the next available UL subframe. SPS collision will occur if UE1 and UE2 are assigned the same frequency.

Fig. 1 The next valid UL subframe is the next UL subframe

- Option b): the next valid UL subframe is the next UL subframe that matches the UE's semiPersistSchedIntervalUL resource. As illustrated in Figure 2, the SPS UL resources of UE1 and UE2 overlap with guard period subframes and are postponed to the next available UL subframe that satisfies its respective semiPersistSchedIntervalUL. For this option, there is no SPS collision issue.

Fig. 2 The next valid UL subframe is the next UL subframe that matches semiPersistSchedIntervalUL

Observation 1: For the agreement “When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe”, there are two interpretations of “the next valid UL subframe”:

- Option a): the next UL subframe;

- Option b): the next UL subframe that matches the UE’s semiPersistSchedIntervalUL.

For option a), based on NW implementation, NW could allocate different SPS frequency resources to different UEs to avoid SPS collisions as much as possible. Additionally, SPS is only used to transmit BSR in NB-IoT and BSR can also be transmitted via dynamic scheduling resources. Therefore, the SPS collision issue does not a critical issue and the specification does not need to address it urgently if SPS collision still exists.

For option b), no SPS collision issue exits. Additionally, the UE can also transmit BSR via dynamic scheduling when it has to wait a long time for the next SPS UL resource in IoT NTN TDD.

Observation 1a: For option a), it relies on NW implementation to avoid SPS collision issue due to SPS postponement as much as possible, e.g., configure different frequency resources for potentially conflicting UEs.

Observation 1b: For option b), there is no SPS collision issue due to SPS postponement.

Proposal 1: For the agreement “When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe”, clarify that “the next valid UL subframe” means:

- Option a): the next UL subframe;

- Option b): the next UL subframe that matches the UE’s semiPersistSchedIntervalUL.

Proposal 2：If option a) in proposal 1 is acceptable, it can be left to NW implementation to schedule uplink to avoid SPS collision issue. Further enhancement is not needed.

Conclusions

In this contribution, we have discussed SPS collision issue due to SPS postponement and provide the following observations and proposals:

Observation 1: For the agreement “When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe”, there are two interpretations of “the next valid UL subframe”:

- Option a): the next UL subframe;

- Option b): the next UL subframe that matches the UE’s semiPersistSchedIntervalUL.

Observation 1a: For option a), it relies on NW implementation to avoid SPS collision issue due to SPS postponement, e.g., configure different frequency resources for potentially conflicting UEs as much as possible.

Observation 1b: For option b), there is no SPS collision issue due to SPS postponement.

Proposal 1: For the agreement “When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe”, clarify that “the next valid UL subframe” means:

- Option a): the next UL subframe;

- Option b): the next UL subframe that matches the UE’s semiPersistSchedIntervalUL.

Proposal 2：If option a) in proposal 1 is acceptable, it can be left to NW implementation to schedule uplink to avoid SPS collision issue. Further enhancement is not needed.

Reference

- RAN2#129bis ChairNotes.

- RAN2#131bis ChairNotes.

- 3GPP TS 36.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 19)".