3GPP TSG-RAN2 Meeting #132                                   R2-2508165

# Dallas, USA, Nov 17th – Nov 21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5495 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AdditionalSpectrumEmission in SL positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Additional spectrum emission requirement is to restrict UE’s transmitting behaviour according to certain regional regulations, to avoid the interference leaked outside the frequency band. All kinds of the UE’s transmission should obey the requirement on the certain frequency in the region. TS38.101 section 6.2E.3.1 has the following requirements for additional spectrum emission for SL frequency: Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [7].. It means each SL frequency band (represented by one or more SL frequencies) should be associated with an additionalSpectrumEmission. In addition, the CR R2-2504710 has been agreed to add the missing additionalSpectrumEmission to each SL frequencies for SL communication (in SIB12, in SL-ConfigDedicatedNR and in SL-PreconfigurationNR). For SL positioning, SIB23, SL-ConfigDedicatedNR (UE dedicated signaling) and SL-PreconfigurationNR (pre-configuration) provide the SL frequencies for SL positioning dedicated pool. The additionalSpectrumEmission of SL-ConfigDedicatedNR is already covered by the CR R2-2504710. However, SIB23 and SL-PreconfigurationNR fails to provide the additionalSpectrumEmission for each provided SL frequencies. |
| Summary of change: |  | - Add the additionalSpectrumEmission for each SL positioning frequencies broadcasted in SIB23; - Add the additionalSpectrumEmission for each SL positioning frequencies indicated in SL-PreconfigurationNR. Impacted functionality: SL positioning Inter-operability: If UE implements the CR and NW does not - there is no inter-operability issue but UE will never get the additionalSpectrumEmission from gNB If NW implements the CR and UE does not - there is no inter-operability issue but UE cannot read the additionalSpectrumEmission provided by the gNB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additionalSpectrumEmission of each SL positioning frequency will be missing, and UE will not know the Additional spectrum emission requirement on that frequency when performing SL positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR .. |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the revision of R2-2506970 |  |  |  |  |  |  |  |  |

START OF CHANGE

6.3	RRC information elements

<omitted>

6.3.1	System information blocks

<omitted>

–	SIB23

SIB23 contains NR sidelink Positioning configuration for dedicated SL-PRS resource pool.

SIB23 information element

-- ASN1START

-- TAG-SIB23-START

SIB23-r18 ::=                         SEQUENCE {

segmentNumber-r18                     INTEGER (0..63),

segmentType-r18                       ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r18                  OCTET STRING

}

SIB23-IEs-r18 ::=                    SEQUENCE {

sl-PosConfigCommonNR-r18              SL-PosConfigCommonNR-r18,

lateNonCriticalExtension              OCTET STRING                                                           OPTIONAL,

...

}

SL-PosConfigCommonNR-r18 ::=          SEQUENCE {

sl-PosFreqInfoList-r18                SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16      OPTIONAL,    -- Need R

sl-PosUE-SelectedConfig-r18           SL-UE-SelectedConfig-r16                                               OPTIONAL,    -- Need R

sl-PosNR-AnchorCarrierFreqList-r18    SL-NR-AnchorCarrierFreqList-r16                                        OPTIONAL,    -- Need R

sl-PosMeasConfigCommon-r18            SL-MeasConfigCommon-r16                                                OPTIONAL,    -- Need R

sl-PosOffsetDFN-r18                   INTEGER (1..1000)                                                      OPTIONAL,    -- Need R

sl-PosSSB-PriorityNR-r18              INTEGER (1..8)                                                         OPTIONAL,    -- Need R

...,

[[

sl-PosFreqInfoListExt-v18xy          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommonExt-v16k0    OPTIONAL    -- Need R

]]

}

-- TAG-SIB23-STOP

-- ASN1STOP

SIB23 field descriptions

segmentContainer This field includes a segment of the encoded SIB23-IEs. The size of the included segment in this container should be small enough that the SI message size is less than or equal to the maximum size of a NR SI, i.e. 2976 bits when SIB23 is broadcast.

segmentNumber This field identifies the sequence number of a segment of SIB23-IEs. A segment number of zero corresponds to the first segment, a segment number of one corresponds to the second segment, and so on.

segmentType This field indicates whether the included segment is the last segment or not.

sl-PosConfigCommonNR This field indicates the NR sidelink positioning configuration.

SL-PosConfigCommonNR field descriptions

sl-PosFreqInfoList, sl-PosFreqInfoListExt This field indicates the NR sidelink positioning carrier frequencies for SL-PRS transmission and reception. In this release, only one entry of SL-FreqConfigCommon is included in the list. If network includes sl-PosFreqInfoListExt, it includes the same number of entries, and listed in the same order, as in sl-PosFreqInfoList. The first entry in the sl-PosFreqInfoListExt corresponds to the AdditionalSpectrumEmission of the frequency of first entry in sl-PosFreqInfoList broadcast in SIB23.

sl-PosMeasConfigCommon This field indicates the measurement configurations (e.g. RSRP) for NR sidelink positioning.

sl-PosNR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink positioning configurations.

sl-PosOffsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

sl-PosSSB-PriorityNR This field indicates the priority of NR sidelink SSB transmission and reception.

sl-PosUE-SelectedConfig Indicates the configuration used for UE autonomous resource selection.

9.3	Sidelink pre-configured parameters

<omitted>

–	SL-PreconfigurationNR

The IE SL-PreconfigurationNR includes the sidelink pre-configured parameters used for NR sidelink communication. Need codes or conditions specified for subfields in SL-PreconfigurationNR do not apply.

SL-PreconfigurationNR information elements

-- ASN1START

-- TAG-SL-PRECONFIGURATIONNR-START

SL-PreconfigurationNR-r16 ::=             SEQUENCE {

sidelinkPreconfigNR-r16                   SidelinkPreconfigNR-r16,

...,

[[

sidelinkPreconfigNR-v16k0                   SidelinkPreconfigNR-v16k0

]]

}

SidelinkPreconfigNR-r16 ::=                 SEQUENCE {

sl-PreconfigFreqInfoList-r16                SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16     OPTIONAL,

sl-PreconfigNR-AnchorCarrierFreqList-r16    SL-NR-AnchorCarrierFreqList-r16                                       OPTIONAL,

sl-PreconfigEUTRA-AnchorCarrierFreqList-r16 SL-EUTRA-AnchorCarrierFreqList-r16                                    OPTIONAL,

sl-RadioBearerPreConfigList-r16             SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16      OPTIONAL,

sl-RLC-BearerPreConfigList-r16              SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16        OPTIONAL,

sl-MeasPreConfig-r16                        SL-MeasConfigCommon-r16                                               OPTIONAL,

sl-OffsetDFN-r16                            INTEGER (1..1000)                                                     OPTIONAL,

t400-r16                                    ENUMERATED{ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL,

sl-MaxNumConsecutiveDTX-r16                 ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32}                         OPTIONAL,

sl-SSB-PriorityNR-r16                       INTEGER (1..8)                                                        OPTIONAL,

sl-PreconfigGeneral-r16                     SL-PreconfigGeneral-r16                                               OPTIONAL,

sl-UE-SelectedPreConfig-r16                 SL-UE-SelectedConfig-r16                                              OPTIONAL,

sl-CSI-Acquisition-r16                      ENUMERATED {enabled}                                                  OPTIONAL,

sl-RoHC-Profiles-r16                        SL-RoHC-Profiles-r16                                                  OPTIONAL,

sl-MaxCID-r16                               INTEGER (1..16383)                                                    DEFAULT 15,

...,

[[

sl-DRX-PreConfigGC-BC-r17                   SL-DRX-ConfigGC-BC-r17                                                OPTIONAL,

sl-TxProfileList-r17                        SL-TxProfileList-r17                                                  OPTIONAL,

sl-PreconfigDiscConfig-r17                  SL-RemoteUE-Config-r17                                                OPTIONAL

]],

[[

sl-PreconfigFreqInfoListSizeExt-v1800       SEQUENCE (SIZE (1..maxNrofFreqSL-1-r18)) OF SL-FreqConfigCommon-r16   OPTIONAL,

sl-RLC-BearerConfigListSizeExt-v1800        SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16        OPTIONAL,

sl-SyncFreqList-r18                         SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-Freq-Id-r16              OPTIONAL,

sl-SyncTxMultiFreq-r18                      ENUMERATED {true}                                                     OPTIONAL,

sl-PreconfigDiscConfig-v1800                SL-PreconfigDiscConfig-v1800                                          OPTIONAL,

sl-PosPreconfigFreqInfoList-r18             SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16     OPTIONAL

]],

[[

t400-U2U-r18                                ENUMERATED {ms200, ms400, ms600, ms800, ms1200, ms2000, ms3000, ms4000} OPTIONAL

]],

[[

sl-PreconfigDiscConfig-v1840                SL-PreconfigDiscConfig-v1840                                          OPTIONAL

]],

[[

sl-PosPreconfigFreqInfoListExt-v18xy       SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommonExt-v16k0   OPTIONAL

]],

[[

sl-PreconfigDiscConfig-v1900                SL-PreconfigDiscConfig-v1900                                          OPTIONAL

]]

}

SidelinkPreconfigNR-v16k0 ::=           SEQUENCE {

sl-PreconfigFreqInfoListExt-v16k0       SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommonExt-v16k0    OPTIONAL

}

SL-TxProfileList-r17 ::=                    SEQUENCE (SIZE (1..256)) OF SL-TxProfile-r17

SL-TxProfile-r17 ::=                        ENUMERATED {drx-Compatible, drx-Incompatible, spare6, spare5, spare4, spare3,spare2, spare1}

SL-PreconfigGeneral-r16 ::=                 SEQUENCE {

sl-TDD-Configuration-r16                    TDD-UL-DL-ConfigCommon                                                OPTIONAL,

reservedBits-r16                            BIT STRING (SIZE (2))                                                 OPTIONAL,

...

}

SL-RoHC-Profiles-r16 ::=              SEQUENCE {

profile0x0001-r16                     BOOLEAN,

profile0x0002-r16                     BOOLEAN,

profile0x0003-r16                     BOOLEAN,

profile0x0004-r16                     BOOLEAN,

profile0x0006-r16                     BOOLEAN,

profile0x0101-r16                     BOOLEAN,

profile0x0102-r16                     BOOLEAN,

profile0x0103-r16                     BOOLEAN,

profile0x0104-r16                     BOOLEAN

}

SL-PreconfigDiscConfig-v1800 ::=      SEQUENCE {

sl-RelayUE-PreconfigU2U-r18           SL-RelayUE-ConfigU2U-r18,

sl-RemoteUE-PreconfigU2U-r18          SL-RemoteUE-ConfigU2U-r18

}

SL-PreconfigDiscConfig-v1840 ::=      SEQUENCE {

sl-RelayUE-PreconfigU2U-v1840         SL-RelayUE-ConfigU2U-v1840,

sl-RemoteUE-PreconfigU2U-v1840        SL-RemoteUE-ConfigU2U-v1830

}

SL-PreconfigDiscConfig-v1900 ::=      SEQUENCE {

sl-RelayUE-PreconfigMH-r19            SL-RelayUE-ConfigMH-r19

}

-- TAG-SL-PRECONFIGURATIONNR-STOP

-- ASN1STOP

SL-PreconfigurationNR field descriptions

sl-DRX-PreConfig-GC-BC This field indicates the sidelink DRX configuration for groupcast and broadcast communication, as specified in TS 38.321 [3].

sl-OffsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on. If the field is absent, no offset is applied.

sl-PosPreconfigFreqInfoList, sl-PosPreconfigFreqInfoListExt This field indicates the NR sidelink positioning carrier frequencies of SL-PRS dedicated resource pool for SL-PRS transmission and reception. In this release, only one entry of SL-FreqConfigCommon is included in the list. If network includes sl-PosPreconfigFreqInfoListExt, it includes the same number of entries, and listed in the same order, as in sl-PosPreconfigFreqInfoList. The first entry in sl-PosPreconfigFreqInfoListExt corresponds to the AdditionalSpectrumEmission of the frequency of first entry in sl-PosPreconfigFreqInfoList.

sl-PreconfigDiscConfig This field indicates the configuration for discovery message transmission used by NR sidelink U2N Remote UE, used by NR sidelink U2U Relay UE or used by NR sidelink U2U Remote UE.

sl-PreconfigEUTRA-AnchorCarrierFreqList This field indicates the EUTRA anchor carrier frequency list, which can provide the NR sidelink communication configuration.

sl-PreconfigFreqInfoList, sl-PreconfigFreqInfoListSizeExt, sl-PreconfigFreqInfoListExt This field indicates the NR sidelink communication and/ or NR sidelink discovery configuration some carrier frequency(ies). In this release, only one SL-FreqConfig can be configured in sl-PreconfigFreqInfoList. More entries of SL-FreqConfig can be configured in sl-PreconfigFreqInfoListSizeExt. If sl-PreconfigFreqInfoListExt is included, it contains the same number of entries, and listed in the same order, as in sl-PreconfigFreqInfoList together with sl-PreconfigFreqInfoListSizeExt. The first entry corresponds to the AdditionalSpectrumEmission of the frequency of first entry in sl-PreconfigFreqInfoList, the second entry corresponds to the AdditionalSpectrumEmission of the frequency of first entry in sl-PreconfigFreqInfoListSizeExt, the third entry corresponds to the AdditionalSpectrumEmission of the frequency of second entry in sl-PreconfigFreqInfoListSizeExt and so on.

sl-PreconfigNR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink communication configuration.

sl-RadioBearerPreConfigList This field indicates one or multiple sidelink radio bearer configurations.

sl-RLC-BearerPreConfigList, sl-RLC-BearerPreConfigListSizeExt This field indicates one or multiple sidelink RLC bearer configurations.

sl-RoHC-Profiles This field indicates the supported RoHC profiles for NR sidelink communications.

sl-SSB-PriorityNR This field indicates the priority of NR sidelink SSB transmission and reception.

sl-SyncFreqList Indicates a list of candidate carrier frequencies that can be used for the synchronisation of NR sidelink communication. For SL-Freq-Id-r16, the value 1 corresponds to the frequency of first entry in sl-PreconfigFreqInfoList, the value 2 corresponds to the frequency of first entry in sl-PreconfigFreqInfoListSizeExt, the value 3 corresponds to the frequency of second entry in sl-PreconfigFreqInfoListSizeExt and so on.

sl-SyncTxMultiFreq Indicates that the UE transmits S-SSB on multiple carrier frequencies for NR sidelink communication. If this field is absent, the UE transmits S-SSB only on the synchronisation carrier frequency.

sl-TxProfileList List of one or multiple Tx profiles, indicating the compatibility of supporting SL DRX as specified in TS 38.321 [3]. Value drx-Compatible means SL DRX is supported, and value drx-Incompatible means SL DRX is not supported. It is up to the UE implementation whether/how to apply this field.

t400 Indicates the value for timer T400 as described in clause 7.1. Value ms100 corresponds to 100 ms, value ms200 corresponds to 200 ms and so on.

t400-U2U Indicates the value for timer T400 to be applied for end-to-end PC5 connection in sidelink U2U relay operation as described in clause 7.1. Value ms200 corresponds to 200 ms, value ms400 corresponds to 400 ms and so on.

END OF CHANGE