3GPP TSG-RAN WG2 Meeting #131bis			         R2-2508179

Dallas, USA, Nov. 17th-21st

Agenda Item:	8.1.2

Source: 	LG Electronics Inc.

Title:         	Remaining issue on Functionality based LCM

Document for: 	Discussion and Decision

Introduction

In RAN2#131 meeting, the AIML for PHY WI is considered complete from RAN2 point of view as follows:

- RAN2#131 Agreements

The AI/ML for PHY WI is considered complete from RAN2 point of view

In the RAN2#131bis meeting, RAN2 has discussed the RILs specified in the RIL review and comments files for AIML PHY [1, 2], and reached agreement on some RILs after the discussion [3].

In this contribution, we will show our views on the remaining RILs and related discuss	ions that are not covered in the previous meeting.

Discussion

Periodic CSI report changing from ‘not applicable’ to ‘applicable’

According to the RAN2#131 agreement, it was agreed that the RRCReconfigurationComplete message could contain previously configured inference configurations where applicability/inapplicability has already been reported and the applicability status has changed since the last report.

RAN2#131 Agreements on applicability/inapplicability 1 RRCReconfigurationComplete shall include applicability/inapplicability status for: a) All inference configurations included in the immediately preceding RRCReconfiguration message, and b) Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report. [CB on how the UE handles previously configured periodic CSI config that becomes applicable] 2 Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

The above agreement merely indicates that the RRCReconfigurationComplete message can include updated applicability/inapplicability status for the configured inference configuration when the corresponding status has been changed. However, it remains unclear when the periodic CSI report can be activated during the transition of the inference configuration from an inapplicable state to an applicable state.

Regarding this issue, extensive discussions were held among many companies through email. As a result, it was concluded to consider the following options for the reporting applicability change of periodic CSI inference configuration from ‘inapplicable’ to ‘applicable’ in RRCReconfigurationComplete.

- Option 1: UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

- Option 2: RRCReconfigurationComplete can include the applicability status for any previously configured inference configurations, except previously configured periodic CSI config, for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

- Option 3: When a UE receives a periodic CSI configuration, and it is inapplicable, the periodic configuration is not reported to the lower layers and the UE autonomously releases it. NW is notified of release via 'inapplicable' status in applicability report.

- Option 4: RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI.

Each option has the following pros and cons:

Option 1:

- Expected Procedure:

  - During the applicability determination procedure upon receiving an RRCReconfiguration message, the UE checks whether the applicable inference configuration is:

    - A newly configured configuration (e.g., addition or modification) in the current RRCReconfiguration message (initial report), or

    - A previously received configuration that has now become applicable (update report)

  - The UE reports all applicable functionalities regardless of whether they are part of an initial or update report.

  - The UE only sends applicable functionalities for configurations newly configured in the current RRCReconfiguration message.

- Pros: Most closely aligned with the existing RAN2#131 agreement

- Cons: More complex for both UE and NW compared to Option 4.

  - UE: Needs to operate with two different sets, one reported to the NW and another reported to the lower layers

  - NW: Must detect updated applicability and perform reconfiguration (release, add, or modify) only for functionalities whose state has changed to “applicable.”

Option 2:

- Expected Procedure:

  - Same procedure as Option 1 for periodic CSI report configuration, but with an additional step for the UE to check whether the configuration is a periodic CSI report configuration.

- Pros: Aligned with the existing RAN2#131 agreement

- Cons: Slightly more complex than Option 1, as the UE and NW need to distinguish operations based on the CSI report type (periodic vs. aperiodic/semi-persistent)

Option 3:

- Expected Procedure:

  - When a periodic CSI report configuration is received and determined to be inapplicable, the UE does not report it to the lower layers and autonomously releases it.

  - NW is notified of the release via an “inapplicability” in the applicable report

- Pros: Clear and immediate release of inapplicable periodic CSI report configurations

- Cons: Since the UE does not store the periodic CSI report configuration, it cannot be used immediately if conditions later change to make it applicable. The network must resend the configuration, making reuse or quick application impossible.

Option 4:

- Expected Procedure:

  - During the applicability determination procedure upon receiving an RRCReconfiguration message, the UE checks the applicability only for newly configured configuration (e.g., addition or modification).

  - The UE reports the applicable functionalities and sends the corresponding applicable configurations accordingly.

- Pros: A simpler approach compared to Option 1, Option 2, and Option 3, reducing complexity in applicability checks.

- Cons: Need to revert previous agreements

After comparing Options 1 through Option 4, We prefer Option 4 because it reduces unnecessary complexity and ensures that applicability reporting remains consistent and focused through a single, dedicated mechanism.

Proposal 1. 	To agree Option 4: RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI.

S050

[Description]: If the UE has send a UAI and within 1 second a handover has occurred or if the UE has send a UAI and a CHO or LTM cell switch has occurred, UE is required to resend the UAI. This is to avoid the transmission of UAI from source to target cell or to prepare CHO/LTM candidate cells, which can interrupt ongoing mobility For applicability reporting, based on the RRC specification, UE will send the new applicability after handover in RRCReconfigurationComplete. Thus, there is no need to retransmit the UAI. [Proposed Change]: TP in [1] [Comments]:

According to the RAN2#131bis agreement, the RIL S050 is expected to be resolved after RAN2#131bis meeting.

RAN2#131bis agreement [S050] RIL S050 to be resolved after RAN2#131bis.

The proposed change in S050 suggests that, for applicability reporting, the UE should not retransmit the UAI after handover, since the UE is expected to send the new applicability through the RRCReconfigurationComplete message following the handover. However, we believe that restraining the retransmission of the UAI is unnecessary. After sending RRCReconfigurationComplete, the UE can safely re-evaluate its applicability, and retransmitting via UAI in such cases does not create any issues, ensuring consistency in reporting and network awareness. We can also think the other scenario. In the previous case, the UE could send the NeedForGapsInfoNR information, which includes interFreq-needForGap and intraFreq-needForGap, through both RRCReconfigurationComplete and UAI. However, even for that preference, there has been no discussion or precedent for restricting its reporting via UAI. Therefore, we don’t think any additional spec change is needed.

Proposal 2.	[S050] Reject S050, i.e., UE is not expected to retransmit UAI for applicability report after handover without any spec change.

TP based on R2-2508018

At the last RAN2#131 meeting, RAN2 has sent an LS [4] to RAN1 inquiring about what IEs are needed for CSI prediction as part of the candidate UE-side data collection configuration. In response, RAN1 provided a reply LS [5], which included the following RAN1 agreement.

RAN1 Agreement Regarding the RAN2 LS from R1-2506715: For the UE-side candidate data collection configuration for CSI prediction, include the following IEs: - CSI-ResourceConfigId For candidate UE-side data collection, following IE(s) in addition to CSI-ResourceConfigId can be provided. - CodebookConfig-r18 set to ‘typeII-Doppler-r18’

RAN2 can conclude that, for candidate data collection configuration within OtherConfig, only a specification change to add CodebookConfig-r18 within OtherConfig would be necessary.

For actual data collection configuration within CSI-ReportConfig, the existing CodebookConfig is to be used, and the configuration for UE-side data collection is explicitly specified in the RAN1 specification as shown below. Accordingly, for the actual data collection configuration, no additional changes are required.

5.2.1.4.2 Report quantity configurations A UE may be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'cri-RI-LI-PMI-CQI', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index', 'tdcp', 'cli-SRS-RSRP', 'cli-RSSI', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', 'rs-pai-r19', 'csi-pai-r19', 'none-csi-r19', 'none-bm-r19', 'cjtc-Dd', 'cjtc-F', 'cjtc-Dd-F' or 'cjtc-P'. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'none-csi-r19' or 'none-bm-r19', then the UE shall not report any quantity for the CSI-ReportConfig. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', the UE shall report a preferred precoder matrix for the entire reporting band, or a preferred precoder matrix per subband, according to Clause 5.2.2.2. If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to ‘none-CSI-r19’, - the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter CodebookConfig-r18 set to ‘typeII-Doppler-r18’. <Text Omitted>

In this regard, we propose the following TP for OtherConfig in TS 38.331 as in the Annex.

Proposal 3.	Adopt the proposed TP in Annex to reflect the IEs needed for the candidate UE-side data collection configuration for CSI prediction in the RRC spec.

Conclusion

Based on the above discussion, we made following proposals:

Proposal 1. 	RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI.

Proposal 2.	[S050] Reject S050, i.e., UE is not expected to retransmit UAI for applicability report after handover without any spec change.

Proposal 3.	Adopt the proposed TP in Annex to reflect the IEs needed for the candidate UE-side data collection configuration for CSI prediction in the RRC spec.

Annex

NEXT CHANGE

6.3	RRC information elements

<Text Omitted>

6.3.4	Other information elements

<Text Omitted>

–	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

<Text Omitted>

OtherConfig-v19xy ::=                   SEQUENCE {

applicabilityReportConfig-r19                SetupRelease {ApplicabilityReportConfig-r19}                   OPTIONAL, -- Need M

dataCollectionPreferenceConfig-r19           SetupRelease {DataCollectionPreferenceConfig-r19}              OPTIONAL, -- Need M

loggedDataCollectionAssistanceConfig-r19     SetupRelease {LoggedDataCollectionAssistanceConfig-r19}        OPTIONAL  -- Need M

}

<Text Omitted>

ApplicabilityReportConfig-r19 ::= SEQUENCE {

reportApplicabilityUAI-r19     ENUMERATED {true}                                                                    OPTIONAL, -- Need R

applicabilityConfigList-r19    SEQUENCE (SIZE (1..maxNrofServingCells)) OF ApplicabilityConfig-r19                  OPTIONAL, -- Need R

...

}

ApplicabilityConfig-r19 ::= SEQUENCE {

applicabilityConfigCellId-r19       ServCellIndex                                                                      OPTIONAL, -- Need R

applicabilitySetConfigList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilitySets-r19)) OF ApplicabilitySetConfig-r19    OPTIONAL, -- Need R

...

}

ApplicabilitySetConfig-r19 ::= SEQUENCE {

applicabilitySetConfigId-r19                ApplicabilitySetConfigId-r19                            OPTIONAL,   -- Need R

resourcesForChannelMeasurement              CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

resourceForCSIPrediction-r19 				CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

reportQuantity-r19                          ReportQuantity-r19                                      OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

}                                                                                                   OPTIONAL,   -- Need R

nrofReportedPredictedRS-r19                 ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                             OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1}          OPTIONAL,   -- Need R

...

}

DataCollectionPreferenceConfig-r19 ::= SEQUENCE {

dataCollectionCandidateConfigList-r19   SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19   OPTIONAL, -- Need R

...

}

DataCollectionCandidateConfig-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19                  ServCellIndex,

dataCollectionCandidateConfigParameterList-r19   SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19  OPTIONAL, -- Need R

...

}

DataCollectionCandidateConfigParameters-r19 ::= SEQUENCE {

dataCollectionCandidateConfigId-r19         DataCollectionCandidateConfigId-r19,

resourcesForChannelMeasurement              CSI-ResourceConfigId                                     OPTIONAL, -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                     OPTIONAL, -- Need R    associatedIdForChannelMeasurement-r19       AssociatedId-r19                                         OPTIONAL, -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                         OPTIONAL, -- Need R

codebookConfig-r18                  		CodebookConfig-r18                                       OPTIONAL, -- Need R...

}

LoggedDataCollectionAssistanceConfig-r19 ::= SEQUENCE {

loggedDataCollectionBufferThreshold-r19     ENUMERATED {kB16, kB32, kB48, spare1} OPTIONAL, -- Need R

...

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

Editor's Note: FFS if any higher values for loggedDataCollectionBufferThreshold are needed depending on UE capability discussion.

Editor's Note: FFS what to add for the candidate UE-side data collection configurations based on RAN1 input.

OtherConfig field descriptions

<Text Omitted>

applicabilityConfigCellId Indicates the serving cell that the applicabilitySetConfigList refers to.

applicabilityReportConfig Configuration for the UE to indicate the applicability of configurations subject to the applicability determination procedure.

applicabilitySetConfigId Indicates the ID of a set of prediction related parameters.

applicabilitySetConfigList Indicates for each serving cell the list of sets of prediction related parameters configured for UE applicability reporting.

<Text Omitted>

dataCollectionCandidateConfigId Indicates the ID of a candidate configuration for UE-side data collection.

dataCollectionCandidateConfigList Indicates for each serving cell the list of candidate radio resources configured for UE-side data collection. The UE is not expected to perform measurements solely based on the configurations provided by this IE.

dataCollectionPreferenceConfig Configuration for the UE to report its preference to be configured with radio resources for UE-side data collection.

dataCollectionServCellIndex Index of the serving cell that the dataCollectionCandidateConfigParameterList refers to.

<Text Omitted>

loggedDataCollectionAssistanceConfig Configuration for the UE to report assistance information related to logging of radio measurements for network-side data collection. When configured with loggedDataCollectionAssistanceConfig the UE reports availability of logged radio measurements for network-side data collection when the buffer reserved for logging of radio measurements for network-side data collection has become full and it reports when it determines that it has entered a low power state.

loggedDataCollectionBufferThreshold Buffer threshold for the UE to report availability of logged radio measurements data for network-side data collection. If the amount of data in the buffer reserved for logging of radio measurements for network-side data collection has become equal to or above the threshold configured in loggedDataCollectionBufferThreshold, the UE reports availability of logged radio measurements for network-side data collection. Value kB16 means the threshold is set to 16 kB and so on.

<Text Omitted>

reportApplicabilityUAI If present, the field indicates the UE shall report applicability in UEAssistanceInformation message.

CodebookConfig-r18 If present, the field can be set to ‘typeII-Doppler-r18’.

<Text Omitted>

<Text Omitted>

NEXT CHANGE

References

- AIML Comments file v029.docx

- AIML Review file v029.docx

- R2_131bis_ChairNotes_25-10-17_17-00_final.docx

- R2-2506470 LS to RAN1_final.docx

- R2-2508018_R1-2508140.docx