3GPP TSG-RAN WG2 Meeting #132			         R2-2508180

Dallas, USA, Nov. 17th-21st

Agenda Item:	8.1.3

Source: 	LG Electronics Inc.

Title:         	Remaining issue on NW side data collection

Document for: 	Discussion and Decision

Introduction

In RAN2#131 meeting, the AIML for PHY WI is considered complete from RAN2 point of view as follows:

- RAN2#131 Agreements

The AI/ML for PHY WI is considered complete from RAN2 point of view

In the RAN2#131bis meeting, RAN2 has discussed the RILs specified in the RIL review and comments files for AIML PHY [1, 2], and reached agreement on some RILs after the discussion [3].

In this contribution, we will show our views on the remaining RILs and related discussions that are not covered in the previous meeting.

Discussion

L002

[Description]: Looking at this, the clause “if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full” already implies that logging will resume once the buffer full condition is resolved, so the very last part (explicit resume operation) seems redundant. [Proposed Change]: TP in [1] [Comments]:

As described in [1], Looking at this, the clause “if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full” already implies that logging will resume once the buffer full condition is resolved, so the very last part (explicit resume operation) seems redundant. Therefore, we propose to remove the last paragraphs.

Proposal 1. 	[L002] Adopt proposed TP from L002 in [1], i.e., remove procedure for the logging resume, to avoid redundant procedure.

S052

[Description]: Event triggered CSI measurement logging can be performed based on L3 events. It is agreed to use CSI framework and not measurement framework and the event is captured as below in RRC CR. CSI-LoggedMeasurementEventTriggerConfig-r19 ::= SEQUENCE { threshold-r19 CHOICE { aboveThreshold-r19 MeasTriggerQuantity, belowThreshold-r19 MeasTriggerQuantity }, hysteresis Hysteresis, timeToTrigger TimeToTrigger, ... } Now it is not clear how does the UE get the quantity configuration for the L3 filtering for deriving L3 measurements for the event evaluation for logging CSI measurements. [Proposed Change]: There can be two options for the quantity configuration for deriving L3 measureemnts for event evaluation for logging CSI measurements. - Quantity configuration is provided in CSI measurement framework. - UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO. This means UE should be provided with the measurement configuration when CSI-LoggedMeasurementEventTriggerConfig is received. TP in [1].

In the RAN2#131bis meeting, RAN2 decided to introduce the reuse of A1 and A2 events into the specification as a choice structure within ‘CSI-LoggedMeasurementEventTriggerConfig-r19’.

- RAN2#131bis Agreements (RIL-N028) Simplify the procedure by adapting the changes as proposed in the Section 2.1.1.

The remaining discussion topic is how the UE obtains the quantity configuration for L3 filtering to derive L3 measurements for event evaluation for logging CSI measurements. ‘Option a’ proposes to introduce a new quantity configuration element within the CSI measurement framework by adding a dedicated field (e.g., csi-LoggedMeasurementConfig-quantityConfig) to the CSI-MeasConfig. This approach would enable the network to explicitly define L3 filtering parameters used by the UE when evaluating event-triggered CSI measurement logging. However, adopting ‘Option a’ requires substantial additions to the ASN.1 definitions and new procedural descriptions to manage the configuration, modification, and resetting of CSI-specific L3 filters. Consequently, the CSI framework would extend beyond its intended scope of CSI resource and reporting configuration into the measurement domain. This overlap increases specification complexity, introduces redundant definitions already present in the existing measurement framework, and may result in potential inconsistencies between CSI-specific and general measurement configurations.

In contrast, ‘Option b’ maintains the existing framework separation by allowing the UE to use the quantity configuration defined in the measurement configuration—such as the quantity configuration associated to measurement object through ServingCellMO—to derive L3 measurements for CSI-logging event evaluation. Under this approach, the CSI framework continues to handle CSI resources, reporting, and logging control, while the measurement framework remains responsible for defining how L3 quantities are filtered and derived. The required specification updates are limited to clarifications in TS 38.331 clauses 5.5.2.8, 5.5.3.1, and 5.5.3.2, ensuring minimal structural impact and preserving backward compatibility.

A key advantage of ‘Option b’ lies in its reuse of the established QuantityConfigNR definitions, ensuring uniform L3 filtering behaviour across both measurement reporting and CSI-logging event evaluations. By avoiding the introduction of an additional CSI-specific quantity configuration, ‘Option b’ eliminates redundancy and guarantees that all event evaluations rely on a single, consistent set of filtering parameters. This approach not only simplifies UE implementation but also prevents configuration misalignments that could otherwise occur between two separate frameworks.

From an implementation and maintenance standpoint, ‘Option b’ is significantly more efficient. The UE can reuse existing RRC data structures and filtering logic, without the need for additional parsing or synchronization mechanisms for CSI-specific filters. This reduces signalling overhead, implementation complexity, and testing burden. Moreover, since ‘Option b’ introduces no new ASN.1 definitions, any future updates to measurement filtering behaviour in TS 38.331 will automatically apply to CSI-logging evaluations, thereby simplifying long-term specification maintenance.

In summary, ‘Option a’ offers an explicit but redundant mechanism that expands the CSI framework and increases specification complexity, while ‘Option b’ provides a streamlined, consistent, and backward-compatible solution. By reusing the existing measurement configuration, ‘Option b’ achieves the same functional objective with minimal change and stronger architectural integrity. Therefore, ‘Option b’ represents the more technically sound and specification-aligned approach for configuring L3 quantity derivation in CSI-logged measurement event evaluation.

Proposal 2.	[S052] Adopt ‘Option b’ for the quantity configuration for deriving L3 measurements for event evaluation for logging CSI measurements.

- ‘Option b’: UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO. This means UE should be provided with the measurement configuration when CSI-LoggedMeasurementEventTriggerConfig is received.

S042

[Description]: According to current text, UE stores NSDC (NW-side data collection) configuration and corresponding UAI configuration in UE Inactive AS Context, before they are released. So, they would be restored during RRCResume procedure. [Proposed Change]: We suggest to follow the legacy principle. i.e., UE stores both configurations in UE Inactive AS Context, and releasese upon RRC resume initiation. TP in [1].

We agree with the intent and the proposed TP in [1]. According to the current specification, the UE stores both the NSDC (Network-side Data Collection) configuration and the UAI configuration in the UE Inactive AS Context before release, and these configurations are restored during the RRCResume procedure. However, this behaviour differs from the legacy principle and may introduce unnecessary complexity in configuration management.

Following the legacy approach, where the UE stores both configurations in the Inactive AS Context but releases them upon RRC Resume initiation, would provide clearer and more consistent behaviour. This ensures alignment with established procedures, avoids redundant retention of configuration data, and simplifies context handling on the UE side. In addition, this approach improves implementation clarity, reduces potential inconsistencies during RRC state transitions, and enhances overall system efficiency. Therefore, we support the proposed change and recommend its adoption to maintain procedural consistency and technical robustness.

Proposal 3.	[S042] Adopt proposed TP from S042 in [1], i.e., UE stores both NSDC (NW-side data collection) configuration and corresponding UAI configuration in UE Inactive AS Context, and releases upon RRC resume initiation.

Conclusion

Based on the above discussion, we made following proposals:

Proposal 1. 	[L002] Adopt proposed TP from L002 in [1], i.e., remove procedure for the logging resume, to avoid redundant procedure.

Proposal 2.	[S052] Adopt ‘Option b’ for the quantity configuration for deriving L3 measurements for event evaluation for logging CSI measurements.

- ‘Option b’: UE uses the Quantity configuration from the measurement configuration, for e.g. the quantity configuration associated to measurement object through ServingCellMO. This means UE should be provided with the measurement configuration when CSI-LoggedMeasurementEventTriggerConfig is received.

Proposal 3.	[S042] Adopt proposed TP from S042 in [1], i.e., UE stores both NSDC (NW-side data collection) configuration and corresponding UAI configuration in UE Inactive AS Context, and releases upon RRC resume initiation.

References

- AIML Comments file v029.docx

- AIML Review file v029.docx

- R2_131bis_ChairNotes_25-10-17_17-00_final.docx