3GPP TSG-RAN WG2 Meeting #132	R2-2508181

Dallas, USA,  Nov. 17th - 21st

Agenda item:	9.3.2

Source:	LG Electronics Inc.

Title:	LCM for UE-sided RRM measurement prediction

Document for:	Discussion and Decision

# 1. Introduction

During R19 AIML SI for NR mobility, there were three use cases: RRM measurement prediction, measurement event prediction and RLF prediction. The former two uses cases were evaluated through simulations.

For RRM measurement prediction, the 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurement results A total of six sub‑use cases are defined: three for cell‑level RRM measurement prediction and three for beam‑level RRM measurement prediction. Evaluation is considered in three domains: temporal, spatial, and frequency. Specifically, the temporal domain comprises two sub‑cases, case A and B according to [1].

In this contribution, we discuss the functionality-based LCM for UE-sided model focusing on the AI/ML mobility.

# 2. Discussion

General

The functionality requirements of RRM measurement prediction can first be considered based on performance, robustness and complexity. According to the study results on AI/ML based mobility [1], for measurement reduction, in FR1 intra-frequency temporal domain case B, reducing measurements (e.g., by 50%) showed no significant handover performance degradation compared to existing L3 procedures. In FR1 inter-frequency prediction, measurement reduction can also improve UE throughput if measurement gaps are avoided or relaxed. For handover performance improvement, in FR2 intra-frequency temporal domain case A, predicting measurement events in advance reduced the handover failure (HOF) rate in most cases.

The functionalities with good performance results may start from FR1 intra‑frequency temporal domain case A and B and inter‑frequency prediction with collocated cells. Additionally, we need to take complexity, robustness and availability into consideration. As for robustness, when operating AI/ML‑based mobility, the UE may be required to achieve performance that is at least equal to, or better than, that provided by non‑AI/ML‑based mobility. Availability is to ensure that the AI/ML model for RRM prediction operates correctly and remains in a usable state at the time it is needed.

Proposal 1. AI/ML-based RRM measurement prediction should be designed to deliver high performance, robustness, and availability, ensuring the model operates correctly and remains usable whenever prediction is needed.

## Definition on Functionality

Functionality can be reported during UE capability exchange procedure, and the network can query the UE regarding its functional capability. AI/ML feature requirements may be defined with different level of granularity, such as use‑case, sub‑use‑case, and domain‑type. In [1], the three use-cases were classified into RRM measurement prediction, RRM measurement event prediction, and RLF. Also, we see that the six sub‑use cases for RRM measurement according to various input and output type for measurement model were identified. Among them, three cases correspond to cell-level prediction and others beam‑level. They are combined with temporal, spatial, and frequency domain type.

In the current work item phase, the focus is on RRM measurement and event prediction using cell‑level prediction in the temporal and frequency domains for the UE‑side model. As for temporal domain, there are two types: temporal domain A and B. In contrast, for the network-side model, RRM measurement and event prediction can involve beam-level prediction or cell-level prediction in the temporal, frequency, and/or spatial domains. The functionality may be discussed as one of these combinations. Before discussing functionality granularity, we need to clarify or define those terms and discuss the functionality.

Proposal 2. RAN2 to use the following terminology, assuming the meanings below:

- Use case: RRM measurement prediction and RRM measurement event prediction

- Sub use case: the six sub‑use cases for RRM measurement according to various input and output type for measurement model were identified in [1]. Among them, three cases correspond to cell-level prediction and others beam‑level

- Prediction domain type: temporal, spatial, and frequency domain types

Proposal 3. RAN2 to assume the following definition on functionality

- Supported functionality: Functionalities that the UE can indicate via UE capability information (e.g., use case / sub use case / prediction domain type related capabilities).

- Applicable functionality: Functionalities that can be applicable by the UE (e.g., unit of measurement configuration associated with a measId, or certain configuration parameters related to use case / sub use case / prediction domain type).

- Activated functionality: Functionalities already enabled for performing inference (e.g., unit of measurement configuration associated with a measId).

## Assumption on Associated ID

In the AI/ML beam management case, for an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Regarding the associated ID defined in [3], the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell.

That is, associated ID was introduced to ensure consistency between training and inference phases. For AI/ML beam management, this ID provides information to UE on whether a same/similar gNB beam is maintained across different DL RS transmission occasions as captured below.

<TS38.214 clause 5.2.1 Channel state information framework> For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE may be configured with one or two Associated ID(s) in CSI-ReportConfig: - if only associatedIDforSetA-r19 is configured, it is associated with the resource sets of the second Resource Setting, and the UE expects that all CSI-RS resources or SS/PBCH block resources in the resource set of the first Resource Setting are among the CSI-RS resources and/or SS/PBCH block resources in the resource set of the second Resource Setting. - if associatedIDforSetA-r19 and associatedIDforSetB-r19 are configured, they are associated with the resource set of the second Resource Setting and of the first Resource Setting, respectively. For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 is set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', if the same Associated ID is configured to be associated with different resource sets, the UE may assume similar properties for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released.

In the study phase, it was agreed that Associated ID can be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations) for AI/ML mobility. As AI/ML mobility spans diverse use cases, sub-use cases, and prediction domain types, RAN2 needs to discuss the meaning of the Associated ID to enable the UE to select and apply the optimal model, maintain training-inference consistency, and avoid cross-use case confusion.

In this regard, the UE could assume the following possible meanings:

- Spatial property: Associated ID indicates same/similar beam or cell configuration for RSs

- Interference metrics: Associated ID indicates same/similar interference environment (e.g., Enables reuse of models across similar interference conditions)

- Time-domain variation: Associated ID indicates same/similar channel/interference time-variation characteristics between measurement and prediction windows (e.g., Supports model selection for static vs. dynamic channel)

- Frequency-domain variation: Associated ID indicates same/similar channel/interference frequency-variation characteristics for given bands/CCs/BWPs (e.g., Support model selection for different FD variation scenario)

Proposal 4. RAN2 to discuss the meaning of Associated ID for AI/ML mobility for considered sub use cases and prediction domain types

## Applicability related reporting

Currently, the capabilities that can be included in UE capability information are static information. Likewise, RAN2 can assume that the UE informs of the static functionality capability, i.e., all supported functionalities. Based on this information, the network can set configuration for the specific functionality.

Based on the functionality configuration, the UE may provide applicable functionality(ies), referred to in RAN2 as an applicability-related report. In [1], the UE can be set up with either a full inference configuration or a partial one, using the inference parameters, which shall be defined further, and delivered through an RRCReconfiguration message. If the UE moves to RRC_IDLE or RRC_INACTIVE state, or experiences RLF, it follows the existing RRC behaviour for deciding whether to keep or release the inference configuration.

When the UE receives an inference configuration, it reports whether it’s applicable in the initial applicability report via RRCReconfigurationComplete. If it’s not applicable, the UE may add a “release configuration” flag to show its preference for release — but the UE itself doesn’t autonomously release it.

Applicability can also be updated via UAI, with a flag in OtherConfig indicating if applicability reporting through UAI is enabled. In order to align signalling framework for different AI use cases, the Rel-19 applicability report framework can be reused for AI‑based mobility as much as possible.

First of all, a decision may be needed whether to support Option A and/or Option B for AI mobility. In our view, Option A has benefit on simplifying operations and Option B has benefit on reduced signalling overhead. Thus, we suggest considering both options for AI mobility. For AI/ML mobility, it is necessary to discuss which configurations could be considered Option A, and which parameters could be considered Option B. For example, in Option A, the measurement configuration associated with measId could be considered, while in Option B, parameters such as associated ID, frequency/cell/beam related information, and/or prediction time related information could be considered. Especially, as FD prediction for AI/ML BM use case has not been considered for Rel-19 AI use cases, FD prediction related configuration and reporting parameters need to be discussed. In the case of RRM measurement prediction in the frequency domain, a pair-wise configuration seems more suitable where a pair refers to the combination of the measurement frequency and the predicted frequency. It is because UE-side capability and applicability on FD prediction can be different for each frequency pair. For Option A, multiple frequency pairs can be configured to UE to determine its applicability. For Option B, UE may report applicable frequency pairs to NW.

Proposal 5. RAN2 to take the Rel-19 applicability report as a baseline, and to discuss which configuration/parameters can be exchanged for option A and option B

For AI mobility, AI model applicability would be dependent on UE mobility (e.g. speed). For example, it was observed that the prediction performance is degraded for high mobility UE

In the study item, the following general trends were observed:

For both FR2 intra-frequency temporal domain case A and FR1 intra-frequency temporal domain case B predictions, the following observations are made: - Higher UE speed correlates with decreased prediction accuracy; - Longer PW length correlates with decreased prediction accuracy; - The gain of cluster approach against single cell approach is not clear. For FR2 intra-frequency temporal domain case A the following observations are made: - Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit; - A majority of the companies observe that RRM sub-use case 2 demonstrates higher prediction accuracy than RRM sub-use case 1 and RRM sub-use case 3 at least with short PW length; - AI algorithm can outperform sample and hold in terms of prediction accuracy. The gain improves with increase of UE speed and PW length within a certain window length;

This means that a model may be applicable under certain UE-side condition only. To address this, two approaches can be considered.

The first approach is to enhance the applicability report framework to accommodate UE-side conditions. More specifically, UE may report applicability according to UE mobility condition so that NW may adopt activated functionality according to UE mobility condition, e.g. reducing measurement/prediction window length and/or the time gap between the two windows. This approach may require NW to track UE mobility condition.

The second approach is to add cause information, e.g. high mobility, when the activated functionality becomes inapplicable. This additional information gives NW a hint how to make the AI prediction become applicable later, e.g. via adjusting window side and gap.

Proposal 5. RAN2 to discuss the impact of UE mobility on the model applicability.

## Inference configuration / reporting

The existing RRM measurement configuration and reporting framework in the RRC layer was considered as the baseline for AI‑based mobility inference configuration and reporting in the study item phase [2]. In the study, the network may configure the UE with the following inference parameters to enable inference and applicability assessment: measured frequency carrier and predicted frequency carrier information for inter‑frequency prediction. parameters for intra‑frequency temporal domain indicating the timing of the network’s SSB configuration, cell information, for which the network expects inference reports, optional associated ID. The report on RRM measurement prediction is intended for measurement reduction, and therefore the discussion can start based on the existing 5G NR periodic report. The configuration and report parameters affecting the measurement reduction gain, UE complexity and trade-off can be discussed. Through configuration parameters that include reporting settings designed to fully leverage the measurement reduction function, the network can optimize mobility in 5G NR.

Proposal 6. RAN2 to discuss inference configuration/reporting procedure based on the periodic measurement report (MR) procedure framework. FFS on detailed prediction related configuration

## AI/ML processing unit (APU)

In RAN1, APU was introduced in Rel-19 in order to address AI/ML processing capability at UE side. This feature was designed based on CPU (CSI processing unit) framework as considered use cases were related to CSI measurement/reporting. Introduction of the APU was motivated by the fact that a dedicated hardware (e.g. GPU, AI engine) would be used for AI/ML processing, and the principle of APU/CPU management is that a UE cannot process multiple tasks simultaneously beyond its capability. The APU capacity defines the maximum number of AI/ML processing tasks that a device can execute concurrently. Depending on the computational requirements of a given task, the processing workload may utilize L APUs for that single task. This parameter is critical for evaluating the device’s ability to support parallel AI/ML operations and for optimizing resource allocation in multi‑tasking scenarios

It should be noted that a common APU pool can be shared for multiple beam/CSI measurement/report tasks due to the fact that the processing hardware can be a common one for these tasks. For AI mobility, therefore, it is a reasonable assumption that AIML processor for mobility use case can be the same one used for BM/CSI use cases, meaning that this use case also shares the APU pool defined in Rel-19. Depending on the internal architecture of the UE, the capability of the AI processing unit should be considered to effectively allocate UE AI/ML resources for supporting multiple use cases. AI/ML processing in the UE is constrained by the capacity of dedicated hardware (e.g., GPU, AI engine), limiting the number of simultaneous use cases. A coordination mechanism between the network and UE should ensure alignment on the number and type of AI/ML instances for beam management, CSI and mobility, thereby maintaining system reliability, avoiding unnecessary resource usage, and ensuring AI/ML availability. To address this, the existing APU framework can be a starting point.

As this may impact on RAN1 specification, close work coordinate with RAN1 is required.

Observation 1. The coordination between the network and UE can be needed to manage AI/ML mobility within the UE’s hardware capacity (e.g., GPU, AI engine) by effectively allocating AI/ML resources, thereby ensuring system reliability, preventing unnecessary resource usage, and achieve AI/ML functional availability.

Proposal 7. RAN2 to discuss AI Processing Unit (APU) determining the computational requirements of a given RRM measurement prediction task. Consider existing APU framework as a starting point.

# 3. Conclusion

Proposal 1. AI/ML-based RRM measurement prediction should be designed to deliver high performance, robustness, and availability, ensuring the model operates correctly and remains usable whenever prediction is needed.

Proposal 2. RAN2 to use the following terminology, assuming the meanings below:

- Use case: RRM measurement prediction and RRM measurement event prediction

- Sub use case: the six sub‑use cases for RRM measurement according to various input and output type for measurement model were identified in [1]. Among them, three cases correspond to cell-level prediction and others beam‑level

- Prediction domain type: temporal, spatial, and frequency domain types

Proposal 3. RAN2 to assume the following definition on functionality

- Supported functionality: Functionalities that the UE can indicate via UE capability information (e.g., use case / sub use case / prediction domain type related capabilities).

- Applicable functionality: Functionalities that can be applicable by the UE (e.g., unit of measurement configuration associated with a measId, or certain configuration parameters related to use case / sub use case / prediction domain type).

- Activated functionality: Functionalities already enabled for performing inference (e.g., unit of measurement configuration associated with a measId).

Proposal 4. RAN2 to discuss the meaning of Associated ID for AI/ML mobility for considered sub use cases and prediction domain types

Proposal 5. RAN2 to take the Rel-19 applicability report as a baseline, and to discuss which configuration/parameters can be exchanged for option A and option B

Proposal 6. RAN2 to discuss inference configuration/reporting procedure based on the periodic measurement report (MR) procedure framework. FFS on detailed prediction related configuration

Proposal 7. RAN2 to discuss AI Processing Unit (APU) determining the computational requirements of a given RRM measurement prediction task. Consider existing APU framework as a starting point.

# 4. References

[1] 3GPP TR 38.744 V19.0.0 (2025-09) Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR;

[2] R2-2506702, Report of 3GPP TSG RAN WG2 meeting #131 Bengaluru, India

[3] 3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface V19.0.0