3GPP TSG RAN WG2 #132	R2-2508194

Dallas, Texas, USA, November 17th - 21st, 2025

Source:	Panasonic

Title:	RRC and Other corrections - Store & Forward

Agenda Item:	8.9.3  Other corrections (corrections to TS 36.300, TS 36.304 and TS 36.306)

Document for:	Discussion, decision

Introduction

In conjunction with the Store & Forward operation of satellites there were a few issues left unresolved during the Prague meeting #131bis. This paper reflects Panasonic’s position regarding one particular issue.

The open issues at the end of the last meeting consisted of – beside others – an indication for the missing need for paging (likely in SIB31 [1]) on the one hand side and the meaning of sf-OperationModeNeigh in SIB33 [1] on the other hand [2]:

- Come back in the next meeting to check whether we can have an indication (likely in SIB31) without mandating any UE behaviour.

- Send an LS to SA2 (cc: RAN3) saying that RAN2 is discussing to have an indication in SIB to inform UEs than no paging is expected (so that the UE may skip paging) and asking them if any paging is expected from MME to RAN when the feeder link is not available (can further work on the LS text in the post-meeting e-mail discussion).

The aim of this document is to propose a solution for the aforementioned open S&F issue.

Discussion on open Store & Forward issue

# 2.1 Is an indication needed in SIB to inform UEs than no paging is expected?

Store and Forward operation is based on regenerative payload on board the satellite, i.e. a full or partial gNB resides on the satellite. This allows local RAN functions, including paging, to be executed onboard. In this case, paging could still occur – even if the feeder link is temporarily unavailable, provided the satellite has cached UE context and can operate semi-autonomously. Only the absence of a service link would disable paging feasibility, which is an obvious case that is not part of the question asked.

Observation 1:	In the presence of regenerative payload on the satellite, paging is principally 
feasible in both modes of operation – normal mode and S&F mode.

Proposal 1:	In conjunction with Store and Forward operation, there is no need to indicate to 
UEs that no paging is expected.

Proposal 2:	RAN2 to neither send any related LS to SA2 nor cc RAN3.

Conclusions

In this document, we discussed an open issue in conjunction with Store and Forward satellite operation mode stemming from the recent Prague meeting #131bis.

The following observations and proposals were made:

Observation 1:	In the presence of regenerative payload on the satellite, paging is principally 
feasible in both modes of operation – normal mode and S&F mode.

Proposal 1:	In conjunction with Store and Forward operation, there is no need to indicate to 
UEs that no paging is expected.

Proposal 2:	RAN2 to neither send any related LS to SA2 nor cc RAN3.

References

[1]	3GPP TS 36.331 V19.0.0 (2025-09)				3rd Generation Partnership Project; Technical
                    	Specification Group Radio Access Network;
                    	Evolved Universal Terrestrial Radio Access (E-
                    	UTRA); Radio Resource Control (RRC);
                    	Protocol specification (Release 19)

[2]	R2-131bis_NES_NR-NTN_IoT-NTN_(Sergio)_EOM