3GPP TSG-RAN WG2 Meeting #132	R2-2508203

# Dallas, US, 17 – 21 Nov. 2025

Agenda item:	10.2.2

Source:	AST SpaceMobile

Title:	Discussion on NTN Requirements for 6GR

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

# Introduction

Per the 6GR Study Item direction, RAN2 seeks a harmonized TN/NTN design: start from the TN baseline and integrate NTN support from Day-1; only where the physical characteristics of satellites require it should NTN-specific behaviors be added.

According to the objectives in 6GR SI [1], in 6GR, we aim at a harmonized 6G radio design for TN and NTN.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

To achieve such a harmonized design for TN and NTN in 6GR NTN study, we may respect the following two rules of thumb: (1) adopt common designs as much as possible, and (2) study functional blocks requiring NTN-specific designs, due to e.g., physical characteristics of NTN and TN-NTN mobility.

This contribution therefore provides AST SpaceMobile’s recommendations on the technology study direction to fulfil the objectives of 6GR, especially for the NTN-specific features. We start by reviewing NTN-specific features introduced in 5G-R, and then discuss our views on NTN essentials for 6G-R.

# Discussion

## NTN Low Elevation Operation

Commercially, the earliest service hours for sparse LEO constellations and a substantial fraction of urban user sessions occur at shallow look angles, where blockage risk and short dwell dominate the user experience. Engineering for best-case (near-zenith) conditions is insufficient to describe real-world performance, because attach reliability, paging reachability, RA efficiency, and mobility continuity are all governed by the worst-case geometry seen most often in the field. Thus, designing RAN2 behavior to be TN/NTN-agnostic (as possible) but explicitly parameterized for long/variable RTT, windowed presence, and confidence in timing/frequency ensures that the same state machines serve both terrestrial cells and low-elevation satellite beams without creating an NTN-only fork.

Observation 1: Low-elevation operation drives NTN economics: Most real usage occurs in short, shallow passes; performance here sets capacity, ARPU, and constellation size.

Observation 2: Realistic day-1 NTN performance is dictated by the low elevation region. In other words, the combination of short dwell, higher path loss, very rapidly varying Doppler, and intermittent visibility at shallow look angles sets the practical floor for attach, paging, access, mobility, and uplink reliability.

In satellite cellular (NTN), the elevation angle is the angle between the local horizon at the user equipment (UE) and the line of sight to the serving satellite. Low-elevation operation refers to service when that angle is shallow, that is typically just a few to a few-tens of degrees above the horizon. At these angles, the slant path from UE to satellite is long, the footprint of a downlink beam is elongated, and the relative motion between satellite and UE projects a large radial velocity component along the line of sight. The radio channel seen by the UE is therefore characterized by higher path loss than at zenith, much faster time variation of frequency error (i.e., Doppler rate), and a greater sensitivity to local obstructions (terrain, buildings, foliage) that lie along the near-horizontal look direction.

At the same time, at the low elevation region, the satellite’s apparent radial velocity is high and changing rapidly, which produces both a large Doppler shift and a high Doppler rate. Even when a nominal frequency pre-compensation is applied, the residual frequency error the UE must track varies more quickly, tightening tolerances on synchronization, demodulation, and uplink transmit accuracy. The one-way propagation delay is also larger and varies during a pass, stressing any procedure that relies on fixed timing windows (e.g., random access response, HARQ timing assumptions, paging occasion alignment), because timer settings that are benign in terrestrial conditions can become either too short (causing timeouts) or wastefully long (consuming scarce dwell time).

Furthermore, at low elevation, the satellite’s usable dwell over a UE can be very short and fragmented by blockage. Handover and “satellite switch with re-sync” decisions must therefore be timely and preferably predictive, informed by when the serving beam will cease to provide service. Purely RSRP-driven triggers react late in this regime: signal strength can degrade slowly while time-to-lose collapses quickly as the beam skirts the horizon. Effective low-elevation mobility therefore benefits from composite triggers that blend time-to-dwell-end, coarse location, and Doppler-rate in addition to classic event-based thresholds, so that execution happens while a viable target is still present.

Also, because timing uncertainty and RTT are way larger and more variable at low elevation, the random-access (RA) handshake is easier to miss if response windows are fix-centric (determined by hard preset windows). Similarly, paging/DRX that is not explicitly tied to predicted service windows forces UEs to potentially wake outside usable intervals (wasting energy) or to miss the few occasions when the satellite can actually reach them. Finally, neighbor measurement behavior that indiscriminately scans many beams consumes battery and often returns stale results in a fast-changing geometry.

Specifically, the uplink is the main limiting leg at the low elevation region. Higher slant loss, device power caps, and frequency-error tolerances are aggregated to reduce PRACH success probability and PUSCH robustness unless access timing and DMRS assumptions are adapted. Where spectrum is shared with terrestrial networks, aggregate terrestrial uplink leakage and inter-beam coupling can further degrade the satellite’s noise floor when the satellite looks tangentially over the horizon. Practical operation in these intervals benefits from uplink gating, well-defined rules for suppressing low-value UL control/data during predicted no-service windows, together with HARQ-off operating rules (DRX alignment to blind re-Tx arrival and RLC/PDCP pacing) that keep throughput predictable despite long RTT.

Thus, in the low elevation region, the following observations are made:

Observation 3: Short, intermittent service windows at low elevation magnify the cost of missed paging occasions, mistimed RA, and late HOs. Without native service-window-aligned behaviors, UEs waste energy waking outside fast service intervals and networks lose scarce “minutes in view”

Observation 4: To preserve device efficiency without hardware changes, NTN procedures should help UEs remain idle or buffered during low-elevation non-serviceable periods instead of performing futile uplink attempts.

Observation 5: Uplink is the key limiting leg. Higher slant loss, tighter frequency-error tolerances, and rapid Doppler rate first erode uplink PRACH success and PUSCH reliability regardless of how misleading ‘good’ the downlink appears.

Observation 6: TN-centric (but NTN-expanded) timers and windows still underperform. RA, DRX/paging, HARQ and related windows tuned for short (or expanded longer), but stable RTTs misfire under long/variable NTN delays. At low elevation, non-delay-aware settings either time out prematurely or squander the limited dwell, directly reducing attach and throughput probability.

Observation 7: Stale measurements degrade mobility. Fast geometry at low elevation shortens the validity of neighbor measurements; RSRP-only triggers respond late as beam time-to-lose collapses. Without freshness limits and composite triggers, UEs execute HOs too late or to the wrong target, increasing failures and ping-pong.

Proposal 1: RAN2 is encouraged to adopt a low-elevation-first design mindset when developing both common TN/NTN features and NTN-specific functions, ensuring that resulting mechanisms work efficiently for all geometries while minimizing added device complexity and energy consumption.

## 6G NTN study

In the following subsection, we set out the key performance constraints of low-elevation NTN operation and outline potential solution directions for the 6GR study.

Shallow angle passes

Shallow-angle passes create short, intermittent service windows. If DRX on-duration and paging occasions are not explicitly aligned to those windows, UEs either wake when the satellite cannot serve them (energy waste) or miss the brief viable intervals (i.e., reachability loss). In this regime, the cost of a single mistimed wake-up or page can be a lost full pass.

The specifications there allow extended DRX cycles, paging time windows, and scheduling flexibility on the network side, and further introduce NTN assistance to inform devices about satellite presence and timing. These tools improve general efficiency and provide the foundation for NTN-aware operation. The current mechanisms are not however normatively tied to predicted time-of-service as experienced by the UE in a specific pass. Without an explicit behavioral link between assistance information and DRX/paging timing, implementations may remain mostly TN-centric and miss low-elevation windows where timing precision is decisive.

Proposal 2: RAN2 studies a service-window-aligned behavioral principle. When assistance indicates a forthcoming service window, DRX on-duration and paging monitoring should preferentially occur inside that window with reasonable guard margins for delay uncertainty.

Uplink-Limited Selection and PRACH Success Biasing

Downlink may appear adequate while uplink is power-limited and interference-sensitive. Camp, reselection, or handover decisions that consider only downlink quality can lead UEs into cells or beams where random access is unlikely to succeed, creating access churn, wasted battery, and degraded user experience. The specifications retain TN-derived selection and reselection logic while introducing NTN-specific assistance and mobility tools. These enable more informed decisions but remain primarily downlink-quality focused in their default form. That is, there is no explicit, standardized uplink-feasibility bias in selection and reselection decisions. In very shallow-angle operation, a downlink-centric view can systematically over-select beams that are uplink-infeasible, increasing failed RA attempts and mobility instability. Thus,

Today’s NR logic = mostly DL-centric.

Low-elevation NTN reality = UL-limited first.

Proposal 3: RAN2 to study a UL-aware selection principle, where available information indicates low probability of uplink success, devices should elevate entry thresholds and seek corroborating evidence before camping or executing handover. The study should focus on general rules and conditions that moderate downlink-only decisions under uplink constraints.

Measurement Budgeting, SMTC Gating, and Freshness

Short dwell and rapid geometry change reduce the useful lifetime of neighbor measurements. Indiscriminate scanning across many beams and carriers drains energy and often produces data that is stale by the time a decision is needed, especially at beam edges. The specifications allow multiple measurement timing configurations, measurement gaps, and use of assistance to better schedule measurement activity. These capabilities improve flexibility in when and where devices measure. Though, there is no supported budgeting doctrine to limit concurrent scan activity under short dwell, nor a common notion of “measurement freshness” that guides when results should be discounted or discarded. These risk fragmented behavior and unnecessary energy consumption.

That is, in terrestrial networks, the radio environment is largely static where cells are fixed, coverage evolves slowly, and measurement results remain valid for seconds or even minutes sometimes. Low-elevation non-terrestrial networks (NTN) completely break that assumption: the serving geometry changes rapidly, beam directions drift, and delay/Doppler evolve continuously. Without clear budgeting and freshness rules, UEs either over-measure (wasting power and signaling capacity) or rely on outdated results, leading to poor handovers or connection drops. For instance, the network, in one option, may mark how long a measurement remains valid, e.g., after 5–10 seconds in low elevation, the link geometry has changed enough that the old results should be discarded (“aging out” stale data). The satellite can broadcast these timing and location hints in a lightweight form (like a SIB or configuration table) so every UE behaves the same way. This avoids a situation where one vendor’s phone burns power constantly scanning, while another misses the next beam entirely.

Proposal 4: RAN2 to study budgeting and freshness principles during low elevation periods, by caping the number of concurrent measurement windows under short dwell conditions, gate measurement activity to periods with a high likelihood of target presence and define a qualitative freshness horizon beyond which measurements should be down-weighted.

Re-evaluating 5G-NTN features for reuse in 6G with emphasis on low-elevation operation

HARQ feedback disabling for DL/UL:

Disabling HARQ feedback (or operating in a HARQ-light regime) remains effective under long/variable RTT, particularly when dwell is short and retransmission cycles risk colliding with the end of service windows. In these intervals, classical HARQ can stall throughput or waste scarce windows waiting for ACK/NACK that arrives too late to be useful. The 5G NR solution is reusable in 6GR, but low-elevation performance benefits from a behavioral playbook that clarifies how devices and networks pace grants, retransmissions, and connected-DRX active time when HARQ is minimized or disabled, so that blind re-transmissions and buffering do not head-of-line block the flow.

Observation 8: HARQ feedback disabling addresses long-RTT stalls and is reusable in 6GR.

RACH-less HO/CHO (low-overhead access):

Frequent transitions and short service windows make low-overhead mobility attractive. RACH-less approaches can avoid the delay of a fresh access attempt, provided residual timing/frequency error remains within safe tolerances.  The 5G concept is reusable, but low-elevation operation may need additional acceptance conditions tied to synchronization confidence (e.g., validity of common/differential TA, frequency drift bounds) and fallback behavior when confidence decays (e.g., return to RA). This is especially relevant if GNSS assistance is intermittent or intentionally avoided (which is under current study).

Observation 9: RACH-less mobility is beneficial and is reusable in 6GR.

Satellite switch with re-synchronization (transparent, quasi-Earth-fixed)

Where the radio “cell” is Earth-fixed but served by successive satellites, satellite switch with re-sync avoids full handover overhead and better preserves scarce dwell. This is especially helpful in short windows, provided residual error bounds are respected. The 5G mechanism is reusable; however, a generic acceptance test (residual TA/frequency error, confidence validity) and a fallback path (revert to RA when out of bounds) would improve inter-vendor predictability in shallow-angle use.

Observation 10: Re-sync-based switching is effective and is reusable in 6GR.

Intermittent coverage and Store-and-Forward

Early constellations and certain service profiles produce intermittent presence; store-and-forward modes further decouple when the UE transmits from when delivery occurs. At low elevation, this reality is common and must be reflected in paging reachability expectations, buffer lifetimes, and UE energy posture. Both concepts are reusable in 6G. What is under-specified today from a RAN2 behavior standpoint are the UE semantics in intermittent regimes: how long to buffer, how to surface “deferred delivery” to upper layers, and when to back-off to conserve energy if windows are repeatedly missed.

Observation 11: Intermittent presence is expected; store-and-forward modes are reusable in 6GR.

Proposal 5: RAN2 to reuse 5GR HARQ-less mode, RACH-less HO, and Store and forward for 6GR, with studying fallback behaviors, device consistency and performance limitations (with GNSS-less operation)

# Conclusion

In this contribution, we have made the following observations and proposals as follows.

Observations:

Observation 1: Low-elevation operation drives NTN economics: Most real usage occurs in short, shallow passes; performance here sets capacity, ARPU, and constellation size.

Observation 2: Realistic day-1 NTN performance is dictated by the low elevation region. In other words, the combination of short dwell, higher path loss, very rapidly varying Doppler, and intermittent visibility at shallow look angles sets the practical floor for attach, paging, access, mobility, and uplink reliability.

Observation 3: Short, intermittent service windows at low elevation magnify the cost of missed paging occasions, mistimed RA, and late HOs. Without native service-window-aligned behaviors, UEs waste energy waking outside fast service intervals and networks lose scarce “minutes in view”

Observation 4: To preserve device efficiency without hardware changes, NTN procedures should help UEs remain idle or buffered during low-elevation non-serviceable periods instead of performing futile uplink attempts.

Observation 5: Uplink is the key limiting leg. Higher slant loss, tighter frequency-error tolerances, and rapid Doppler rate first erode uplink PRACH success and PUSCH reliability regardless of how misleading ‘good’ the downlink appears.

Observation 6: TN-centric (but NTN-expanded) timers and windows still underperform. RA, DRX/paging, HARQ and related windows tuned for short (or expanded longer), but stable RTTs misfire under long/variable NTN delays. At low elevation, non-delay-aware settings either time out prematurely or squander the limited dwell, directly reducing attach and throughput probability.

Observation 7: Stale measurements degrade mobility. Fast geometry at low elevation shortens the validity of neighbor measurements; RSRP-only triggers respond late as beam time-to-lose collapses. Without freshness limits and composite triggers, UEs execute HOs too late or to the wrong target, increasing failures and ping-pong.

Observation 8: HARQ feedback disabling addresses long-RTT stalls and is reusable in 6GR.

Observation 9: RACH-less mobility is beneficial and is reusable in 6GR.

Observation 10: Re-sync-based switching is effective and is reusable in 6GR.

Observation 11: Intermittent presence is expected; store-and-forward modes are reusable in 6GR.

Proposals:

Proposal 1: RAN2 is encouraged to adopt a low-elevation-first design mindset when developing both common TN/NTN features and NTN-specific functions, ensuring that resulting mechanisms work efficiently for all geometries while minimizing added device complexity and energy consumption.

Proposal 2: RAN2 studies a service-window-aligned behavioral principle. When assistance indicates a forthcoming service window, DRX on-duration and paging monitoring should preferentially occur inside that window with reasonable guard margins for delay uncertainty.

Proposal 3: RAN2 to study a UL-aware selection principle, where available information indicates low probability of uplink success, devices should elevate entry thresholds and seek corroborating evidence before camping or executing handover. The study should focus on general rules and conditions that moderate downlink-only decisions under uplink constraints.

Proposal 4: RAN2 to study budgeting and freshness principles during low elevation periods, by caping the number of concurrent measurement windows under short dwell conditions, gate measurement activity to periods with a high likelihood of target presence and define a qualitative freshness horizon beyond which measurements should be down-weighted.

Proposal 5: RAN2 to reuse 5GR HARQ-less mode, RACH-less HO, and Store and forward for 6GR, with studying fallback behaviors, device consistency and performance limitations (with GNSS-less operation)

# References

- RP-252912	New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN#109, Sep. 2025.