3GPP TSG-RAN WG2 Meeting #132	R2-2508204

Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	8.12.2

Source:	Samsung

Title:	Discussion on remaining MAC open issues

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss MAC open issues for maintenance.

# Discussion

## Issue-1

Issue-1: UE behavior for the case: the TAT for mode-B PUSCH is expired while the TAT for PUCCH is running Rapporteur Proposal 1: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, Option 2: UE does not release the PUCCH, UE does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

As shared in several rounds of previous discussions, we disagree to release PUCCH resource due to the following issues of this optimization.

- UE autonomously releasing PUCCH resource may cause impact on other UCI (e.g., HARQ feedback). Although some insist such impact can be avoided by NW, considering the very complicated RAN1 design of HARQ feedback and UCI multiplexing, any potential impacts to PHY UCI should be avoided.

- It has been discussed that potential impacts on other UCI can be avoided by adding a condition that “if this PUCCH is not associated with any other UCI”. However, from UE perspective, how to check “this PUCCH is not associated with any other UCI” is not clear. HARQ-ACK/NACK for dynamic DL scheduling is scheduled by DCI and the assigned the PUCCH resource is indicated in DCI which is invisible to MAC since there is no indication between MAC and PHY regarding the DCI scheduling of HARQ-ACK/NACK.

- If the PUCCH resource is released, from NW perspective, in order to check whether a HARQ feedback can be scheduled on a PUCCH resource on a serving cell with TAT still running, NW has to check the TAT of another TAG for a different cell on which the type-1 CG is configured. This is new complexity compared to legacy.

The other options are either UE transmits UEIRI on PUCCH regardless, or UE does not transmit UEIRI on PUCCH.

We see the merit that by transmitting the Reporting Indication on PUCCH, the NW may know the configured event is met even without the report on PUSCH. And this has no MAC specification impact in our view.

Some raise the concern for this option that as the current TS 38.214 specifies that UE transmits mode-B UEI-CSI report in the first available PUSCH occasion after transmitting the UEI Report Indication on PUCCH, This option makes this UE behavior not clear since UE cannot transmit the report on PUSCH due to the expired TAT. We think it is clear that the legacy behavior of no UL transmission when TAT expired is always applied, and the procedure of any new functionality is performed complying this legacy behavior, meaning that the second step of UEI CSI report on PUSCH will not be transmitted in this case.

For the other option as in the Rapporteur’s proposal, it can be acceptable as a compromise. For this, we need to specify UE behavior clearly in MAC. For example, in clause 5.2 (Maintenance of Uplink Time Alignment): For mode-B UE-initiated CSI reporting, if either the PUCCH or the associated PUSCH is associated with a TAG for which the timeAlignmentTimer is not running, UE does not transmit the UEIRI on the PUCCH and the UEI-CSI report on the associated PUSCH.

Proposal 1: For mode-B UEI reporting, when the TAT for PUSCH is expired while the TAT for PUCCH is running, not release PUCCH. Support either

- (preferred) transmit UEIRI on PUCCH, no MAC impact, or

- (acceptable) not transmit UEIRI on PUCCH, and specify UE behaviour in MAC.

## Issue-2

Issue-2: DRX active time for mode-A UEI reporting Rapporteur Proposal 2: For DRX, after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling the mode-A UEI CSI report on PUSCH or until the next PUCCH occasion for UEIRI, whichever comes the first. TP baseline: When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]), a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received and the next PUCCH occasion to transmit UE Initiated Report Indication has not come yet.

As discussed in the offline, the point to be clarified for this issue is how UE transmits UEIRI for a triggered report, i.e., whether UE continuously transmit UEIRI on each PUCCH occasions for a triggered report or not. However, as checked with RAN1, it seems there is no common understanding from specification point of view, which leaves UE behaviour up to implementation.

If UE continuously transmits UEIRI on each PUCCH occasions for a triggered report, after each UEIRI transmission, the DRX active time will be prolonged to the next PUCCH occasion until UE receives the PDCCH. For this case, there should be no need to change the current specification (i.e., UE stays in active time until receiving PDCCH scheduling a PUSCH for UEI CSI report), which is actually more precise on the active time ending point.

Otherwise, UE may transmit only one UEIRI or intermittent UEIRIs for a triggered report. In this case based on the above proposal, after a UEIRI transmission, if PDCCH is not received, UE stays in active until the next PUCCH occasion. This can be beneficial for UE power saving in DRX despite the potential issue of small PUCCH periodicity.

Although the issue of missing PDCCH scheduling can be rare, we can accept the Rapporteur’s proposal to solve the issue.

# Conclusion

Based on the discussion, we have the following proposal.

Issue-1:

Proposal 1: For mode-B UEI reporting, when the TAT for PUSCH is expired while the TAT for PUCCH is running, not release PUCCH. Support either

- (preferred) transmit UEIRI on PUCCH, no MAC impact, or

- (acceptable) not transmit UEIRI on PUCCH, and specify UE behaviour in MAC.

Issue-2:

Proposal 2: For DRX, after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling the mode-A UEI CSI report on PUSCH or until the next PUCCH occasion for UEIRI, whichever comes the first.

# References

- RP‑242394	WID revision: NR MIMO Phase 5.

- R2-2508129 Report of Rel-19 MIMO MAC open issues