3GPP TSG-RAN WG2 Meeting #132	R2-2508213

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	9.3.3

Source:	OPPO

Title:	Discussion on functionality management for measurement event prediction

Document for:	Discussion, Decision

Introduction

In RAN#109 the WID of AI mobility is further updated in [1]. The working scope is limited to intra-frequency domain case A and case B, co-located inter-frequency prediction for L3 cell level measurement prediction i.e. without L3 beam level measurement prediction. And the WID [1] also clarified that spatial domain prediction is out of the scope.

The paper focus on indirect measurement event prediction and common part between indirect and direct measurement event prediction. Direct event prediction specific aspects will be discussed in February meeting.

General issue

The general issue related to reference signal, measurement quantity and network architecture scope in our contribution [2] are also applicable for measurement event prediction.

Proposal 1: Proposals 1~5 in contribution [2] are applicable also for measurement event prediction

Applicability procedure

The proposals in our contribution [2] for applicability procedure are also applicable for measurement event prediction.

Proposal 2: Proposals 6~16 are also applicable for measurement event prediction

Inference configuration and reporting

The agreements specific to measurement event is captured in the TR [3]:

On top of inference parameters captured in section 6.1.2.1.2, event-related parameters e.g. event type are part of inference configuration of measurement event prediction.

UE can be configured with event triggered reporting based on predicted and/or actual measurement result(s). As baseline event type A1~A6 can be predicted and reported. UE can report measurement results, predicted measurement event together with its timing related information. When UE is configured with intra-frequency temporal domain case B or inter-frequency prediction, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact.

Table 4-1

The legacy IE EventTriggerConfig within reportConfigNR can already indicate measurement event type. It means no new parameter is needed if EventTriggerConfig can be reused to configure a full inference configuration. However, for partial inference configuration, obvious a whole EventTriggerConfig IE is too heavy. So, a new parameter to indicate measurement event type is needed for partial inference configuration.

Proposal 3: For full inference configuration, EventTriggerConfig is reused to indicate measurement event type

Proposal 4: For partial inference configuration, a new parameter indicating measurement event type should be introduced to cover measurement event A1~A6.

The 2nd part of agreement “UE can report measurement results, predicted measurement event together with its timing related information” is only applicable for temporal domain case A. The issue is how to indicate the timing information. Considering there is delay introduced by transmitting and receiving measurementReport message, there could be two options:

Option 1: Relative timing information compared to current time slot i.e. upon transmitting/receiving the measuremnetReport message

Option 2: Absolute timing information

The ambiguity introduced by RRC procedure could be up to few tens of millisecond due to potential HARQ and ARQ procedure in user plane. During simulation phase the typical PW length is 320ms for FR2 temporal domain case A with high F1 score. So compared to UE’s capability, the worst case i.e. few tens of millisecond don’t introduce too much ambiguity. In addition, option 1 obviously demands less signaling overhead compared to option 2. Hence option 1 is better choice.

Proposal 5: Relative timing information is introduced to indicate the future time point of predicted measurement event

For temporal domain case B, we can quickly confirm that existing measurement message can be reused i.e. no spec impact.

Proposal 6: When UE is configured with intra-frequency temporal domain case B, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact

For inter-frequency case, “no spec impact” is based on assumption that UE can report cell identity e.g. PCI of the concerned cell on predicted frequency carrier when reporting predicted measurement event and measurement result. It also implies network need provide some assistant information about co-location between cell on measured frequency carrier and cell on predicted frequency carrier. RAN2 need confirm whether this is the case.

Proposal 7: RAN2 is kindly to requested to confirm that network will provide assistant information about co-location between cell on measured frequency carrier and cell on predicted frequency carrier.

Conclusion

Here are the proposals:

Proposal 1: Proposals 1~5 in contribution [2] are applicable also for measurement event prediction

Proposal 2: Proposals 6~16 are also applicable for measurement event prediction

Proposal 3: For full inference configuration, EventTriggerConfig is reused to indicate measurement event type

Proposal 4: For partial inference configuration, a new parameter indicating measurement event type should be introduced to cover measurement event A1~A6.

Proposal 5: Relative timing information is introduced to indicate the future time point of predicted measurement event

Proposal 6: When UE is configured with intra-frequency temporal domain case B, UE reports measurement event by following existing measurementReport message. This can be achieved without specification impact

Proposal 7: RAN2 is kindly to requested to confirm that network will provide assistant information about co-location between cell on measured frequency carrier and cell on predicted frequency carrier.

Reference

[1] RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2] R2-2508213 Discussion on functionality management for RRM measurement prediction