3GPP TSG-RAN WG2 Meeting #132												R2-2508220

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.2.2

Source: 	Sharp

Title:  	RRC structure and configuration

Document for: 	Discussion

Introduction

In RAN2#131bis meeting [1], RAN2 discussed the RRC structure and configuration, and has made the related agreements as follows:

Agreements 1. ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR). 2. Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling 3. Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling). 4. RAN2 will study modular design of RRC for 6G. Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

This document discusses RRC structure/configuration.

Discussion

RRC Protocol and Architecture

Building upon the experience from 5G NR, the RRC protocol for 6GR is expected to evolve into a more flexible and context-aware architecture that can support a wider range of communication scenarios and service requirements. While 5G NR offered a robust RRC design, its procedures were largely static and uniform across different service types and connectivity conditions. In contrast, 6GR is anticipated to introduce greater adaptability and allow RRC behavior to adjust based on link quality, service characteristics, and/or network topology. Core RRC functions such as connection control, configuration management, and mobility handling are expected to remain essential, but the architecture should be able to accommodate more diverse deployment models and operational conditions.

To support this evolution, 6GR RRC can adopt a modular and scalable design that enables compatibility with various network topologies and access paradigms but still maintain simplicity for conventional deployments. This may involve clearer separation of control responsibilities across network nodes. Such modularity also facilitates integration with emerging architectural concepts in both RAN and core network, e.g., flexible function placement and resource orchestration.

From the procedural viewpoint, 5G NR RRC signaling has proven reliable, but there is room to improve signaling efficiency and responsiveness. In 6GR, minimizing control-plane latency and overhead will be especially important in environments with variable link quality, e.g., underserved areas that are candidates for complementary NTN coverage. Enhancements may include lightweight or opportunistic procedures for state transitions, adaptive configuration of RRC timers and parameters, and improved handling of connectivity events without requiring full connection reestablishment.

RRC state management is another area for potential improvement. 5G NR defined multiple states (Idle, Inactive, Connected) to balance responsiveness and resource efficiency. In 6GR, more refined state transitions, for example including intermediate or context-driven sub-states, can improve continuity and reduce signaling overhead, especially in dynamic or intermittent connectivity conditions.

The RRC information model and configuration granularity can also evolve to support more context-aware operation. This can involve enhanced configuration templates, better grouping of parameters, and conditional activation of configurations that allow the UEs and network nodes to adapt efficiently to changing link conditions or service conditions with reduced signaling overhead; see delta signaling in the next subsection for more details. These enhancements are especially valuable in scenarios where consistent CP interaction is difficult due to variable link conditions or network topology.

Additionally, interaction between RRC and higher-layer management and control functions such as network slicing, service exposure, and AI-assisted optimization is expected to become more prominent in 6GR. The RRC protocol can expose relevant configuration states and enable coordination between access and core layers.

Proposal 1: For studying 6GR RRC protocol and architecture, aim at improving signaling efficiency, supporting diverse deployments, and improving responsiveness under varying link conditions.

Considerations on Module Design

Building on the architectural principles outlined in Section 2.1, the design of RRC configuration modules for 6GR should align with the goals of flexibility, context-awareness, and signaling efficiency. While 5G NR relies on a single Reconfiguration message for RRC configuration, it results in inefficient processing—such as the UE having to decode fields it does not support—and increases UE implementation complexity.

Therefore, in principle, RRC configuration messages should be defined so that they are self‑contained per feature/vertical supported by the UE, and to enable selective activation and minimizing unnecessary processing.

In this context, configuration parameters that are common or overlapping across multiple features/verticals are considered to consolidate as a base configuration, which serves as a stable reference point. Feature/vertical-specific modules can then be layered on top of this base configuration, reducing duplication and improving clarity.

However, future releases may introduce new features whose required parameters partially overlap with existing configurations. To avoid inconsistencies and excessive signaling when harmonizing such parameters, careful consideration of modularization granularity is essential. Modules should be designed not only by feature or vertical but also by the nature and dependency of parameters, ensuring that common elements remain centralized while feature-specific variations are isolated.

Furthermore, to minimize signaling overhead and improve responsiveness, delta signaling should be applied for non-common, feature/vertical-specific configurations. Instead of transmitting full parameter sets, the network should convey only incremental changes relative to the base configuration. This approach reduces message size, accelerates configuration updates, and aligns with the context-aware operation model described in Section 2.1. Delta signaling also facilitates conditional activation of configurations, allowing UEs and network nodes to adapt efficiently to dynamic link conditions or service requirements without requiring complete reconfiguration.

Observation 1: A single monolithic Reconfiguration message, as in 5G NR, forces UEs to process unsupported fields, which increases complexity and signaling overhead.

Observation 2: Without a clear mechanism for incremental updates, introducing new features or harmonizing overlapping parameters across releases can result in large message sizes and inconsistent behaviour.

Proposal 2: Define a base configuration for common parameters and layer feature/vertical-specific modules on top of this base configuration to reduce duplication and improve clarity.

Proposal 3: Study the design of message (module) units not only based on features/verticals, but also on the nature and dependency of parameters to ensure modularization granularity supports future extensibility.

Proposal 4: Apply delta signaling for non-common, feature-specific configurations so that only incremental changes relative to the base configuration are conveyed, minimizing signaling overhead and improving responsiveness.

Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Proposal 1: For studying 6GR RRC protocol and architecture, aim at improving signaling efficiency, supporting diverse deployments, and improving responsiveness under varying link conditions.

Observation 1: A single monolithic Reconfiguration message, as in 5G NR, forces UEs to process unsupported fields, which increases complexity and signaling overhead.

Observation 2: Without a clear mechanism for incremental updates, introducing new features or harmonizing overlapping parameters across releases can result in large message sizes and inconsistent behaviour.

Proposal 2: Define a base configuration for common parameters and layer feature/vertical-specific modules on top of this base configuration to reduce duplication and improve clarity.

Proposal 3: Study the design of message (module) units not only based on features/verticals, but also on the nature and dependency of parameters to ensure modularization granularity supports future extensibility.

Proposal 4: Apply delta signaling for non-common, feature-specific configurations so that only incremental changes relative to the base configuration are conveyed, minimizing signaling overhead and improving responsiveness.

References

[1]  RAN2#131-bis Chair Notes, Prague, Czech Republic, October 2025.