3GPP TSG-RAN WG2 Meeting #132                            R2-2508224

Dallas, USA, Nov.17th–21th, 2025

Agenda Item:	10.3.1.2

Source: 	Transsion Holdings

Title:  	Discussion on RAN QoS in 6G

Document for:	Discussion and Decision

Introduction

In the last meeting, the UP for 6G were discussed, and the following agreements were made[1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we provide our views on these aspects to proceed the study item.

Discussion

RAN QoS for 6G

The 5G QoS model is based on QoS Flows and supports both GBR QoS Flows and non-GBR QoS Flows. At NAS level, the QoS flow is the finest granularity of QoS differentiation in a PDU session. A QoS flow is identified within a PDU session by a QFI. With the introduction of XR-Awareness which relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information, the PDU Set QoS Parameters are provided as part of the QoS profile of the QoS flow, however for a given QoS flow, the PDU Set QoS parameters are common for all PDU Sets but can be different for UL and DL, that means the network does not allow different QoS profile for different PDU sets within the same QoS flow. However 6G aims to support a variety of data types and applications, the data packets within a flow may have different QoS requirement, so 6G QoS should support differentiated QoS treatment for distinct PDU Set within a QoS Flow to achieve finer-grained QoS control.

- Proposal 1:	 The service awareness techniques introduced in 5G should be supported and the finer QoS granularity treatment should be considered to support differentiate data handling in 6G.

In 5G, only basic multi-modality awareness is supported via the provisioning of Multi-modal Service ID (MMSID) from 5GC to NG-RAN when establishing and/or updating the corresponding QoS Flows, and it is up to NG-RAN implementation to handle the inter-dependence Qos flows. However for immersive multi-modal VR applications, synchronization between different media components is critical in order to avoid having a negative impact on the user experience (i.e. viewers detecting lack of synchronization), particularly when the synchronization threshold between two or more modalities is less than the latency KPI for the application.The multi-modal service should be enhanced to support inter-flow synchronization in 6G QoS.

- Proposal 2:	 The multi-modal service should be enhanced to support inter-flow synchronization in 6G QoS to meet the requirement of immersive multi-modal VR application.

Conclusion

In this contribution we discussed the QoS for 6G and made the following proposals:

- Proposal 1:	 The service awareness techniques introduced in 5G should be supported and the finer QoS granularity treatment should be considered to support differentiate data handling in 6G.

- Proposal 2:	 The multi-modal service should be enhanced to support inter-flow synchronization in 6G QoS to meet the requirement of immersive multi-modal VR application.

Reference

- “Draft_RAN2_131bis_MeetingReport_v1.docx”