3GPP TSG-RAN WG2 Meeting #132	R2-2508246

Dallas, USA, Nov. 17th - 21st

Agenda Item:	9.3.3

Source: 	NEC

Title:  	Discussion on UE-side event prediction

Document for:	Discussion and Decision

- 1 Introduction

The Rel-20 AI/ML mobility WID [1] related to UE-side event prediction part is shown as below:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we discuss LCM functionality management for RRM event prediction including applicability report procedure, inference configuration and report.

- 2 Discussion

- 2.1 Applicability report procedure

In terms of applicability report procedure, no matter it is for RRM measurement prediction or event prediction, a common framework should be considered, and the framework should be taking Rel-19 AI PHY as baseline, we have discussed it in our companion paper (R2-2508245) under agenda 9.3.2 RRM measurement prediction. The difference is that when NW provide NW-side additional condition, the content between RRM measurement prediction and event prediction could be different in some cases, and can be further studied later.

Proposal-1: the applicability report procedure in Rel-19 AI/ML PHY can be taken as baseline for AI/ML mobility including both measurement prediction and event prediction.

- 2.2 Inference configuration

Based on WID, it is noted that whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions and check in RAN#112. Therefore, we only discuss indirect event prediction here.

Indirect prediction means the UE still performs inference based on RRM measurement prediction and then based on predicted results to evaluate event as legacy. Therefore, the inference configuration related to RRM measurement prediction is still needed. Furthermore, in order to indicate UE that which event should be used for prediction, the NW also need to provide event-related information, e.g., event type, event related parameters including Hyst, TTT. On the other hand, event evaluation like event A3/5 is normally used for the UE at cell edge to evaluate potential handover, which is the reason why we have s-Measure mechanism to stop measurements for event evaluation when the UE is in cell centre, as there is no need for UE to evaluate potential handover. Same principle can be considered for event prediction, some conditions can be introduced into inference configuration, and the UE performs inference for event prediction only when some conditions are fulfilled for power saving reason.

Proposal-2: besides the contents introduced into RRM measurement prediction, the following contents can be considered in inference configuration for event prediction:

- Event related information, e.g., event type, event related parameters

- Condition to trigger UE performing event prediction

- 2.3 Report

In the content of measurement reporting for event prediction, several aspects may warrant further considerations:

Firstly besides the content introduced for RRM measurement prediction report, e.g., multiple of predicted measurement results with time information, we also need to consider event prediction related parameters. When the UE predicts that a specific event will be fulfilled in the future based on predicted measurement results, the initiated event prediction report via e.g., MR should also include detailed information about the triggered event and the expected time of event occurrence. This can help the network better understand the UE situation and facilitate timely mobility management actions such as handover.

Proposal-3: besides the contents introduced into RRM measurement prediction, the following content can be considered in measurement report for event prediction:

- The triggered event and the expected time of event occurrence

Last but not least, in legacy the UE only initiates measurement report when the event is fulfilled within the TTT period, when it comes to AI-driven event prediction, the UE may anticipate event fulfilment in advance. Therefore, it may be beneficial to further consider the procedures for initiating event prediction reports in such cases. For example, whether reporting should be initiated immediately after a prediction has been made, or after a certain offset, or only when additional conditions (e.g., real measurement-based conditions) are satisfied. Such considerations could help ensure that reporting occurs principally in situations where the predicted results are likely to be reliable and valuable for network operation, thereby optimizing resource usage.

Proposal-4: RAN2 to discuss the following options for the UE-sided event prediction report after predicting event is fulfilled:

Option-1: event prediction report is triggered immediately after a positive prediction

Option-2: event prediction report is triggered after the fulfilment of additional conditions (e.g. a certain offset, real measurement based) and based on a positive prediction

- 3 Conclusions

Proposal-1: the applicability report procedure in Rel-19 AI/ML PHY can be taken as baseline for AI/ML mobility including both measurement prediction and event prediction.

Proposal-2: besides the contents introduced into RRM measurement prediction, the following contents can be considered in inference configuration for event prediction:

- Event related information, e.g., event type, event related parameters

- Condition to trigger UE performing event prediction

Proposal-3: besides the contents introduced into RRM measurement prediction, the following content can be considered in measurement report for event prediction:

- The triggered event and the expected time of event occurrence

Proposal-4: RAN2 to discuss the following options for the UE-sided event prediction report after predicting event is fulfilled:

Option-1: event prediction report is triggered immediately after a positive prediction

Option-2: event prediction report is triggered after the fulfilment of additional conditions (e.g. a certain offset, real measurement based) and based on a positive prediction

- 4 References

[1] RP-252899 	Revision of WID on AI mobility		OPPO, Interdigital