3GPP TSG-RAN WG2 Meeting #132		R2- 2508250

Dallas, USA, 17th – 21st Nov., 2025

Source:	vivo

Title:	Discussion on RAN1 and SA2 LS for LP-WUS WUR

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the LS [1] received from SA2 on allocation of CN assigned subgroup ID for LP-WUS.

Besides, we will discuss the open issue identified in the email discussion on [Post131bis][207][LPWUS] CR for TS 38.304:

| Index | Issue description |
| --- | --- |
| 38304-3 | Discuss whether and how to capture the agreement in RAN1 LS R1-2508170. |

- Discussion

  - SA2 LS on PO-to-LO association

In RAN2#131bis meeting, RAN2 received an LS from SA2 [1]. SA2 has discussed the allocation of CN assigned subgroup ID for LP-WUS and would like to receive some further information about PO-to-LO association from RAN3 and RAN2. The detail issues in the LS are shown below:

SA2 discussed the issue and would like to receive some further information from RAN3 and RAN2: a) What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose. b) Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less? c) Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213. SA2 would appreciate replies to the above questions. 2. Actions: To RAN3, RAN2 ACTION: SA2 kindly asks RAN3 to provide answers for a, c) and RAN2 to provide answers for b).

For question a), we understand SA2’s concern is that different PO-to-LO association configuration may lead to non-uniformity of CN assigned subgroup ID range within the registration area. However, we think this is not a new issue. In Rel-17 on PEI discussion, we have the same concern on different cells may support different CN assigned subgroups, in this case, the CN can’t assign a unified subgroup ID in the registration area.  This similar issue was discussed in RAN2#115-e and reached the following conclusion [2]:

R2 assumes that All the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found).

Anyway, the typical area of the PO-to-LO association configuration could be up to network implementation, that is operators could design the applied area of a PO-to-LO association configuration, which could be per cell or registration area. But without any restriction, additional enhancements should be introduced to solve the inconsistent CN assigned subgroup ID issue. Thus, in our view, it is straightforward to follow the similar method for PEI as mentioned above. RAN2 assumes that all the cells within the registration area supports the same PO-to-LO association. In this case, the CN assigned subgroup ID could be assigned same within the registration area for UE. If a UE moves from a cell to another other cell in the same registration area, there is no need to recheck the CN assigned subgroup ID or coordinate with CN, which can reduce the workload of the UE and CN.

Proposal 1: On Question a), the typical area of the PO-to-LO association configuration is up to implementation, RAN2 assumes that all the cells within the registration area supports the same PO-to-LO association and the number of CN assigned subgroups.

For question b), it is overly restrictive to prohibit changes to the PO-to-LO association configuration, while the PO-to-LO association configuration should be allowed to adapt based on factors, such as the situation of paging transfer and the number of UE supporting LP-WUS. On the other hand, since the configuration of the PO-to-LO association configuration is also related to the traffic type, UE characteristic, network load, etc., the gNB could acquire this information by the historical statistics or the gNB function, we think this information won’t be changed frequently. Besides, frequent change on the PO-to-LO association will result in the frequent changes of CN assigned subgroup ID and UE_ID based subgroup ID for LP-WUS and lead to the coordination between UE and CN. Therefore, the PO-to-LO association configuration will not change often.

Proposal 2: On Question b), the PO-to-LO association configuration could be changed, but it will not change often, as it impacts on the UE_ID subgroup configuration and CN assigned subgroup ID.

For question c), this is the work exclusive to RAN3, and it is up to RAN3 to reply.

Proposal 3: On Question c), it is up to RAN3 to reply.

  - (38304-3) LO determination

In RAN2#131bis meeting, RAN2 received a LS [3] from RAN1 on LP-WUS, in which it pointed out the detailed mechanism of the LO determination based on lpwus-LoFrameOffsetList for LP-WUS operation in RRC_IDLE/INACTIVE mode. The agreement of the LS is shown below:

Agreement The LO determination based on lpwus-LoFrameOffsetList is clarified as follows: - If the number of POs associated with a LO is less than Ns (the number of POs per PF), the UE uses the (i_sNPOWO+1)-th value in offsetForLongerWakeUpDelay (if configured) and the (i_sNPOWO+1)-th value in offsetForShorterWakeUpDelay (if configured) provided by lpwus-LoFrameOffsetList. - For the case when both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured, - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay; - Otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay; - Otherwise, the UE monitors legacy PO. - For the case when only one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay is configured, - If the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay or offsetForShorterWakeUpDelay (whichever is configured) and the start of the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO; - Otherwise, the UE monitors legacy PO.

In the current version for TS 38.304, the description related to the time location of LO is shown below [4]：

If single value is configured for lpwus-LoFrameOffsetList, and if the gap between the LO and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2. If more than one values are configured for lpwus-LoFrameOffsetList, and if the gap between the LO associated with at least one offset and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the smallest offset value with which the gap between the LO and the corresponding PO is no less than the wake-up delay, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

The current text only describes the selection of the lpwus-LoFrameOffsetList, but further clarification on the selection of the parameters in the chosen list is missing based on RAN1 LS [1]. The corresponding TP is provided in Annex.

Proposal 4: (38304-3) RAN2 to adopt the TP for LO determination as shown in Annex based on RAN1 LS.

- Conclusion

In this contribution, we discuss the LS (R1-2508170) from RAN1 and the LS (S2-2509834) received from SA2. Based on the discussion, we have the following proposals.

PO-to-LO association

Proposal 1: On Question a), the typical area of the PO-to-LO association configuration is up to implementation, RAN2 assumes that all the cells within the registration area supports the same PO-to-LO association and the number of CN assigned subgroups.

Proposal 2: On Question b), the PO-to-LO association configuration could be changed, but it will not change often, as it impacts on the UE_ID subgroup configuration and CN assigned subgroup ID.

Proposal 3: On Question c), it is up to RAN3 to reply.

(38304-3) LO determination

Proposal 4: (38304-3) RAN2 to adopt the TP for LO determination as shown in Annex based on RAN1 LS.

- References

- S2-2509834, Reply LS on allocation of CN assigned subgroup ID for LP-WUS

- RAN2#115-e Chair minutes

- R1-2508170, LS on the LO determination based on lpwus-LoFrameOffsetList for LP-WUS operation in IDLE/INACTIVE mode

- TS 38.304, v19.0.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- Annex

## Text proposal for Rel-19 TS 38.304:

Start OF the CHANGE

7.5	LP-WUS monitoring

7.5.0	General

The UE supporting LP-WUS may monitor LP-WUS in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption.

------omit-------

The time location of an LO for UE’s PO is determined by a reference PF/PO and the configured frame-level offset:

-	The reference PF/PO is the start of the PF, or the first PF of the PF or PFs (if mapping of POs from multiple PFs to one LO is configured), associated with the LO. The reference PF/PO for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) *T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4C in TS 38.213 [4], T, Ns, and N are determined in clause 7.1.

-	The frame-level offset between the LO and the reference PF/PO is provided by lpwus-LoFrameOffsetList in SIB1. If the number of POs associated with a LO is less than Ns, the UE uses the (i_sNPOWO+1)-th value in offsetForLongerWakeUpDelay, if configured, or the (i_sNPOWO+1)-th value in offsetForShorterWakeUpDelay, if configured, provided by lpwus-LoFrameOffsetList.

If one of offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay  is configured for lpwus-LoFrameOffsetList, and if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay, if configured, or offsetForShorterWakeUpDelay, if configured, and the start of the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

If both offsetForLongerWakeUpDelay and offsetForShorterWakeUpDelay are configured, and if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForShorterWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the value in offsetForShorterWakeUpDelay, otherwise if the gap between the end of the last LP-WUS MO the UE would monitor in the LO associated with the value in offsetForLongerWakeUpDelay and the start of the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the value in offsetForLongerWakeUpDelay, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

If the UE in RRC_IDLE and RRC_INACTIVE is unable to monitor the LP-WUS occasion in all MO corresponding to its PO, the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

In RRC_INACTIVE state, when the UE uses the same i_s as for RRC_IDLE state as specified in clause 7.1, the UE shall use the same iPO for LP-WUS as for RRC_IDLE state. Otherwise, the UE determines the iPO for LP-WUS based on the i_s for RRC_INACTIVE state.

------omit-------

End OF the CHANGE