3GPP TSG-RAN WG2#132	R2-2508266

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	9.3.2

Title:	Discussion on RRM measurement prediction

Document for:	Discussion and decision

# Introduction

The WID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

In this contribution, we would like to provide our views on RRM measurement prediction.

# Discussion

## UE capability on RRM measurement prediction

In Rel-19, AI/ML for mobility has been studied including RRM measurement prediction. According to the corresponding technical report [2], 3 RRM sub-use cases for cell-level RRM measurement prediction and 3 RRM sub-use cases for beam-level RRM measurement prediction were identified. The evaluation for RRM measurement was considered based on 6 scenarios for 2 different goals, from which 3 scenarios with high priority had been evaluated by the simulation, e.g., for intra-frequency temporal domain case B, the evaluation was based on 3 different filtering options, taking L3-RSRP measurement results as reference KPI.

According to the Rel-20 WID on AI/ML for mobility in NR, signalling and specification aspects on UE capability on RRM measurement prediction are to be addressed, but the category for the UE capability on RRM measurement prediction has not yet been clarified. RRM measurement prediction can be temporal domain prediction, but it can also be an inter-frequency prediction or spatial domain prediction. The prediction can be cell level, and it can also be a beam level prediction. In addition, the prediction was evaluated in Rel-19 based on L3-RSRP measurement results, but the predictable RRM measurement results can also be other KPIs, e.g., RSRQ, SINR, or CQI and etc. Each type of prediction may be corresponding to a UE capability. Some capabilities may not need to be known by the network side, but some capabilities may be required by the network to adjust the following behaviours, e.g., reference signal transmission, handover decision or performance monitoring. Only indicating UE being able to perform RRM measurement prediction may be not sufficient for the network to decide following control. Therefore, RAN2 needs to decide the granularity of UE capability to be signalled to the network before working on the signalling of UE capability request and response procedure.

Proposal 1: RAN2 to discuss the granularity of UE capability for RRM measurement prediction.

## LCM functionality management

For the AI/ML based beam management or CSI prediction, whether UE side model is applicable for a specific prediction can highly depend on network configuration. Therefore, network is expected to provide inference configuration and associated ID for UE to decide the applicability. But for RRM measurement prediction, UE may predict measurement results without additional configuration from the network. For instance, in intra-frequency temporal domain case A, UE may predict future measurements using historical results, and the UE may only need to report whether a measurement report is based on predicted results, or to indicate when the UE will start the inference for RRM measurement. On the other hand, UE may decide whether an inference configuration is applicable once the configuration is received but not start the inference immediately, which is up to UE implementation. If UE responses an inference configuration message with corresponding applicability report but delay the inference start time, there can be a scenario that the network has stopped reference signal transmission according to a skip pattern or predictable frequency, while actual measurement on the reference signal is still ongoing by UE, and the UE may consider the channel quality being too bad to perform inference. Basing on the above discussion, we think RAN2 needs to clarify whether UE reports applicability when it is ready to start inference, or whether UE is expected to start inference once it has reported applicability to network.

Proposal 2: RAN2 to decide whether UE starts inference once applicability is reported to NW.

For temporal domain prediction case B, actual measurements on partial RS of time instance can be skipped by UE using the AI/ML prediction results instead. When the skipped RS is a UE specific RS, e.g., UE specific CSI-RS, the network may not necessarily transmit it; if the skipped RS is a cell specific RS, e.g., SSB, the network may also decide the RS transmission as unnecessary when all the UE are performing prediction on the RS, then such RS transmission can be cancelled to save the network energy. Therefore, we consider for temporal domain prediction case B, UE should assume the RS will be transmitted according to the skip pattern to decide applicability report, and based on the UE applicability report, the network is allowed to transmit SSB/CSI-RS according to the skip pattern.

Proposal 3: For temporal domain prediction case B, NW is allowed to transmit SSB/CSI-RS according to the skip patterns and UE applicability report.

For temporal domain prediction case B, when UE detects an applicable measurement prediction has turned to be inapplicable, and reports the inapplicability report, the UE may need to recover the actual measurements on the RS which have been skipped, as the inference results may not be trustable anymore. Without the RS transmission, UE cannot perform actual measurement to obtain valid measurement results associated with the RS. Therefore, when inapplicability report associated with configuration of temporal domain prediction case B, the NW should recover the SSB or CSI-RS transmission.

Proposal 4: For temporal domain prediction case B, RAN2 to clarify that NW is expected to recover SSB/CSI-RS transmission after receiving inapplicability report.

## Measurement report

For temporal domain case A based RRM measurement prediction, UE may predict measurement results without inference configuration indicated by network, since UE will perform actual measurement as legacy RRM measurement. The different thing is that UE can predict future measurements using historical results, and the UE may report the predicted results before the associated RS is really measured. Therefore, when the UE assembles measurement report, both predicted results for future time instance and latest results of latest measurement can be available. Considering that the network may take the usage of both actual measurement results and predicted results for different purposes, and separate measurement reports may introduce more air interface signalling, we think UE can report the latest actual measurement results along with the predicted measurement results in the same message.

Proposal 5: For temporal domain prediction case A, UE reports the latest actual measurement results along with the predicted measurement results via the same message.

Without indication, network may consider the reported result based on the latest UE actual measurement, which may impact network decision, e.g., network may want to delay the corresponding actions based on predicted result, but start the actions immediately according to the actual measured results. Therefore, we think UE needs to indicate whether the measurement result is based on AI/ML prediction.

Proposal 6: UE indicates whether a measurement result is based on AI/ML prediction.

# Conclusion

In this paper we presented some views on RRM measurement prediction for AI mobility and have come up with the following proposals:

Proposal 1: RAN2 to discuss the granularity of UE capability for RRM measurement prediction.

Proposal 2: RAN2 to decide whether UE starts inference once applicability is reported to NW.

Proposal 3: For temporal domain prediction case B, NW is allowed to transmit SSB/CSI-RS according to the skip patterns and UE applicability report.

Proposal 4: For temporal domain prediction case B, RAN2 to clarify that NW is expected to recover SSB/CSI-RS transmission after receiving inapplicability report.

Proposal 5: For temporal domain prediction case A, UE reports the latest actual measurement results along with the predicted measurement results via the same message.

Proposal 6: UE indicates whether a measurement result is based on AI/ML prediction.

# References

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR