3GPP TSG-RAN WG2 Meeting #132	R2-2508278

Dallas, USA, 17 – 21 November, 2025

Agenda Item:	8.17

Source: 	Huawei, HiSilicon

Title: 	Remaining issues for IoT NTN TDD

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we further discuss the remaining issues for IoT NTN TDD including the SPS postponement issue and the neighbour cell measurement issue.

# 2	Discussion

2.1 SPS collision

In RAN2#129bis meeting, RAN2 made the following agreement:

Agreement When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

In RAN2#131bis meeting [1], SPS postponement was discussed based on some proposals worrying that the postponement may cause collision issue, as multiple UL SPS transmission occasions may be postponed to the same valid UL subframe:

R2-2507773 Report of [AT131bis][311][IoT NTN TDD] RRC open issues Huawei discussion Rel-19 IoT_NTN_TDD Proposal 1: Come back to the SPS collision issue (due to SPS postponement) in the next meeting. - Come back to the SPS collision issue (due to SPS postponement) in the next meeting.

There maybe two kinds of collision: 1) collision between different SPS configurations and 2) collision between different SPS occasions for the same SPS. There are some solutions raised but companies thought they would bring huge complexity for the configuration. After some further study, we think NW implementation may solve this issue to a large extent.

For the collision 1), it can be up to NW implementation to avoid overlapped SPS occasions between different SPS configurations as much as possible. For the collision 2), since NW can predict the occasions falling on the invalid subframes, it can know about the potential collision. In this case, NW can configure a larger SPS resource to accommodate multiple packets that collision with each other after postponement. Some may worry about the resource waste, but the SPS occasion falling on invalid occasions can be reserved for other use actually. In this case, the only thing we need to make sure is to allow multiple postponed packets to be combined and transmitted in a single valid SPS occasion.

Proposal 1: In case of SPS collision, multiple packets are allowed to be re-combined and transmitted on the single SPS occasion.

Another issue we have not discussed is that how to handle the HARQ process after SPS postponement. In legacy, HARQ ID related to an SPS occasion is calculated based on the position of the SPS occasion. In this case, we need to consider which HARQ ID to be used if a packet is postponed to a different SPS occasion. We see two ways:

- The original HARQ process ID before postponement is used.

- The new HARQ process ID after postponement is used.

For 1), we see a big specification impact since it changes the principle of HARQ ID determination for SPS. For 2), we need only to clarify the MAC PDU after postponement is transmitted via the new HARQ process. And if the generated MAC PDU can be moved to the buffer of this new HARQ process.

Proposal 2: In case of SPS postponement, the generated MAC PDU should be moved to the buffer of the new HARQ process calculated based on the SPS occasion after postponement, i.e., no change of legacy HARQ ID determination for SPS.

2.2 Neighbour cell measurement

In RAN2#131bis meeting [1], there was an issue left for neighbour cell measurement:

R2-2507773 Report of [AT131bis][311][IoT NTN TDD] RRC open issues Huawei discussion Rel-19 IoT_NTN_TDD Proposal 2: Check RAN4 progress and see if anything needs to be clarified for UE behaviour on neighbour measurement using TDD frame offset in the next meeting. - Check RAN4 progress and see if anything needs to be clarified for UE behaviour on neighbour measurement using TDD frame offset in the next meeting.

After checking RAN4 progress, there was no discussion related to this issue. We think usually UE behaviour related to measurement can be specified in RAN2 while RAN4 is responsible for making the corresponding requirement. Therefore, RAN2 need to clarify how the neighbour measurement is done with the TDD radio frame offset between serving cell and neighbor cell(s). Now it is clear that the offset is to help UE detect the Frame boundary of the neighbour cell. Based on the Frame Structure specified in RAN1, there will be a set of consecutive valid downlink frames (8.28ms) within one 90ms frame structure. Therefore, in order to measure the neighbour cell, UE should monitor the neighbour downlink reference signal from the start of the consecutive valid downlink frames which lasts 8.28ms, based on the TDD radio frame offset. Performing DL measurement out of this duration will lead to UE power waste. It is better to clarify this to avoid ambiguity.

Observation: UE behaviour related to measurement can be specified in RAN2 while RAN4 is responsible for making the corresponding requirement.

Proposal 3: RAN2 to clarify that UE should perform neighbour cell measurement within the 8.28ms duration from the start of the valid downlink duration.

# 3	Conclusion

The observations and proposals made in this contribution are summarized below:

SPS collision

Proposal 1: In case of SPS collision, multiple packets are allowed to be re-combined and transmitted on the single SPS occasion.

Proposal 2: In case of SPS postponement, the generated MAC PDU should be moved to the buffer of the new HARQ process calculated based on the SPS occasion after postponement, i.e., no change of legacy HARQ ID determination for SPS.

Neighbour cell measurement

Observation: UE behaviour related to measurement can be specified in RAN2 while RAN4 is responsible for making the corresponding requirement.

Proposal 3: RAN2 to clarify that UE should perform neighbour cell measurement within the 8.28ms duration from the start of the valid downlink duration.

# 4	References

- R2-2507703, Report from Break-out session on NES, NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 131bis, Prague, Czech Republic, Oct 13th – 17th, 2025